3GPP TSG-WG4 Meeting #94-e 	R4-2000565

# Online, 24th February – 6th March 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0036 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of DL physical channels |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2020-02-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The DL physical channel tables in Annex C require more details regarding the channel mapping. Since the channel matrix H for a given requirement may affect the power levels differently, it is required to specify how the channels are mapped to physical antennas at the base station side. Otherwise it will be unclear and ambiguous how to guarantee the correct power levels at the DUT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated tables C.3.1-1 and C.5.1-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Channel powers and mapping remain unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex C.3.1, Annex C.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | A discussion paper further explaining the issue is provided in R4-2000564. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

Annex C (normative):
Downlink physical channels

# C.1	General

This annex specifies the downlink physical channels that are needed for setting a connection and channels that are needed during a connection.

# C.2	Setup (Conducted)

Table C.2-1 describes the downlink Physical Channels that are required for connection set up.

Table C.2-1: Downlink Physical Channels required for connection set-up

Physical Channel

PBCH

SSS

PSS

PDCCH

PDSCH

PBCH DMRS

PDCCH DMRS

PDSCH DMRS

CSI-RS

# C.3	Connection (Conducted)

The following clauses, describes the downlink Physical Channels that are transmitted during a connection i.e., when measurements are done.

## C.3.1	Measurement of Performance requirements

<Editor's note: OCNG for DMRS is FFS in Annex A.>

Table C.3.1-1 is applicable for measurements in which uniform RS-to-EPRE boosting for all downlink physical channels, unless otherwise stated.

Table C.3.1-1: Downlink Physical Channels transmitted during a connection (FDD and TDD)

| Parameter | Unit | Value before applying channel matrix | Mapping to antenna before applying channel matrix |
| --- | --- | --- | --- |
| SSS transmit power | W | Test specific |  |
| EPRE ratio of PSS to SSS | dB | 0 | PBCH transmitted over Single antenna |
| EPRE ratio of PBCH to SSS | dB | 0 |  |
| EPRE ratio of PBCH to PBCH DMRS | dB | 0 |  |
| EPRE ratio of PDCCH to SSS | dB | 0 | PDCCH transmitted over same antenna as PBCH |
| EPRE ratio of PDCCH to PDCCH DMRS | dB | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS | dB | Test specific | TC specific |
| EPRE ratio of PDSCH to PDSCH DMRS | dB | Test specific (Note 1) |  |
| EPRE ratio of CSI-RS to SSS | dB | Test specific (Note 2) | TC specific |
| EPRE ratio of PDSCH OCNG to SSS | dB | Test specific (Note 3) | TC specific |
| EPRE ratio of PDCCH OCNG to SSS | dB | 0 | Transmitted over same antenna as PBCH |
| Note 1: Value is derived from Table 4.1-1 in TS 38.214 [12] based on “Number of DM-RS CDM groups without data” and “DMRS Type” parameters specified for each test. Note 2: CSI-RS is not beamformed. Therefore in case of beamforming in general it will experience a gain from the channel matrix H which is different to the gain that a beamformed channel/signal will experience. Note 3: Since OCNG on different transmit antennas is always uncorrelated according to Annex A.5 it in general will experience a different power gain from the channel matrix H than PDSCH. This is because PDSCH on different transmit antennas might be correlated, e.g. in case of beamforming or Tx diversity. |  |  |  |

# C.4	Setup (Radiated)

Table C.4-1 describes the downlink Physical Channels that are required for connection set up.

Table C.4-1: Downlink Physical Channels required for connection set-up

Physical Channel

PBCH

SSS

PSS

PDCCH

PDSCH

PBCH DMRS

PDCCH DMRS

PDSCH DMRS

CSI-RS

PTRS

# C.5	Connection (Radiated)

The following clauses, describes the downlink Physical Channels that are transmitted during a connection i.e., when measurements are done.

## C.5.1	Measurement of Receiver Characteristics

<Editor's note: OCNG for DMRS is FFS in Annex A.>

Table C.5.1-1 is applicable for measurements in which uniform RS-to-EPRE boosting for all downlink physical channels, unless otherwise stated.

Table C.5.1-1: Downlink Physical Channels transmitted during a connection (TDD)

| Parameter | Unit | Value before applying channel matrix | Mapping to antenna before applying channel matrix |
| --- | --- | --- | --- |
| SSS transmit power | W | Test specific |  |
| EPRE ratio of PSS to SSS | dB | 0 | PBCH transmitted over Single antenna |
| EPRE ratio of PBCH to SSS | dB | 0 |  |
| EPRE ratio of PBCH to PBCH DMRS | dB | 0 |  |
| EPRE ratio of PDCCH to SSS | dB | 0 | PDCCH transmitted over same antenna as PBCH |
| EPRE ratio of PDCCH to PDCCH DMRS | dB | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS | dB | Test specific | TC specific |
| EPRE ratio of PDSCH to PDSCH DMRS | dB | Test specific (Note 1) |  |
| EPRE ratio of CSI-RS to SSS | dB | Test specific (Note 2) | TC specific |
| EPRE ratio of PTRS to PDSCH | dB | Test specific | TC specific |
| EPRE ratio of PDSCH OCNG to SSS | dB | Test specific (Note 3) | TC specific |
| EPRE ratio of PDCCH OCNG to SSS | dB | 0 | Transmitted over same antenna as PBCH |
| Note 1: Value is derived from Table 4.1-1 in TS 38.214 [12] based on “Number of DM-RS CDM groups without data” and “DMRS Type” parameters specified for each test. Note 2: CSI-RS is not beamformed. Therefore in case of beamforming in general it will experience a gain from the channel matrix H which is different to the gain that a beamformed channel/signal will experience. Note 3: Since OCNG on different transmit antennas is always uncorrelated according to Annex A.5 it in general will experience a different power gain from the channel matrix H than PDSCH. This is because PDSCH on different transmit antennas might be correlated, e.g. in case of beamforming or Tx diversity. |  |  |  |

< End of changes >