3GPP TSG RAN WG4 Meeting #94-e	R4-2000566

Online, 24th February – 6th March 2020

Source:		Rohde & Schwarz

Title:	Simulation results for URLCC device test times

Agenda Item:	8.9.1.1

Document for:	Discussion

# Introduction

In this contribution we present our simulation results for the test times of URLLC devices based on the agreements in during the ad-hoc discussions at RAN4 #93 [1].

# Discussion

## Simulation results for number of samples

Based on the agreements from last RAN4 meeting we took the following assumptions into when running the simulations:

- Target BLER: 1 e-5

- Confidence level: 99.999%

- Bad DUT factor M: 1.5

- N: 1 (pass/fail decision after each error)

When running the simulations we simulated many different DUTs with different BLERs to be able to estimate not only the TC runtime for UEs that are somewhat close to the limit but also which are much better or much worse. Figure 1 & 2 below show the calculated tests times (in number of samples) for a DUT with a given BLER. As can be seen from the diagrams, the test time peaks for DUT which are close to the limit and drops the further away from the limit the real DUT BLER is. However for DUTs with a really low BLER the test time goes up again, which can be attributed to the fact that it may take a long time for the first error to occur in case the DUT has a very low error rate to begin with.

Figure 1 Mean number of samples vs DUT BLER

Figure 2 Max number of samples vs DUT BLER

| Table 1 shows the standard deviation of the test time in addition to the information shown in the graphs. DUT BLER; Mean number of samples; Standard deviation; Max number of samples. 1.00E-07; 1.10E+07; 1.00E+07; 1.43E+08. 2.00E-07; 5.98E+06; 5.00E+06; 6.82E+07. 3.00E-07; 4.33E+06; 3.33E+06; 4.95E+07. 4.00E-07; 3.51E+06; 2.50E+06; 3.33E+07. 5.00E-07; 3.03E+06; 2.00E+06; 2.93E+07. 6.00E-07; 2.71E+06; 1.67E+06; 2.77E+07. 7.00E-07; 2.49E+06; 1.43E+06; 1.88E+07. 8.00E-07; 2.33E+06; 1.26E+06; 2.17E+07. 9.00E-07; 2.21E+06; 1.12E+06; 1.64E+07. 1.00E-06; 2.11E+06; 1.01E+06; 1.42E+07. 2.00E-06; 1.81E+06; 5.71E+05; 8.31E+06. 3.00E-06; 1.90E+06; 5.19E+05; 6.36E+06. 4.00E-06; 2.14E+06; 5.99E+05; 6.20E+06. 5.00E-06; 2.52E+06; 7.61E+05; 7.98E+06. 6.00E-06; 3.07E+06; 1.00E+06; 1.01E+07. 7.00E-06; 3.86E+06; 1.36E+06; 1.39E+07. 8.00E-06; 5.04E+06; 1.88E+06; 1.82E+07. 9.00E-06; 6.87E+06; 2.72E+06; 2.69E+07. 1.00E-05; 9.92E+06; 4.12E+06; 3.44E+07. 2.00E-05; 2.13E+06; 1.21E+06; 1.13E+07. 3.00E-05; 6.25E+05; 3.77E+05; 4.41E+06. 4.00E-05; 3.17E+05; 1.97E+05; 1.92E+06. 5.00E-05; 1.99E+05; 1.24E+05; 1.44E+06. 6.00E-05; 1.42E+05; 8.72E+04; 9.54E+05. 7.00E-05; 1.09E+05; 6.49E+04; 7.07E+05. 8.00E-05; 8.86E+04; 5.06E+04; 6.17E+05. 9.00E-05; 7.46E+04; 4.05E+04; 4.69E+05. 1.00E-04; 6.47E+04; 3.35E+04; 4.63E+05. 2.00E-04; 3.00E+04; 1.24E+04; 1.59E+05. 3.00E-04; 2.00E+04; 8.16E+03; 8.58E+04. 4.00E-04; 1.50E+04; 6.12E+03; 6.73E+04. 5.00E-04; 1.20E+04; 4.90E+03; 4.97E+04. 6.00E-04; 9.99E+03; 4.08E+03; 4.37E+04. 7.00E-04; 8.58E+03; 3.50E+03; 3.69E+04. 8.00E-04; 7.50E+03; 3.07E+03; 3.32E+04. 9.00E-04; 6.67E+03; 2.72E+03; 2.83E+04. 1.00E-03; 6.00E+03; 2.45E+03; 2.45E+04. 2.00E-03; 3.00E+03; 1.22E+03; 1.33E+04. 3.00E-03; 2.00E+03; 8.16E+02; 8.68E+03. 4.00E-03; 1.50E+03; 6.11E+02; 6.71E+03. 5.00E-03; 1.20E+03; 4.89E+02; 5.05E+03. 6.00E-03; 1.00E+03; 4.07E+02; 4.58E+03. 7.00E-03; 8.58E+02; 3.48E+02; 3.57E+03. 8.00E-03; 7.51E+02; 3.05E+02; 3.22E+03. 9.00E-03; 6.67E+02; 2.71E+02; 2.79E+03. Table 1 Simulation results for number of samples vs. DUT BLER |  |  |  |
| --- | --- | --- | --- |
| ## Test time estimation From the results shown in chapter 2.1 we have also calculated the test times for different subcarrier spacings and different assumed RMCs. As outlined in [1] we have assumed 15 and 30 kHz SCS for FDD and 30 kHz for TDD operation. SCS: For UE: 30kHz for TDD, FDD: 15kHz, [FFS 30kHz]. For FDD the assumption is that all slots in the RMC can be used. For TDD there are no agreements so far on the actual UL-DL pattern to be used for URLLC testing. We chose the UL-DL pattern FR1.30-1 according to Annex A of 38.101-4 [2] as an example, since this pattern utilizes 14 DL slots per frame. Mean Test Time in Minutes; Maximum Test Time in Minutes. BER_0; FDD 15 kHz; FDD 30 kHz; TDD FR1.30-1; FDD 15 kHz; FDD 30 kHz; TDD FR1.30-1. 1.00E-07; 182.75; 91.37; 130.53; 2384.20; 1192.10; 1703.00. 2.00E-07; 99.73; 49.87; 71.24; 1137.28; 568.64; 812.35. 3.00E-07; 72.10; 36.05; 51.50; 825.14; 412.57; 589.39. 4.00E-07; 58.52; 29.26; 41.80; 555.03; 277.52; 396.45. 5.00E-07; 50.42; 25.21; 36.01; 488.10; 244.05; 348.64. 6.00E-07; 45.15; 22.58; 32.25; 462.08; 231.04; 330.06. 7.00E-07; 41.51; 20.76; 29.65; 312.65; 156.32; 223.32. 8.00E-07; 38.79; 19.39; 27.70; 362.28; 181.14; 258.77. 9.00E-07; 36.76; 18.38; 26.26; 272.74; 136.37; 194.82. 1.00E-06; 35.18; 17.59; 25.13; 236.91; 118.46; 169.22. 2.00E-06; 30.19; 15.10; 21.57; 138.48; 69.24; 98.92. 3.00E-06; 31.67; 15.83; 22.62; 106.07; 53.04; 75.77. 4.00E-06; 35.73; 17.86; 25.52; 103.34; 51.67; 73.81. 5.00E-06; 42.06; 21.03; 30.05; 132.95; 66.47; 94.96. 6.00E-06; 51.23; 25.61; 36.59; 168.50; 84.25; 120.36. 7.00E-06; 64.41; 32.21; 46.01; 232.25; 116.12; 165.89. 8.00E-06; 84.00; 42.00; 60.00; 302.83; 151.42; 216.31. 9.00E-06; 114.51; 57.26; 81.79; 447.57; 223.78; 319.69. 1.00E-05; 165.37; 82.69; 118.12; 573.37; 286.69; 409.55. 2.00E-05; 35.51; 17.76; 25.36; 188.36; 94.18; 134.54. 3.00E-05; 10.42; 5.21; 7.44; 73.51; 36.75; 52.50. 4.00E-05; 5.28; 2.64; 3.77; 31.92; 15.96; 22.80. 5.00E-05; 3.32; 1.66; 2.37; 23.99; 11.99; 17.13. 6.00E-05; 2.37; 1.18; 1.69; 15.90; 7.95; 11.36. 7.00E-05; 1.82; 0.91; 1.30; 11.79; 5.90; 8.42. 8.00E-05; 1.48; 0.74; 1.05; 10.28; 5.14; 7.34. 9.00E-05; 1.24; 0.62; 0.89; 7.82; 3.91; 5.58. 1.00E-04; 1.08; 0.54; 0.77; 7.72; 3.86; 5.52. 2.00E-04; 0.50; 0.25; 0.36; 2.65; 1.33; 1.89. 3.00E-04; 0.33; 0.17; 0.24; 1.43; 0.72; 1.02. 4.00E-04; 0.25; 0.12; 0.18; 1.12; 0.56; 0.80. 5.00E-04; 0.20; 0.10; 0.14; 0.83; 0.41; 0.59. 6.00E-04; 0.17; 0.08; 0.12; 0.73; 0.36; 0.52. 7.00E-04; 0.14; 0.07; 0.10; 0.61; 0.31; 0.44. 8.00E-04; 0.13; 0.06; 0.09; 0.55; 0.28; 0.39. 9.00E-04; 0.11; 0.06; 0.08; 0.47; 0.24; 0.34. 1.00E-03; 0.10; 0.05; 0.07; 0.41; 0.20; 0.29. 2.00E-03; 0.05; 0.03; 0.04; 0.22; 0.11; 0.16. 3.00E-03; 0.03; 0.02; 0.02; 0.14; 0.07; 0.10. 4.00E-03; 0.03; 0.01; 0.02; 0.11; 0.06; 0.08. 5.00E-03; 0.02; 0.01; 0.01; 0.08; 0.04; 0.06. 6.00E-03; 0.02; 0.01; 0.01; 0.08; 0.04; 0.05. 7.00E-03; 0.01; 0.01; 0.01; 0.06; 0.03; 0.04. 8.00E-03; 0.01; 0.01; 0.01; 0.05; 0.03; 0.04. 9.00E-03; 0.01; 0.01; 0.01; 0.05; 0.02; 0.03. Table 2 Test times vs DUT BLER As can be seen from the table above the maximum for mean test time is in the region of 2.5 hours for a marginal DUT and testing with 15 kHz SCS for FDD. The maximum test time is around 9.5 hours under the same conditions. This can be shortened significantly by either utilizing 30 kHz SCS for FDD or testing in TDD. Observation 1: The absolute maximum test time is around 9.5 hours. However it should be noted that for DUTs that are only slightly better or worse than the limit the test time already drops drastically, so that the actual test times become much shorter. The real test times which can be expected therefor depend on how the actual DUTs behave. Observation 2: The actual test times depend on DUT behaviour, but are likely much shorter than the maximum. As a comparison, in the current version of the LTE UE test specification 36.521-1 several TCs already require several million samples to be measured. As an example the minimum test times for Cat-M1 and NB-IoT TCs are shown below. So there should be no concern regarding the long test times from a test system point of view. ## Number of samples vs pass/fail criteria |  |  |  |
| In addition to the simulations above, we also simulated the effect of checking the pass/fail criteria every 1, 10 or 100 errors as outlined in [1]: Baseline parameters for methodology 1: N (i.e. pass/fail criterion reported after observing N errors): [1, 10, 100] From our simulations, there is no benefit in reducing the pass/fail criteria checks from checks after every error to checking only every 10 or 100 errors, as can be seen from figure 2 below. This result however does not take into account other parameters, like reduced effort for calculations, etc. during TC execution, which may reduce the advantage for calculating the pass/fail decision after every error. However this is difficult to quantify without running the fully implemented TC. |  |  |  |
| Observation 3: The shortest test time can be achieved when checking the pass/fail criteria after every error. |  |  |  |

# Conclusion

In this paper we provided our simulation results for the URLCC device test times and make the following observations.

Observation 1: The absolute maximum test time is around 9.5 hours.

Observation 2: The actual test times depend on DUT behaviour, but are likely much shorter than the maximum.

Observation 3: The shortest test time can be achieved when checking the pass/fail criteria after every error.

# References

- R4-1915866, Ad-hoc minutes for NR URLLC test feasibility, Ericsson, RAN4 #93, Reno, November 2019

- TS 38.101-4 v15.4.0