3GPP TSG-RAN4 Meeting #94-e 		R4-2000575

Online, 24th Feb – 6th Mar, 2020

Title:	Further discussion on RRM measurement relaxation for NR power saving

Source:	CATT

Agenda Item:	8.7.3.1

Document for:	Discussion

- Introduction

In RAN4#93 meeting, we had discussed RRM measurement relaxation for UE power saving, and the scenarios and the options used for measurement relaxation are captured in the following agreed WF [1].

- For information: Scenarios - #1: Low mobility scenario - #2: Not in cell-edge scenario - #3: Low-mobility + Not in cell-edge scenario - RRM measurement relaxation methods for UE power saving in RRC_idle/inactive state - Option 1: Allow RRM measurements with longer intervals - Option 2: UE is not required to meet the intra-frequency and inter-frequency neighbor cell measurement requirements - Applicability of RRM relaxation methods when the relaxation criteria are fulfilled Applicable RRM relaxation methods. Scenarios #1; FFS: Option 1 and/or 2. Scenarios #2; FFS: Option 1 and/or 2. Scenarios #3; Option 2. - Selection of option 1 or option 2 will depend on the details of the criteria as developed by RAN2. - Encourage companies to bring the analysis on how to relax measurement interval if option 1 is proposed in next meeting - Encourage companies to bring the analysis for option 2 if option 2 is proposed in next meeting - RRM measurement relaxation by reducing the number of frequency layer to be measured is FFS - The impact on early measurement reporting due to power saving mode: - Option 1: UE is not allowed to relax or enter any relaxed measurement modes (e.g. based on scenarios defined in slide 2) if UE is configured with early measurement reporting (EMR) and T331 timer is running. - Other options is not precluded - RAN4 to study the RRM impact, e.g. timeline of DCI based BWP switch, due to cross-slot scheduling power saving technique if there is any impact to the DCI processing delay based on RAN1 agreements.

In this contribution, we further discuss the remaining issue for RRM measurement relaxation in IDLE/INACTIVE mode for NR UE power saving.

- Discussion

Measurement relaxation for scenario #1 and #2

According to LS [2] from RAN2, RAN2 has agreed to support RRM measurement relaxation for low mobility UE and UE not at cell edge. When the relaxation criteria are satisfied, how to relax intra-frequency and inter-frequency measurement need further be discussed in RAN4. In last meeting, two option of UE measurement behavior are discussed for RRM measurement relaxation. And for the scenario #3, it was agreed that option 2 is adapted for measurement relaxation, and for scenario #1 and 2, it was FFS.

In either scenario #1 or scenario #2, when the relaxation criteria are fulfilled, UE may limit the needed measurement activities in idle/inactive mode in order to achieve power saving gain. In LTE MTC/NB-IoT case, if the relaxation criteria are fulfilled, UE may drop the neighbor carrier measurement for up to 24 hours, since most of NB-Iot UEs are stationary, and the mobility performance is not as important as power energy. However, mobility performance is one of important aspect which should be considered for NR UEs. In some case, NR UEs in power saving mode may enter normal mode, UE can use the neighbor cell measurement measured during power saving mode for potential cell reselection. If UEs drop the neighbor cell measurement during power saving mode, the mobility performance will not be guaranteed when UEs enter from power saving mode to normal mode. Thus, the measurement relaxation should be evaluated in a tradeoff between power saving gain and mobility performance. And option 1 is more appropriate way to define the RRM measurement relaxation requirement for power saving.

Proposal 1: For scenario #1 and #2, option1 is proposed to define RRM measurement relaxation for UE power saving in NR.

According to TR 38.840 [3], increasing the measurement period can be considered under the certain conditions to save the UE power consumption. When DRX is configured, UE RRM measurements are associated with the DRX cycle in RRC_IDLE/Inactive modes. The RRM measurement period is relaxed to associate with the period of the DRX cycle, e.g., 1*DRX cycle changed to N*DRX cycle for RRC_IDLE/RRC_INACTIVE, where N is configurable. If the serving cell with better signal quality and the highest frequency priority, UE could not need cell reselection for a while, then the frequent RRM measurement could be unnecessary. And extending the measurement period may be an efficient method to save UE power consumption. The evaluation for UE power saving in time domain including impact on mobility performance was studied in RRC_IDLE/INACTIVE modes, and the outcome of the evaluation for RRC_IDLE/INACTIVE is shown as follows:

-	By increasing measurement period, the UE power saving gain at least depends on the number of I-DRX/paging cycles to be skipped for measurement and the number of SSB burst sets to be used for measurement and periodic activities. When measurement is performed once every 4 paging cycles, the gain is 17.9%-19.7% (one SSB burst set for measurement and periodic activities), 0.89%-5.36% (two or three SSB burst sets for measurement and periodic activities).

Proposal 2: If the measurement relaxation criteria are met, the measurement period can be extended to 4 times to achieve power saving gain.

Inter-frequency carrier with higher priority

According to the measurement rules for cell re-selection defined in TS 38.304 [4], for inter-frequency measurement, if the NR inter-frequency has a higher reselection priority than the current NR frequency, the UE shall perform measurements of higher priority NR inter-frequency at least every (60 * Nlayers) seconds regardless of the cell selection criterion S. In addition, RAN2 agreed in [2] whether higher priority frequencies can be relaxed is up to network configuration.

Agreements in R2#108 Whether higher priority frequencies can be relaxed is up to network configuration. FFS on how the configuration is done.

If network is configured to relax inter-frequency measurement with higher priority, RAN4 should discuss how to relax the measurement requirement. Currently, RAN4 defined the requirement in TS38.133 [5] for inter-frequency carrier with higher priority as follows.

“The UE shall search every layer of higher priority at least every Thigher_priority_search = (60 * Nlayers) seconds, where Nlayers is the total number of higher priority NR and E-UTRA carrier frequencies broadcasted in system information.”

Since the inter-frequency carrier with higher priority configured by network is considered to have a higher priority to be selected to, the mobility performance should be considered from RRM aspect.

In good coverage case, i.e. Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the higher priority frequency measurements are already at their lowest rate, i.e. every 60s for each frequency layer, which is considerd as the maximum allowed relaxation for load balancing to work properly. In addition, the power saving gain is limit to further relax the measurement interval from 60s to a larger value. Thus, there is no relaxation for the higher priority frequency layers.

In not good coverage case, i.e. Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, In Rel-15 legacy requirement, the measurement delay requirement for higher, equal and lower frequency layer are the same value (Tmeasure,NR_Inter). So, if network is configured to relax inter-frequency measurement with higher priority and the relaxed criteria are met, the relaxed requirement for the frequency layer of higher priority shall have the same Tmeasure,NR_Inter_relax as those for the frequency layer of equal/lower priority.

Proposal 3: When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, if the relaxation criterions are fulfilled, no relaxation of the current measurement delay requirement is expected for inter-frequency measurement with higher priority.

Proposal 4: When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, if the relaxation criterions are fulfilled, the relaxed requirement for the frequency layer of higher priority shall have the same Tmeasure,NR_Inter_relax as those for the frequency layer of equal/lower priority.

Reducing number of carriers

Reducing the number of carriers to be measured for intra and inter-frequency is another aspect need to be considered for UE power saving. According to TS38.133 [5], the measurement requirements for inter-frequency layer are scaling with the number of carriers configured to be measured. It means that the power will not be increased with the same measurement interval for multiple carriers. Thus, from RRM measurements aspect, reducing the number of carriers to be measured cannot save power consumption.

Proposal 5: Reducing the number of carriers to be measured cannot save power consumption.

EMR impact

The motivation for early measurement reporting introduced in WI of multi-RAT DC and CA enhancement is to allow network to obtain measurement results for the candidate PSCell and SCell when UE enters CONNECTED state, which will reduce the delay to set up MR-DC and/or CA. EMR is configured by the network via RRC Connection Release or SIB. Network will configure a list of frequencies and optionally a list of cells for UE to measure in Idle or Inactive. In addition, a timer T331 can be configured RRC Connection Release which is used to limit the time when UE is required to perform early measurement. Early measurement is performed only when the target frequency can form a valid CA band combination of the UE with the serving carrier.

According to definition of overlapping carrier in agreed WF [6], an overlapping carrier is a carrier which the UE is actively measuring for mobility and EMR, and a non-overlapping carrier is a carrier which the UE is actively measuring for EMR only. In last meeting, the impact on early measurement reporting due to power saving mode was discussed in [7]. Since measurement relaxation is only for intra-frequency and inter-frequency measurement for mobility purpose, there will be no impact on EMR for non-overlapping carriers in power saving mode. For overlapping carriers, it is not required to be measured or can be down selected when serving cell Rxlev is not good, and the related side condition is FFS. If UE is required to perform EMR on overlapping carriers in power saving mode, e.g. in case of not in cell-edge, the UE shall perform at least a single measurement on detected cell prior to the measurement report. Whether UE is allowed to enter power saving mode is up to UE implementation. According to the running CR 38.304, the UE may relax choose to perform relaxed measurements if RRM measurement relaxation criterion is met. It is not mandatory. Thus, it is up to UE implementation whether to relax RRM measurement for fast CA/DC setup. If the UE wants to achieve fast CA/DC setup, it will not relax RRM measurement for early measurement reporting. Otherwise, it can relax RRM measurement for power saving.

Figure 1: Measurement for EMR in different cell area

Proposal 6: For overlapping carriers, it is up to UE implementation whether to relax RRM measurement for fast CA/DC setup.

Proposal 7: For non-overlapping carriers, there is no impact on EMR in power saving mode.

RRM impact due to cross-slot scheduling power saving technique

In power saving WI [8], the cross-slot scheduling technique is introduced to achieve power saving gain, since UE will consume less power with a longer time to parse the DCI command. Thus, RAN1 discussed and agreed the scheduling offset for K0 in DL and K2 in UL as follows:

R1-1913537 Agreements: - For K0/K2 under same-carrier scheduling, possible suggested values by the UE are: - 15kHz/30kHz SCS: {1, 2, 4, 6} slots - 60kHz/120kHz SCS: {2, 4, 8, 12} slots

From RRM perspective, the configurable value of K0/K2 will impact DCI based delay requirements, i.e. DCI based BWP switching delay and DCI based TCI switching delay. Figure 2 show the timeline of DCI based BWP switch defined in Rel-15 and DCI based BWP switch with cross-slot scheduling, if the scheduling offset K0/K2 is configured as 4 slots, then the scheduling offset will be longer than DCI-based BWP switch delay defined in current 38.133. Thus, these delay requirements should be revisited when the related DCI command carrying cross-slot scheduling configuration.

For DCI based BWP switching, the current requirement in 38.133 are defined as follows:

For DCI-based BWP switch, after the UE receives BWP switching request at DL slot n on a serving cell, UE shall be able to receive PDSCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch on the first DL or UL slot occurs right after the beginning of DL slot n+ TBWPswitchDelay.

If cross-slot scheduling is configured in the same DCI command which is configured to active BWP switching, the related delay should be extended by the configured value of K0/K2. And the BWP switch delay should be defined as the maximum between TBWPswitchDelay and configured K0/K2 value.

Figure 2: Timeline for active BWP switch

Proposal 8: When UE receives DCI command with the configuration of cross-slot scheduling and active BWP switch, the active BWP switch delay should be Max(TBWPswitchDelay, K0/K2), where K0/K2 is the configured scheduling offset for cross-slot scheduling.

For DCI based TCI state switch, the delay requirement is defined as timeDurationForQCL, which is the time required by the UE to perform PDCCH reception and applying spatial QCL information received in DCI for PDSCH processing. If the cross-slot scheduling offset is configured in the same DCI command, the TCI state switching delay should be the maximum between timeDurationForQCL and K0/K2 value.

Proposal 9: When UE receives DCI command with the configuration of cross-slot scheduling and TCI state switch, the TCI state switch delay should be Max(timeDurationForQCL, K0/K2), where K0/K2 is the scheduling offset for cross-slot scheduling.

- Conclusion

In this contribution, we provide our analysis on RRM measurement relaxation in IDLE/INACTIVE mode for NR UE power saving, and provide the proposals as follows:

Proposal 1: For scenario #1 and #2, option1 is proposed to define RRM measurement relaxation for UE power saving in NR.

Proposal 2: If the measurement relaxation criteria are met, the measurement period can be extended to 4 times to achieve power saving gain.

Proposal 3: When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, if the relaxation criterions are fulfilled, no relaxation of the current measurement delay requirement is expected for inter-frequency measurement with higher priority.

Proposal 4: When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, if the relaxation criterions are fulfilled, the relaxed requirement for the frequency layer of higher priority shall have the same Tmeasure,NR_Inter_relax as those for the frequency layer of equal/lower priority.

Proposal 5: Reducing the number of carriers to be measured cannot save power consumption.

Proposal 6: For overlapping carriers, it is up to UE implementation whether to relax RRM measurement for fast CA/DC setup.

Proposal 7: For non-overlapping carriers, there is no impact on EMR in power saving mode.

Proposal 8: When UE receives DCI command with the configuration of cross-slot scheduling and active BWP switch, the active BWP switch delay should be Max(TBWPswitchDelay, K0/K2), where K0/K2 is the configured scheduling offset for cross-slot scheduling.

Proposal 9: When UE receives DCI command with the configuration of cross-slot scheduling and TCI state switch, the TCI state switch delay should be Max(timeDurationForQCL, K0/K2), where K0/K2 is the configured scheduling offset for cross-slot scheduling.

- Reference

[1]	R4-1915946,	WF on RRM measurement relaxation for Power Saving,		CATT

[2]	R2-1916375,	LS on RRM Measurement Relaxation for UE Power Saving in NR, RAN2

[3]	TS 38.840

[4]	TS 38.304

[5]	TS 38.133

[6]	R4-1915842,	WF on MR-DC Early measurement reporting, Nokia, Nokia Shanghai Bell

[7]	R4-1915295,	Discussions on RRM impact of NR UE power saving, Ericsson

[8]	RP-191607,	UE Power Saving in NR,	CATT