3GPP TSG-RAN4 Meeting #94-e		R4-2000582

Online, 24th Feb – 6th Mar, 2020

Title:	Further discussion on CSI-RS measurement configuration for RRM measurement requirement

Source:	CATT

Agenda Item:	8.16.1.1

Document for:	Discussion

- Introduction

In RAN4 #93 meeting, RAN4 had discussed CSI-RS based measurement configuration for defining RRM measurement requirement. Regarding the number of sets of CSI-RS configurations for RRM measurement requirements, RAN4 had reached the following consensus in the agreed WF [1].

- CSI-RS configurations (bandwidth of CSI-RS and density of CSI-RS resource) used to define requirements - Define requirements at least for 1 set of configurations - Option 1: 48PRBs and D = 1 - Option 2: 48PRBs and D = 3 - FFS whether to define requirements for additional configurations - 48PRBs and D = 1 - 48PRBs and D = 3 - 96PRBs and D = 1 - 96PRBs and D = 3

In this contribution, we further discuss CSI-RS configurations (bandwidth of CSI-RS and density of CSI-RS) used to define CSI-RS based measurement requirement, and give our proposal.

- Discussion

For each CSI-RS resource, the number of PRB can be configured as 24, 48, 96, 192 or 264 in frequency domain, the density of CSI-RS can be configured as 1 or 3 RE(s) in one PRB. And the simulation results for different CSI-RS configurations (48PRBs and D = 1, 48PRBs and D = 3 and 96PRBs and D = 1) are provided in the following table 1, 2, 3, respectively.

Table 1: Delta RSRP, SINR=-5.97 dB, sync, 15 KHz, D=1, 48PRB

| Channel | Sample num | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AWGN | 3 | -1.82 | 0.51 | 2.44 | 2.33 | 1.93 | 2.13 |
|  | 5 | -1.37 | 0.50 | 2.25 | 1.87 | 1.75 | 1.81 |
|  | 10 | -0.64 | 0.50 | 2.05 | 1.14 | 1.55 | 1.34 |
| EPA5 | 3 | -2.79 | 0.83 | 3.68 | 3.62 | 2.85 | 3.23 |
|  | 5 | -2.11 | 0.76 | 3.02 | 2.87 | 2.26 | 2.56 |
|  | 10 | -1.32 | 0.66 | 2.54 | 1.98 | 1.88 | 1.93 |

Table 2: Delta RSRP, SINR=-5.97 dB, sync, 15 KHz, D=3, 48PRB

| Channel | Sample num | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AWGN | 3 | -1.43 | 0.31 | 1.71 | 1.74 | 1.4 | 1.57 |
|  | 5 | -0.71 | 0.17 | 1.55 | 0.88 | 1.38 | 1.13 |
|  | 10 | -0.64 | 0.31 | 1.28 | 0.95 | 0.97 | 0.96 |
| EPA5 | 3 | -2.11 | 0.40 | 2.22 | 2.51 | 1.82 | 2.16 |
|  | 5 | -1.65 | 0.23 | 1.94 | 1.88 | 1.71 | 1.79 |
|  | 10 | -0.91 | 0.24 | 1.41 | 1.15 | 1.17 | 1.16 |

Table 3: Delta RSRP, SINR=-5.97 dB, syn,15kHz, D=1, 96PRB

| Channel | Sample num | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AWGN | 3 | -1.74 | 0.31 | 1.96 | 2.05 | 1.65 | 1.85 |
|  | 5 | -1.31 | 0.28 | 1.52 | 1.59 | 1.24 | 1.41 |
|  | 10 | -0.95 | 0.28 | 1.25 | 1.23 | 0.97 | 1.10 |
| EPA5 | 3 | -2.04 | 0.67 | 3.22 | 2.71 | 2.55 | 2.63 |
|  | 5 | -1.47 | 0.65 | 2.55 | 2.12 | 1.9 | 2.01 |
|  | 10 | -0.96 | 0.67 | 2.15 | 1.63 | 1.48 | 1.55 |

According to the simulation results, it can be generally seen that the CSI-RSRP measurement accuracy is improved by increasing the measurement bandwidth of CSI-RS and the CSI-RS resource density. For the configuration of {PRB=48 and D=1}, it is required minimum 5 samples of measurement to achieve the acceptable RSRP accuracy requirement, and for the configuration of {PRB=48 and D=3} and {PRB=96 and D=1}, only 3 measurement samples can achieve the acceptable RSRP accuracy requirement. Therefore, it is beneficial to define two sets of configurations for measurement requirement. One is for legacy requirements which 5 measurement samples are needed to define the measurement delay requirement, and the other is for enhancement requirements which 3 measurement are required to define the requirement. In addition, two set of configuration can also reflect the flexibility of CSI-RS measurement from RRM point of view.

Observation 1: It was observed that 5 samples of measurement can achieve the acceptable accuracy requirement when CSI-RS configuration is configured with {PRB=48 and D=1}.

Observation 2: It was observed that 3 samples of measurement can achieve the acceptable accuracy requirement when CSI-RS configuration is configured with {PRB=48 and D=3} or {PRB=96 and D=1}.

Thus, in order to achieve better measurement delay requirement and flexibility of CSI-RS configuration based on the common acceptable accuracy performance, it is proposed that 2 set of CSI-RS based measurement delay requirement shall be defined based on different combination of measurement bandwidth of CSI-RS and CSI-RS resource density.

Proposal: 2 set of CSI-RS based measurement requirements shall be defined based on the combination of measurement bandwidth of CSI-RS and CSI-RS resource density, which is expressed in Table 1.

Table 1: TCSI-RS_measurement_Period with different configurations

| Configuration | Measurement bandwidth [RB] | CSI-RS resource Density | TCSI-RS_measurement_Period |
| --- | --- | --- | --- |
| 0 | 48 | 1 | 5 samples |
| 1 | 48 | 3 | 3 samples |

- Conclusion

In this contribution, we discussed the CSI-RS based measurement bandwidth in order to define CSI-RS based measurement requirement, and give our proposal as follows.

Proposal: 2 set of CSI-RS based measurement requirements shall be defined based on the combination of measurement bandwidth of CSI-RS and CSI-RS resource density, which is expressed in Table 1.

Table 1: TCSI-RS_measurement_Period with different configurations

| Configuration | Measurement bandwidth [RB] | CSI-RS resource Density | TCSI-RS_measurement_Period |
| --- | --- | --- | --- |
| 0 | 48 | 1 | 5 samples |
| 1 | 48 | 3 | 3 samples |

- Reference

[1]	R4-1913749	WF on CSI-RS L3 RRM requirements, 	CATT