3GPP TSG-RAN WG4 Meeting #94-e	R4-2000600

# Online, 24th Feb. – 6th Mar. 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | 0183 | rev | - | Current version: | 16.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS38.101-3, Introduce Rx requirements for NR V2X concurrent operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2020-2-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduce Rx requirements for NR V2X concurrent operation to TS 38.101-3 based on agreed TR38.886 v0.5.0. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specifying Rx requirements for NR V2X inter band concurrent operation with NR license band in TS 38.101-3: |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Rx requirements for NR V2X concurrent operation will not be defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New sections: 7.3E, 7.4E, 7.5E, 7.6E, 7.7E, 7.8E |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

========================= First Change Request ===========================

7.3E	Reference sensitivity for NR V2X concurrent operation

For NR V2X UE operating on V2X band con-current with one NR Uu carrier assigned to lecensed band. The REFSENS requirement specified in table 7.3E-1 will be applied on CC of NR licensed band and applied on CC of NR Band n47 if there was no self-interference problems in own receiver frequency band by own uplink and sidelink transmission.

Table 7.3E-1: Reference sensitivity for NR V2X QPSK PREFSENS

| Inter-band V2X reception |  | Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR V2X Band | NR band | NR Band | SCS (kHz) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | Duplex Mode |
| n47 | NR Band nX | nX | 15 | TBD | TBD | TBD | TBD |  |  | TDD or FDD |
|  |  |  | 30 | TBD | TBD | TBD | TBD |  |  |  |
|  |  |  | 60 | TBD | TBD | TBD | TBD |  |  |  |
|  |  | n47 | 15 | TBD |  | TBD |  | TBD | TBD | HD |
|  |  |  | 30 | TBD |  | TBD |  | TBD | TBD |  |
|  |  |  | 60 | TBD |  | TBD |  | TBD | TBD |  |

Table 7.3E-2 and Table 7.3E-3 are defined as the uplink test configurations for inter-band con-current NR V2X REFSENS requirements. For the uplink configuration, 10MHz Channel bandwidth is used.

Table 7.3E-2: Uplink configuration for reference sensitivity of NR V2X bands (PC5)

| Inter-band NR V2X con-current band configuration |  | NR UL band / Channel BW / NRB / Duplex mode |  |  |  |
| --- | --- | --- | --- | --- | --- |
| NR V2X band (PC5) | NR band (Uu) | NR UL band | Channel Bandwidth (MHz) | NRB | Duplex Mode |
| n47 | nX | nX | 10 | 50 | TDD or FDD |

Table 7.3E-3: SL Tx configuration for reference sensitivity of NR V2X bands (Uu)

| Inter-band NR V2X con-current band configuration |  | NR UL band / Channel BW / NRB / Duplex mode |  |  |  |
| --- | --- | --- | --- | --- | --- |
| NR V2X band (PC5) | NR band (Uu) | NR V2X band (PC5) | Channel Bandwidth (MHz) | NRB | Duplex Mode |
| n47 | nX | nX | 10 | 50 | HD |

========================= Second Change Request ===========================

7.4E	Maximum input level for NR V2X concurrent operation

For NR V2X UE operating on V2X band con-current with one NR Uu carrier assigned to lecensed band. The maximum input level requirement specified in clause 7.4 in TS38.101-1 [2] will be applied on CC of NR licensed band and specified in clause 7.4E in TS38.101-1 [2] applied on CC of NR Band n47.

========================= Third Change Request ===========================

7.5E	Adjacent channel selectivity for NR V2X concurrent operation

For NR V2X UE operating on V2X band con-current with one NR Uu carrier assigned to lecensed band. The ACS requirement specified in clause 7.5 in TS38.101-1 [2] will be applied on CC of NR licensed band and specified in clause 7.5E in TS38.101-1 [2] applied on CC of NR Band n47.

========================= Fourth Change Request ===========================

7.6E	Blocking characteristics for NR V2X concurrent operation

For NR V2X UE operating on V2X band con-current with one NR Uu carrier assigned to lecensed band. The in-band blocking and out of band blocking requirement specified in clause 7.6 in TS38.101-1 [2] will be applied on CC of NR licensed band and specified in clause 7.6E in TS38.101-1 [2] applied on CC of NR Band n47. PInterferer power is increased by ΔRIB,c in the requirement.

No narrow band blocking requirement applied for NR band n47

========================= Fifth Change Request ===========================

7.7E	Spurious response for NR V2X

For NR V2X UE operating on V2X band con-current with one NR Uu carrier assigned to lecensed band. The spurious response requirements specified in clause 7.7 in TS38.101-1 [2] will be applied on CC of NR licensed band and specified in clause 7.7E in TS38.101-1 [2] applied on CC of NR Band n47.

========================= Sixth Change Request ===========================

7.8E	Intermodulation characteristics for NR V2X

For NR V2X UE operating on V2X band con-current with one NR Uu carrier assigned to licensed band. The wideband inter-modulation requirement specified in clause 7.7 in TS38.101-1 [2] will be applied on CC of NR licensed band and specified in clause 7.8E in TS38.101-1 [2] applied on CC of NR Band n47. PInterferer power is increased by ΔRIB,c in the requirement.