3GPP TSG-RAN WG4 Meeting #94-e	R4-2000605

Online, 24th Feb. – 6th Mar. 2020

Title: 	Further discussion on impact of positioning measurement on RRM requirements

Source: 	CATT

Agenda item:	8.8.2.3

Document for:	Discussion

- Introduction

In last RAN4#93 meeting, some paper discussed the impact of positioning measurement on RRM requirements [1][2].

This document will discuss the issues and give our understanding and proposals.

- Discussion

Based on the proposals and observations in [1][2], following items may be consensus:

1): No new measurement gap pattern needs to be defined for NR positioning. PRS periodicities shorter than a measurement gap periodicity can still be used for positioning measurements if measurement gap is not required.

2): Existing HO requirements apply while UE performs PRS measurements, and the measurement period of RSTD measurement will be extended.

3): Existing requirements on Tx timing including accuracy and application of TA apply even during an LPP session.

4): The requirements for Rx – Tx time difference measurement is not applied if the transmit timing change by TA adjust signal.

Following issues need discussed further:

Issue # 1: UE Rx-Tx measurement impact on UE RRM requirements (general)

Based on the definition of UE Rx-Tx time difference in RAN1 specification [3], the Rx time measurement is done from a positioning node, and may be intra frequency or inter frequency measurement. The measurement can be done not only on serving cell but also neighbor cell. But it can be measured on same radio resource with RSRP/RSRQ measurement on SSB, CSI-RS, or DMRS of serving cell or neighbour cell when no PRS is configured. When PRS is configured, the UE Rx-Tx time difference can be measured on same PRS radio resource with PRS-RSRP and RSTD measurement. No more additional radio resources are needed comparing with normal mobility measurement and OTDOA positioning measurement. So the UE Rx – Tx difference measurement is not additional capability and should not impact on existing RRM requirements. The requirements for measurement period, accuracy and reporting mapping should be defined in RRM specification.

Proposal 1: The UE Rx – Tx time difference measurement is not additional capability and is not impact on existing RRM requirements. The requirements for measurement period, accuracy and reporting mapping will be defined in RRM specification.

Due to the measurement accuracy of Rx timing is related with the reference signal bandwidth, so the requirements for measurement period and accuracy of UE Rx – Tx time difference measurement should be defined based on the working bandwidth.

Proposal 2: The requirements for measurement period and accuracy of UE Rx – Tx time difference measurement should be defined based on the working bandwidth.

Issue # 2: UE RSTD measurement impact on UE RRM requirements

As above agreement 1, no new measurement gap pattern needs to be defined for NR positioning. But the positioning measurement using GAP will share the measurement gap configured with RRM measurement using GAP. It will impact UE RRM measurement requirements. If the measurement on PRS can’t done at same time with RLM/BFD/CBD/L1-RSRP measurement due collision, measurement gap may be needed for the RSTD measurement. Following observations and proposal are given.

Observation 1: How to define intra/inter frequency measurement and whether measurement gap is needed should be alignment with other RRM measurement.

Proposal 3: One additional PRS measurement layer is defined for UE measurement.

- Conclusion

This paper discussed impacts of NR positioning measurement on existing RRM requirements. It is summarized following items can be agreed:

1): No new measurement gap pattern needs to be defined for NR positioning. PRS periodicities shorter than a measurement gap periodicity can still be used for positioning measurements if measurement gap is not required.

2): Existing HO requirements apply while UE performs PRS measurements, and the measurement period of RSTD measurement will be extended.

3): Existing requirements on Tx timing including accuracy and application of TA apply even during an LPP session.

4): The requirements for Rx – Tx time difference measurement is not applied if the transmit timing change by TA adjust signal.

Following observation and proposals are presented:

Observation 1: How to define intra/inter frequency measurement and whether measurement gap is needed should be alignment with other RRM measurement.

Proposal 1: The UE Rx – Tx time difference measurement is not additional capability and is not impact on existing RRM requirements. The requirements for measurement period, accuracy and reporting mapping will be defined in RRM specification.

Proposal 2: The requirements for measurement period and accuracy of UE Rx – Tx time difference measurement should be defined based on the working bandwidth.

Proposal 3: One additional PRS measurement layer is defined for UE measurement.

- Reference

[1]	R4-1913705, Impact of positioning measurement on RRM requirements, CATT

[2]	R4-1915194, On impact of NR positioning on existing RRM requirements, Qualcomm Incorporated

[3]	3GPP TS 38.215 V16.0.1, NR Physical layer measurements