3GPP TSG-RAN WG4 Meeting #94-e                                                               R4-2000608

# Electronic Meeting, 24th Feb. – 6th Mar., 2020

Source: 	CATT

Title: 	Discussion on front loaded DMRS start symbol for NR HST PUSCH

Agenda item:	8.17.2.2.1

Document for:	Agreement

- Introduction

In the recent RAN4#93 meeting, a WF [1] on NR HST PUSCH demodulation requirements was approved. There are still some remaining issues regarding PUSCH demodulation, e.g. l0 for mapping type A. This contribution will provide our view on this topic based on simulation results in our companion paper [2] for NR HST PUSCH demodulation.

- Discussion

According to the WF [1] on NR HST PUSCH demodulation, the agreements on l0 for mapping type A are shown as follows:

- l0 for PUSCH mapping type A - l0 = 2 (For simulation alignment) - If no performance different between l0 = 2 and l0 = 3, define performance requirements based on l0 = 2 - l0 value for testing is based on BS declaration

Table 2-1 Evaluation results for 350km/h

| Propagation condition | Carrier frequency (GHz) | SCS | CBW | Maximum Doppler shift | Tx/Rx | MCS | SNR @70% of maximum throughput for l0=2 | SNR @70% of maximum throughput for l0=3 | gain of l0=3 relative to l0=2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tunnel | 2.1 | 15KHz | 10MHz | 1340Hz | 1T1R[FFS] | 2 | -3.47 | -3.39 | -0.08 |
|  |  |  |  |  |  | 16 | 8.65 | 8.68 | -0.03 |
|  |  |  |  |  | 1T2R | 2 | -6.2 | -6.1 | -0.1 |
|  |  |  |  |  |  | 16 | 5.68 | 5.71 | -0.03 |
|  | 3.6 | 30KHz | 40MHz | 2334Hz | 1T1R[FFS] | 2 | -3.37 | -3.29 | -0.08 |
|  |  |  |  |  |  | 16 | 8.73 | 8.81 | -0.08 |
|  |  |  |  |  | 1T2R | 2 | -6.11 | -5.93 | -0.18 |
|  |  |  |  |  |  | 16 | 5.79 | 5.86 | -0.07 |
| Open space | 2.1 | 15KHz | 10MHz | 1340Hz | 1T2R | 2 | -6.21 | -6.1 | -0.11 |
|  |  |  |  |  |  | 16 | 5.67 | 5.7 | -0.03 |
|  |  |  |  |  | 1T8R | 2 | -11.2 | -10.99 | -0.21 |
|  |  |  |  |  |  | 16 | -0.24 | -0.12 | -0.12 |
|  | 3.6 | 30KHz | 40MHz | 2334Hz | 1T2R | 2 | -6.12 | -5.93 | -0.19 |
|  |  |  |  |  |  | 16 | 5.76 | 5.85 | -0.09 |
|  |  |  |  |  | 1T8R | 2 | -11.05 | -10.62 | -0.43 |
|  |  |  |  |  |  | 16 | -0.11 | 0.09 | -0.2 |

Table 2-2 Evaluation results for 500km/h

| Propagation condition | Carrier frequency (GHz) | SCS | CBW | Maximum Doppler shift | Tx/Rx | MCS | SNR @70% of maximum throughput for l0 = 2 | SNR @70% of maximum throughput for l0 =3 | gain of l0=3 relative to l0=2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tunnel | 1.8 | 15KHz | 10MHz | 1740Hz | 1T1R[FFS] | 2 | -3.46 | -3.38 | -0.08 |
|  |  |  |  |  |  | 16[FFS] | 8.67 | 8.69 | -0.02 |
|  |  |  |  |  | 1T2R | 2 | -6.2 | -6.09 | -0.11 |
|  |  |  |  |  |  | 16[FFS] | 5.7 | 5.74 | -0.04 |
|  | 3.6 | 30KHz | 40MHz | 3334Hz | 1T1R[FFS] | 2 | -3.37 | -3.29 | -0.08 |
|  |  |  |  |  |  | 16[FFS] | 8.77 | 8.84 | -0.07 |
|  |  |  |  |  | 1T2R | 2 | -6.11 | -5.93 | -0.18 |
|  |  |  |  |  |  | 16[FFS] | 5.83 | 5.89 | -0.06 |
| Open space | 1.8 | 15KHz | 10MHz | 1740Hz | 1T2R | 2 | -6.21 | -6.1 | -0.11 |
|  |  |  |  |  |  | 16 | 5.68 | 5.71 | -0.03 |
|  |  |  |  |  | 1T8R | 2 | -11.2 | -10.99 | -0.21 |
|  |  |  |  |  |  | 16 | TBD | TBD | TBD |
|  | 3.6 | 30KHz | 40MHz | 3334Hz | 1T2R | 2 | -6.12 | -5.93 | -0.19 |
|  |  |  |  |  |  | 16 | 5.79 | 5.86 | -0.07 |
|  |  |  |  |  | 1T8R | 2 | -11.04 | -10.62 | -0.42 |
|  |  |  |  |  |  | 16 | -0.1 | 0.1 | -0.2 |

The evaluation results for 500km/h at 2.1GHz (15kHz SCS) and for 500km/h at 3.6GHz (30kHz SCS) are shown in Table 2-3 and Table 2-4 respectively. According to the evaluation results in Table 2-1 and Table 2-2, the performance difference between l0=2 and l0=3 is minor and the performance of l0=2 slightly outperforms that of l0=3. The performance gain of l0=2 relative to l0=3 is within the range of 0~0.5dB.

Proposal 1: The performance difference between l0=2 and l0=3 is minor, and the performance gain of l0=2 relative to l0=3 is within the range of 0~0.5dB.

- Conclusion

This contribution provides analysis on l0 for mapping type A for NR HST PUSCH demodulation based on simulation results. The following proposal is concluded as follows:

Proposal 1: The performance difference between l0=2 and l0=3 is minor, and the performance gain of l0=2 relative to l0=3 is within the range of 0~0.5dB.

- References

- R4-1915886, Way forward on NR HST PUSCH demodulation requirements, Huawei, HiSilicon, RAN4#93

- R4-2000610, Initial simulation results for NR HST PUSCH demodulation requirement, CATT, RAN4#94-e