3GPP TSG-RAN WG4 Meeting #94-e                                                               R4-2000612

# Online, 24th February – 6th March, 2020

Source: 	CATT

Title: 	Initial simulation results for NR HST PRACH demodulation requirement with PRACH short sequence format

Agenda item:	8.17.2.2.2

Document for:	Agreement

- Introduction

In the recent RAN4#93 meeting, a WF [1] on NR HST BS demodulation requirements was approved. This paper will provide our evaluation results for NR HST PRACH with short sequence format for simulation alignment based on agreed simulation assumptions in WF.

- Evaluation results

The agreed simulation assumptions in WF [1] are summarized and shown in Table A-1 in Annex.

The evaluation results for PRACH short sequence format at 15kHz SCS and 30kHz SCS are shown in Table 2-1 and Table 2-2 respectively.

Table 2-1 Evaluation results for PRACH short sequence format at 15kHz SCS

| Format | SCS | Ncs | Logical sequence index | v | Tx/Rx | Carrier frequency(GHz) | Propagation condition | Frequency offset (Hz) | Timing error tolerance | SNR @ 99% detection probability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | Ideal | Impairment |
| Format A2 | 15kHz | 23 | 0 | 0 | 1T2R | 1.8 | AWGN | 1740 | 0.52us | -13.29 | -11.29 |
| Format A2 | 15kHz | 23 | 0 | 0 | 1T2R | 1.8 | TDLC300-100 | 400 | 2.03us | -6.57 | -4.57 |
| Format B4 | 15kHz | 23 | 0 | 0 | 1T2R | 1.8 | AWGN | 1740 | 0.52us | -16.51 | -14.51 |
| Format B4 | 15kHz | 23 | 0 | 0 | 1T2R | 1.8 | TDLC300-100 | 400 | 2.03us | -9.82 | -7.82 |
| Format C2 | 15kHz | 23 | 0 | 0 | 1T2R | 1.8 | AWGN | 1740 | 0.52us | -13.31 | -11.31 |
| Format C2 | 15kHz | 23 | 0 | 0 | 1T2R | 1.8 | TDLC300-100 | 400 | 2.03us | -6.81 | -4.81 |
| Format A2 | 15kHz | 23 | 0 | 0 | 1T4R | 1.8 | AWGN | 1740 | 0.52us | -15.32 | -13.32 |
| Format A2 | 15kHz | 23 | 0 | 0 | 1T4R | 1.8 | TDLC300-100 | 400 | 2.03us | -11.22 | -9.22 |
| Format B4 | 15kHz | 23 | 0 | 0 | 1T4R | 1.8 | AWGN | 1740 | 0.52us | -18.49 | -16.49 |
| Format B4 | 15kHz | 23 | 0 | 0 | 1T4R | 1.8 | TDLC300-100 | 400 | 2.03us | -14.24 | -12.24 |
| Format C2 | 15kHz | 23 | 0 | 0 | 1T4R | 1.8 | AWGN | 1740 | 0.52us | -15.34 | -13.34 |
| Format C2 | 15kHz | 23 | 0 | 0 | 1T4R | 1.8 | TDLC300-100 | 400 | 2.03us | -11.23 | -9.23 |
| Format A2 | 15kHz | 23 | 0 | 0 | 1T8R | 1.8 | AWGN | 1740 | 0.52us | -17.35 | -15.35 |
| Format A2 | 15kHz | 23 | 0 | 0 | 1T8R | 1.8 | TDLC300-100 | 400 | 2.03us | -14.29 | -12.29 |
| Format B4 | 15kHz | 23 | 0 | 0 | 1T8R | 1.8 | AWGN | 1740 | 0.52us | -20.26 | -18.26 |
| Format B4 | 15kHz | 23 | 0 | 0 | 1T8R | 1.8 | TDLC300-100 | 400 | 2.03us | -17.27 | -15.27 |
| Format C2 | 15kHz | 23 | 0 | 0 | 1T8R | 1.8 | AWGN | 1740 | 0.52us | -17.33 | -15.33 |
| Format C2 | 15kHz | 23 | 0 | 0 | 1T8R | 1.8 | TDLC300-100 | 400 | 2.03us | -14.26 | -12.26 |

Table 2-1 Evaluation results for PRACH short sequence format at 30kHz SCS

| Format | SCS | Ncs | Logical sequence index | v | Tx/Rx | Carrier frequency(GHz) | Propagation condition | Frequency offset (Hz) | Timing error tolerance | SNR @ 99% detection probability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | Ideal | Impairment |
| Format A2 | 30kHz | 46 | 0 | 0 | 1T2R | 3.6 | AWGN | 3334 | 0.26us | -13.19 | -11.19 |
| Format A2 | 30kHz | 46 | 0 | 0 | 1T2R | 3.6 | TDLC300-100 | 400 | 1.77us | -7.03 | -5.03 |
| Format B4 | 30kHz | 46 | 0 | 0 | 1T2R | 3.6 | AWGN | 3334 | 0.26us | -16.41 | -14.41 |
| Format B4 | 30kHz | 46 | 0 | 0 | 1T2R | 3.6 | TDLC300-100 | 400 | 1.77us | -10.09 | -8.09 |
| Format C2 | 30kHz | 46 | 0 | 0 | 1T2R | 3.6 | AWGN | 3334 | 0.26us | -13.19 | -11.19 |
| Format C2 | 30kHz | 46 | 0 | 0 | 1T2R | 3.6 | TDLC300-100 | 400 | 1.77us | -7.02 | -5.02 |
| Format A2 | 30kHz | 46 | 0 | 0 | 1T4R | 3.6 | AWGN | 3334 | 0.26us | -15.25 | -13.25 |
| Format A2 | 30kHz | 46 | 0 | 0 | 1T4R | 3.6 | TDLC300-100 | 400 | 1.77us | -11.32 | -9.32 |
| Format B4 | 30kHz | 46 | 0 | 0 | 1T4R | 3.6 | AWGN | 3334 | 0.26us | -18.33 | -16.33 |
| Format B4 | 30kHz | 46 | 0 | 0 | 1T4R | 3.6 | TDLC300-100 | 400 | 1.77us | -14.14 | -12.14 |
| Format C2 | 30kHz | 46 | 0 | 0 | 1T4R | 3.6 | AWGN | 3334 | 0.26us | -15.25 | -13.25 |
| Format C2 | 30kHz | 46 | 0 | 0 | 1T4R | 3.6 | TDLC300-100 | 400 | 1.77us | -11.1 | -9.1 |
| Format A2 | 30kHz | 46 | 0 | 0 | 1T8R | 3.6 | AWGN | 3334 | 0.26us | -17.27 | -15.27 |
| Format A2 | 30kHz | 46 | 0 | 0 | 1T8R | 3.6 | TDLC300-100 | 400 | 1.77us | -14.26 | -12.26 |
| Format B4 | 30kHz | 46 | 0 | 0 | 1T8R | 3.6 | AWGN | 3334 | 0.26us | -20.2 | -18.2 |
| Format B4 | 30kHz | 46 | 0 | 0 | 1T8R | 3.6 | TDLC300-100 | 400 | 1.77us | -17.23 | -15.23 |
| Format C2 | 30kHz | 46 | 0 | 0 | 1T8R | 3.6 | AWGN | 3334 | 0.26us | -17.24 | -15.24 |
| Format C2 | 30kHz | 46 | 0 | 0 | 1T8R | 3.6 | TDLC300-100 | 400 | 1.77us | -14.32 | -12.32 |

- Conclusion

This paper provides evaluation results for NR HST PRACH with format 0 for simulation alignment based on agreed simulation assumptions in WF.

- References

- R4-1915871, Way forward on NR HST PRACH demodulation requirement, Samsung, RAN4#93

- R4-1910984,Discussion on HST PRACH demodulation requirement, CATT, RAN4#92bis

- R4-1913726, Discussion on HSTPRACH demodulation requirement, CATT, RAN4#93

- Annex

Table A-1 Simulation assumptions for NR HST PRACH with PRACH short sequence format

| Parameter | Value |
| --- | --- |
| Number of Tx | 1 |
| Number of Rx | 2, 4, 8 |
| Preamble format | Format A2, B4, C2 |
| SCS | 15kHz, 30kHz |
| Ncs | 15kHz: 23 30kHz: 46 |
| v | 0 |
| Logical sequence index | 0 |
| Frequency offset | AWGN: 15kHz: 1740Hz 30kHz: 3334Hz TDLC300-100 : 400Hz |
| Propagation condition | AWGN, TDLC300-100 |
| Proposed time estimation error for AWGN/TDL | AWGN: 15kHz: 0.52us 30kHz: 0.26us TDLC300-100: 15kHz: 2.03us 30kHz: 1.77us |
| Carrier frequency (GHz) | 15kHz: 1.8 30kHz: 3.6 |
| Test metric | SNR @ 99% detection probability |