3GPP TSG-RAN WG4 Meeting #94-e                                                               R4-2000614

# Online, 24th February– 6th March, 2020

Source: 	CATT

Title: 	Discussion on PUSCH performance requirement with 30% throughput metric

Agenda item:	8.18.2.1

Document for:	Agreement

- Introduction

In recent RAN4#93 meeting, the agreed issues and open issues for 30% throughput testing point inAd Hoc minutes for NR Rel-15 and Rel-16 BS demodulation[2] are shown below:

Agreements in RAN4#93: Antenna for FR1: 1T2R Use MCS 16 instead of MCS 2 for full PRB allocation. Waveform: only CP-OFDM for MCS 16 DM-RS configuration - FR2: - Option 1: 1+1 - Option 2: 1+0 - Option 3: 1+1 and 1+0 PT-RS for FR2: - Option 1: with and without PT-RS - Option 2: without PT-RS - Option 3: with PT-RS Channel Model: - FR1: TDLC300-100 (MCS 16) - FR2:TDLA30-300 (MCS 16) Aim to finalize all the test requirements in the next meeting.

This paper will provide our views on PUSCH performance requirement with 30% throughput testing metric.

- Discussion

The open issues in Ad Hoc minutes for NR Rel-15 and Rel-16 BS demodulation in RAN4#93[2] are disscused as follows:

DM-RS configuration for FR2:

In R15 NR normal PUSCH demodulation for FR2, both the DM-RS configuration 1+0 and 1+1 were defined. So DM-RS configurationfor 30% throughput testing point can be defined same as the 70% throughput cases.

Proposal 1: Choose Option 3: 1+1 and 1+0 for FR2.

PT-RS for FR2

The Test Parameters with PTRS and without PTRS were defined in R15 normal PUSCH demodulation for FR2, but the phase noise was not considered in R15 NR normal PUSCH demodulation for FR2, and whether to configure PTRS is not important. To limit test case, the option2: without PTRS is applicable.

Proposal 2: Choose Option 2: without PTRS.

Based on our poposals above and agreed issues in chairman note in RAN4#92bis[1] and in Ad Hoc minutes for NR Rel-15 and Rel-16 BS demodulation in RAN4#93[2], the test parameters for NR PUSCHwith 30% throughput metric are summarized in Table A5-1in Annex.

The evaluation results for NR PUSCH with 30% throughput metric based on proposed test parameters in Table A5-1 are shown in Table 2-1 and Table 2-2.

Table 2-1 Evaluation results for NR PUSCH with 30% throughput metric with CP-OFDM at 4GHz

| waveform | Carrier Freqency(GHz) | Tx/Rx | Time domain resource allocation type | SCS and BW | MCS level | Propagation condition | DMRS | SNR @30% of maximum throughput |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Ideal | Impairment |
| CP-OFDM | 4 | 1T2R | type A | 15kHz, 5MHz | MCS16 | TDLC300-100 | DMRS 1+1 | 0.53 | 2.53 |
| CP-OFDM | 4 | 1T2R | type A | 30kHz, 10MHz | MCS16 | TDLC300-100 | DMRS 1+1 | 0.44 | 2.44 |
| CP-OFDM | 4 | 1T2R | type B | 15kHz, 5MHz | MCS16 | TDLC300-100 | DMRS 1+1 | 0.49 | 2.49 |
| CP-OFDM | 4 | 1T2R | type B | 30kHz, 10MHz | MCS16 | TDLC300-100 | DMRS 1+1 | 0.37 | 2.37 |

Table 2-2 Evaluation results for NR PUSCH with 30% throughput metric with CP-OFDM at 30GHz

| waveform | Carrier Freqency(GHz) | Tx/Rx | Time domain resource allocation type | SCS and BW | MCS level | Propagation condition | DMRS | PTRS | SNR @30% of maximum throughput |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Ideal | Impairment |
| CP-OFDM | 30 | 1T2R | type B | 60kHz, 50MHz | MCS16 | TDLA30-300 | DMRS 1 | No | 0.98 | 2.98 |
| CP-OFDM | 30 | 1T2R | type B | 120kHz, 50MHz | MCS16 | TDLA30-300 | DMRS 1 | No | 1.16 | 3.16 |
| CP-OFDM | 30 | 1T2R | type B | 60kHz, 50MHz | MCS16 | TDLA30-300 | DMRS 1+1 | No | 0.45 | 2.45 |
| CP-OFDM | 30 | 1T2R | type B | 120kHz, 50MHz | MCS16 | TDLA30-300 | DMRS 1+1 | No | 0.41 | 2.41 |

Based on above proposals and evaluation results for 30% throughput testing point, the CR to TS 38.104, TS38.141-1 and TS38.141-2 are provided for approval.

- Conclusion

In this contribution, we discuss the test parameters for NR PUSCH with 30% throughput metric and give our proposals as follows:

Proposal 1: Choose Option 3: 1+1 and 1+0 for FR2.

Proposal 2: Choose Option 2: without PTRS.

Based on above proposals and evaluation results for 30% throughput testing point, the CR to TS 38.104[3], TS38.141-1[4] and TS38.141-2 [5] are provided for approval.

- References

- RAN4_92bis_RD_chairman_report_10_Fri_EOM, RAN4#92bis

- R4-1915804,Ad Hoc minutes for NR Rel-15 and Rel-16 BS demodulation, China Telecom,  Huawei, HiSilicon,RAN4#93

- R4-2000615, CR for TS38.104: Introduce PUSCH performance requirements at 30% throughput testing point, CATT, RAN4#94-e

- R4-2000616, CR for TS38.141-1: Introduce PUSCH performance requirements at 30% throughput testing point, CATT, RAN4#94-e

- R4-2000617, CR for TS38.141-2: Introduce PUSCH performance requirements at 30% throughput testing point, CATT, RAN4#94-e

- Annex

Table A5-1 Test Parameters for CP-OFDM with 30% throughput metric

| Parameter | value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Transform precoding | Disabled | Disabled |
| Number of Tx | 1 | 1 |
| Number of Rx | 2 | 2 |
| Number of layers | 1 | 1 |
| Transmission scheme | Identity matrix (TPMI index 0) | Identity matrix (TPMI index 0) |
| DMRS type | type 1 | type 1 |
| Number of DMRS symbols | 1+1 | 1+0, 1+1 |
| symbols length | 14 | 10 |
| start symbol index | 0 | 0 |
| Time domain resource allocation type | type A and B | type B |
| Frequency domain resource | Full applicable test bandwidth | Full applicable test bandwidth |
| MCS index | 16 | 16 |
| Carrier frequency (GHz) | 4 | 30 |
| Propagation condition | MCS16:TDLC300-100 | MCS16:TDLA30-300 |
| SCS and BW | 15kHz: 5MHz; 30kHz: 10MHz; | 60kHz: 50MHz; 120kHz: 50MHz; |
| PTRS | Not configured | Not configured |
| Timing offset | 0 | 0 |
| Frequency offset | 0 | 0 |
| Code block group, Frequency hopping, Limited buffer rate matching | Disabled | Disabled |
| Number of HARQ transmissions | 4 | 4 |
| Testing metric | SNR @30% of maximum throughput | SNR @30% of maximum throughput |