3GPP TSG-RAN4 Meeting #94-e	R4-2000783

# Online, 24th Feb 2020 - 6th Mar 2020

Agenda item:	8.8.2.1.2

Source:	Apple

Title:		On RSTD measurement report mapping for NR positioning

Document for:	Discussion

# Introduction

In RAN4 #93, the RSTD measurement report mapping table has been discussed and some agreements have been captured in chairman notes,

Agreement - RSTD reporting range - The Max/Min reported values are same as in LTE for FR1 and FR2

At the meantime, RAN1 also had some conclusions from their RAN1 #99 meeting, and the agreements have been captured in the LS R1-1913522 [1].

Agreement: - The reporting granularity for the UE/gNB timing measurements (DL RSTD, the UE Rx-Tx time difference, UL RTOA, gNB Rx-Tx time difference) is defined as , where k is a configuration parameter with a minimum value of at most 0. - Note: RAN4 can determine if -1 can be a minimum value - RAN1 assumes that the details of the reporting granularity and ranges for the UE/gNB timing measurements (DL RTSD, the UE Rx-Tx time difference, UL RTOA, gNB Rx-Tx time difference) will be determined by RAN4, including the potential relation of the parameter k to DL PRS bandwidth.

In this contribution, we discuss how to use the RSTD measurement granularity to define the mapping table and a LS is proposed to RAN1 and RAN2 for their information.

# Discussion

Even for positioning measurement, the UE capability on time chip granularity shall still be carefully considered since PRS based RSTD is also one kind of timing measurement. The UE implementation on time chip granularity is decided by both SCS, FFT size and oversampling rate, as shown in the following equation (1).

Time chip granularity=1∆f*Nf

Where ∆f denotes SCS size and Nf denotes FFT size. The smallest FFT size (without oversampling) for NR system could be derived from the BW and SCS (look up the tables 5.3.2-1 in TS38.101-1 and TS38.101-2 for the RB number).

For instance, if SCS=15kHz and BW=30MHz, based on the Table 5.3.2-1 in TS38.101-1, the RB number is 160. The total SCS number is 160*12=1920, and therefore the smallest FFT size is 2048 for this case without oversampling.

The PRS BW definition from RAN1 conclusion is duplicated as below;

With respect to DL PRS signal design, the following set of agreements was made regarding BW, Start PRB and DL PRS Point A for DL PRS configurations: Start PRB parameter for DL PRS configuration has granularity of one PRB with a minimum of 0 and a maximum of [2176] PRBs 4 PRB granularity is used for DL PRS BW configuration Maximum BW for DL PRS in PRBs does not exceed 272 PRBs Minimum BW for DL PRS in PRBs is not less than 24 PRBs The DL-PRS-Point A can take values given by ARFCN-ValueNR

And the the table of higher layer parameters for NR positioning is captured in R1-1913536[2],

| DL-PRS-SubcarrierSpacing | Subcarrier Spacing for DL PRS Resource | 15, 30, 60 kHz for FR1 60, 120 kHz for FR2 |  |
| --- | --- | --- | --- |
| DL-PRS-ResourceBandwidth | Number of PRBs allocated for DL PRS Resource (allocated DL PRS bandwidth) All DL PRS Resources of the DL PRS Resource Set have the same bandwidth All DL PRS Resource Sets belonging to the same Positioning Frequency Layer have the same value of DL PRS Bandwidth and Start PRB | (24, 28, 32, 36, 40, …, 268, 272) | • 4 PRB granularity is used for DL PRS BW configuration • Maximum BW for DL PRS in PRBs does not exceed 272 PRBs • Minimum BW for DL PRS in PRBs is not less than 24 PRBs |

The CBW limitation table from TS38.101-1 and TS38.101-2 are duplicated as below,

FR1 Table 5.3.2-1: Maximum transmission bandwidth configuration NRB SCS (kHz); 5 MHz; 10 MHz; 15 MHz; 20 MHz; 25 MHz; 30 MHz; 40 MHz; 50 MHz; 60 MHz; 70 MHz; 80 MHz; 90 MHz; 100 MHz. NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB. 15; 25; 52; 79; 106; 133; 160; 216; 270; N/A; N/A; N/A; N/A; N/A. 30; 11; 24; 38; 51; 65; 78; 106; 133; 162; 189; 217; 245; 273. 60; N/A; 11; 18; 24; 31; 38; 51; 65; 79; 93; 107; 121; 135. FR2 Table 5.3.2-1: Maximum transmission bandwidth configuration NRB SCS (kHz); 50 MHz; 100 MHz; 200 MHz; 400 MHz. NRB; NRB; NRB; NRB. 60; 66; 132; 264; N.A. 120; 32; 66; 132; 264.

Considering both the CBW limitation and PRS BW, the PRS PRB number for different SCS can be summarized as below,

Table 1. Summary of PRS measurement BW

|  |  | PRS measurement BW range (PRBs) | Note |
| --- | --- | --- | --- |
| SCS FR1 | 15kHz | 24~268 | The limitation of CBW is up to 270RBs, so up to 268RBs PRS could be configured |
|  | 30kHz | 24~272 | The limitation of CBW is up to 273RBs, so up to 272RBs PRS could be configured. We think UE shall not be required to meet requirement for RSTD measurement in case of “SCS=30kHz && CBW=5MHz” |
|  | 60kHz | 24~132 | The limitation of CBW is up to 135RBs, so up to 132RBs PRS could be configured. We think UE shall not be required to meet requirement for RSTD measurement in case of “SCS=60kHz && CBW=10MHz” and “SCS=60kHz && CBW=15MHz” |
| SCS FR2 | 60kHz | 24~264 | The limitation of CBW is up to 264RBs, so up to 264RBs PRS could be configured |
|  | 120kHz | 24~264 | The limitation of CBW is up to 264RBs, so up to 264RBs PRS could be configured |

With the above PRS measurement BW, the FFT size and time chip granularity could be derived as well, which is summarized in the table 2. In LTE TS36.133, the basic RSTD measurement report granularity is 1Ts and the higher resolution is 0.5Ts for the range from RSTD_2260 to RSTD_10451. So in NR we would like to at least keep 1Ts (i.e., 64Tc) as a lowest resolution level.

Table 2. Summary of smallest FFT size and basic time chip granularity

| FFT size without OS |  | PRS measurement BW range (PRBs) | Smallest FFT size | Basic time chip granularity without oversampling | Note |
| --- | --- | --- | --- | --- | --- |
| SCS FR1 | 15kHz | 24~40 | 512 | 64Tc | 1/(512*15k) = 256Tc > 64Tc, so we choose 64Tc as baseline |
|  |  | 44~84 | 1024 | 64Tc | 1/(1024*15k) = 128Tc > 64Tc, so we choose 64Tc as baseline |
|  |  | 88~168 | 2048 | 64Tc |  |
|  |  | 172~268 | 4096 | 32Tc | For 15kHz case it cannot achieve 16Tc as baseline granularity if max FFT is 4096. |
|  | 30kHz | 24~40 | 512 | 64Tc | 1/(512*30k) = 128Tc> 64Tc, so we choose 64Tc as baseline |
|  |  | 44~84 | 1024 | 64Tc |  |
|  |  | 88~168 | 2048 | 32Tc |  |
|  |  | 168~272 | 4096 | 16Tc |  |
|  | 60kHz | 24~40 | 512 | 1/(512*60k) = 64Tc |  |
|  |  | 44~84 | 1024 | 32Tc |  |
|  |  | 88~132 | 2048 | 16Tc |  |
| SCS FR2 | 60kHz | 24~40 | 512 | 1/(512*60k) = 64Tc |  |
|  |  | 44~84 | 1024 | 32Tc |  |
|  |  | 88~168 | 2048 | 16Tc |  |
|  |  | 172~264 | 4096 | 8Tc | For 60kHz case it cannot achieve 4Tc as baseline granularity if max FFT is 4096. |
|  | 120kHz | 24~40 | 512 | 1/(512*120k) = 32Tc |  |
|  |  | 44~84 | 1024 | 16Tc |  |
|  |  | 88~168 | 2048 | 8Tc |  |
|  |  | 172~264 | 4096 | 4Tc |  |

From the above table 2, we found that the granularity is decided by SCS, PRS BW and FR type and therefore there might be three options for consideration to define the RSTD report mapping table,

Option 1: differentiate by SCS, BW and FR type Note: How to differentiate BW in RSTD report mapping table might be based on accuracy requirement in which RAN4 may differentiate the accuracy performance associated with different BW sizes. Option 2: differentiate by SCS and FR only, and choose the finest or worst granularity for each SCS among different BW Note: How to differentiate BW in RSTD report mapping table might be based on accuracy requirement in which RAN4 may differentiate the accuracy performance associated with different BW sizes. Note: RAN4 may choose finest or worst granularity among different BW for the group of identical SCS and identical FR Option 3: differentiate by FR and choose the finest or worst granularity within each FR Note: RAN4 may choose finest or worst granularity among different BW and SCS for each FR

The option 1 is the most precise way to define the mapping table but it will make the mapping table very complicated to standardize and very difficult to interpret. Option 3 is the simplest way to define the requirement, however, if we choose the finest granularity in each FR it will be too tight for UE with low SCS. For instance, in FR1, if we choose 16Tc as the baseline granularity for mapping table, the UE who’s working on SCS=15kHz may need 8192 FFT to achieve so fine granularity and it’s not realistic. Similar situation for FR2 60kHz case, if we choose 4Tc as the baseline granularity for mapping table, the UE who’s working on SCS=60kHz may need 8192 FFT to achieve so fine granularity and it’s not realistic. So as a compromise, option 2 would be the best choice to define the RSTD report mapping table, i.e., define the RSTD report mapping table per SCS in each FR, and also we propose to choose the finest granularity within a SCS to achieve the reliable positioning quantization performance (in LTE 1.4MHz and 20MHz are sharing the same RSTD mapping table).

With this approach, the specification will be relatively simple and the quantization gain can also maintain at a relative good level.

Proposal 1: The RSTD report mapping table will be defined per SCS per FR and choose the finest granularity among different BWs within each SCS.

With the proposal 1, the basic granularity can be summarized in table 3.

Proposal 2: The basic granularity of RSTD report mapping table is as defined in table 3.

Table 3. basic time chip granularity without oversampling for different SCS

| FFT size without OS |  | Basic time chip granularity without oversampling |
| --- | --- | --- |
| SCS FR1 | 15kHz | 32Tc |
|  | 30kHz | 16Tc |
|  | 60kHz | 16Tc |
| SCS FR2 | 60kHz | 8Tc |
|  | 120kHz | 4Tc |

Since in LTE we have oversampling rate 2 for the range from RSTD_2260 to RSTD_10451, here we also prefer to reuse the same oversampling rate assumption for NR case (how to perform this oversampling is purely UE implementation, but at least the higher resolution table will not hurt the UE who has no oversampling implementation). So we propose to have the granularity from RSTD_2260 to RSTD_10451 as summarized in table 4 for FR1 and FR2.

Proposal 3: The oversampling rate of RSTD report mapping table is same as LTE, i.e., 2, for the range from RSTD_2260 to RSTD_10451.

Proposal 4: The granularity from RSTD_2260 to RSTD_10451 after oversampling is summarized in table 4 for FR1 and FR2.

Table 4. time chip granularity with oversampling rate=2 for different SCS

| FFT size with OS=2 |  | Basic time chip granularity without oversampling |
| --- | --- | --- |
| SCS FR1 | 15kHz | 16Tc |
|  | 30kHz | 8Tc |
|  | 60kHz | 8Tc |
| SCS FR2 | 60kHz | 4Tc |
|  | 120kHz | 2Tc |

Besides the principles above, we also need to consider if RSTD measurement is performed between the frequency layers with different SCSs. Since the granularity assumption in our design is differentiate between SCSs, the granularity of RSTD measurement between PRSs with different SCS shall be the worst one between two SCSs, that is, granularity in lower SCS with lower FR.

Proposal 5: if RSTD measurement is performed between PRSs with different SCS, then UE can follow the worst granularity between two SCSs to report.

In last RAN4 meeting, it was agreed that the Max/Min reported values of RSTD mapping table are same as in LTE for FR1 and FR2. So we think that the reporting range will still be from RSTD_0000 to RSTD_12711. Another thing to consider for the requirement is the positioning coverage. We prefer to reuse the same methodology in LTE to design the mapping tables and also prefer to keep the signalling structure similar as LTE, that is to say, we propose to use a RSTD report mapping table together with a relative quantity mapping table for NR RSTD report.

Proposal 6: similar as LTE, RAN4 will use a RSTD report mapping table together with a relative quantity mapping table for NR RSTD report.

For NR FR1, we think the positioning coverage shall be similar as LTE, and therefore we propose to use the same unit as LTE for the RSTD report mapping table, i.e., 1Ts=64Tc. Since the finest basic granularity of NR FR1 is now 16Tc as in table 3, it’s necessary to introduce more values of “Relative Quantity Value” compared with LTE.

Proposal 7: For different SCSs in FR1, use the same unit as LTE for the baseline RSTD report mapping table, i.e., 1Ts=64Tc

In TS36.133, the relative quantity value for higher resolution case is as below, and obviously the 0.5Ts and 1Ts resolution is not sufficient for NR FR1 case any more.

Table 9.1.10.4-1: Relative quantity mapping for higher-resolution RSTD measurement reporting Reported Relative Quantity Value; Measured Relative Quantity Value,; Unit. RSTD_delta_0; 0; Ts. RSTD_delta_1; 0.5; Ts. RSTD_delta_2; 1.0; Ts. RSTD_delta_3; 2.0; Ts. RSTD_delta_4; 3.0; Ts. RSTD_delta_5; 4.0; Ts.

So we propose to extend the relative quantity mapping table with more values to cover more granularity for different SCS in FR1 RSTD report mapping.

Proposal 8: Compared with relative RSTD quantity mapping table in LTE, Extend the relative quantity mapping table with more values to cover the different SCS in FR1 RSTD report mapping.

Compare the finest SCS of FR1 and finest SCS of FR2, there are 4 times difference between FR1 and FR2 case, so if we use 16Tc for the unit in FR2 cases, it means same the signalling can be shared for both FR1 and FR2 cases, and we only need to define the interpretation of granularity of those signalling. For FR2, the positioning coverage can be smaller than FR1, due to the network coverage of FR2. Since we suggest using ¼ unit from FR1 for FR2 (that means the unit for FR2 case can be 16Tc), the boundary of RSTD report range can be 16Tc*15391 = 246256Tc, and it can cover positioning measurement for distance difference up to 37.6km. We believe this positioning coverage is sufficient in theory for FR2.

Proposal 9: For different SCSs in FR2, use the 16Tc as unit for the baseline RSTD report mapping table.

Also we propose to extend the relative quantity mapping table with more values to cover more granularity for different SCS in FR2 RSTD report mapping. As analysis for proposal 9, the same number of reporting values can be used for both FR1 and FR2 relative quantity mapping table.

Proposal 10: for different SCS in FR2 RSTD report mapping, use the same number of values for relative quantity mapping table as FR1.

Based on all the analysis above, the principle to define the mapping table is:

- Like in LTE, RAN4 will have two kinds of tables for RSTD report mapping: baseline RSTD report mapping table and relative mapping table

- Baseline RSTD report mapping table is designed per FR, where the unit of FR1 is 64Tc and unit of FR2 is 16Tc

- Relative mapping table have the same number of signaling for all the SCSs in FR1 and FR2 but the applicability for each signaling value shall be defined per SCS per FR

E.g., in FR1, 15kHz SCS has 16Tc oversampled granularity for range from RSTD_2260 to RSTD_10451 and 32Tc basic granularity; while 30kHz/60kHz SCS has 8Tc oversampled granularity for range from RSTD_2260 to RSTD_10451 and 16Tc basic granularity. So to design the relative quantity mapping table, if the relative quantity   can be exactly divided by 8Tc but not by 16Tc, that means it can only be applied for 30kHz/60kHz SCS with oversampling in range from RSTD_2260 to RSTD_10451. If the relative quantity   can be exactly divided by 16Tc, that means it can be applied for 15kHz SCS with oversampling in range from RSTD_2260 to RSTD_10451 and also can be applied for 30kHz/60kHz SCS with whole RSTD report range.

In order to consider both positioning coverage and the new granularity for NR FR1, the following mapping table and relative quantity mapping table are proposed,

Proposal 11: The RSTD report mapping table and relative quantity mapping table for different SCSs in FR1 is designed as,

For FR1, the UE shall report a reference quantity based on Table x-1 and a relative quantity defined in Table x-2, so that the difference between the measured RSTD quantity and the lower bound of the corresponding range from Table x-1 is between and + resolutionStep. resolutionStep is 16Tc for 15kHz SCS, 8Tc for 30kHz SCS and 8Tc for 60kHz in the range from RSTD_2260 to RSTD_10451, and 32Tc for 15kHz SCS, 16Tc for 30kHz SCS and 16Tc SCS for 60kHz in the range from RSTD_0000 to RSTD_2259 or in the range from RSTD_10452 to RSTD_12711. The applicability of relative quantity value is defined in Table x-2. Table x-1 RSTD report mapping for FR1 Reported Value; Measured Quantity Value; Unit. RSTD_0000; -15391 > RSTD; 64Tc. RSTD_0001; -15391 RSTD < -15386; 64Tc. …; …. RSTD_2258; -4106 RSTD < -4101; 64Tc. RSTD_2259; -4101 RSTD < -4096; 64Tc. RSTD_2260; -4096 RSTD < -4095; 64Tc. RSTD_2261; -4095 RSTD < -4094; 64Tc. …. RSTD_6353; -3 RSTD -2; 64Tc. RSTD_6354; -2 RSTD -1; 64Tc. RSTD_6355; -1 RSTD 0; 64Tc. RSTD_6356; 0 < RSTD 1; 64Tc. RSTD_6357; 1 < RSTD 2; 64Tc. RSTD_6358; 2 < RSTD 3; 64Tc. …; …; …. RSTD_10450; 4094 < RSTD 4095; 64Tc. RSTD_10451; 4095 < RSTD 4096; 64Tc. RSTD_10452; 4096 < RSTD 4101; 64Tc. RSTD_10453; 4101 < RSTD 4106; 64Tc. …; …; …. RSTD_12709; 15381 < RSTD 15386; 64Tc. RSTD_12710; 15386 < RSTD 15391; 64Tc. RSTD_12711; 15391 < RSTD; 64Tc. Table x-2 Relative quantity mapping for different SCS in FR1 Reported Relative Quantity Value; Measured Relative Quantity Value,; Unit; Applicable SCS(kHz); Applicable RSTD report range in table x-1. RSTD_delta_0; 0; Tc; 15; all RSTD reported values in table x-1. 30/60. RSTD_delta_1; 8; Tc; 15; NA. 30/60; from RSTD_2260 to RSTD_10451 in table x-1. RSTD_delta_2; 16; Tc; 15; from RSTD_2260 to RSTD_10451 in table x-1. 30/60; all RSTD reported values in table x-1. RSTD_delta_3; 24; Tc; 15; NA. 30/60; from RSTD_2260 to RSTD_10451 in table x-1. RSTD_delta_4; 32; Tc; 15; all RSTD reported values in table x-1. 30/60. RSTD_delta_5; 40; Tc; 15; NA. 30/60; from RSTD_2260 to RSTD_10451 in table x-1. RSTD_delta_6; 48; Tc; 15; from RSTD_2260 to RSTD_10451 in table x-1. 30/60; all RSTD reported values in table x-1. RSTD_delta_7; 56; Tc; 15; NA. 30/60; from RSTD_2260 to RSTD_10451 in table x-1. RSTD_delta_8; 64; Tc; 15; all RSTD reported values in table x-1. 30/60. RSTD_delta_9; 80; Tc; 15; NA. 30/60; all RSTD reported values in table x-1. RSTD_delta_10; 96; Tc; 15; all RSTD reported values in table x-1. 30/60. RSTD_delta_11; 112; Tc; 15; NA. 30/60; all RSTD reported values in table x-1. RSTD_delta_12; 128; Tc; 15; all RSTD reported values in table x-1. 30/60. RSTD_delta_13; 144; Tc; 15; NA. 30/60; all RSTD reported values in table x-1. RSTD_delta_14; 160; Tc; 15; all RSTD reported values in table x-1. 30/60. RSTD_delta_15; 176; Tc; 15; NA. 30/60; all RSTD reported values in table x-1. RSTD_delta_16; 192; Tc; 15; all RSTD reported values in table x-1. 30/60. RSTD_delta_17; 208; Tc; 15; NA. 30/60; all RSTD reported values in table x-1. RSTD_delta_18; 224; Tc; 15; all RSTD reported values in table x-1. 30/60. RSTD_delta_19; 240; Tc; 15; NA. 30/60; all RSTD reported values in table x-1. RSTD_delta_20; 256; Tc; 15; all RSTD reported values in table x-1. 30/60. RSTD_delta_21; 272; Tc; 15; NA. 30/60; all RSTD reported values in table x-1. RSTD_delta_22; 288; Tc; 15; all RSTD reported values in table x-1. 30/60. RSTD_delta_23; 304; Tc; 15; NA. 30/60; all RSTD reported values in table x-1.

In order to consider both positioning coverage and the new granularity for NR FR2, the following mapping table and relative quantity mapping table are proposed,

Proposal 12: The RSTD report mapping table and relative quantity mapping table for FR2 is designed as,

For FR2, the UE shall report a reference quantity based on Table y-1 and a relative quantity defined in Table y-2, so that the difference between the measured RSTD quantity and the lower bound of the corresponding range from Table y-1 is between and + resolutionStep. resolutionStep is 4Tc for 60kHz SCS and 2Tc for 120kHz SCS in the range from RSTD_2260 to RSTD_10451, and 8Tc for 60kHz SCS and 4Tc for 120kHz SCS in the range from RSTD_0000 to RSTD_2259 or in the range from RSTD_10452 to RSTD_12711. The applicability of relative quantity value is defined in Table y-2. Table y-1 RSTD report mapping for FR2 Reported Value; Measured Quantity Value; Unit. RSTD_0000; -15391 > RSTD; 16Tc. RSTD_0001; -15391 RSTD < -15386; 16Tc. …; …. RSTD_2258; -4106 RSTD < -4101; 16Tc. RSTD_2259; -4101 RSTD < -4096; 16Tc. RSTD_2260; -4096 RSTD < -4095; 16Tc. RSTD_2261; -4095 RSTD < -4094; 16Tc. …. RSTD_6353; -3 RSTD -2; 16Tc. RSTD_6354; -2 RSTD -1; 16Tc. RSTD_6355; -1 RSTD 0; 16Tc. RSTD_6356; 0 < RSTD 1; 16Tc. RSTD_6357; 1 < RSTD 2; 16Tc. RSTD_6358; 2 < RSTD 3; 16Tc. …; …; …. RSTD_10450; 4094 < RSTD 4095; 16Tc. RSTD_10451; 4095 < RSTD 4096; 16Tc. RSTD_10452; 4096 < RSTD 4101; 16Tc. RSTD_10453; 4101 < RSTD 4106; 16Tc. …; …; …. RSTD_12709; 15381 < RSTD 15386; 16Tc. RSTD_12710; 15386 < RSTD 15391; 16Tc. RSTD_12711; 15391 < RSTD; 16Tc. Table y-2 Relative quantity mapping for FR2 Reported Relative Quantity Value; Measured Relative Quantity Value,; Unit; Applicable SCS(kHz); Applicability. RSTD_delta_0; 0; Tc; 60; all RSTD reported values in table y-1. 120. RSTD_delta_1; 2; Tc; 60; NA. 120; from RSTD_2260 to RSTD_10451 in table y-1. RSTD_delta_2; 4; Tc; 60; from RSTD_2260 to RSTD_10451 in table y-1. 120; all RSTD reported values in table y-1. RSTD_delta_3; 6; Tc; 60; NA. 120; from RSTD_2260 to RSTD_10451 in table y-1. RSTD_delta_4; 8; Tc; 60; all RSTD reported values in table y-1. 120. RSTD_delta_5; 10; Tc; 60; NA. 120; from RSTD_2260 to RSTD_10451 in table y-1. RSTD_delta_6; 12; Tc; 60; from RSTD_2260 to RSTD_10451 in table y-1. 120; all RSTD reported values in table y-1. RSTD_delta_7; 14; Tc; 60; NA. 120; from RSTD_2260 to RSTD_10451 in table y-1. RSTD_delta_8; 16; Tc; 60; all RSTD reported values in table y-1. 120. RSTD_delta_9; 20; Tc; 60; NA. 120; all RSTD reported values in table y-1. RSTD_delta_10; 24; Tc; 60; all RSTD reported values in table y-1. 120. RSTD_delta_11; 28; Tc; 60; NA. 120; all RSTD reported values in table y-1. RSTD_delta_12; 32; Tc; 60; all RSTD reported values in table y-1. 120. RSTD_delta_13; 36; Tc; 60; NA. 120; all RSTD reported values in table y-1. RSTD_delta_14; 40; Tc; 60; all RSTD reported values in table y-1. 120. RSTD_delta_15; 44; Tc; 60; NA. 120; all RSTD reported values in table y-1. RSTD_delta_16; 48; Tc; 60; all RSTD reported values in table y-1. 120. RSTD_delta_17; 52; Tc; 60; NA. 120; all RSTD reported values in table y-1. RSTD_delta_18; 56; Tc; 60; all RSTD reported values in table y-1. 120. RSTD_delta_19; 60; Tc; 60; NA. 120; all RSTD reported values in table y-1. RSTD_delta_20; 64; Tc; 60; all RSTD reported values in table y-1. 120. RSTD_delta_21; 68; Tc; 60; NA. 120; all RSTD reported values in table y-1. RSTD_delta_22; 72; Tc; 60; all RSTD reported values in table y-1. 120. RSTD_delta_23; 76; Tc; 60; NA. 120; all RSTD reported values in table y-1.

With the above mapping tables, RAN2 only needs to extend the signalling for relative quantity mapping, but the same signalling can be completely shared by both FR1 and FR2 cases.

# Conclusion

In this contribution, we discuss how to use the RSTD measurement granularity to define the mapping table and a LS is proposed to RAN1 and RAN2 for their information.

Proposal 1: The RSTD report mapping table will be defined per SCS per FR and choose the finest granularity among different BWs within each SCS.

Proposal 2: The basic granularity of RSTD report mapping table is as defined in table 3.

Table 3. basic time chip granularity without oversampling for different SCS

| FFT size without OS |  | Basic time chip granularity without oversampling |
| --- | --- | --- |
| SCS FR1 | 15kHz | 32Tc |
|  | 30kHz | 16Tc |
|  | 60kHz | 16Tc |
| SCS FR2 | 60kHz | 8Tc |
|  | 120kHz | 4Tc |

Proposal 3: The oversampling rate of RSTD report mapping table is same as LTE, i.e., 2, for the range from RSTD_2260 to RSTD_10451.

Proposal 4: The granularity from RSTD_2260 to RSTD_10451 after oversampling is summarized in table 4 for FR1 and FR2.

Table 4. time chip granularity with oversampling rate=2 for different SCS

| FFT size with OS=2 |  | Basic time chip granularity without oversampling |
| --- | --- | --- |
| SCS FR1 | 15kHz | 16Tc |
|  | 30kHz | 8Tc |
|  | 60kHz | 8Tc |
| SCS FR2 | 60kHz | 4Tc |
|  | 120kHz | 2Tc |

Proposal 5: if RSTD measurement is performed between PRSs with different SCS, then UE can follow the worst granularity between two SCSs to report.

Proposal 6: similar as LTE, RAN4 will use a RSTD report mapping table together with a relative quantity mapping table for NR RSTD report.

Proposal 7: For different SCSs in FR1, use the same unit as LTE for the baseline RSTD report mapping table, i.e., 1Ts=64Tc

Proposal 8: Compared with relative RSTD quantity mapping table in LTE, Extend the relative quantity mapping table with more values to cover the different SCS in FR1 RSTD report mapping.

Proposal 9: For different SCSs in FR2, use the 16Tc as unit for the baseline RSTD report mapping table.

Proposal 10: for different SCS in FR2 RSTD report mapping, use the same number of values for relative quantity mapping table as FR1.

Based on all the analysis above, the principle to define the mapping table is:

- Like in LTE, RAN4 will have two kinds of tables for RSTD report mapping: baseline RSTD report mapping table and relative mapping table

- Baseline RSTD report mapping table is designed per FR, where the unit of FR1 is 64Tc and unit of FR2 is 16Tc

- Relative mapping table have the same number of signaling for all the SCSs in FR1 and FR2 but the applicability for each signaling value shall be defined per SCS per FR

Proposal 11: The RSTD report mapping table and relative quantity mapping table for different SCSs in FR1 is designed as table x-1 and x-2.

Proposal 12: The RSTD report mapping table and relative quantity mapping table for FR2 is designed as table y-1 and y-2.

# References

[1] R1-1913522, LS on agreements related to NR Positioning, Nokia, RAN1 #99 meeting

[2] R1-1913536, Update to the table of higher layer parameters for NR Positioning (RAN1#99), Intel, RAN1 #99 meeting