3GPP TSG-RAN WG4 Meeting #94-e	R4-2000810

Electronic Meeting, 24th Feb - 6th Mar 2020

Source:	ZTE Corporation

Title:	Further consideration on uplink carrier switching

Agenda Item:	8.13.1.6

Document for:	Approval

# Introduction

RAN4#93 saw further progress regarding two uplink carrier switching with a WF [1] and an LS on UE capability and RRC signalling [2]. There are still several open issues and hopefully can be resolved in this meeting.

- Length of UL switching period for defining UE RF requirements: - Non-zero value - Option A: {35us, 140 us, 250us} or {1, 4, and 7} OFDM symbols for 30kHz SCS - Option B: {35us, 140 us} or {1, 4} OFDM symbols for 30kHz SCS - Option C: only 35us - FFS whether to define 0us switching period - Handling of DL reception interruption - Option A: Define different capabilities for UEs with and without DL reception interruption - If UE does not report this capability, it means there is no DL reception interruption. - Option B: DL reception interruption is not allowed. - Option C: - No DL reception interruption for the following duplex mode combinations: (carrier 1 + carrier 2) - SUL+TDD - FDD+TDD - TDD+TDD with the same UL-DL pattern - Other duplex mode combinations: allowed for one or two DL carriers based on UE capability reporting

Furthermore, there is no consensus on whether or not to introduce RRM interrupt requirements either [3].

•	Further study if interruption requirements shall be defined

# Discussion

Switching period

The duration of switching period has been discussed intensively. The intermediate consensus is that no more than three Tx switching periods as UE capability. Under such agreements, the decision on whether or not to include 0us switching period is closely related to the selection of the three options on non-zero switching period values. That is to say, if option A is adopted, then it actually means exclusion of 0us switching period, and only option B and C allows for a 0us switching period. In our understanding, 250us switching period may leads to no system gain in many cases, which has been shown by some analysis. A reported 250us switching period capability is equivalent to that the UE does not support the feature, thus, it is a redundant information to have 250us switching period and UE capability signalling and can be removed from the value set.

Proposal 1: 250us switching period is redundant to UE capability signalling, so it can be removed from the value set.

It is commonly understood that 0us switching period requires 3 transmitters at UE side, but the three transmitters are not supposed to transmit at the same time. We understand the strong concern from UE implementation aspects, but there may be other none hand-held terminals which can be equipped with three transmitters. In addition, with never stopping processing progress in semi-conduction industry, its feasibility is foreseeable. As a trade-off, we propose to include 0us into the value set for future proof, but does not define corresponding requirements within Rel-16.

Proposal 2: Include 0us into the value set for none hand-held devices and future proof, and not define requirements corresponding to 0us within Rel-16.

Downlink interruption

Downlink interruption does not exist in some duplex mode combinations of the two uplink carriers, e.g.,

•	SUL+TDD

•	FDD+TDD

•	TDD+TDD with the same UL-DL pattern

For SUL+FDD, the switching may happen from SUL transmission to NR UL transmission, or vice versa. When SUL is transmitted, UE is required to support NR DL receiving, and when UE transmits NR UL, it is also required to receive the NR DL, therefore, no matter what type of UE implementation, the switching between SUL and NR UL does not introduce additional downlink interruption.

For TDD+TDD case with different UL-DL pattern, the switching is only allowed for the overlapped UL slots, in which there is no DL reception. Thus there is no downlink interruption issue.

Therefore, there is no downlink interruption issue in both SUL+FDD and TDD+TDD with different UL-DL pattern, and we propose downlink interruption is not allowed due to the switching between two uplink carriers.

Proposal 3: Downlink interruption is not allowed due to the switching between two uplink carriers.

For the potential RRM interruption requirements, we do not see the necessity of introducing such requirements for the switching of two uplink carriers.

Proposal 4: No RRM interruption requirements defined for the switching between two uplink carriers.

# Conclusion

In this contribution, we have the following proposals for the switching of two uplink carriers:

Proposal 1: 250us switching period is redundant to UE capability signalling, so it can be removed from the value set.

Proposal 2: Include 0us into the value set for none hand-held devices and future proof, and not define requirements corresponding to 0us within Rel-16.

Proposal 3: Downlink interruption is not allowed due to the switching between two uplink carriers.

Proposal 4: No RRM interruption requirements defined for the switching between two uplink carriers.

# Reference

R4-1916084, Way forward on Tx switching between two uplink carriers, China Telecom

R4-1916083, LS on UE capability and RRC signalling for Tx switching, Apple

- [3]  RAN4#93 RD Chairman notes