3GPP TSG-RAN WG4 Meeting #94-e                                                R4-2000925

# E-meeting, 24 Feb. – 6 Mar., 2020

Agenda Item:	8.1.4.1

Source:	MediaTek Inc.

Title:	Simulation results for SI reading

Document for:		Discussion

- Introduction

In this paper, it provides the simulation result for SI reading.

- Discussion

Figure 1 shows the simulation result of SI Reading in RRC Release with Redirection, RRC Re-establishment, and Paging Interruption Requirements, according to the simulation assumption provided in [1]. In addition, the BLER and the number of PDSCH samples to meet 95% of decoding success rate are provided in Table 1 and Table 2, respectively.

To meet 95% of decoding success rate in fading channels, it can be observed that the number of PDSCH samples will be 8, 5, 3, and 2 for SNR of -3dB, -2dB, -1dB, and -0 dB, respectively.

Observation 1: To meet 95% of decoding success rate, 8, 5, 3, and 2 PDSCH samples are required under SNR condition of -3dB, -2dB, -1dB and 0 dB, respectively.

Figure 1. BLER of SI reading with TBS of 1864 bits.

Table 1. BLER of SI reading with TBS of 1864 bits.

| SNR | -3 | -2 | -1 | 0 |
| --- | --- | --- | --- | --- |
| AWGN | 0.84 | 0.34 | 0.00 | 0.00 |
| TDLA | 0.73 | 0.61 | 0.48 | 0.35 |
| TDLB | 0.80 | 0.65 | 0.50 | 0.34 |
| TDLC | 0.77 | 0.62 | 0.46 | 0.30 |

Table 2. Number of PDSCH samples to meet BLER to meet 95% of decoding success rate.

| SNR | -3 | -2 | -1 | 0 |
| --- | --- | --- | --- | --- |
| AWGN | 3 | 1 | 1 | 1 |
| TDLA | 7 | 5 | 3 | 3 |
| TDLB | 8 | 5 | 3 | 2 |
| TDLC | 7 | 4 | 3 | 2 |

- Simulation Assumptions

The simulation assumptions are given by [1].

| Parameters | Unit | Value |
| --- | --- | --- |
| Channel bandwidth | MHz | 20 |
| SCS | kHz | 15 |
| Number of transmitter antennas |  | 1 |
| Number of receive antennas |  | 2 |
| Allocated resource blocks for PDSCH |  | 24 (allocated in the lower edge of the carrier) |
| MCS index |  | 4 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 0.33 |
| Number of control symbols |  | 2 |
| PDSCH mapping type |  | Type A |
| Information Bit Payload |  |  |
| For slots with RMSI | Bits | 1864 |
| Number of Code Blocks per slot |  | 1 |
| Binary Channel Bits Per slot |  |  |
| For slots with RMSI | Bits | 6048 |
| Side condition (PDSCH SNR) | dB | -6 |
| Propagation channel model |  | AWGN, TDLA30-10, TDLB100-400, TDLC300-100 |
| Receiver assumption |  | - Single shot detection - Soft-combining (2, 4 and 8 samples) |
| SIB1 transmission period | ms | 20 |
| Metric |  | 95% of SIB1 acquisition success rate |
| Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 1. |  |  |

- Summary

In this paper, simulation result for SI reading has been provided. And the observation is given:

Observation 1: To meet 95% of decoding success rate, 8, 5, 3, and 2 PDSCH samples are required under SNR condition of -3dB, -2dB, -1dB and 0 dB, respectively.

References

[1] R4-1915777, WF on NR-U RRM Requirements, Ericsson.