3GPP TSG-RAN WG4 Meeting #94-e                                                R4-2000927

# E-meeting, 24 Feb. – 6 Mar., 2020

Agenda Item:	8.1.4.5

Source:	MediaTek Inc.

Title:	Discussion on PSCell addition for NR-U

Document for:		Discussion

- Introduction

In this paper, it provides the view on PSCell addition requirements for NR-U.

- Discussion

The maximum delay extension and the UE behavior for PSCell addition has been discussed [1], attached below for reference.

PSCell Addition Delay

- L1max=3 for SMTC periodicity >40 ms, L1max=5 for SMTC periodicity ≤40 ms

- L2max=2 for SMTC periodicity >40 ms, L2max=3 for SMTC periodicity ≤40 ms

- Upon exceeding L1max or L2max or upon T304 timer, whichever comes first, the UE shall stop the PSCell addition procedure

It can be observed that when exceeding L1max (for search) L2max (for sync), UE could stop PSCell addition procedure even T304 has not been expired. However, it would have RAN2 specification impact. The PSCell addition (reconfiguration with sync) specified in 5.3.5 [2] will start T304 timer, and upon T304 expiry, UE will report SCG reconfiguration with sync failure. Stopping PSCell addition procedure upon exceeding L1max and L2max will introduce new procedures for reconfiguration with sync and it would lead to unexpected UE behavior.

Observation 1: Stopping PSCell addition procedure upon exceeding L1max and L2max will have RAN2 impact. LS to RAN2 would be required.

Proposal 1: PSCell addition procedure should not be terminated due to the number of missing DRS occasions exceeds the upper limit.

Proposal 2: Not to specify L1max and L2max.

- Summary

In this paper, PSCell addition requirements for NR-U have been discussed. We have the following observations and proposals:

Observation 1: Stopping PSCell addition procedure upon exceeding L1max and L2max will have RAN2 impact.

Proposal 1: PSCell addition procedure should not be terminated due to the number of missing DRS occasions exceeds the upper limit.

Proposal 2: Not to specify L1max and L2max.

References

[1] R4-1915777, WF on NR-U RRM Requirements, Ericsson.

[2] TS 38.331, NR; Radio Resource Control (RRC) protocol specification, V15.8.0.