3GPP TSG-RAN WG4 Meeting #94-e                                                R4-2000937

# E-meeting, 24 Feb. – 6 Mar., 2020

Agenda Item:	8.11.2.1

Source:	MediaTek Inc.

Title:	Discussion on RRM requirements for L1-SINR accuracy requirement

Document for:		Discussion

- Introduction

In the last meeting, L1-SINR accuracy requirement have been discussed [1] and some views are provided to define L1-SINR accuracy requirement.

- Discussion

In the last meeting, L1-SINR simulation assumption is provided. According to the specification shown as below, ZP-IMR must be the CSI-IM.

For L1-SINR measurement: - When two Resource Settings are configured, the first one Resource Setting (given by higher layer parameter resourcesForChannelMeasurement) is for channel measurement on SSB or 1 port NZP CSI-RS with density 3 REs/RB and the second one (given by either higher layer parameter csi-IM-ResourcesForInterference or higher layer parameter nzp-CSI-RS-ResourcesForInterference) is for interference measurement performed on CSI-IM or on 1 port NZP CSI-RS with density 3 REs/RB, where each SSB or NZP CSI-RS resource for channel measurement is associated with one CSI-IM resource or one NZP CSI-RS resource for interference measurement by the ordering of the SSB or NZP CSI-RS resource for channel measurement and CSI-IM resource or NZP CSI-RS resource for interference measurement in the corresponding resource sets. The number of SSB(s) or CSI-RS resources for channel measurement equals to the number of CSI-IM resources or the number of NZP CSI-RS resource for interference measurement.

Therefore, for the case involved ZP-IMR configuration in the simulation assumption, the interference measurement resource configuration should be corrected to CSI-IM, instead of NZP CSI-RS Thus, CSI-IM resource are used for all scenarios involved ZP-IMR configuration in this paper.

Proposal 1: For L1-SINR simulation assumption, the ZP-IMR should be corrected to CSI-IM.

For CMR only scenario, the simulation results of L1-SINR measurement accuracy are provided as below.

- L1-SINR accuracy evaluation simulation assumption for CMR only: Parameters; Values. SCS; 15kHz (FR1), 30kHz (FR1), 120kHz (FR2). Channel measurement resource (CMR); CSI-RS. Interference measurement resource (IMR) configuration; N/A. Side condition (SNR) on CMR; -3dB. Density (D); 3. Number of samples (M); 1, 3, 5. Number of PRBs; 48.

Table 1. CSI-RS based CMR L1-SINR absolute accuracy (dB) @ SNR=-3dB, 1, 3, 5 sample(s)

| SCS | # of samples | 5%-tile | 50%-tile | 95%-tile |
| --- | --- | --- | --- | --- |
| 15 kHz | 1 | -1.30 | 0.43 | 1.95 |
|  | 3 | -0.51 | 0.44 | 1.30 |
|  | 5 | -0.29 | 0.43 | 1.14 |
| 30 kHz | 1 | -1.22 | 0.51 | 1.96 |
|  | 3 | -0.43 | 0.51 | 1.40 |
|  | 5 | -0.19 | 0.49 | 1.20 |
| 120 kHz | 1 | -1.26 | 0.46 | 1.98 |
|  | 3 | -0.47 | 0.46 | 1.35 |
|  | 5 | -0.26 | 0.48 | 1.14 |

From Table 1, for 1 sample SSB measurement at SNR of -3dB, the measurement accuracy about 2 dB can be achieved. Considering implementation margin, accuracy of 3.5 dB is proposed in FR1. In FR2, no extra margin is assumed as SS-SINR measurement.

Proposal 2: For SSB based CMR L1- SINR for reporting, the absolute measurement accuracy is +/- 3.5 dB for FR1; +/- 3.5 dB for FR2 with side condition on CMR=-3dB.

For CMR + NZP-IMR scenario, the simulation results of L1-SINR measurement accuracy are provided. There are two options of SNR side conditions are further discussed:

- Option-1: Side condition (SNR) on CMR = 0dB, Side condition (SNR) on IMR = 0dB (as agreed simulation assumption)

- Option-2: Side condition (SNR) on CMR = -3dB, Side condition (SNR) on IMR = -3dB

For Option-1: Side condition (SNR) on CMR = 0dB, Side condition (SNR) on IMR = 0dB (as agreed simulation assumption), the simulation assumption and the simulation results are provided as below.

- L1-SINR accuracy evaluation simulation assumption for SSB-based CMR + NZP-IMR: Parameters; Values. SCS; 15kHz (FR1), 30kHz (FR1), 120kHz (FR2). Channel measurement resource (CMR); SSB. Interference measurement resource (IMR) configuration; CSI-RS. Periodicity; CMR Periodicity = IMR Periodicity. Ideal SINR; -3dB. Side condition (SNR) on CMR; option-1: 0dB, other options not excluded. Side condition (SNR) on IMR; option-1: 0dB, other options not excluded. Density (D) for IMR; 3. Number of samples (M) for IMR/CMR; 1, 3, 5. Number of PRBs for IMR; 48. Propagation condition; AWGN.

Table 2. SSB based CMR and NZP IMR L1-SINR absolute accuracy (dB), CMR and IMR at SNR of 0dB with 1, 3, 5 sample(s)

| SCS | # of samples | 5%-tile | 50%-tile | 95%-tile |
| --- | --- | --- | --- | --- |
| 15 kHz | 1 | -1.15 | 0.03 | 1.15 |
|  | 3 | -0.64 | 0.03 | 0.66 |
|  | 5 | -0.49 | 0.01 | 0.54 |
| 30 kHz | 1 | -1.11 | 0.03 | 1.19 |
|  | 3 | -0.63 | 0.03 | 0.65 |
|  | 5 | -0.47 | 0.05 | 0.53 |
| 120 kHz | 1 | -1.15 | 0.01 | 1.13 |
|  | 3 | -0.63 | 0.02 | 0.63 |
|  | 5 | -0.48 | 0.00 | 0.52 |

However, in this case, the side conditions among different L1-SINR scenario are not consistent, and it would lead to inconsistent SNR operation among scenarios.

Proposal 3: For L1-SINR simulation assumption, the side condition on CMR across cases should be the same.

In option-2, CMR and IMR at SNR of -3dB to align with other scenarios are suggested.

- Option-2: Side condition (SNR) on CMR = -3dB, Side condition (SNR) on IMR = -3dB

| Parameters | Values |
| --- | --- |
| SCS | 15kHz (FR1), 30kHz (FR1), 120kHz (FR2) |
| Channel measurement resource (CMR) | SSB |
| Interference measurement resource (IMR) configuration | CSI-RS |
| Periodicity | CMR Periodicity = IMR Periodicity |
| Ideal SINR | -4.77dB |
| Side condition (SNR) on CMR | option-2: -3dB, other options not excluded |
| Side condition (SNR) on IMR | option-2: -3dB, other options not excluded |
| Density (D) for IMR | 3 |
| Number of samples (M) for IMR/CMR | 1, 3, 5 |
| Number of PRBs for IMR | 48 |
| Propagation condition | AWGN |

Table 3. SSB based CMR and NZP IMR L1-SINR absolute accuracy (dB), CMR and IMR at SNR of -3 dB with 1, 3, 5 sample(s)

| SCS | # of samples | 5%-tile | 50%-tile | 95%-tile |
| --- | --- | --- | --- | --- |
| 15 kHz | 1 | -1.30 | 0.21 | 1.62 |
|  | 3 | -0.66 | 0.20 | 1.01 |
|  | 5 | -0.44 | 0.19 | 0.85 |
| 30 kHz | 1 | -1.28 | 0.20 | 1.63 |
|  | 3 | -0.60 | 0.21 | 0.99 |
|  | 5 | -0.45 | 0.22 | 0.82 |
| 120 kHz | 1 | -1.30 | 0.18 | 1.58 |
|  | 3 | -0.61 | 0.19 | 0.96 |
|  | 5 | -0.41 | 0.19 | 0.80 |

From Table 3, for 1 sample SSB based CMR and NZP IMR measurement at SNR of -3 dB, the measurement accuracy about 2 dB can be achieved. Considering implementation margin, accuracy of 3.5dB is proposed in FR1. In FR2, no extra value is assumed as SS-SINR measurement.

Proposal 4: For SSB based CMR and NZP IMR L1-SINR for reporting, the absolute measurement accuracy is +/- 3.5 dB for FR1; +/- 3.5 dB for FR2 with side condition on CMR=-3dB and IMR=-3dB.

For SSB based CMR + ZP-IMR scenario, the simulation assumption and the simulation results are provided as below.

- L1-SINR accuracy evaluation simulation assumption for SSB-based CMR + ZP-IMR: Parameters; Values. SCS; 15kHz (FR1), 30kHz (FR1), 120kHz (FR2). Channel measurement resource (CMR); SSB. Interference measurement resource (IMR) configuration; CSI-RS. Periodicity; CMR Periodicity = IMR Periodicity. Ideal SINR; -3dB. Side condition (SNR) on CMR; -3dB. Side condition (SNR) on IMR; N/A (only AWGN noise). Density (D) for IMR; 3. Number of samples (M) for IMR/CMR; 1, 3, 5. Number of PRBs for IMR; 48. Propagation condition; AWGN.

Table 4. SSB based CMR and ZP-IMR L1-SINR absolute accuracy (dB) @ SNR=-3 dB, 1, 3, 5 sample(s)

| SCS | # of samples | 5%-tile | 50%-tile | 95%-tile |
| --- | --- | --- | --- | --- |
| 15 kHz | 1 | -1.00 | 0.40 | 1.69 |
|  | 3 | -0.40 | 0.38 | 1.14 |
|  | 5 | -0.22 | 0.41 | 0.99 |
| 30 kHz | 1 | -0.97 | 0.39 | 1.69 |
|  | 3 | -0.40 | 0.42 | 1.18 |
|  | 5 | -0.14 | 0.41 | 0.98 |
| 120 kHz | 1 | -1.02 | 0.40 | 1.65 |
|  | 3 | -0.43 | 0.39 | 1.13 |
|  | 5 | -0.22 | 0.39 | 0.99 |

From Table 4, for 1 sample SSB based CMR and ZP-IMR measurement at SNR of -3dB, the measurement accuracy about 2 dB can be achieved. Considering implementation margin, accuracy of 3.5dB is proposed in FR1. In FR2, no extra value is assumed as SS-SINR measurement.

Proposal 5: For SSB based CMR and ZP-IMR L1-SINR for reporting, the absolute measurement accuracy is +/- 3.5 dB for FR1; +/- 3.5 dB for FR2 with side condition on CMR=-3dB and IMR=-3dB.

For CSI-RS based CMR + NZP-IMR scenario, the simulation assumption and the simulation results are provided as below.

Firstly, the SNR of 0dB is assumed as agreed simulation assumption.

- Option-1: Side condition (SNR) on CMR = 0dB, Side condition (SNR) on IMR = 0dB

- L1-SINR accuracy evaluation simulation assumption for CSI-RS-based CMR + NZP-IMR: Parameters; Values. SCS; 15kHz (FR1), 30kHz (FR1), 120kHz (FR2). Channel measurement resource (CMR); CSI-RS. Interference measurement resource (IMR) configuration; CSI-RS. Periodicity; CMR Periodicity = IMR Periodicity. Ideal SINR; -3dB. Side condition (SNR) on CMR; option-1: 0dB, other options not excluded. Side condition (SNR) on IMR; option-1: 0dB, other options not excluded. Density (D) for CMR/IMR; 3. Number of samples (M) for CMR/IMR; 1, 3, 5. Number of PRBs for CMR/IMR; 48. Propagation condition; AWGN.

Table 5. CSI-RS based CMR and NZP IMR L1-SINR absolute accuracy (dB) @ SNR=0 dB, 1, 3, 5 sample(s)

| SCS | # of samples | 5%-tile | 50%-tile | 95%-tile |
| --- | --- | --- | --- | --- |
| 15 kHz | 1 | -1.29 | -0.12 | 0.97 |
|  | 3 | -0.78 | -0.12 | 0.48 |
|  | 5 | -0.62 | -0.13 | 0.35 |
| 30 kHz | 1 | -1.23 | -0.08 | 0.97 |
|  | 3 | -0.69 | -0.10 | 0.51 |
|  | 5 | -0.57 | -0.08 | 0.37 |
| 120 kHz | 1 | -1.26 | -0.11 | 0.95 |
|  | 3 | -0.74 | -0.11 | 0.46 |
|  | 5 | -0.59 | -0.11 | 0.33 |

In addition, based on Poposal 3, the simulation assumptions and results for CMR and IMR at SNR of -3dB are also provided below (option 2).

- Option-2: Side condition (SNR) on CMR = -3dB, Side condition (SNR) on IMR = -3dB

| Parameters | Values |
| --- | --- |
| SCS | 15kHz (FR1), 30kHz (FR1), 120kHz (FR2) |
| Channel measurement resource (CMR) | CSI-RS |
| Interference measurement resource (IMR) configuration | CSI-RS |
| Periodicity | CMR Periodicity = IMR Periodicity |
| Ideal SINR | -4.77dB |
| Side condition (SNR) on CMR | option-1: -3dB, other options not excluded |
| Side condition (SNR) on IMR | option-1: -3dB, other options not excluded |
| Density (D) for CMR/IMR | 3 |
| Number of samples (M) for CMR/IMR | 1, 3, 5 |
| Number of PRBs for CMR/IMR | 48 |
| Propagation condition | AWGN |

Table 6. CSI-RS based CMR and NZP IMR L1-SINR absolute accuracy (dB) @ SNR=-3dB, 1, 3, 5 sample(s)

| SCS | # of samples | 5%-tile | 50%-tile | 95%-tile |
| --- | --- | --- | --- | --- |
| 15 kHz | 1 | -1.54 | 0.09 | 1.51 |
|  | 3 | -0.83 | 0.08 | 0.91 |
|  | 5 | -0.59 | 0.09 | 0.72 |
| 30 kHz | 1 | -1.43 | 0.13 | 1.52 |
|  | 3 | -0.66 | 0.13 | 0.94 |
|  | 5 | -0.52 | 0.14 | 0.77 |
| 120 kHz | 1 | -1.49 | 0.10 | 1.47 |
|  | 3 | -0.77 | 0.10 | 0.88 |
|  | 5 | -0.61 | 0.11 | 0.71 |

From Table 6, for 1 sample CSI-RS based CMR and NZP IMR measurement at SNR of -3 dB, the measurement accuracy about 2 dB can be achieved. Considering implementation margin, accuracy of 3.5 dB is proposed in FR1. In FR2, no extra value is assumed as SS-SINR measurement.

Proposal 6: For CSI-RS based CMR and NZP IMR L1- SINR for reporting, the absolute measurement accuracy is +/- 3.5 dB for FR1; +/- 3.5 dB for FR2 with side condition on CMR=-3dB and IMR=-3dB.

For CSI-RS based CMR + ZP-IMR scenario, the simulation assumption and the simulation results are provided as below.

- L1-SINR accuracy evaluation simulation assumption for CSI-RS-based CMR + ZP-IMR: Parameters; Values. SCS; 15kHz (FR1), 30kHz (FR1), 120kHz (FR2). Channel measurement resource (CMR); CSI-RS. Interference measurement resource (IMR) configuration; CSI-RS. Periodicity; CMR Periodicity = IMR Periodicity. Ideal SINR; -3dB. Side condition (SNR) on CMR; -3dB. Side condition (SNR) on IMR; N/A (only AWGN noise). Density (D) for CMR/IMR; 3. Number of samples (M) for CMR/IMR; 1, 3, 5. Number of PRBs for CMR/IMR; 48. Propagation condition; AWGN.

Table 7. CSI-RS based CMR and ZP-IMR L1-SINR absolute accuracy (dB) @ SNR=-3 dB, 1, 3, 5 sample(s)

| SCS | # of samples | 5%-tile | 50%-tile | 95%-tile |
| --- | --- | --- | --- | --- |
| 15 kHz | 1 | -1.34 | 0.28 | 1.70 |
|  | 3 | -0.62 | 0.30 | 1.10 |
|  | 5 | -0.38 | 0.29 | 0.93 |
| 30 kHz | 1 | -1.26 | 0.32 | 1.72 |
|  | 3 | -0.59 | 0.34 | 1.19 |
|  | 5 | -0.33 | 0.31 | 0.97 |
| 120 kHz | 1 | -1.28 | 0.30 | 1.70 |
|  | 3 | -0.55 | 0.31 | 1.14 |
|  | 5 | -0.35 | 0.32 | 0.96 |

From Table 7, for 1 sample CSI-RS based CMR and ZP-IMR measurement at SNR of -3dB, the measurement accuracy about 2 dB can be achieved. Considering implementation margin, accuracy of 3.5dB is proposed in FR1. In FR2, no extra value is assumed as SS-SINR measurement.

Proposal 7: For CSI-RS based CMR and ZP-IMR L1- SINR for reporting, the absolute measurement accuracy is +/- 3.5 dB for FR1; +/- 3.5 dB for FR2 with side condition on CMR=-3dB and IMR=-3dB.

- Summary

In this paper, the update of L1-SINR accuracy requirement are provided. We have the following proposals:

Proposal 1: For L1-SINR simulation assumption, the ZP-IMR should be corrected to CSI-IM.

Proposal 2: For SSB based CMR L1- SINR for reporting, the absolute measurement accuracy is +/- 3.5 dB for FR1; +/- 3.5 dB for FR2 with side condition on CMR=-3dB.

Proposal 3: For L1-SINR simulation assumption, the side condition on CMR across cases should be the same.

Proposal 4: For SSB based CMR and NZP IMR L1-SINR for reporting, the absolute measurement accuracy is +/- 3.5 dB for FR1; +/- 3.5 dB for FR2 with side condition on CMR=-3dB and IMR=-3dB.

Proposal 5: For SSB based CMR and ZP-IMR L1-SINR for reporting, the absolute measurement accuracy is +/- 3.5 dB for FR1; +/- 3.5 dB for FR2 with side condition on CMR=-3dB and IMR=-3dB.

Proposal 6: For CSI-RS based CMR and NZP IMR L1- SINR for reporting, the absolute measurement accuracy is +/- 3.5 dB for FR1; +/- 3.5 dB for FR2 with side condition on CMR=-3dB and IMR=-3dB.

Proposal 7: For CSI-RS based CMR and ZP-IMR L1- SINR for reporting, the absolute measurement accuracy is +/- 3.5 dB for FR1; +/- 3.5 dB for FR2 with side condition on CMR=-3dB and IMR=-3dB.

References

[1] R4-1915850_WF MIMO RRM_1