3GPP TSG-RAN WG4 Meeting #94-e	R4-2000944

Electronic Meeting, February 24 – March 6, 2020

Agenda item:	8.9.1.2

Source: 		NTT DOCOMO, INC.

Title: 	Discussion on UE performance requirements for URLLC

Document for:	Approval

- Introduction

In RAN4 #93, WF for NR URLLC was approved [1]. In this contribution, we provide views on UE performance requirements for URLLC.

2.	Discussion

2.1	TDD configurations

There are several requirements for TDD patterns utilized for URLLC. Firstly, we need to consider synchronization to the existing cells in order to avoid cross-link interference (CLI). If TDD configs are not aligned to the existing cell, we need to deploy URLLC cells only in isolated areas, which causes large restriction for the URLLC deployment. Secondly, in order to achieve low latency, e.g., low ACK/NACK delay, it is better that downlink symbols and uplink symbols switches in a short period. Thirdly, it is important that DL:UL ratio of the TDD config is aligned to the URLLC traffic. In RAN1, DL:UL is assumed to be 1:1 for many scenarios. Among them, deployment flexibility is really important for promoting URLLC service. For this motivation, it is beneficial to apply same TDD configurations, which are supported for eMBB. It is also beneficial for simplicity of the test. Other TDD configs can be additionally supported that can be more suitable for URLLC.

Proposal 1: Following TDD configs should be supported for URLLC in order to avoid CLI.

- 1st priority

  - 30kHz SCS: DDDSUUDDDD, S=6D:4G:4U

  - 120kHz SCS: DDDSU, S=10D:2G:2U

- 2nd priority

  - 30kHz SCS: DSUU, S=12D:2G

2.2.	Non-slot based transmission

For low latency, non-slot based PUSCH with shorter symbol should be considered. According to TS 38.211, for mapping type B, the number of symbols is configurable from{2, 4, 7} for PDSCH and {1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14} for PUSCH.

Table 5.1.2.1-1: Valid S and L combinations (PDSCH)

| PDSCH mapping type | Normal cyclic prefix |  |  | Extended cyclic prefix |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | S | L | S+L | S | L | S+L |
| Type A | {0,1,2,3} (Note 1) | {3,…,14} | {3,…,14} | {0,1,2,3} (Note 1) | {3,…,12} | {3,…,12} |
| Type B | {0,…,12} | {2,4,7} | {2,…,14} | {0,…,10} | {2,4,6} | {2,…,12} |
| Note 1: S = 3 is applicable only if dmrs-TypeA-Position = 3 |  |  |  |  |  |  |

Table 6.1.2.1-1: Valid S and L combinations (PUSCH)

| PUSCH mapping type | Normal cyclic prefix |  |  | Extended cyclic prefix |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | S | L | S+L | S | L | S+L |
| Type A | 0 | {4,…,14} | {4,…,14} | 0 | {4,…,12} | {4,…,12} |
| Type B | {0,…,13} | {1,…,14} | {1,…,14} | {0,…, 11} | {1,…,12} | {1,…,12} |

In Rel. 15 UE demodulation requirements, non-slot performance requirement with L =7 is supported. It is better to have other non-slot duration for URLLC requirements, which is L=2 and 4.

Proposal 2: For non-slot based transmission, L= 2 and 4 should be supported.

3.	Conclusion

In this contribution, we provide views on UE performance requirements for URLLC. The following proposals are obtained.

Proposal 1: Following TDD configs should be supported for URLLC in order to avoid CLI.

- 1st priority

  - 30kHz SCS: DDDSUUDDDD, S=6D:4G:4U

  - 120kHz SCS: DDDSU, S=10D:2G:2U

- 2nd priority

  - 30kHz SCS: DSUU, S=12D:2G

Proposal 2: For non-slot based transmission, L= 2 and 4 should be supported.

References

- R4-1915913, “Way forward on NR URLLC demodulation and CSI requirements”, Huawei, RAN4 #93, November 2019.