3GPP TSG-RAN WG4 Meeting #94-e	R4-2000950

Electronic Meeting, 24 Feb. – 6 Mar., 2020

Agenda item:			8.17.2.1.3

Source:				NTT DOCOMO, INC.

Title:		Views on HST single tap for NR

Document for:	Discussion

1. Introduction

At the previous meeting, we discussed UE demodulation requirements for Rel.16 NR HST test and agreed a WF [1]. This contribution presents our views on HST single tap for NR downlink.

2. Discussion

2.1. Target maximum Doppler frequency

In the last RAN4 meeting, we agreed two options on maximum Doppler shift values as follows.

- Maximum Doppler frequency - For 15KHz SCS, 500km/h - Option 1: 1250Hz - Option 2: 875Hz

Theoretically, maximum Doppler shift that can be compensated at receiver side can be presented as

fd= 12∆t ,

where ∆t is the minimum time interval between two RS symbols that is used for frequency tracking. From this equation, TRS design with 4 symbol separation can compensate maximum Doppler shift up to 3500 Hz for 30 kHz SCS and 1750 Hz for 15 kHz SCS. It means that 1250 Hz Maximum Doppler frequency can be compensated.

Proposal 1: Target Doppler frequency in the Single-tap test as follows.

- Maximum Doppler frequency

  - For 15KHz SCS, 500km/h

    - Option 1: 1250Hz

2.2. Single-tap requirements definition

In the last RAN4 meeting, we agreed some options about Single-tap requirements definition.

- Single tap requirements definition - Option A: Define requirements based on worst case and UE performs multi-shot TRS-based time/freq tracking - Option B: Define requirements under assumption UE is aware on HST single tap conditions and can adjust time/freq tracking algorithms - Further study how UE can become aware on conditions - Option 1: UE detects the conditions - Option 2: Rely on agreed NR HST RRM enhancement network assistance signaling - Option 3: Additional network assistance is provided - Further study UE demodulation performance under HST single tap conditions for the case of single-shot and multi-shot TRS-based tracking

If variation of the Doppler shift is large, it is better to use single-shot TRS-based tracking in terms of the performance. However, we already made requirements for single-tap with multi-shot TRS-based tracking in Rel.15 NR. In this sense, making multi-shot TRS-based tracking requirements is the first priority.

Proposal 2: Introduce the multi-shot TRS-based requirements is baseline.

3. Conclusion

In this contribution, we present our views on HST single-tap tests for NR. Observation and proposals are presented as follows.

Proposal 1: Target Doppler frequency in the Single-tap test as follows.

- Maximum Doppler frequency

  - For 15KHz SCS, 500km/h

    - Option 1: 1250Hz

Proposal 2: Introduce the multi-shot TRS-based requirements is baseline.

References

- R4-1915926, CMCC , “WF on UE demodulation for NR HST,” Nov. 2019