3GPP TSG-RAN WG4 Meeting #94-e	R4- 2000951

Electronic Meeting, 24 Feb. – 6 Mar., 2020

Agenda item:			8.17.2.1.4

Source:				NTT DOCOMO, INC.

Title:		Views on High-speed multi-path fading channels

Document for:	Discussion

1. Introduction

This contribution summarizes simulation results for multi-path fading channel. The simulation assumption in this contribution is based on the agreed WF [1].

2. Simulation results

This section provides the simulation results for TDD Rank1. Table A1 provides general simulation assumption in this contribution. Other major parameters are same as Rel-15 parameters for UE demodulation requirements.

2.1. TDD Rank 1 with 2x2 and 2x4

Fig 1. Simulation results

From the link simulation results, we have following observation.

Observation: 70% maximum throughput can be achieved with a reasonable SNR with TDD Rank 1

| Antenna configuration | Rank | MCS | SNR @ 70% maximum Throughput (dB) |
| --- | --- | --- | --- |
| 2x2 | 1 | 4 | -2.7 |
|  |  | 13 | 5.8 |
|  |  | 17 | 9.6 |
| 2x4 |  | 4 | -5.8 |
|  |  | 13 | 2 |
|  |  | 17 | 5.5 |

In the previous meeting, we discussed whether to use Rank 1 or Rank 2. From TR 38.822 [2], 2 additional DMRS+Rank1 is mandatory feature. For this reason and observation 1, Rank 1 is preferable.

Proposal: Performance requirements for HST under multi-path fading channel should be designed with Rank=1, since rank2 is mandatory with capability signaling for 2 additional DM-RS.

3. Conclusion

In this contribution, we present our views on HST tests for NR.

Observation 1: 70% maximum throughput can be achieved with a reasonable SNR with TDD Rank 1

| Antenna configuration | Rank | MCS | SNR @ 70% maximum Throughput (dB) |
| --- | --- | --- | --- |
| 2x2 | 1 | 4 | -2.7 |
|  |  | 13 | 5.8 |
|  |  | 17 | 9.6 |
| 2x4 |  | 4 | -5.8 |
|  |  | 13 | 2 |
|  |  | 17 | 5.5 |

Proposal: Performance requirements for HST under multi-path fading channel should be designed with Rank=1, since rank2 is mandatory with capability signaling for 2 additional DM-RS.

References

- R4-1915926, CMCC , “WF on UE demodulation for NR HST,” Nov. 2019

- TR 38.822, 3GPP

Appendix

Table A1. General simulation assumption

| Parameter | Value |  |
| --- | --- | --- |
|  | FDD 15KHz SCS | TDD 30KHz SCS |
| Antenna configuration | 2x2, 2x4 |  |
| DMRS type | type 1 |  |
| Number of DMRS symbols | DMRS 1+1+1 |  |
| TDD pattern | 7D1S2U, S: 6D 4G 4U |  |
| MCS | MCS 4; MCS 13; MCS 17 based on 64QAM table |  |
| Propagation condition | TDL-C 300ns |  |
| TRS periodicity | 10ms, 2slot pattern |  |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |  |
| Rank | Rank = 1 and 2 |  |
| BW | 10MHz | 40MHz |
| Maximum Doppler shift | 600Hz | 1200Hz |
| Testing metric | SNR @70% of maximum throughput |  |
| Note : The other parameters are same as Rel-15 |  |  |