TSG-RAN Working Group 4 (Radio) meeting #94-e	R4-2001020

Electronic Meeting, February 24th – March 06th 2020

Source:	Ericsson

Title:	TP to TR 38.xxx: Addition of background information for FR2 reference sensitivity in subclause 10.3

Agenda item:	8.5.4.2.2

Document for:	Approval

# Introduction

At the last RAN4 meeting (RAN4#93 in Reno) technical background information was included in the IAB technical report [1]. Following the RAN4 IAB WI schedule it is time to start to capture technical background information in the technical report to facilitate efficient reqirement drafting of the technical specification (TS 38.174).

At last meeting, initial examples of draft specification text were presented in [2]. From the discussion it can be concluded that background information needs to be captured in the technical report before the specification can be created. But due to the WI progress the text for the TS and TR will be discussed in parallel.

In this contribution a text proposal with technical background information relevant for the IAB-Node FR2 reference sensitivity requirement have been created. The text proposal is attached at the end of this contribution and is presented for approval to be included in the IAB technical report (TR 38.xxx).

# Discussion

For an IAB-Node the antenna array characteristics are essential to be able to provide access coverage as well as providing backhaul transmission. Since the reference sensitivity requirement is used as foundation for other uplink requirements the design of the reference sensitivity requirement should account for that.

The FR2 BS requirement was designed to include the array antenna characteristics. To be implementation neutral the requirement is based on a declaration of the OTA sensitivity level within a declared spatial angular range, referred to as OTA REFSENS RoAoA. The spatial angular region is defined as the Range of Angles of Arrival (RoAoA) determined by the contour defined by the points at which the achieved EIS is 3 dB higher than the achieved EIS in the reference direction. In practice the contour will be related to the average element/sub-array radiation pattern 3dB beam-width(s).

The OTA sensitivity level is defined as an EIS level corresponding to 95% link throughput for a defined Fixed Reference Channel (FRC). The EIS value declared by the vendor is mapped towards the class as described in Table 2-1.

Table 2-1: EIS class ranges

| Class | EISREFSENS_50M range (dBm) |
| --- | --- |
| Wide Area | -96 to -119 |
| Medium Range | -91 to -114 |
| Local Area | -86 to -109 |

By using this approach, the IAB-Node OTA reference sensitivity can be specified for the IAB-DU functionality and the IAB-MT functionality. Hence the IAB-DU and IAB-MT reference sensitivity is specified independently with different FRC and separate declarations.

Summary;

- In subclause 10.1, a general section is added for FR1 and FR2 common issues.

- A subclause 10.3.1 is added as a place holder for FR1 technical background information.

- A subclause 10.3.2 is added for FR2 technical background.

# Conclusion

At the end of this contribution a text proposal with technical background information relevant for OTA reference sensitivity is attached. The text proposal is presented for approval.

# References

[1]	R4-1913332, “Updated IAB TR”, Samsung, RAN4#93

[2] 	R4-1914525, “Draft specification text for IAB reference sensitivity in subclause 7.2 and subclause 10.3”, Ericsson, RAN4#93

[3]	R4-2001019, “Technical background for IAB-Node reference sensitivity”, Ericsson

TEXT PROPOSAL:

# 10	Radiated receiver characteristics

## 10.1	General

Radiated receiver reference sensitivity is specified at RIB for IAB type 1-O, or IAB type 2-O, with full complement of transceivers for the configuration in normal operating condition. For OTA reference sensitivity, the following arrangements apply;

-	Requirements apply during the IAB-Node receive period.

-	Requirements shall be met for any transmitter setting.

-	Throughput requirements defined for the radiated receiver characteristics do not assume HARQ retransmissions.

-	When the IAB-Node is configured to receive multiple carriers, all the throughput requirements are applicable for each received carrier.

-	For ACS, blocking and intermodulation characteristics, the negative offsets of the interfering signal apply relative to the lower RF Bandwidth edge or sub-block edge inside a sub-block gap, and the positive offsets of the interfering signal apply relative to the upper RF Bandwidth edge or sub-block edge inside a sub-block gap.

-	Each requirement shall be met over the RoAoA specified.

In normal operating condition the IAB-Node in TDD operation is configured to TX OFF power during receive period.

The OTA reference sensitivity is defined in an angular region referred to as OTA REFSENS RoAoA, where the RoAoA determined by the contour defined by the points at which the achieved EIS is 3dB higher than the achieved EIS in the reference direction assuming that for any AoA, the receiver gain is optimized for that AoA. In practice the contour will be related to the average element/sub-array radiation pattern 3dB beamwidth.

## 10.2	OTA sensitivity

Detailed structure of the subclause is TBD.

## 10.3	OTA reference sensitivity level

Detailed structure of the subclause is TBD.

### 10.3.1	IAB Node 1-O

Detailed structure of the subclause is TBD.

### 10.3.2	IAB Node 2-O

The OTA sensitivity requirement is defined in an angular region called OTA REFSENS RoAoA. The OTA REFSENS RoAoA defines the angular region over which the EISREFSENS value is fulfilled.

For FR2 requirements which are to be met over the OTA REFSENS RoAoA absolute requirement values are offset by the following term:

∆FR2_REFSENS=-3 dB for the reference direction

and

∆FR2_REFSENS=0 dB for all other directions

In order to allow for base station implementations suitable for different deployment scenarios a range of antenna gain is identified for each BS class. This range of antenna gain can then be used along with the implementation margin and SINR requirements to calculate a range of potential minimum EIS values. The vendor declares the OTA reference sensitivity within this range, depending on their view on the needed array size considering the propagation scenarios under which they expect their BS to operate.

The sensitivity level can be expressed in dBm as:

EISREFSENS=10log10kTB+F+IM+SNR-G		(Eq. 10.3.2-1)

, where k is Boltzmanns constant equal to 1.38.10-23 J/K, T is the temperature in K, B is the noise bandwidth of the FRC, F is the noise figure, IM is the implementation margin, SNR is the required signal-to-noise ratio and G is the antenna gain.

Since the peak gain depends on the array size, it was concluded to defined multiple ranges mapped to IAB classes account for different types of implementations. The declared EIS level needs to be with the EIS range specified for the IAB class as described in table 10.3.2-1.

Table 10.3.2-1: IAB-Node EIS ranges

| IAB Class | EISREFSENS_50M range (dBm) |
| --- | --- |
| Wide Area | -96 to -119 |
| Medium Range | -91 to -114 |
| Local Area | -86 to -109 |

The IAB-DU reference sensitivity requirement level for specific parameters are listed in table 10.3.2-2.

Table 10.3.3-1: IAB-DU reference sensitivity requirement

| Channel bandwidth (MHz) | Sub-carrier spacing (kHz) | FRC | EISREFSENS level (dBm) |
| --- | --- | --- | --- |
| 50, 100, 200 | 60 | G-FR2-A1-1 | EISREFSENS_50M + ΔFR2_REFSENS |
| 50 | 120 | G-FR2-A1-2 | EISREFSENS_50M + ΔFR2_REFSENS |
| 100, 200, 400 | 120 | G-FR2-A1-3 | EISREFSENS_50M + 3 + ΔFR2_REFSENS |
| NOTE 1: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full IAB-Node channel bandwidth. NOTE 2: The declared EISREFSENS_50M shall be within the range specified above. NOTE 3: The FRCs are defined in TBD. |  |  |  |

The IAB-MT reference sensitivity requirement level for specific parameters are listed in table 10.3.2-3.

Table 10.3.2-3: IAB-MT reference sensitivity requirement

| Channel bandwidth (MHz) | Sub-carrier spacing (kHz) | FRC | EISREFSENS level (dBm) |
| --- | --- | --- | --- |
| 50, 100, 200, 400 | 60 | G-FR2-TBD | EISREFSENS_50M + ΔFR2_REFSENS |
| 50, 100, 200, 400 | 120 | G-FR2-TBD | EISREFSENS_50M + ΔFR2_REFSENS |
| NOTE 1: The declared EISREFSENS_50M shall be within the range specified above. NOTE 2: The FRCs are defined in TBD. |  |  |  |