3GPP TSG-RAN WG4 Meeting #94-e                                        	R4-2001128

Online, 24th Feb. – 06th March 2020

Source:	LG Electronics

Title:	TP on update of summary of self-interference analysis for LTE x bands DL (x=1,2,3,4) and NR 2 bands DL DC band combinations in rel-16

Agenda item:			9.7.2

Document for:	Approval

1. Introduction

In this contribution, we captured the approved self- desense results in the individual EN-DC band combos in summary Table 5.3-1 and Table 5.3-5 based on the approved TPs at RAN4 #92BIS meeting for EN-DC of LTE (x bands DL/1UL, x=1,2,3,4) and NR (2 bands DL/1UL) band combinations for rel-16.

2. Text Proposal for TR 37.716-21-21

5.3	summary of self-interference analysis

Table 5.3-1 summarizes the EN- DC band combinations with self-interference problems for 3DL/2UL EN-DC operation.

Table 5.3-1: Summary of Self-interference analysis for LTE 1 band & NR 2 bands DL and 2 bands UL EN-DC operation

| Downlink band configuration | Uplink DC Configuration |  | Harmonic relation issues | intermodulation to own rx band | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- | --- |
| DC_1_n1-n78 | DC_1A_n78A |  |  | 4th IMD into n1 |  | - The 4th IMD already covered in 2DL/2UL DC_1A_n78A |
| DC_1_n3-n78 | DC_1A_n3A |  | 2nd harmonic from n3 into n78 | 2nd & 4th IMDs into n78 | Yes | - Harmonic problems already covered in DC_n3A-n78A. - 28.4 dB by 2nd IMD in rel-15 TR - 11.2 dB by 4th IMD in Rel-15 TR. - Small freq. gap was covered in Table 7.3.1A-0bA in TS36.101 |
|  | DC_1A_n78A |  | - | 2nd IMD into n3 | - | - 27.9 dB by 2nd IMD in rel-15 TR. |
| DC_1_n7-n78 | DC_1A_n7A DC_1A_n7B |  | - | 4th & 5th IMDs into n78 | - | - 10.1 dB by 4th IMD problem. - 3.8 dB by 5th IMD problem That were already covered in DC_7_n1-n78 in rel-15 TR |
|  | DC_1A_n78A, DC_1A_n78(2A) |  | - | 4th IMD into B7 | - | -9.1dB by 4th IMD problem that was already covered in DC_1-7_n78 in rel-15 TR |
| DC_1_n78-n257 | DC_1A_n78A |  | 7th & 8th Harmonics from n78 into n257 | - | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I |  | 2nd harmonic from B1 into n78 | - | - | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
| DC_3_n1-n77 | DC_3A_n1A |  | 2nd harmonic from B3 into n77 | 2nd & 4th IMDs into n77 | Yes | - Harmonic problems will be covered in DC_3A_n77A. - 2nd & 4th IMDs problems was same as DC_3A_n1A-n78A - Small freq. gap was covered in Table 7.3.1A-0bA in TS36.101 |
|  | DC_3A_n77A |  | - | 2nd & 5th IMD into n1 | - | - 31.0 dB by 2nd IMD that follow DC_1A-3A_n77A in TR37.863-02-01 in rel-15. - Follow DC_3A_n1A-n78A for 5th IMD problem. |
| DC_3_n1-n78 | DC_3A_n1A |  | 2nd harmonic from 3 into n78 | 2nd & 4th IMDs into n78 | Yes | - Harmonic problems was already covered in DC_3A_n78A. - 2nd &4th IMDs problems same as DC_1A_n3A-n78A. - Small freq. gap was covered in Table 7.3.1A-0bA in TS36.101 |
|  | DC_3A_n78A |  | - | 5th IMD into n1 | - | - 3.5 dB by 5th IMD in rel-15 TR |
| DC_3_n3-n77 | DC_3A_n3A1 |  | 2nd harmonic into n77 | - | - | - This harmonic impact already covered in 2DL/2UL DC_3A-n77A |
|  | DC_3A_n77A |  | 2nd harmonic into n77 | - | - | - This harmonic impact already covered in 2DL/2UL DC_3A-n77A In rel-15 |
| DC_3_n3-n78 | DC_3A_n3A1 |  | 2nd harmonic into n78 | 2nd , 3rd ,4th & 5th IMDs into n3 | - | - This harmonic & IMD problems are already covered in 2DL/2UL DC_3A-n78A |
|  | DC_3A_n78A |  | 2nd harmonic into n78 | - | - | - This harmonic impact already covered in 2DL/2UL DC_3A-n78A In rel-15. |
| DC_3_n7-n78 | DC_3A_n7A DC_3A_n7B DC_3C_n7A DC_3C_n7B |  | 2nd harmonic from 3 into n78 | 3rd IMD into n78 | - | - Harmonic problem already solved in DC_3A_n78A - 16.1dB by 3rd IMD problem that already covered in DC_7A_n3A-n78A in rel-15 TR. |
|  | DC_3A_n78A, DC_3A_n78(2A) DC_3C_n78A |  | - | - | - | No issues |
| DC_3-3_n7-n78 | DC_3A_n7A DC_3A_n7B |  | 2nd harmonic from 3 into n78 | 3rd IMD into n78 | - | - Harmonic problem already solved in DC_3A_n78A - 16.1dB by 3rd IMD problem that already covered in DC_7A_n3A-n78A in rel-15 TR. |
|  | DC_3A_n78A |  | - | - | - | No issues |
| DC_3_n20-n78 | DC_3A_n20A |  | 2nd & 4th harmonics into n78 | 3rd & 5th IMDs into n78 | - | These harmonic issues are already covered in DC_3A_n78A and DC_20A_n78A - 16.1 dB by 3rd IMD in rel-15 TR - 4.5 dB by 5th IMD in rel-15 TR |
|  | DC_3A_n78A |  | - | - | - | No issues |
| DC_3_n28-n78 | DC_3A_n28A |  | 2nd harmonic into n78 5th harmonic into n78 | 5th IMD into n78 | - | - Harmonic problem already discussed and will be solved in DC_3A_n78A or DC_28A_n78A - 4.5dB by 5th IMD problem in rel-15 TR |
|  | DC_3A_n78A |  | 2nd harmonic into n78 | 2nd,4th & 5th IMDs into B3 4th & 5th IMDs into n78 | - | - 2nd harmonic problem already discussed and will be solved in DC_3A_n78A - 2nd,4th & 5th IMDs problems already discussed and will be solved in DC_3A_n78A - 4th & 5th IMDs problems are no issue for TDD band |
| DC_3_n78-n257 DC_3-3_n78-n257 | DC_3A_n78A |  | 7th & 8th harmonics from n78 into n257 | - | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I |  | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
| DC_5_n78-n257 | DC_5A_n78A |  | 7th & 8th harmonics from n78 into n257 | - | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  | DC_5A_n257A |  | 4th harmonic from B5 into n78 | - | - | - Harmonic problem will be solved in DC_5A_n78A |
| DC_7_n1-n3 | DC_7A_n1A |  |  |  | Yes | No issues |
|  | DC_7A_n3A |  |  |  | - | No issues |
| DC_7_n1-n78 | DC_7A_n1A, DC_7C_n1A |  |  | 4th & 5th IMDs into n78 | - | - 10.1 dB by 4th IMD in rel-15 TR. - 3.8 dB by 5th IMD in rel-15 TR. |
|  | DC_7A_n78A, DC_7C_n78A |  |  | 4th IMD into n1 | - | - 9.0 dB by 4th IMD in rel-15 TR. |
| DC_7_n3-n78 | DC_7A_n3A |  | 2nd harmonic from n3 into n78 | 3rd IMD into n78 | - | - Harmonic problem already solved in CA_n3A-n78A - 16.1 dB by 3rd IMD in rel-15 TR |
|  | DC_7A_n78A |  | - | 3rd & 4th IMDs into n3 | - | - 15.6 dB by 3rd IMD in rel-15 TR - 9.2 dB by 4th IMD in rel-15 TR. |
| DC_7_n28-n78 | DC_7A_n28A |  | 5th harmonic into n78 | 2nd,4th IMDs into n78 | - | - 5th harmonic problem will be solved in DC_28A_n78A - 29.7 dB by 2nd IMD in rel-15 TR - 10.3 dB by 4th IMD problem in rel-15 TR |
|  | DC_7A_n78A |  | - | 2nd & 5th IMDs into B28 4th IMD into n78 | - | - 28.8 dB by 2nd IMD in rel-15 TR - 4.0 dB by 5th IMD problems in rel-15 TR - 4th IMD problem is no issue for TDD band |
| DC_7-7_n78-n257 | DC_7A_n78A |  | 7th harmonic from B7 into n257 |  | - | - Up to 7th harmonic order will be not any impact to n257 |
|  | DC_7A_n257A |  | - | - | - | No issues |
| DC_7_n78-n257 | DC_7A_n78A |  | 7th & 8th harmonics from n78 into n257 | - | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  | DC_7A_n257A |  | - | - | - | No issues |
| DC_8_n8-n78 | DC_8A_n78A |  | - | 4th IMD into n8 | - | The 4th IMD issue already covered in DC_8A-n78A |
| DC_8_n78-n257 | DC_8A_n78A |  | 7th & 8th harmonics from n78 into n257 | - | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  | DC_8A_n257A |  | 4th harmonic from B8 into n78 | - | - | - Harmonic problem will be solved in DC_5A_n78A |
| DC_13_n5-n260 | DC_13A_n5A |  | - | - | - | No issue |
|  | DC_13A_n260A |  | - | - | - | No issue |
| DC_20_n1-n3 | DC_20A_n1A |  |  |  | - | No issues |
|  | DC_20A_n3A |  |  |  | - | No issues |
| DC_20_n1-n78 | DC_20A_n1A |  | 4th harmonic from B20 into n78 | 3rd IMD into n78 | - | - Harmonic problem already solved in DC_20A_n78A - 16.0 dB by 3rd IMD in rel-15 TR |
|  | DC_20A_n78A |  | - | 3rd IMD into n1 | - | - 15.3 dB by 3rd IMD in rel-15 TR |
| DC_20_n3-n78 | DC_20A_n3A |  | - 2nd harmonic from n3 into n78 - 4th harmonic from B20 into n78 | 3rd & 5th IMDs into n78 | - | - 16.1 dB by 3rd IMD in rel-15 TR - 4.5 dB by 5th IMD in rel-15 TR |
|  | DC_20A_n78A |  | - | 3rd IMD into n3 | - | - 15.7 dB by 3rd IMD in rel-15 TR |
| DC_20_n20-n78 | DC_20A_n78A |  | - | 4th IMD into n20 | - | This IMD impact already covered in 2DL/2UL DC_20A-n78A |
| DC_28_n8-n78 | DC_28A_n8A |  | 4th harmonic into n78 5th harmonic into n78 | 4th & 5th IMDs into n78 | - | - 4th harmonic problem will be solved in DC_8A-n78A - 5th harmonic problem will be solved in DC_28A-n78A - 9.1 dB by 4th IMD in rel-15 TR - 3.7 dB by 5th IMD in rel-15 TR |
|  | DC_28A_n78A |  | 5th harmonic into n78 | 5th IMD into B28 5th IMD into n8 | - | - 5th harmonic problem will be solved in DC_28A-n78A - 5th IMD problem already discussed and will be solved in DC_28A-n78A - 4.3 dB by 5th IMD in rel-15 TR |
| DC_28_n8-n258 | DC_28A_n8A |  | 33th harmonics into n258 27th harmonics into n258 | - | - | - No harmonic problems by 27th & 33th |
|  | DC_28A_n258A |  | - | - | - | - No issue |
| DC_28_n28-n78 | DC_28A_n78A |  | - | 5th IMD into n28 | - | This IMD impact already covered in 2DL/2UL DC_28A-n78A |
| DC_66_n5-n260 | DC_66A_n5A |  | 22th harmonics into n260 | - | - | Up to 7th harmonic order will be not any impact to n260. |
|  | DC_66A_n260A |  | - | - | - | No issues |
| DC_66_n71-n260 DC_66_n71-n260(2A) |  | DC_66A_n71A | 22th harmonics into n260 | - | - | Up to 7th harmonic order will be not any impact to n257. |
|  |  | DC_66A_n260A | - | - | - | No issues |
| DC_66_n71-n261 DC_66_n71-n261(2A) |  | DC_66A_n71A | 16th harmonics into n261 | - | - | Up to 7th harmonic order will be not any impact to n261. |
|  |  | DC_66A_n261A | - | - | - | No issues |
| DC_1_n77-n78 |  | DC_1A_n77A |  | 4th IMD & 5th IMD | - | These MSD issues are same in DC_1A_n77A. So the MSD level by 4th IMD can follow 2DL/2UL_DC_1A_n77A |
|  |  | DC_1A_n78A | 2nd harmonic | 4th IMD & 5th IMD |  | 2nd harmonic issue already specified in TS38.101-1 for DC_1A_n77A These MSD issues are same in DC_1A_n78A. So the MSD level by 4th IMD can follow 2DL/2UL_DC_1A_n78A |
| DC_3_n77-n78 |  | DC_3A_n77A | 2nd harmonic | 4th IMD & 5th IMD |  | 2nd harmonic issue already specified in TS38.101-1 for DC_3A_n78A These MSD issues are same in DC_3A_n77A. So the MSD level by 4th IMD can follow 2DL/2UL_DC_3A_n77A |
|  |  | DC_3A_n78A | 2nd harmonic | 4th IMD & 5th IMD |  | 2nd harmonic issue already specified in TS38.101-1 for DC_3A_n77A These MSD issues are same in DC_3A_n77A. So the MSD level by 4th IMD can follow 2DL/2UL_DC_3A_n78A |
| DC_19_n77-n78 |  | DC_19A_n77A | 4th harmonic | 2nd IMD |  | 4th harmonic & 2nd IMD will be impact to n78 band. However, consider specific band for n77 in Japan, there was no IMD problem |
|  |  | DC_19A_n78A | 4th & 5th harmonic | 2nd IMD |  | 5th IMD already specified for DC_19A_n77A in TS38.101-3 Consider specific band for n78 in Japan, there was no IMD problem |
| DC_21_n77-n78 |  | DC_21A_n77A |  | 4th IMD & 5th IMD | - | These MSD issues are same in DC_21A_n77A. However, RAN4 consider the specific band for n77 in Japan, there was no IMD problem |
|  |  | DC_21A_n78A |  | 4th IMD |  | These MSD issue is same in DC_21A_n78A. However, RAN4 consider the specific band for n78 in Japan, there was no IMD problem |
| DC_1_n40-n78 |  | DC_1A_n40A |  | 4th IMD & 5th IMD | - | - 9.8dB by 4th IMD - 4.2dB by 5th IMD |
|  |  | DC_1A_n78A |  | 4th IMD | - | - 10.6dB by 4th IMD |
| DC_3_n1-n79 |  | DC_3A_n1A |  | 5th IMD | Yes | - 4.7dB by 5th IMD |
|  |  | DC_3A_n79A |  | 5th IMD | - | - 3.6dB by 5th IMD |
| DC_3_n40-n78 |  | DC_3A_n40A | 2nd harmonic | 5th IMD | - | Harmonic issue already covered in DC_3A_n78A - 4.8dB by 5th IMD |
|  |  | DC_3A_n78A |  | 5th IMD | - | - 4.4dB by 5th IMD |
| DC_66_n25-n41 |  | DC_66A_n25A | - | - | - | No issue |
|  |  | DC_66A_n41A |  | 4th IMD | - | - 11.0dB by 4th IMD |
| DC_1_n77-n257 DC_1_n77(2A)-n257 |  | DC_1A_n77A | 7th & 8th Harmonics from n77 into n257 | - | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  |  | DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I | 2nd harmonic from B1 into n77 | - | - | - Harmonic problem already covered in DC_1A_n77A |
| DC_1_n77-n258 |  | DC_1A_n77A | 6th, 7th & 8th Harmonics from n77 into n258 | - | - | - No harmonic problems by 6th, 7th & 8th order between FR1 and FR2 |
|  |  | DC_1A_n258A | 2nd harmonic from B1 into n77 | - | - | - Harmonic problem already covered in DC_1A_n77A |
| DC_1_n78-n258 |  | DC_1A_n78A | 7th & 8th Harmonics from n78 into n258 | - | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  |  | DC_1A_n258A | - | - | - | No issue |
| DC_1_n79-n258 |  | DC_1A_n79A | 5th & 6th Harmonics from n79 into n258 | - | - | - Harmonic problem will be solved in DC_n79A-n258A (5th order) |
|  |  | DC_1A_n258A | - | - | - | No issue |
| DC_3_n1-n257 |  | DC_3A_n1A | - | - | Yes | No issue |
|  |  | DC_3A_n257A | - | - | - | No issue |
| DC_3_n77-n258 |  | DC_3A_n77A | 6th, 7th & 8th harmonics from n77 into n258 | - | - | - No harmonic problems by 6th ~ 8th order between FR1 and FR2 |
|  |  | DC_3A_n258A | 2nd harmonic from B3 into B77 | - | - | - Harmonic problem already covered in DC_3A_n77A |
| DC_3_n78-n258 |  | DC_3A_n78A | 7th & 8th harmonics from n78 into n258 | - | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  |  | DC_3A_n258A | 2nd harmonic from B3 into B78 | - | - | - Harmonic problem already covered in DC_3A_n78A |
| DC_3_n79-n258 |  | DC_3A_n79A | 5th & 6th Harmonics from n79 into n258 | - | - | - Harmonic problem will be solved in DC_n79A-n258A (5th order) |
|  |  | DC_3A_n258A | - | - | - | No issue |
| DC_8_n77-n257 DC_8_n77(2A)-n257 |  | DC_8A_n77A | 7th & 8th Harmonics from n77 into n257 | - | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  |  | DC_8A_n257A DC_8A_n257D DC_8A_n257G DC_8A_n257H DC_8A_n257I | 4th harmonic from B8 into n77 | - | - | - Harmonic problem already covered in DC_8A_n77A |
| DC_1_n5-n78 |  | DC_1A_n5A | - | 3rd IMD | - | - 18.1dB by 3rd IMD |
|  |  | DC_1A_n78A | - | 5th IMD | - | Already specified in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
| DC_3_n5-n78 |  | DC_3A_n5A, DC_3C_n5A | 2nd harmonic from B3 into n78 | 3rd & 5th IMDs | - | - 2nd harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - 16.1dB by 3rd IMD - FFS |
|  |  | DC_3A_n78A, DC_3C_n78A | - | - | - | No issue |
| DC_7_n5-n78 |  | DC_7A_n5A, DC_7C_n5A | - | 2nd & 4th IMDs | - | - 29.7dB by 3rd IMD - FFS |
|  |  | DC_7A_n78A, DC_7C_n78A | - | 2nd & 5th IMDs | - | Already specified these IMDs problems for DC_5-7_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
| DC_28_n5-n78 |  | DC_28A_n5A | 4th harmonic from n5 into n78 5th harmonic from B28 into n78 |  |  | - These harmonic problems already specified in Table 7.3B.2.3.1-1 in TS38.101-3 |
|  |  | DC_28A_n78A | - | 5th IMD | - | - 3.8dB by 5th IMD |
| DC_39_n41-n79 |  | DC_39A_n41A | 2nd harmonic from n41 into n79 | 2nd I& 5th MDs | - | - Over 4992MHz frequency in n79 was impacted by 2nd harmonic, but not used - 29.8dB by 2nd IMD - 4.4dB by 5th IMD |
|  |  | DC_39A_n79A | - | 2nd & 5th IMDs | - | - 30.2dB by 2nd IMD - 4.3dB by 5th IMD |
| DC_40_n41-n79 |  | DC_40A_n41A | 2nd harmonic from n40 into n79 2nd harmonic from n41 into n79 | 2nd & 4th IMDs | - | - FFS by 2nd harmonic from B40 - Over 4992MHz frequency in n79 was impacted by 2nd harmonic, but not used - 30.5dB by 2nd IMD - No impact to china band since over 4800MHz is considered in NR |
|  |  | DC_40A_n79A | - | 2nd , 4th & 5th IMDs | - | Band 40 and n41 should be operate the TDD synchronous operation No MSD into n41 |
| DC_2_n7-n66 |  | DC_2A_n7A | - | - | - | No issue |
|  |  | DC_2A_n66A | - | - | - | No issue |
| DC_12_n7-n66 |  | DC_12A_n7A | 3rd harmonic from B12 into n66 | - | - | - The harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 |
|  |  | DC_12A_n66A | - | - | - | No issue |
| DC_5_n7-n78 |  | DC_5A_n7A | - | 2nd & 4th IMDs | - | - These MSD issue follow DC_7_n5-n78 |
|  |  | DC_5A_n78A | - | 2nd IMD | Yes | - 2nd IMD problems was covered in DC_5A-7A_n78A. - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |
| DC_7_n7-n78 |  | DC_7A_n7A1 | - | - | - | No issue |
|  |  | DC_7A_n78A | - | - | Yes | - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |
| DC_12_n7-n78 |  | DC_12A_n7A | 5th harmonic from B12 into n78 | 4th IMD |  | - 5th harmonic issue will be covered in DC_12A_n78A - 8.4dB by 4th IMD |
|  |  | DC_12A_n78A | - | 2nd IMD | Yes | - 28.7dB by 2nd IMD - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |
| DC_66_n7-n78 |  | DC_66A_n7A | 2nd harmonic from B66 into n78 | 3rd IMD | - | - The harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 -reuse MSD of DC_3A_n7-n78 for 3rd IMD problem |
|  |  | DC_66A_n78A | - | - | Yes | - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |
| DC_2_n66-n78 |  | DC_2A_n66A | 2nd harmonic from B2 into n78 2nd harmonic from n66 inton78 | 2nd & 4th IMDs | - | - These harmonic problems already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - 29.4dB by 2nd IMD - 8.9 dB by 4th IMD |
|  |  | DC_2A_n78A | - | 4th IMD | - | 4th IMD problem same as DC_2A-66A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
| DC_12_n66-n78 |  | DC_12A_n66A | 2nd harmonic from n66 inton78 5th harmonic from B12 into n78 | 5th IMD |  | - 2nd harmonic is same as DC_66A_n78A and 5th harmonic will be covered in DC_12A_n78A - 4.1dB by 5th IMD |
|  |  | DC_12A_n78A | 3rd harmonic from B12 into n66 | 3rd IMD |  | - The harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - 16.5dB by 3rd IMD |
| DC_3-3_n1_n78 |  | DC_3A_n1A | 2nd harmonic from 3 into n78 | 2nd & 4th IMDs | - | - Harmonic problems was already covered in DC_3A_n78A. - 2nd &4th IMDs problems same as DC_1A_n3A-n78A. |
|  |  | DC_3A_n78A | - | 5th IMD | Yes | - 3.5 dB by 5th IMD in rel-15 TR - Small freq. gap was covered in Table 7.3.1A-0bA in TS36.101 |
| DC_7-7_n1_n78 |  | DC_7A_n1A |  | 4th & 5th IMDs | Yes | - 10.1 dB by 4th IMD in rel-15 TR. - 3.8 dB by 5th IMD in rel-15 TR. - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |
|  |  | DC_7A_n78A |  | 4th IMD | - | - 9.0 dB by 4th IMD in rel-15 TR. |
| DC_2_n41-n260 |  | DC_2A_n41A | High order harmonic into n260 | - | - | - No harmonic problem |
| DC_2_n41-n261 |  | DC_2A_n41A | High order harmonic into n261 | - | - | - No harmonic problem |
| DC_66_n41_n260 |  | DC_66A_n41A | High order harmonic into n260 | - | - | - No harmonic problem |
| DC_66_n41_n261 |  | DC_66A_n41A | High order harmonic into n261 | - | - | - No harmonic problem |
| DC_2_n5-n258 |  | DC_2A_n5A | High order harmonic into n258 | - | - | - No harmonic problem |
|  |  | DC_2A_n258A | - | - | - | No issue |
| DC_2_n5-n260 |  | DC_2A_n5A | High order harmonic into n260 | - | - | - No harmonic problem |
|  |  | DC_2A_n260A | - | - | - | No issue |
| DC_2_n5-n261 |  | DC_2A_n5A | High order harmonic into n261 | - | - | - No harmonic problem |
|  |  | DC_2A_n261A | - | - | - | No issue |
| DC_2_n71-n258 |  | DC_2A_n71A | High order harmonic into n258 | - | - | - No harmonic problem |
|  |  | DC_2A_n258A | - | - | - | No issue |
| DC_2_n71-n260 |  | DC_2A_n71A | High order harmonic into n260 | - | - | - No harmonic problem |
|  |  | DC_2A_n260A | - | - | - | No issue |
| DC_2_n71-n261 |  | DC_2A_n71A | High order harmonic into n261 | - | - | - No harmonic problem |
|  |  | DC_2A_n261A | - | - | - | No issue |
| DC_66_n5-n258 |  | DC_66A_n5A | High order harmonic into n258 | - | - | - No harmonic problem |
|  |  | DC_66A_n258A | - | - | - | No issue |
| DC_66_n5-n261 |  | DC_66A_n5A | High order harmonic into n261 | - | - | - No harmonic problem |
|  |  | DC_66A_n261A | - | - | - | No issue |
| DC_66_n71-n258 |  | DC_66A_n71A | High order harmonic into n258 | - | - | - No harmonic problem |
|  |  | DC_66A_n258A | - | - | - | No issue |
| DC_3_n77-n257 DC_3_n77(2A)-n257 |  | DC_3A_n77A | 7th & 8th harmonics from n77 into n257 | - | - | - No harmonic problems by 6th ~ 8th order between FR1 and FR2 |
|  |  | DC_3A_n257A, DC_3A_n257D, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into B77 | - | - | - Harmonic problem already covered in DC_3A_n77A |
| DC_7_n66-n78 |  | DC_7A_n66A | 2nd harmonic from n66 into n78 | 3rd IMD |  | - Harmonic problem already covered in DC_66A_n78A - 16.0dB by 3rd IMD |
|  |  | DC_7A_n78A | - | 4th IMD | Yes | - 4th IMD problem same as DC_7A-66A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
| DC_66_n66-n78 |  | DC_66A_n66A1 | 2nd harmonic from B66 into n78 | 2nd & 4th IMDs |  | - Harmonic problem already covered in DC_66A_n78A - FFS - FFS |
|  |  | DC_66A_n78A | - | 5th IMD |  | - Follow MSD result for DC_66A_n78A |
| DC_20_n1-n7 |  | DC_20A_n1A | - | - | - | No issue |
|  |  | DC_20A_n7A | - | - | - | No issue |
| DC_3_n1-n7 |  | DC_3A_n1A, DC_3C_n1A | - | - | - | No issue |
|  |  | DC_3A_n7A, DC_3C_n7A | - | - | - | No issue |
| DC_20_n1-n28 |  | DC_20A_n1A | - | - | - | No issue |
|  |  | DC_20A_n28A | 3rd harmonic from n28 into n1 | 5th IMD | - | - Harmonic problem already covered in DC_1A_n28A - No need 5th IMD problem due to restricted n28 freq. range in Europe |
| DC_3_n1-n28 |  | DC_3A_n1A, DC_3C_n1A | - | - | - | No issue |
|  |  | DC_3A_n28A, DC_3C_n28A | 3rd harmonic from n28 into n1 | 4th IMD | - | - Harmonic problem already covered in DC_1A_n28A - 4th IMD problem same as DC_1A-3A_28A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
| DC_20_n7-n28 |  | DC_20A_n7A | - | 3rd & 5th IMDs | - | - 15.7dB by 3rd IMD - 2.8dB by 5th IMD |
|  |  | DC_20A_n28A | - | 5th IMD | - | - 5th IMD problem same as DC_7A-20A_n28A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
| DC_20_n7-n78 |  | DC_20A_n7A | 4th harmonic from B20 into n78 | 2nd & 4th IMDs | - | - The harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - 28.8dB by 2nd IMD - 8.5dB by 4th IMD |
|  |  | DC_20A_n78A | - | 2nd IMD | Yes | - 2nd IMD problem same as DC_7A-20A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |
| DC_3_n7-n28 |  | DC_3A_n7A, DC_3C_n7A |  | 2nd IMD | - | - 29.0dB by 2nd IMD |
|  |  | DC_3A_n28A, DC_3C_n28A | - | 3rd IMD | - | - 2nd IMD problem same as DC_3A-7A_n28A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
| DC_2_n41-n71 |  | DC_2A_n41A | - | 2nd & 5th IMDs | - | - 28.7dB by 2nd IMD - 3.8dB by 5th IMD |
|  |  | DC_2A_n71A | 4th harmonic from n71 into n41 | 2nd IMD | - | The harmonic problem will be treated CA_n41A_n71A - 29.2dB by 2nd IMD |
| DC_66_n41-n71 |  | DC_66A_n41A | - | - | - | No issue |
|  |  | DC_66A_n71A | 4th harmonic from n71 into n41 | - | - | The harmonic problem will be treated CA_n41A_n71A |
| DC_18_n3-n78 |  | DC_18A_n3A | 2nd harmonic from n3 into n78 4th harmonic from B18 into n78 | 3rd & 5th IMDs | - | These harmonic problem already specified in in Table 7.3B.2.3.1-1 in TS38.101-3 - 15.2dB by 3rd IMD - 4.0dB by 5th IMD |
|  |  | DC_18A_n78A | - | 3rd IMD |  | - Only one Japan operation use B18 and the n78 freq. range is allocated 3600 to 3800 MHz. So No IMD problem |
| DC_28_n3-n78 |  | DC_28A_n3A | 2nd harmonic from n3 into n78 5th harmonic from B28 into n78 | 5th IMD | - | - 5th IMD problem same as DC_3A_n28A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
|  |  | DC_28A_n78A | 5th harmonic from B28 into n78 | 3rd IMD | - | - The harmonic problem already specified in in Table 7.3B.2.3.1-1 in TS38.101-3 - 3rd IMD already covered in DC_3A-28A_n78A |
| DC_8_n1-n78 |  | DC_8A_n1A | 4th harmonic from B8 into n78 | 3rd IMD | - | - The harmonic problem already specified in in Table 7.3B.2.3.1-1 in TS38.101-3 - 14.9dB by 3rd IMD |
|  |  | DC_8A_n78A | - | - | - | No issue |
| DC_3_n40-n41 |  | DC_3A_n40A | 3rd harmonic from B3 into n41 | - | - | CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band |
|  |  | DC_3A_n41A | - | - | Yes | CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |
| DC_3_n40-n79 |  | DC_3A_n40A | 2nd harmonic from n40 into n79 | 5th IMD | - | - The 2nd harmonic problem will be studied in CA_n40-n79. - 4.7dB by 5th IMD |
|  |  | DC_3A_n79A | - | 5th IMD | - | - 3.2dB by 5th IMD |
| DC_3_n41-n79 |  | DC_3A_n41A | 2nd harmonic from n41 into n79 | 2nd & 5th IMDs | Yes | - The 2nd harmonic problem will be studied in CA_n41-n79. - 30.8dB by 2nd IMD - 5.2dB by 5th IMD - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |
|  |  | DC_3A_n79A | - | 2nd & 5th IMDs | - | - Already MSD analysis are covered in DC_3A-41A_n79A |
| DC_8_n40-n41 |  | DC_8A_n40A | 3rd harmonic from B8 into n41 | - | - | - Harmonic problem will be studied in DC_8A_n41A. - CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band |
|  |  | DC_8A_n41A | - | 5th IMD | - | - CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band |
| DC_8_n40-n79 |  | DC_8A_n40A | 2nd harmonic from n40 into n79 5th harmonic from B8 into n79 | 4th IMD | - | - These harmonic problem will be treated lower order DC or CA band combos. - 10.7dB by 4th IMD |
|  |  | DC_8A_n79A | - | 4th IMD | - | - 9.2dB by 4th IMD |
| DC_8_n41-n79 |  | DC_8A_n41A | 2nd harmonic from n41 into n79 5th harmonic from B8 into n79 | 3rd IMD | - | - These harmonic problem will be treated lower order DC or CA band combos. - 16.3dB by 3rd IMD |
|  |  | DC_8A_n79A | 3rd harmonic from B8 into n41 | 3rd IMD | - | - 15.5dB by 3rd IMD |
| DC_39_n40-n41 |  | DC_39A_n40A | - | 3rd IMD | - | - CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band |
|  |  | DC_39A_n41A | - | - | - | - CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band |
| DC_39_n40-n79 |  | DC_39A_n40A | 2nd harmonic from n40 into n79 | 4th IMD | - | - The harmonic problem will be treated CA_n40-n79 band combos. - 5.8 dB by 4th IMD |
|  |  | DC_39A_n79A | - | - | - | No issue |
| DC_41_n78-n257 |  | DC_41A_n78A | 7th & 8th harmonics from n78 into n257 | - | - | No harmonic impact to n257 |
|  |  | DC_41A_n257A, DC_41A_n257G, DC_41A_n257H, DC_41A_n257I | - | - | - | No issue |
| DC_42_n78-n257 |  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - | - | - | No issue |
| DC_18_n78-n257 |  | DC_18A_n78A | 7th & 8th harmonics from n78 into n257 | - | - | No harmonic impact to n257 |
|  |  | DC_18A_n257A, DC_18A_n257G, DC_18A_n257H, DC_18A_n257I | - | - | - | No issue |
| DC_28_n78-n257 |  | DC_28A_n78A | 7th & 8th harmonics from n78 into n257 | - | - | No harmonic impact to n257 |
|  |  | DC_28A_n257A, DC_28A_n257G, DC_28A_n257H, DC_28A_n257I | - | - | - | No issue |
| DC_1_n3-n257 |  | DC_1A_n3A | High order harmonic into n257 | - | -- | No harmonic impact to n257 |
|  |  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | - | - | Yes | Cross isolation issue already covered in Table 7.3.1A-0bA in TS36.101 |
| DC_18_n3-n257 |  | DC_18A_n3A | High order harmonics into n257 | - | - | No harmonic problem to n257 |
|  |  | DC_18A_n257A, DC_18A_n257G, DC_18A_n257H, DC_18A_n257I | - | - | - | No issue |
| DC_28_n3-n257 |  | DC_28A_n3A | High order harmonics into n257 | - | - | No harmonic problem to n257 |
|  |  | DC_28A_n257A, DC_28A_n257G, DC_28A_n257H, DC_28A_n257I | - | - | - | No issue |
| DC_1_n3-n28 |  | DC_1A_n3A | - | - | - | No issue |
|  |  | DC_1A_n28A | - | 5th IMD | Yes | - The 5th IMD problem same as DC_1A-3A_n28A in Table 7.3B.2.3.5.2-1 in TS38.101-3 - The cross band isolation already treated in TS36.101 |
| DC_8_n3-n28 |  | DC_8A_n3A | - | 2nd & 5th IMDs | - | - 30.4dB by 2nd IMD - 2.6dB by 5th IMD |
|  |  | DC_8A_n28A | 2nd harmonic from B8 into n3 | - | - | The 2nd harmonic problem will be covered in DC_8A_n3A |
| DC_11_n77-n257 DC_11_n77(2A)-n257 |  | DC_11A_n77A | 7th & 8th harmonics from n77 into n257 | - | - | No harmonic problem to n257 |
|  |  | DC_11A_n257A DC_11A_n257D DC_11A_n257G DC_11A_n257H DC_11A_n257I | - | - | - | No issue |
| DC_2_n12-n258 |  | DC_2A_n12A | High order harmonic into n258 | - | - | No harmonic problem to n258 |
|  |  | DC_2A_n258A | - | - | - | No issue |
| DC_2_n12-n260 |  | DC_2A_n12A | High order harmonic into n260 | - | - | No harmonic problem to n260 |
|  |  | DC_2A_n260A | - | - | - | No issue |
| DC_2_n12-n261 |  | DC_2A_n12A | High order harmonic into n261 | - | - | No harmonic problem to n261 |
|  |  | DC_2A_n261A | - | - | - | No issue |
| DC_66_n12-n258 |  | DC_66A_n12A | High order harmonic into n258 | - | - | No harmonic problem to n258 |
|  |  | DC_66A_n258A | - | - | - | No issue |
| DC_66_n12-n260 |  | DC_66A_n12A | High order harmonic into n260 | - | - | No harmonic problem to n260 |
|  |  | DC_66A_n260A | - | - | - | No issue |
| DC_66_n12-n261 |  | DC_66A_n12A | High order harmonic into n261 | - | - | No harmonic problem to n261 |
|  |  | DC_66A_n261A | - | - | - | No issue |
| DC_7-7_n66-n78 |  | DC_7A_n66A | 2nd harmonic from n66 into n78 | 3rd IMD | - | The harmonic problem already covered in Table 7.3B.2.3.1-1 in TS38.101-3 - 15.1dB by 3rd IMD |
|  |  | DC_7A_n78A | - | 4th IMD | Yes | - The 4th IMD problem same as DC_7A-66A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |
| DC_13_n5-n48 |  | DC_13A_n48A | - | - | - | No issue |
| DC_13_n48-n66 |  | DC_13A_n48A | - | 3rd IMD | - | - FFS |
|  |  | DC_13A_n66A | 2nd harmonic from n66 into n48 | 5th IMD | - | - 2nd harmonic issue will be solved in CA_n48-n66 - FFS |
| DC_66_n5-n48 |  | DC_66A_n5A | 2nd harmonic from n66 into n48 | 5th IMD | - | - 2nd harmonic issue will be solved in CA_n48-n66 - FFS |
|  |  | DC_66A_n48A | - | - | - | - No issue |
| DC_2_n41-n66 |  | DC_2A_n41A | - | - | - | - No issue |
|  |  | DC_2A_n66A | - | - | - | - No issue |
| DC_2_n66-n71 |  | DC_2A_n66A | - | - | - | - No issue |
|  |  | DC_2A_n71A | - | - | - | - No issue |
| DC_66_n25-n71 |  | DC_66A_n25A | - | - | - | - No issue |
|  |  | DC_66A_n71A | 3rd harmonic from n71 into n25 | - | - | 3rd harmonic issue will be solved in CA_n25-n71 |
| DC_28_n7-n78 |  | DC_28A_n7A DC_28A_n7B | 5th harmonic from B28 into n78 | 2nd & 4th IMDs | - | - 5th harmonic issue will be solved in DC_28-n78 - These IMD issue were covered in DC_7_n28-n78 |
|  |  | DC_28A_n78A | - | 2nd IMD |  | - 2nd IMD issue was covered in DC_7-28_n78 |
| DC_11_n3-n28 |  | DC_11A_n3A | - | 5th IMD |  | - FFS |
|  |  | DC_11A_n28A | - | - | - | - No issue |
| DC_1_n8-n78 |  | DC_1A_n8A | 4th harmonic from n8 into n78 | 3rd IMD | - | - The harmonic problem already specified in in Table 7.3B.2.3.1-1 in TS38.101-3 - 14.9dB in DC_8A_n1A-n78A |
|  |  | DC_1A_n78A | - | 5th IMD | - | - 5th IMD already covered in DC_1A-8A_n78A |
| DC_1_n8-n40 |  | DC_1A_n8A | - | - | - | - No issue |
|  |  | DC_1A_n40A | - | 4th & 5th IMDs |  | - FFS - FFS |
| DC_20_n20-n75 |  | DC_20A_n20A2 | - | - | - | - No issue |
| DC_1_n28-n40 |  | DC_1A_n28A | - | 4th IMD | - | - FFS |
|  |  | DC_1A_n40A | - | 4th IMD | - | - FFS |
| DC_3_n1-n40 |  | DC_3A_n1A | - | 5th IMD | - | - FFS |
|  |  | DC_3A_n40A | - | - | - | - No issue |
| DC_3_n8-n40 |  | DC_3A_n8A | - | - | - | - No issue |
|  |  | DC_3A_n40A | - | - | - | - No issue |
| DC_3_n8-n78 |  | DC_3A_n8A | 4th harmonic from n8 into n78 | 3rd & 5th IMDs | - | - 4th harmonic issue was solved in DC_8A_n78A - FFS - FFS |
|  |  | DC_3A_n78A | - | - | - | - No issue |
| DC_3_n28-n40 |  | DC_3A_n28A | - | - | - | - No issue |
|  |  | DC_3A_n40A | - | - | - | - No issue |
| DC_7_n1-n40 |  | DC_7A_n1A | - | - | - | - No issue |
|  |  | DC_7A_n40A | - | 3rd & 5th IMDs | - | - FFS - FFS |
| DC_7_n8-n40 |  | DC_7A_n8A | - | 5th IMD | - | - FFS |
|  |  | DC_7A_n40A | - | - | - | - No issue |
| DC_7_n8-n78 |  | DC_7A_n8A | 4th harmonic from n8 into n78 | 2nd & 4th IMDs |  | - 4th harmonic issue was solved in DC_8A_n78A - FFS - FFS |
|  |  | DC_7A_n78A | - | 2nd & 5th IMDs |  | - FFS - FFS |
| DC_7_n28-n40 |  | DC_7A_n28A | - | - | - | - No issue |
|  |  | DC_7A_n40A | - | - | - | - No issue |
| DC_2_n7-n78 |  | DC_2A_n7A | 2nd harmonic from B2 into n78 | 5th IMD |  | - FFS |
|  |  | DC_2A_n78A | - | - | - | - No issue |
| DC_13_n7-n78 |  | DC_13A_n7A | - | 2nd & 4th IMDs | - | - FFS - FFS |
|  |  | DC_13A_n78A | - | 2nd IMD | - | - FFS |
| DC_2_n71-n261 |  | DC_2A_n71A | 15th harmonic from B2 into n261 | - | - | - No harmonic problem to n261 |
| DC_28_n77-n257 DC_28_n77(2A)-n257 |  | DC_28A_n77A | 7th & 8th harmonics from n77 into n257 | - | - | - No harmonic problem to n257 |
|  |  | DC_28A_n257A DC_28A_n257D DC_28A_n257G DC_28A_n257H DC_28A_n257I | 5th harmonic from B28 into n77 | - | - | - 5th harmonic problem was solved in DC_28A_n77A |
| DC_19_n77-n257 |  | DC_19A_n77A | 7th & 8th harmonics from n77 into n257 | - | - | - No harmonic problem to n257 |
|  |  | DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I | 4th harmonic from B19 into n77 | - | - | - 4th harmonic problem was solved in DC_19A_n77A |
| DC_21_n77-n257 |  | DC_21A_n77A | 7th & 8th harmonics from n77 into n257 | - | - | - No harmonic problem to n257 |
|  |  | DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I | - | - | - | - No issue |
| DC_42_n77-n257 |  | DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | - | - | - | - No issue |
| DC_19_n78-n257 |  | DC_19A_n78A | 7th & 8th harmonics from n78 into n257 | - | - | - No harmonic problem to n257 |
|  |  | DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I | 4th harmonic from B19 into n78 | - | - | - 4th harmonic problem was solved in DC_19A_n78A |
| DC_21_n78-n257 |  | DC_21A_n78A | 7th & 8th harmonics from n78 into n257 | - | - | - No harmonic problem to n257 |
|  |  | DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I | - | - | - | - No issue |
| DC_1_n79-n257 |  | DC_1A_n79A | 6th harmonic from n79 into n257 | - | - | - No harmonic problem to n257 |
|  |  | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I | - | - | - | - No issue |
| DC_3_n79-n257 |  | DC_3A_n79A | 6th harmonic from n79 into n257 | - | - | - No harmonic problem to n257 |
|  |  | DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I | - | - | - | - No issue |
| DC_19_n79-n257 |  | DC_19A_n79A | 6th harmonic from n79 into n257 | - | - | - No harmonic problem to n257 |
|  |  | DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I | 6th harmonic from B19 into n79 | - | - | - No harmonic problem to n79 |
| DC_21_n79-n257 |  | DC_21A_n79A | 6th harmonic from n79 into n257 | - | - | - No harmonic problem to n257 |
|  |  | DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I | - | - | - | - No issue |
| DC_42_n79-n257 |  | DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | - | - | - | - No issue |
| Note1: These band combinations should be completed the dual uplink EN-DC firstly Note 2: only single switched UL is supported |  |  |  |  |  |  |

Table 5.3-2 summarizes the EN-DC band combinations with self-interference problems for 4DL/2UL DC operation.

Table 5.3-2: Summary of Self-interference analysis for LTE 2 bands & NR 2 bands DL and 2 bands UL EN-DC operation

| Downlink band configuration | Uplink DC Configuration |  | Harmonic relation issues |  | intermodulation to own rx band |  | interference due to small frequency separation | MSD |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1-3_n7-n78 | DC_1A_n7A DC_1A_n7B |  | - |  | 4th & 5th IMDs into n78 |  | Yes | These IMD problems were already covered in DC_7_n1-n78 with 2UL_DC_7A_n1A |  |
|  | DC_1A_n78A, DC_1A_n78(2A) |  | - |  | 2nd IMD into n3 4th IMD into n7 |  | Yes | These IMD issues were already covered in DC_1A-3A_n78A and DC_1A_n7A-n78A. |  |
|  | DC_3A_n7A DC_3A_n7B |  | 2nd harmonic from 3 into n78 |  | 3rd IMD into n78 |  | - | - Harmonic problem was solved in DC_3A_n78A - 3rd IMD problem already covered in DC_3A_n7A-n78A. |  |
|  | DC_3A_n78A, DC_3A_n78(2A) |  |  |  | 5th IMD into B1 |  |  | -5th IMD problem was solved in 3DL_DC_1A-3A_n78A |  |
| DC_1-3_n28-n78 | DC_1A_n28A |  | 3rd harmonic into B1 5th harmonic into n78 |  | 3rd IMD into n78 5th IMD into B3 |  | Yes | - Harmonic problems already solved in DC_1A_n28A or DC_28A_n78A - 3rd IMD problem was solved in 3DL_DC_1A_n28A-n78A - 5th IMD problem was solved in 3DL_DC_1A-3A_n28A |  |
|  | DC_1A_n78A |  | - |  | 2nd IMD into B3 4th IMD into B1 4th IMD into n78 5th IMD into n28 |  | Yes | - 2nd IMD problem was solved in 3DL_DC_1A-3A_n78A - 4th IMD problem already covered in DC_1A_n78A - 4th IMD problem is no issue for TDD band - 5th IMD problem was solved in 3DL_DC_1A_n28A-n78A |  |
|  | DC_3A_n28A |  | 2nd harmonic into n78 3rd harmonic into B1 5th harmonic into n78 |  | 4th IMD into B1 5th IMD into n78 |  |  | - Harmonic problems already solved in DC_3A_n78A or DC_1A_n28A or DC_28A_n78A - 4th IMD problem was solved in 3DL_DC_1A-3A_n28A - 5th IMD problem was solved in 3DL_DC_3A_n28A-n78A |  |
|  | DC_3A_n78A |  | 2nd harmonic into n78 |  | 2nd,4th & 5th IMDs into B3 4th & 5th IMDs into n78 5th IMD into B1 |  |  | - 2nd harmonic problem already solved in DC_3A_n78A - 2nd,4th & 5th IMDs problems already solved in DC_3A_n78A - 4th & 5th IMDs problems are no issue for TDD band -5th IMD problem was solved in 3DL_DC_1A-3A_n78A |  |
| DC_1-3_n78-n257 | DC_1A_n78A |  | 7th & 8th Harmonics from n78 into n257 |  | 2nd IMD into B3 |  | Yes | - Up to 7th harmonic order will be not any impact to n257 - This IMD problem will be covered in DC_1A-3A_n78A - Proximity also follow LTE_CA_1A-3A |  |
|  | DC_1A_n257A/D/G/H/I |  | 2nd harmonic from B1 into n78 |  | - |  | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |  |
|  | DC_3A_n78A |  | 7th & 8th harmonics from n78 into n257 |  | 5th IMD into B1 |  | - | - Up to 7th harmonic order will be not any impact to n257 - 5th IMD problems will be covered in DC_1A-3A_n78A. |  |
|  | DC_3A_n257A/D/G/H/I |  | 2nd harmonic from B3 into n78 |  | - |  | - | - Harmonic problem will be solved in DC_3A_n78A |  |
| DC_1-5_n78-n257 | DC_1A_n78A |  | 7th & 8th Harmonics from n78 into n257 |  | 5th IMD into B5 |  | - | - Up to 7th harmonic order will be not any impact to n257 - 5th IMD problems will be covered in DC_1A-5A_n78A. |  |
|  | DC_1A_n257A |  | 2nd harmonic from B1 into n78 |  | - |  | - | - Harmonic problem already discussed and will be solved in DC_1A_n78A |  |
|  | DC_5A_n78A |  | 7th & 8th harmonics from n78 into n257 |  | 3rd IMD into B1 |  | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 3rd IMD problems will be covered in DC_1A-5A_n78A. |  |
|  | DC_5A_n257A |  | 4th harmonic from B5 into n78 |  | - |  | - | - Harmonic problem will be solved in DC_5A_n78A |  |
| DC_1-7_n28-n78 | DC_1A_n28A |  | 3rd harmonic into B1 5th harmonic into n78 |  | 2nd IMD into B7 3rd IMD into n78 |  |  | - Harmonic problems already solved in DC_1A_n78A or DC_28A_n78A - 2nd IMD problem was solved in 3DL_DC_1A-7A_n28A - 3rd IMD problem was solved in 3DL_DC_1A_n28A-n78A |  |
|  | DC_1A_n78A |  | - |  | 4th IMD into B1 4th IMD into n7 5th IMD into n28 |  |  | - 4th IMD problem already solved in DC_1A_n78A - 4th IMD problem is no issue for TDD band - 5th IMD problem was solved in 3DL_DC_1A_n28A-n78A |  |
|  | DC_7A_n28A |  | 3rd harmonic into B1 5th harmonic into n78 |  | 2nd,4th IMDs into n78 |  | - | - Harmonic problem was solved in DC_1A_n28A or DC_28A_n78A - 2nd & 4th IMDs problems were solved in 3DL_DC_7A_n28A-n78A |  |
|  | DC_7A_n78A |  | - |  | 2nd & 5th IMDs into B28 4th IMD into n78 4th IMD into B1 |  | - | - 2nd & 5th IMDs problems were solved in 3DL_DC_7A-28A_n78A - 4th IMD problem is no issue for TDD band - 4th IMD problem was solved in 3DL_DC_1A-7A_n78A |  |
| DC_1-7-7_n78-n257 | DC_1A_n78A |  | 7th & 8th Harmonics from n78 into n257 |  | 4th IMD into B7 |  | - | - Up to 7th harmonic order will be not any impact to n257 - 4th IMD problems will be covered in DC_1A-7A_n78A. |  |
|  | DC_1A_n257A |  | 2nd harmonic from B1 into n78 |  | - |  | - | - Harmonic problem already discussed and will be solved in DC_1A_n78A |  |
|  | DC_7A_n78A |  | 7th harmonic from B7 into n257 |  | 4th IMD into B1 |  | - | - Up to 7th harmonic order will be not any impact to n257 - The IMD problem will be covered in DC_1A_7A-n78A |  |
|  | DC_7A_n257A |  | - |  | - |  | - | No issues |  |
| DC_1-7_n78-n257 | DC_1A_n78A |  | 7th & 8th Harmonics from n78 into n257 |  | 4th IMD into B7 |  | - | - Up to 7th harmonic order will be not any impact to n257 - 4th IMD problems will be covered in DC_1A-7A_n78A. |  |
|  | DC_1A_n257A |  | 2nd harmonic from B1 into n78 |  | - |  | - | - Harmonic problem already discussed and will be solved in DC_1A_n78A |  |
|  | DC_7A_n78A |  | 7th harmonic from B7 into n257 |  | 4th IMD into B1 |  | - | - Up to 7th harmonic order will be not any impact to n257 - The IMD problem will be covered in DC_1A_7A-n78A |  |
|  | DC_7A_n257A |  | - |  | - |  | - | No issues |  |
| DC_1-8_n78-n257 | DC_1A_n78A |  | 7th & 8th Harmonics from n78 into n257 |  | 5th IMD into B8 |  | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 5th IMD already covered in DC_1A-8A_n78A |  |
|  | DC_1A_n257A |  | 2nd harmonic from B1 into n78 |  | - |  | - | - Harmonic problem already discussed and will be solved in DC_1A_n78A |  |
|  | DC_8A_n78A |  | 7th & 8th Harmonics from n78 into n257 |  | - |  | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |  |
|  | DC_8A_n257A |  | 4th harmonic from B8 into n78 |  | - |  | - | - Harmonic problem will be solved in DC_8A_n78A |  |
| DC_3-5_n78-n257 | DC_3A_n78A |  | 7th & 8th harmonics from n78 into n257 |  | - |  | - | - Up to 7th harmonic order will be not any impact to n257 |  |
|  | DC_3A_n257A |  | 2nd harmonic from B3 into n78 |  | - |  | - | - Harmonic problem will be solved in DC_3A_n78A |  |
|  | DC_5A_n78A |  | 7th & 8th harmonics from n78 into n257 |  | 3rd IMD into B3 |  | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 3rd IMD problems will be covered in DC_3A-5A_n78A. |  |
|  | DC_5A_n257A |  | 4th harmonic from B5 into n78 |  | - |  | - | - Harmonic problem will be solved in DC_5A_n78A |  |
| DC_3-7_n28-n78 | DC_3A_n28A |  | 2nd harmonic into n78 5th harmonic into n78 |  | 3rd IMD into B7 5th IMD into n78 |  | - | - Harmonic problems already solved in DC_3A_n78A or DC_28A_n78A - 3rd IMD problem was solved in 3DL_DC_3A-7A_n28A - 5th IMD problem was solved in 3DL_DC_3A_n28A-n78A |  |
|  | DC_3A_n78A |  | 2nd harmonic into n78 |  | 2nd,4th & 5th IMDs into B3 4th & 5th IMDs into n78 |  |  | - 2nd harmonic problem already solved in DC_3A_n78A - 2nd,4th & 5th IMDs problems already solved in DC_3A_n78A - 4th & 5th IMDs problems are no issue for TDD band |  |
|  | DC_7A_n28A |  | 5th harmonic into n78 |  | 2nd IMD into B3 2nd,4th IMDs into n78 |  | - | - Harmonic problem was solved in DC_28A_n78A - 2nd IMD problems was solved in 3DL_DC_3A-7A_n28A - 2nd & 4th IMDs problems were solved in 3DL_DC_7A_n28A-n78A |  |
|  | DC_7A_n78A |  | - |  | 2nd & 5th IMDs into B28 3rd & 4th IMDs into B3 4th IMD into n78 |  | - | - 2nd & 5th IMDs problems were solved in 3DL_DC_7A_n28A-n78A - 3rd & 4th IMDs problems were solved in 3DL_DC_3A-7A_n78A - 4th IMD problem is no issue for TDD band |  |
| DC_3-7-7_n78-n257 DC_3-3-7_n78-n257 DC_3-3-7-7_n78-n257 | DC_3A_n78A |  | 7th & 8th harmonics from n78 into n257 |  | - |  | - | - Up to 7th harmonic order will be not any impact to n257 |  |
|  | DC_3A_n257A |  | 2nd harmonic from B3 into n78 |  | - |  | - | - Harmonic problem will be solved in DC_3A_n78A |  |
|  | DC_7A_n78A |  | 7th harmonic from B7 into n257 |  | 3rd & 4th IMDs into B3 |  | - | - Up to 7th harmonic order will be not any impact to n257 - The IMD problem will be covered in DC_3A-7A_n78A |  |
|  | DC_7A_n257A |  | - |  | - |  | - | No issues |  |
| DC_3-7_n78-n257 | DC_3A_n78A |  | 7th & 8th harmonics from n78 into n257 |  | - |  | - | - Up to 7th harmonic order will be not any impact to n257 |  |
|  | DC_3A_n257A |  | 2nd harmonic from B3 into n78 |  | - |  | - | - Harmonic problem will be solved in DC_3A_n78A |  |
|  | DC_7A_n78A |  | 7th harmonic from B7 into n257 |  | 3rd & 4th IMDs into B3 |  | - | - Up to 7th harmonic order will be not any impact to n257 - The IMD problem will be covered in DC_3A-7A_n78A |  |
|  | DC_7A_n257A |  | - |  | - |  | - | No issues |  |
| DC_3-8_n78-n257 | DC_3A_n78A |  | 7th & 8th harmonics from n78 into n257 |  | - |  | - | - Up to 7th harmonic order will be not any impact to n257 |  |
|  | DC_3A_n257A |  | 2nd harmonic from B3 into n78 |  | - |  | - | - Harmonic problem will be solved in DC_3A_n78A |  |
|  | DC_8A_n78A |  | 2nd harmonic from B8 into B3 |  | 3rd IMD into B3 |  | - | - Harmonic problem already be solved in CA_3A_8A - 3rd IMD already covered in DC_3A-8A_n78A |  |
|  | DC_8A_n257A |  | 4th harmonic from B8 into n78 |  | - |  | - | - Harmonic problem will be solved in DC_8A_n78A |  |
| DC_5-7-7_n78-n257 | DC_5A_n78A |  | 7th & 8th harmonics from n78 into n257 |  | 2nd IMD into B7 |  | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 2nd IMD problems will be covered in DC_5A-7A_n78A. |  |
|  | DC_5A_n257A |  | 4th harmonic from B5 into n78 |  | - |  | - | - Harmonic problem will be solved in DC_5A_n78A |  |
|  | DC_7A_n78A |  | 7th harmonic from B7 into n257 |  | 2nd & 5th IMDs into B5 |  | - | - Up to 7th harmonic order will be not any impact to n257 - The IMD problems will be covered in DC_5A-7A_n78A |  |
|  | DC_7A_n257A |  | - |  | - |  | - | No issues |  |
| DC_5-7_n78-n257 | DC_5A_n78A |  | 7th & 8th harmonics from n78 into n257 |  | 2nd IMD into B7 |  | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 2nd IMD problems will be covered in DC_5A-7A_n78A. |  |
|  | DC_5A_n257A |  | 4th harmonic from B5 into n78 |  | - |  | - | - Harmonic problem will be solved in DC_5A_n78A |  |
|  | DC_7A_n78A |  | 7th harmonic from B7 into n257 |  | 2nd & 5th IMDs into B5 |  | - | - Up to 7th harmonic order will be not any impact to n257 - The IMD problems will be covered in DC_5A-7A_n78A |  |
|  | DC_7A_n257A |  | - |  | - |  | - | No issues |  |
| DC_1-3_n77-n78 | DC_1A_n77A |  | - |  | 2nd, 4th & 5th IMDs into B3 4th & 5th IMD into n78 |  | Yes | These MSD issues are already covered in 3DL_DC_1A-3A_n77A with 2UL_DC_1A_n77A. These MSD issues are already covered in 3DL_DC_1A_n77A-n78A with 2UL_DC_1A_n77A. |  |
|  | DC_1A_n78A |  | 2nd harmonic from B1 into n77 |  | 2nd IMD into B3 4th & 5th IMDs into n77 |  | Yes | Harmonic problem already covered in DC_1A_n77A 2nd IMD problem already covered in 3DL_DC_1A-3A_n78A with 2UL_DC_1A_n78A 4th & 5th IMDs problem already covered in 3DL_DC_1A_n77A-n78A with 2UL_DC_1A_n78A |  |
|  | DC_3A_n77A |  | 2nd harmonic from B3 into n78 |  | 2nd & 5th IMD into B1 4th IMD & 5th IMD into n78 |  | - | 2nd harmonic issue already covered in DC_3A_n78A These MSD issues are already covered in 3DL_DC_1A-3A_n77A with 2UL_DC_3A_n77A and 3DL_3A_n77A-n78A with 2UL_DC_3A_n77A |  |
|  | DC_3A_n78A |  | 2nd harmonic from B3 into n77 |  | 4th & 5th IMDs into n77 5th IMD into B1 |  | - | 2nd harmonic issue already covered in DC_3A_n77A These MSD issues are already covered in 3DL_DC_1A-3A_n78A with 2UL_DC_3A_n78A and 3DL_3A_n77A-n78A with 2UL_DC_3A_n78A |  |
| DC_1-3_n77-n79 | DC_1A_n77A |  | - |  | 2nd, 4th & 5th IMDs into B3 3rd, 4th & 5th IMDs into n79 |  | Yes | These MSD issues are already covered in 3DL_DC_1A-3A_n77A with 2UL_DC_1A_n77A. These MSD issues will be covered in 3DL_DC_1A_n77A-n79A with 2UL_DC_1A_n77A. |  |
|  | DC_1A_n79A |  | 2nd harmonic from B1 into n77 |  | 5th IMD into n77 |  | Yes | 2nd harmonic issue already covered in DC_1A_n77A The MSD issue is already covered in 3DL_DC_1A_n77A-n79A with 2UL_DC_1A_n79A |  |
|  | DC_3A_n77A |  | - |  | 2nd & 5th IMD into B1 3rd & 4th IMD into n79 |  | - | These MSD issues are already covered in 3DL_DC_1A-3A_n77A with 2UL_DC_3A_n77A and 3DL_3A_n77A-n79A with 2UL_DC_3A_n79A |  |
|  | DC_3A_n79A |  | 2nd harmonic from B3 into n77 |  | 5th IMD into B1 5th IMD into n77 |  | - | 2nd harmonic issue already covered in DC_3A_n77A The MSD issue will be covered in 3DL_DC_1A-3A_n79A with 2UL_DC_3A_n79A and 3DL_3A_n77A-n79A with 2UL_DC_3A_n79A |  |
| DC_1-3_n78-n79 | DC_1A_n78A |  | - |  | 2nd IMD into B3 3rd & 5th IMDs into n79 |  | Yes | 2nd IMD problem already covered in 3DL_DC_1A-3A_n78A with 2UL_DC_1A_n78A 3rd & 5th IMDs problem will be covered in 3DL_DC_1A_n78A-n79A with 2UL_DC_1A_n78A |  |
|  | DC_1A_n79A |  | - |  | 5th IMD into n78 |  | Yes | The MSD issue is already covered in 3DL_DC_1A_n78A-n79A with 2UL_DC_1A_n79A |  |
|  | DC_3A_n78A |  | - |  | 5th IMD into B1 3rd IMD into n79 |  | - | These MSD issues are already covered in 3DL_DC_1A-3A_n78A with 2UL_DC_3A_n78A and 3DL_3A_n78A-n79A with 2UL_DC_3A_n78A |  |
|  | DC_3A_n79A |  | 2nd harmonic from B3 into n78 |  | 5th IMD into B1 5th IMD into n78 |  | - | 2nd harmonic issue already covered in DC_3A_n78A The MSD issue will be covered in 3DL_DC_1A-3A_n79A with 2UL_DC_3A_n79A and 3DL_3A_n78A-n79A with 2UL_DC_3A_n79A |  |
| DC_1-19_n77-n79 | DC_19A_n77A |  | 6th harmonic from B19 into n79 |  | 3rd IMD into B1 2nd, 3rd, 4th & 5th IMDs into n79 |  | - | 6th harmonic will not any impact to n79 The MSD issue will be covered in 3DL_DC_1A-19A_n77A with 2UL_DC_19A_n77A and 3DL_19A_n77A-n79A with 2UL_DC_19A_n77A |  |
|  | DC_19A_n79A |  | 4th harmonic from B19 into n77 |  | 4th IMD into B1 2nd & 3rd IMDs into n77 |  | - | 4th harmonic will be covered in DC_19A_n77A The MSD issue will be covered in 3DL_DC_1A-19A_n79A with 2UL_DC_19A_n79A and 3DL_19A_n77A-n79A with 2UL_DC_19A_n79A |  |
| DC_1-19_n78-n79 | DC_19A_n78A |  | 6th harmonic from B19 into n79 |  | 3rd IMD into B1 2nd, 3rd, 4th & 5th IMDs into n79 |  |  | 6th harmonic will not any impact to n79 The MSD issue will be covered in 3DL_DC_1A-19A_n78A with 2UL_DC_19A_n78A and 3DL_19A_n78A-n79A with 2UL_DC_19A_n78A |  |
|  | DC_19A_n79A |  | 4th harmonic from B19 into n78 |  | 4th IMD into B1 2nd & 3rd IMDs into n78 |  | - | 4th harmonic will be covered in DC_19A_n78A The MSD issue will be covered in 3DL_DC_1A-19A_n79A with 2UL_DC_19A_n79A and 3DL_19A_n78A-n79A with 2UL_DC_19A_n79A |  |
| DC_1-21_n77-n78 | DC_1A_n77A |  | - |  | 2nd & 5th IMDs into B21 4th & 5th IMD into n78 |  | - | These MSD issues are already covered in 3DL_DC_1A-21A_n77A with 2UL_DC_1A_n77A. These MSD issues are already covered in 3DL_DC_1A_n77A-n78A with 2UL_DC_1A_n77A. |  |
|  | DC_1A_n78A |  | 2nd harmonic from B1 into n77 |  | 2nd & 5th IMDs into B21 4th & 5th IMD into n77 |  | - | These MSD issues are already covered in 3DL_DC_1A-21A_n78A with 2UL_DC_1A_n78A. These MSD issues are already covered in 3DL_DC_1A_n77A-n78A with 2UL_DC_1A_n78A. |  |
| DC_1-21_n77-n79 | DC_1A_n77A |  | - |  | 2nd & 5th IMDs into B21 3rd, 4th & 5th IMD into n79 |  | - | These MSD issues are already covered in 3DL_DC_1A-21A_n77A with 2UL_DC_1A_n77A. These MSD issues are already covered in 3DL_DC_1A_n77A-n79A with 2UL_DC_1A_n77A. |  |
|  | DC_1A_n79A |  | 2nd harmonic from B1 into n77 |  | 4th IMD into B21 5th IMD into n77 |  | - | 2nd harmonic issue already covered in DC_1A_n77A The MSD issues are already covered in 3DL_DC_1A-21A_n79A with 2UL_DC_1A_n79A and 3DL_DC_1A_n77A-n79A with 2UL_DC_1A_n79A |  |
| DC_1-21_n78-n79 | DC_1A_n78A |  | - |  | 2nd & 5th IMDs into B21 3rd & 5th IMD into n79 |  | - | These MSD issues are already covered in 3DL_DC_1A-21A_n78A with 2UL_DC_1A_n78A. These MSD issues are already covered in 3DL_DC_1A_n78A-n79A with 2UL_DC_1A_n78A. |  |
|  | DC_1A_n79A |  | - |  | 4th IMD into B21 5th IMD into n78 |  | - | The MSD issues are already covered in 3DL_DC_1A-21A_n79A with 2UL_DC_1A_n79A and 3DL_DC_1A_n78A-n79A with 2UL_DC_1A_n79A |  |
| DC_1-42_n77-n78 | DC_1A_n77A |  | - |  | 4th & 5th IMD into B42 4th & 5th IMD into n78 |  | - | These MSD issues are already covered in 3DL_DC_1A-42A_n77A with 2UL_DC_1A_n77A. These MSD issues are already covered in 3DL_DC_1A_n77A-n78A with 2UL_DC_1A_n77A. |  |
|  | DC_1A_n78A |  | 2nd harmonic from B1 into n77 |  | 4th IMD into B42 4th & 5th IMD into n77 |  | - | These MSD issues are already covered in 3DL_DC_1A-42A_n78A with 2UL_DC_1A_n78A. These MSD issues are already covered in 3DL_DC_1A_n77A-n78A with 2UL_DC_1A_n78A. |  |
| DC_1-42_n77-n79 | DC_1A_n77A |  | - |  | 4th & 5th IMD into B42 3rd, 4th & 5th IMD into n79 |  | - | These MSD issues are already covered in 3DL_DC_1A-42A_n77A with 2UL_DC_1A_n77A. These MSD issues are already covered in 3DL_DC_1A_n77A-n79A with 2UL_DC_1A_n77A. |  |
|  | DC_1A_n79A |  | 2nd harmonic from B1 into n77 |  | 5th IMD into B42 5th IMD into n77 |  | - | 2nd harmonic issue already covered in DC_1A_n77A The MSD issue are already covered in 3DL_DC_1A-42A_n79A with 2UL_DC_1A_n79A and 3DL_DC_1A_n77A-n79A with 2UL_DC_1A_n79A |  |
| DC_1-42_n78-n79 | DC_1A_n78A |  | - |  | 4th IMD into B42 3rd & 5th IMD into n79 |  | - | These MSD issues are already covered in 3DL_DC_1A-42A_n78A with 2UL_DC_1A_n78A. These MSD issues are already covered in 3DL_DC_1A_n78A-n79A with 2UL_DC_1A_n78A. |  |
|  | DC_1A_n79A |  | - |  | 5th IMD into B42 5th IMD into n78 |  | - | The MSD issue are already covered in 3DL_DC_1A-42A_n79A with 2UL_DC_1A_n79A and 3DL_DC_1A_n78A-n79A with 2UL_DC_1A_n79A |  |
| DC_3-19_n77-n78 | DC_19A_n77A |  | 4th harmonic from B19 into n78 |  | 3rd IMD into B3 2nd IMD into n78 |  | - | 4th harmonic problem already covered in DC_19A_n78A These MSD issues are already covered in 3DL_DC_3A-19A_n77A with 2UL_DC_19A_n77A and 3DL_DC_19A_n77A-n78A with 2UL_DC_19A_n77A. |  |
|  | DC_19A_n78A |  | 4th harmonic from B19 into n77 |  | 3rd IMD into B3 2nd & 5th IMDs into n77 |  | - | 4th harmonic problem already covered in DC_19A_n77A These MSD issues are already covered in 3DL_DC_3A-19A_n78A with 2UL_DC_19A_n78A and 3DL_DC_19A_n77A-n78A with 2UL_DC_19A_n78A. |  |
| DC_3-19_n77-n79 | DC_19A_n77A |  | 6th harmonic from B19 into n79 |  | 3rd IMD into B3 2nd, 3rd, 4th & 5th IMD into n79 |  | - | 6th harmonic problem already covered in DC_19A_n78A These MSD issues are already covered in 3DL_DC_3A-19A_n77A with 2UL_DC_19A_n77A and 3DL_DC_19A_n77A-n78A with 2UL_DC_19A_n77A. |  |
|  | DC_19A_n79A |  | 4th harmonic from B19 into n77 |  | 4th IMD into B3 2nd & 3rd IMDs into n77 |  | - | 4th harmonic will be covered in DC_19A_n77A The MSD issue will be covered in 3DL_DC_3A-19A_n79A with 2UL_DC_19A_n79A and 3DL_19A_n77A-n79A with 2UL_DC_19A_n79A |  |
| DC_3-19_n78-n79 | DC_19A_n78A |  | 6th harmonic from B19 into n79 |  | 3rd IMD into B3 2nd, 3rd, 4th & 5th IMD into n79 |  | - | 6th harmonic problem already covered in DC_19A_n79A These MSD issues are already covered in 3DL_DC_3A-19A_n78A with 2UL_DC_19A_n78A and 3DL_DC_19A_n78A-n79A with 2UL_DC_19A_n78A. |  |
|  | DC_19A_n79A |  | 4th harmonic from B19 into n78 |  | 4th IMD into B3 2nd & 3rd IMDs into n78 |  | - | 4th harmonic will be covered in DC_19A_n78A The MSD issue will be covered in 3DL_DC_3A-19A_n79A with 2UL_DC_19A_n79A and 3DL_19A_n78A-n79A with 2UL_DC_19A_n79A |  |
| DC_3-21_n77-n78 | DC_3A_n77A |  | 2nd harmonic from B3 into n78 |  | 4th & 5th IMDs into B21 4th & 5th IMDs into n78 |  | - | 2nd harmonic issue already covered in DC_3A_n78A These MSD issues are already covered in 3DL_DC_3A-21A_n77A with 2UL_DC_3A_n77A and 3DL_3A_n77A-n78A with 2UL_DC_3A_n77A |  |
|  | DC_3A_n78A |  | 2nd harmonic from B3 into n77 |  | 4th & 5th IMDs into B21 4th & 5th IMDs into n77 |  | - | 2nd harmonic issue already covered in DC_3A_n77A These MSD issues are already covered in 3DL_DC_3A-21A_n78A with 2UL_DC_3A_n78A and 3DL_3A_n77A-n78A with 2UL_DC_3A_n78A |  |
|  | DC_21A_n77A |  | - |  | 2nd & 5th IMDs into B3 4th & 5th IMDs into n78 |  | - | These MSD issues are already covered in 3DL_DC_3A-21A_n77A with 2UL_DC_21A_n77A and 3DL_21A_n77A-n78A with 2UL_DC_21A_n77A |  |
|  | DC_21A_n78A |  | - |  | 2nd IMDs into B3 4th IMD into n77 |  | - | These MSD issue is already covered in 3DL_DC_3A-21A_n78A with 2UL_DC_21A_n78A and 3DL_21A_n77A-n78A with 2UL_DC_21A_n78A |  |
| DC_3-21_n77-n79 | DC_3A_n77A |  | - |  | 4th & 5th IMDs into B21 3rd & 4th IMD into n79 |  | - | These MSD issues are already covered in 3DL_DC_3A-21A_n77A with 2UL_DC_3A_n77A and 3DL_3A_n77A-n79A with 2UL_DC_3A_n77A |  |
|  | DC_3A_n79A |  | 2nd harmonic from B3 into n77 |  | 3rd IMD into B21 5th IMD into n77 |  | - | 2nd harmonic issue already covered in DC_3A_n77A The MSD issue will be covered in 3DL_DC_3A-21A_n79A with 2UL_DC_3A_n79A and 3DL_3A_n77A-n79A with 2UL_DC_3A_n79A |  |
|  | DC_21A_n77A |  |  |  | 2nd & 5th IMDs into B3 2nd & 4th IMDs into n79 |  | - | These MSD issues are already covered in 3DL_DC_3A-21A_n77A with 2UL_DC_21A_n77A and 3DL_21A_n77A-n79A with 2UL_DC_21A_n77A |  |
|  | DC_21A_n79A |  | - |  | 3rd IMD into B3 2nd IMD into n77 |  |  | These MSD issues are already covered in 3DL_DC_3A-21A_n79A with 2UL_DC_21A_n79A and 3DL_21A_n77A-n79A with 2UL_DC_21A_n79A |  |
| DC_3-21_n78-n79 | DC_3A_n78A |  | - |  | 4th & 5th IMDs into B21 3rd IMD into n79 |  | - | These MSD issues are already covered in 3DL_DC_3A-21A_n78A with 2UL_DC_3A_n78A and 3DL_3A_n78A-n79A with 2UL_DC_3A_n78A |  |
|  | DC_3A_n79A |  | 2nd harmonic from B3 into n78 |  | 3rd IMD into B21 5th IMD into n78 |  | - | 2nd harmonic issue already covered in DC_3A_n78A The MSD issue will be covered in 3DL_DC_3A-21A_n79A with 2UL_DC_3A_n79A and 3DL_3A_n78A-n79A with 2UL_DC_3A_n79A |  |
|  | DC_21A_n78A |  | - |  | 2nd IMDs into B3 2nd & 4th IMDs into n79 |  | - | These MSD issue is already covered in 3DL_DC_3A-21A_n78A with 2UL_DC_21A_n78A and 3DL_21A_n77A-n78A with 2UL_DC_21A_n78A |  |
|  | DC_21A_n79A |  | - |  | 3rd IMD into B3 2nd IMD into n78 |  | - | These MSD issues are already covered in 3DL_DC_3A-21A_n79A with 2UL_DC_21A_n79A and 3DL_21A_n78A-n79A with 2UL_DC_21A_n79A |  |
| DC_3-42_n77-n78 | DC_3A_n77A |  | 2nd harmonic from B3 into B42 & n78 |  | 4th & 5th IMDs into B42 4th & 5th IMDs into n78 |  | - | 2nd harmonic issue already covered in DC_3A-42A and DC_3A_n78A These MSD issues are already covered in 3DL_DC_3A-42A_n77A with 2UL_DC_3A_n77A and 3DL_3A_n77A-n78A with 2UL_DC_3A_n77A |  |
|  | DC_3A_n78A |  | 2nd harmonic from B3 into B42 & n77 |  | 4th & 5th IMDs into B42 4th & 5th IMDs into n77 |  | - | 2nd harmonic issue already covered in DC_3A-42A and DC_3A_n77A These MSD issues are already covered in 3DL_DC_3A-42A_n78A with 2UL_DC_3A_n78A and 3DL_3A_n77A-n78A with 2UL_DC_3A_n78A |  |
| DC_3-42_n77-n79 | DC_3A_n77A |  | 2nd harmonic from B3 into B42 |  | 4th & 5th IMDs into B42 3rd & 4th IMD into n79 |  | - | 2nd harmonic issue already covered in DC_3A-42A These MSD issues are already covered in 3DL_DC_3A-42A_n77A with 2UL_DC_3A_n77A and 3DL_3A_n77A-n79A with 2UL_DC_3A_n77A |  |
|  | DC_3A_n79A |  | 2nd harmonic from B3 into B42 & n77 |  | 5th IMD into B42 5th IMD into n77 |  | - | 2nd harmonic issue already covered in DC_3A-42A and DC_3A_n77A The MSD issue will be covered in 3DL_DC_3A-42A_n79A with 2UL_DC_3A_n79A and 3DL_3A_n77A-n79A with 2UL_DC_3A_n79A |  |
| DC_3-42_n78-n79 | DC_3A_n78A |  | 2nd harmonic from B3 into B42 |  | 4th & 5th IMDs into B42 3rd IMD into n79 |  | - | 2nd harmonic issue already covered in DC_3A-42A These MSD issues are already covered in 3DL_DC_3A-42A_n78A with 2UL_DC_3A_n78A and 3DL_3A_n78A-n79A with 2UL_DC_3A_n78A |  |
|  | DC_3A_n79A |  | 2nd harmonic from B3 into B42 & n78 |  | 5th IMD into B42 5th IMD into n78 |  | - | 2nd harmonic issue already covered in DC_3A-42A and DC_3A_n78A The MSD issue will be covered in 3DL_DC_3A-42A_n79A with 2UL_DC_3A_n79A and 3DL_3A_n78A-n79A with 2UL_DC_3A_n79A |  |
| DC_19-21_n77-n78 | DC_19A_n77A |  | 4th harmonic from B19 into n78 |  | 4th IMD into B21 2nd IMD into n78 |  | - | 4th harmonic problem already covered in DC_19A_n78A These MSD issues are already covered in 3DL_DC_19A-21A_n77A with 2UL_DC_19A_n77A and 3DL_DC_19A_n77A-n78A with 2UL_DC_19A_n77A. |  |
|  | DC_19A_n78A |  | 4th harmonic from B19 into n77 |  | 2nd & 5th IMDs into n77 |  | - | 4th harmonic problem already covered in DC_19A_n77A These MSD issues are already covered in 3DL_DC_19A_n77A-n78A with 2UL_DC_19A_n78A. |  |
| DC_19-21_n77-n79 | DC_19A_n77A |  | 6th harmonic from B19 into n79 |  | 4th IMD into B21 2nd, 3rd , 4th & 5th IMDs into n79 |  | - | 6th harmonic problem already covered in DC_19A_n79A These MSD issues are already covered in 3DL_DC_19A-21A_n77A with 2UL_DC_19A_n77A and 3DL_DC_19A_n77A-n79A with 2UL_DC_19A_n77A. |  |
|  | DC_19A_n79A |  | 4th harmonic from B19 into n77 |  | 5th IMD into B21 2nd & 3rd IMDs into n77 |  | - | 4th harmonic will be covered in DC_19A_n77A The MSD issue will be covered in 3DL_DC_19A-21A_n79A with 2UL_DC_19A_n79A and 3DL_19A_n77A-n79A with 2UL_DC_19A_n79A |  |
| DC_19-21_n78-n79 | DC_19A_n78A |  | 6th harmonic from B19 into n79 |  | 2nd, 3rd, 4th & 5th IMD into n79 |  | - | 6th harmonic problem already covered in DC_19A_n79A These MSD issues are already covered in 3DL_DC_19A_n78A-n79A with 2UL_DC_19A_n78A. |  |
|  | DC_19A_n79A |  | 4th harmonic from B19 into n78 |  | 5th IMD into B21 2nd & 3rd IMDs into n78 |  | - | 4th harmonic will be covered in DC_19A_n78A The MSD issue will be covered in 3DL_DC_19A-21A_n79A with 2UL_DC_19A_n79A and 3DL_19A_n78A-n79A with 2UL_DC_19A_n79A |  |
| DC_19-42_n77-n78 | DC_19A_n77A |  | 4th harmonic from B19 into n78 |  | 2nd IMD into n78 |  | - | 4th harmonic problem already covered in DC_19A_n78A These MSD issues are already covered in 3DL_DC_19A_n77A-n78A with 2UL_DC_19A_n77A. |  |
|  | DC_19A_n78A |  | 4th harmonic from B19 into n77 |  | 2nd & 5th IMDs into n77 |  | - | 4th harmonic problem already covered in DC_19A_n77A These MSD issues are already covered in 3DL_DC_19A_n77A-n78A with 2UL_DC_19A_n78A. |  |
| DC_19-42_n77-n79 | DC_19A_n77A |  | 6th harmonic from B19 into n79 |  | 2nd, 3rd , 4th & 5th IMDs into n79 |  | - | 6th harmonic problem already covered in DC_19A_n79A These MSD issues are already covered in 3DL_DC_19A_n77A-n79A with 2UL_DC_19A_n77A. |  |
|  | DC_19A_n79A |  | 4th harmonic from B19 into n77 |  | 2nd IMD into B42 2nd & 3rd IMDs into n77 |  | - | 4th harmonic will be covered in DC_19A_n77A The MSD issue will be covered in 3DL_DC_19A-42A_n79A with 2UL_DC_19A_n79A and 3DL_19A_n77A-n79A with 2UL_DC_19A_n79A |  |
| DC_19-42_n78-n79 | DC_19A_n78A |  | 6th harmonic from B19 into n79 |  | 2nd, 3rd, 4th & 5th IMD into n79 |  | - | 6th harmonic problem already covered in DC_19A_n79A These MSD issues are already covered in 3DL_DC_19A_n78A-n79A with 2UL_DC_19A_n78A. |  |
|  | DC_19A_n79A |  | 4th harmonic from B19 into n78 |  | 2nd IMD into B42 2nd & 3rd IMDs into n78 |  | - | 4th harmonic will be covered in DC_19A_n78A The MSD issue will be covered in 3DL_DC_19A-42A_n79A with 2UL_DC_19A_n79A and 3DL_19A_n78A-n79A with 2UL_DC_19A_n79A |  |
| DC_21-42_n77-n78 | DC_21A_n77A |  | - |  | 5th IMD into B42 4th & 5th IMDs into n78 |  | - | These MSD issues are already covered in 3DL_DC_21A-42A_n77A with 2UL_DC_21A_n77A and 3DL_21A_n77A-n78A with 2UL_DC_21A_n77A |  |
|  | DC_21A_n78A |  | - |  | 4th IMD into n77 |  | - | These MSD issue is already covered in 3DL_21A_n77A-n78A with 2UL_DC_21A_n78A |  |
| DC_21-42_n77-n79 | DC_21A_n77A |  | - |  | 5th IMD into B42 2nd & 4th IMDs into n79 |  | - | These MSD issues are already covered in 3DL_DC_21A-42A_n77A with 2UL_DC_21A_n77A and 3DL_21A_n77A-n79A with 2UL_DC_21A_n77A |  |
|  | DC_21A_n79A |  | - |  | 2nd IMD into B42 2nd IMD into n77 |  |  | These MSD issues are already covered in 3DL_DC_21A-42A_n79A with 2UL_DC_21A_n79A and 3DL_21A_n77A-n79A with 2UL_DC_21A_n79A |  |
| DC_21-42_n78-n79 | DC_21A_n78A |  | - |  | 2nd & 4th IMDs into n79 |  | - | These MSD issue is already covered in 3DL_21A_n78A-n79A with 2UL_DC_21A_n78A |  |
|  | DC_21A_n79A |  | - |  | 2nd IMD into B42 2nd IMD into n78 |  | - | These MSD issues are already covered in 3DL_DC_21A-42A_n79A with 2UL_DC_21A_n79A and 3DL_21A_n78A-n79A with 2UL_DC_21A_n79A |  |
| DC_1-28_n28-n78 | DC_1A_n28A |  | 5th harmonic from n28 into n78 |  | 3rd IMD into n78 |  | - | - Harmonic problems already solved in CA_n28A-n78A - 3rd IMD problem was solved in 3DL_DC_1A_n28A-n78A |  |
|  | DC_1A_n78A |  | - |  | 5th IMD into B28 & n28 |  | - | - 5th IMD problem was solved in 3DL_DC_1A_n28A-n78A or 3DL_DC_1A-28A_n78A |  |
|  | DC_28A_n28A1 |  | 3rd harmonic into B1 |  | 3rd & 5th IMDs into B1 5th IMD into n78 |  | - | 4th harmonic will be covered in DC_28A_n1A The MSD issue will be covered in 3DL_DC_1A-28A_n28A with 2UL_DC_28A_n28A and 3DL_28A_n28A-n78A with 2UL_DC_28A_n28A |  |
|  | DC_28A_n78A |  | 3rd harmonic from B28 into B1 |  | 3rd IMD into B1 5th IMD into n28 |  | - | 3rd harmonic will be covered in CA_1A-28A The MSD issue will be covered in 3DL_DC_1A-28A_n78A with 2UL_DC_28A_n78A and 3DL_28A_n28A-n78A with 2UL_DC_28A_n78A |  |
| DC_3-7_n1-n78 | DC_3A_n1A |  | 2nd harmonic from 3 into n78 |  | 2nd & 4th IMDs into n78 |  | Yes | 2nd harmonic will be covered in DC_3A_n78A The MSD issue will be covered in 3DL_3A_n1A-n78A with 2UL_DC_3A_n1A |  |
|  | DC_3A_n78A |  | - |  | 5th IMD into n1 |  | - | These MSD issues are already covered in 3DL_DC_3A_n1A-n78A with 2UL_DC_3A_n78A |  |
|  | DC_7A_n1A, DC_7C_n1A |  | - |  | 4th & 5th IMDs into n78 |  | Yes | These IMD problems were already covered in DC_7A_n1A-n78A with 2UL_DC_7A_n1A |  |
|  | DC_7A_n78A, DC_7C_n78A |  | - |  | 3rd & 4th IMDs into B3 4th IMD into n1 |  | - | These IMD problems were already covered in 3DL_DC_3A-7A_n78A with 2UL_DC_7A_n78A and 3DL_DC_7A_n1A-n78A with 2UL_DC_7A_n78A |  |
| DC_3-28_n28-n78 | DC_3A_n28A DC_3C_n28A |  | 2nd harmonic into n78 5th harmonic into n78 |  | 5th IMD into n78 |  | - | - Harmonic problems already solved in DC_3A_n78A or CA_n28A-n78A - 5th IMD problem was solved in 3DL_DC_3A_n28A-n78A with 2UL_DC_3A_n28A |  |
|  | DC_3A_n78A DC_3C_n78A |  | - |  | - |  | - | No issue |  |
|  | DC_28A_n28A1 |  | - |  | 5th IMD into n78 |  | - | The MSD issue will be covered in 3DL_28A_n28A-n78A with 2UL_DC_28A_n28A |  |
|  | DC_28A_n78A |  | - |  | 3rd IMD into B3 5th IMD into n28 |  | - | The MSD issue will be covered in 3DL_DC_3A-28A_n78A with 2UL_DC_28A_n78A and 3DL_28A_n28A-n78A with 2UL_DC_28A_n78A |  |
| DC_7-28_n28-n78 | DC_7A_n28A DC_7C_n28A |  | 5th harmonic into n78 |  | 2nd,4th IMDs into n78 |  | - | - Harmonic problem was solved in DC_28A_n78A - 2nd & 4th IMDs problems were solved in 3DL_DC_7A_n28A-n78A with 2UL_DC_7A_n28A |  |
|  | DC_7A_n78A DC_7C_n78A |  | - |  | 2nd & 5th IMDs into B28 & n28 |  | - | - 2nd & 5th IMDs problems were solved in 3DL_DC_7A_n28A-n78A or 3DL_DC_7A-28A_n78A |  |
|  | DC_28A_n28A1 |  | - |  | 5th IMD into n78 |  | - | The MSD issue will be covered in 3DL_28A_n28A-n78A with 2UL_DC_28A_n28A |  |
|  | DC_28A_n78A |  | - |  | 2nd IMD into B7 5th IMD into n28 |  | - | The MSD issue will be covered in 3DL_DC_7A-28A_n78A with 2UL_DC_28A_n78A and 3DL_28A_n28A-n78A with 2UL_DC_28A_n78A |  |
| DC_1-8_n77-n257 | DC_1A_n77A |  | 7th & 8th Harmonics from n77 into n257 |  | 5th IMD into B8 |  | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 5th IMD already covered in DC_1A-8A_n77A |  |
|  | DC_1A_n257A |  | 2nd harmonic from B1 into n77 |  | - |  | - | - Harmonic problem already discussed and will be solved in DC_1A_n77A |  |
|  | DC_8A_n77A |  | 7th & 8th Harmonics from n77 into n257 |  | 3rd IMD into B1 |  | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 The MSD issue will be covered in 3DL_DC_1A-8A_n77A with 2UL_DC_8A_n77A |  |
|  | DC_8A_n257A |  | 4th harmonic from B8 into n77 |  | - |  | - | - Harmonic problem will be solved in DC_8A_n77A |  |
| DC_1-3_n5-n78 | DC_1A_n5A | 4th harmonic from n5 into n78 |  | 3rd IMD into n78 |  | Yes |  |  | - 4th harmonic already covered in Table 7.3B.2.3.1-1 in TS38.101-3 - The 3rd IMD issue will be covered in DC_1_n5A-n78A |
|  | DC_3A_n5A, DC_3C_n5A | 2nd harmonic from B3 into n78 |  | 3rd & 5th IMDs into n78 |  | - |  |  | - 2nd harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - These IMD issue will be covered in DC_3_n5A-n78A |
|  | DC_1A_n78A | - |  | 2nd IMD into B3 5th IMD into n5 |  | Yes |  |  | - The 2nd IMD issue will be covered in DC_1A-3A_n78A - The 5th IMD issue will be covered in DC_1A_n5A-n78A |
|  | DC_3A_n78A, DC_3C_n78A | - |  | 5th IMD into B1 |  | - |  |  | - The 5th IMD issue will be covered in DC_1A-3A_n78A |
| DC_1-7_n5-n78 | DC_1A_n5A | 4th harmonic from n5 into n78 |  | 3rd IMD into n78 |  | - |  |  | - 4th harmonic already covered in Table 7.3B.2.3.1-1 in TS38.101-3 - The 3rd IMD issue will be covered in DC_1_n5A-n78A |
|  | DC_7A_n5A, DC_7C_n5A | - |  | 2nd & 4th IMDs into n78 |  | - |  |  | These IMDs problem will be covered in DC_7A_n5A-n78A |
|  | DC_1A_n78A | - |  | 5th IMD into n5 4th IMD into B7 |  | Yes |  |  | The 5th IMD will be covered in DC_1A_n5A-n78A The 4th IMD will be covered in DC_1A-7A_n78A |
|  | DC_7A_n78A, DC_7C_n78A | - |  | 4th IMD into B1 2nd & 5th IMDs into n5 |  | Yes |  |  | The 4th IMD will be covered in DC_1A-7A_n78A The 2nd & 5th IMDs will be covered in DC_7A_n5A-n78A |
| DC_1-28_n5-n78 | DC_1A_n5A | 4th harmonic from n5 into n78 |  | 3rd IMD into n78 |  | - |  |  | - 4th harmonic already covered in Table 7.3B.2.3.1-1 in TS38.101-3 - The 3rd IMD issue will be covered in DC_1_n5A-n78A |
|  | DC_28A_n5A | 3rd harmonic from B28 into B1 4th harmonic from n5 into n78 5th harmonic from B28 into n78 |  | 5th IMD into B1 |  |  |  |  | - The 3rd harmonic already covered in CA_1A-28A. - The other harmonic problems already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - The 5th IMD problem will be covered in DC_1A-28A_n5A |
|  | DC_1A_n78A | - |  | 5th IMD into n5 5th IMD into B28 |  | - |  |  | - The 5th IMD problem will be covered in DC_1A_n5A-n78A - The 5th IMD problem will be covered in DC_1A-28A_n78A |
|  | DC_28A_n78A | 3rd harmonic from B28 into B1 |  | 3rd IMD into B1 5th IMD into n5 |  | - |  |  | 3rd harmonic will be covered in CA_1A-28A The 3rd IMD issue will be covered in DC_1A-28A_n78A The 5th IMD issue will be covered in DC_28A_n5A-n78A |
| DC_3-7_n5-n78 | DC_3A_n5A, DC_3C_n5A | 2nd harmonic from B3 into n78 |  | 2nd & 3rd IMDs into B7 3rd & 5th IMDs into n78 |  | - |  |  | - 2nd harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - These IMD issue will be covered in DC_3A-7A_n5A - These IMD issue will be covered in DC_3A_n5A-n78A |
|  | DC_7A_n5A, DC_7C_n5A | - |  | 2nd & 4th IMDs into n78 |  | - |  |  | These IMDs problem will be covered in DC_7A_n5A-n78A |
|  | DC_3A_n78A, DC_3C_n78A | - |  | - |  | - |  |  | No issue |
|  | DC_7A_n78A, DC_7C_n78A | - |  | 2nd & 5th IMDs into n5 3rd & 4th IMDs into B3 |  | Yes |  |  | The 2nd & 5th IMDs will be covered in DC_7A_n5A-n78A The 3rd & 4th IMDs will be covered in DC_3A-7A_n78A |
| DC_3-28_n5-n78 | DC_3A_n5A, DC_3C_n5A | 2nd harmonic from B3 into n78 |  | 3rd & 5th IMDs into n78 4th IMD into B28 |  | - |  |  | - 2nd harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - These IMD issue will be covered in DC_3A_n5A-n78A The 4th IMD issue will be covered in DC_3A-28A_n5A |
|  | DC_28A_n5A | 4th harmonic from n5 into n78 5th harmonic from B28 into n78 |  | 4th IMD into B3 |  |  |  |  | - These harmonic problems already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - The 4th IMD will be covered in DC_3A-28A_n5A |
|  | DC_3A_n78A, DC_3C_n78A | - |  | - |  | - |  |  | No issue |
|  | DC_28A_n78A | - |  | 3rd IMD into B3 5th IMD into n5 |  | - |  |  | - The 3rd IMD will be covered in DC_3A-28A_n78A - The 5th IMD will be covered in DC_28A_n5A-n78A |
| DC_7-28_n5-n78 | DC_7A_n5A, DC_7C_n5A | - |  | 2nd & 4th IMDs into n78 3rd & 5th IMDs into B28 |  | - |  |  | - These IMDs problem will be covered in DC_7A_n5A-n78A – These IMDs problems will be covered in DC_7A-28A_n5A |
|  | DC_28A_n5A | 4th harmonic from n5 into n78 5th harmonic from B28 into n78 |  | 5th IMD into B7 |  | - |  |  | - These harmonic problems already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - The 5th IMD issue will be covered in 7A-28A_n5A |
|  | DC_7A_n78A, DC_7C_n78A | - |  | 2nd & 5th IMDs into n5 2nd & 5th IMDs into B28 |  | Yes |  |  | 2nd & 5th IMDs will be covered in DC_7A_n5A-n78A Other 2nd & 5th IMDs will be covered in DC_7A-28A_n78A |
|  | DC_28A_n78A | - |  | 5th IMD into n5 2nd IMD into B7 |  | Yes |  |  | - The 5th IMD will be covered in DC_28A_n5A-n78A - 2nd IMD will be covered in DC_7A-28A_n78A |
| DC_1-7_n3-n78 | DC_1A_n3A | 2nd harmonic from 3 into n78 |  | 2nd & 4th IMDs into n78 |  | Yes |  |  | - Harmonic problems was already covered in DC_3A_n78A. - 2nd &4th IMDs problems same as DC_1A_n3A-n78A. |
|  | DC_1A_n78A | - |  | 4th IMD into B7 2nd IMD into n3 |  | Yes |  |  | The 4th IMD will be covered in DC_1A-7A_n78A The 2nd IMD will be covered in DC_1A_n3A-n78A |
|  | DC_7A_n3A | 2nd harmonic from n3 into n78 |  | 3rd IMD into n78 |  | - |  |  | - Harmonic problem already solved in CA_n3A-n78A - The IMD problem already covered in DC_7A_n3A-n78A |
|  | DC_7A_n78A |  |  | 3rd & 4th IMDs into n3 4th IMD into B1 |  | Yes |  |  | - The 3rd & 4th IMDs will be covered in DC_3A-7A_n78A - The 4th IMD will be covered in DC_1A-7A_n78A |
| DC_2-7_n38-n66 | DC_2A_n66A | - |  | - |  | - |  |  | No issue |
| DC_2-7_n38-n78 | DC_2A_n78A | - |  | - |  | - |  |  | No issue |
| DC_2-66_n41-n260 | DC_2A_n41A | High order harmonic into n260 |  | - |  | - |  |  | - No harmonic problem |
|  | DC_66A_n41A | High order harmonic into n260 |  | 4th IMD into B2 |  | - |  |  | - No harmonic problem - The 4th IMD issue will be covered in DC_2A-66A_n41A |
| DC_2-66_n41-n261 | DC_2A_n41A | High order harmonic into n261 |  | - |  | - |  |  | - No harmonic problem |
|  | DC_66A_n41A | High order harmonic into n261 |  | 4th IMD into B2 |  | - |  |  | - No harmonic problem - The 4th IMD issue will be covered in DC_2A-66A_n41A |
| DC_7-66_n38-n78 | DC_66A_n78A | - |  | - |  | - |  |  | No issue |
| DC_1-3_n77-n257 | DC_1A_n77A | 7th & 8th Harmonics from n77 into n257 |  | 2nd, 4th & 5th IMDs into B3 |  | Yes |  |  | - No harmonic problems by 7th & 8th order between FR1 and FR2 - These IMDs issue will be covered in DC_1A-3A_n77A |
|  | DC_3A_n77A | 7th & 8th harmonics from n77 into n257 |  | 2nd & 5th IMD into B1 |  | - |  |  | - No harmonic problems - These IMD problem will be covered in DC_1A-3A_n77A_ |
|  | DC_1A_n257A/D/G/H/I | 2nd harmonic from B1 into n77 |  | - |  | Yes |  |  | - Harmonic problem already discussed and will be solved in DC_1A_n77A |
|  | DC_3A_n257A//D/G/H/I | 2nd harmonic from B3 into B77 |  | - |  | - |  |  | - Harmonic problem already covered in DC_3A_n77A |
| DC_3-20_n1-n7 | DC_3A_n1A, DC_3C_n1A | - |  | - |  | - |  |  | No issue |
|  | DC_3A_n7A, DC_3C_n7A | - |  | 2nd IMD into B20 |  | - |  |  | - 2nd IMD issue will be covered in DC_3A-20A_n7A |
|  | DC_20A_n1A | - |  | - |  | Yes |  |  | No issue |
|  | DC_20A_n7A | - |  | - |  | - |  |  | No issue |
| DC_3-20_n1-n28 | DC_3A_n1A, DC_3C_n1A | - |  | - |  | - |  |  | No issue |
|  | DC_3A_n28A, DC_3C_n28A | 3rd harmonic from n28 into n1 |  | 4th IMD into n1 |  |  |  |  | - Harmonic problems already solved in DC_1A_n28A - 4th IMD problem was solved in 3DL_DC_1A-3A_n28A |
|  | DC_20A_n1A | - |  | - |  | Yes |  |  | No issue |
|  | DC_20A_n28A | 3rd harmonic from n28 into n1 |  | 5th IMD into n1 |  | - |  |  | - Harmonic problem already covered in DC_1A_n28A - 5th IMD issue also covered in DC_20A_n1A-n28A |
| DC_3-20_n7-n28 | DC_3A_n7A, DC_3C_n7A | - |  | 2nd IMD into B20 2nd IMD into n28 |  | - |  |  | - 2nd IMD issue will be covered in DC_3A-20A_n7A - 2nd IMD issue will be covered in DC_3A_n7A-n28A |
|  | DC_3A_n28A, DC_3C_n28A | - |  | 3rd IMD into n7 |  | - |  |  | - 3rd IMD problem was solved in 3DL_DC_3A_n7A-n28A |
|  | DC_20A_n7A | - |  | 3rd & 5th IMDs into n28 |  | - |  |  | - These IMDs problem were solved in DC_20A_n7A-n28A |
|  | DC_20A_n28A | - |  | 5th IMD into n7 |  | - |  |  | - 5th IMD problem same as DC_7A-20A_n28A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
| DC_3-20_n7-n78 | DC_3A_n7A | 2nd harmonic from 3 into n78 |  | 2nd IMD into B20 3rd IMD into n78 |  | - |  |  | - Harmonic problem was solved in DC_3A_n78A - 2nd IMD issue will be covered in DC_3A-20A_n7A - 3rd IMD problem already covered in DC_3A_n7A-n78A. |
|  | DC_3A_n78A | - |  | - |  | Yes |  |  | No issue |
|  | DC_20A_n7A | 4th harmonic from B20 into n78 |  | 2nd & 4th IMDs into n78 |  | - |  |  | - The harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - These IMD issue will be covered in DC_20A_n7A-n78A |
|  | DC_20A_n78A |  |  | 2nd IMD into n7 3rd IMD into B3 |  | Yes |  |  | - 2nd IMD problem same as DC_7A-20A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 - 3rd IMD issue will be covered in DC_3A-20A_n78A |
| DC_3-20_n1-n78 | DC_3A_n1A | 2nd harmonic from 3 into n78 |  | 2nd & 4th IMDs into n78 |  | - |  |  | - 2nd harmonic will be covered in DC_3A_n78A - The MSD issue will be covered in 3DL_3A_n1A-n78A |
|  | DC_3A_n78A | - |  | 5th IMD into n1 |  | - |  |  | - The 5th IMD issue will be covered in DC_3A_n1A-n78A |
|  | DC_20A_n1A | 4th harmonic from B20 into n78 |  | 3rd IMD into n78 |  | - |  |  | - Harmonic problem already solved in DC_20A_n78A - 3rd IMD will be covered in DC_20A_n1A-n78A |
|  | DC_20A_n78A | - |  | 3rd IMD into B3 3rd IMD into n1 |  | Yes |  |  | - 3rd IMD issue will be covered in DC_3A-20A_n78A - 3rd IMD issue will be covered in DC_20A_n1A-n78A |
| DC_3-20_n28-n78 | DC_3A_n28A | 2nd harmonic into n78 5th harmonic into n78 |  | 5th IMD into n78 |  | - |  |  | - Harmonic problems already solved in DC_3A_n78A or CA_n28A-n78A - 5th IMD problem was solved in 3DL_DC_3A_n28A-n78A with 2UL_DC_3A_n28A |
|  | DC_3A_n78A | - |  | - |  | - |  |  | No issue |
|  | DC_20A_n28A | 4th harmonic from B20 into n78 5th harmonic from n28 into n78 |  | 4th & 5th IMDs into n78 |  |  |  |  | - Harmonic problems already solved in DC_20A_n78A or CA_n28A-n78A - These IMD issue will be covered in DC_20A_n28A-n78A |
|  | DC_20A_n78A | - |  | 3rd IMD into B3 4th IMD into n28 |  | - |  |  | - 3rd IMD issue will be covered in DC_3A-20A_n78A - 4th IMD issue will be covered in DC_20A_n28A-n78A |
| DC_2-66_n41-n71 DC_2-66_n41(2A)-n71 | DC_2A_n41A | - |  | 2nd & 5th IMDs into n71 |  | - |  |  | - These IMD issue will be covered in DC_2A_n41A-n71A |
|  | DC_2A_n71A | 4th harmonic from n71 into n41 |  | 2nd IMD into n41 |  | - |  |  | - The harmonic problem will be treated CA_n41A_n71A - 2nd IMD issue will be covered in DC_2A_n41A-n71A |
|  | DC_66A_n41A |  |  | 4th IMD into B2 |  | - |  |  | - 4th IMD issue will be covered in DC_2A-66A_n41A |
|  | DC_66A_n71A | 4th harmonic from n71 into n41 |  | - |  | - |  |  | The harmonic problem will be treated CA_n41A_n71A |
| DC_1-18_n3-n78 | DC_1A_n3A | 2nd harmonic from 3 into n78 |  | 2nd & 4th IMDs into n78 |  | Yes |  |  | - Harmonic problems was already covered in DC_3A_n78A. - 2nd &4th IMDs problems will be covered in DC_1A_n3A-n78A. |
|  | DC_1A_n78A |  |  | 2nd IMD into n3 5th IMD into B18 |  | Yes |  |  | - The 2nd IMD will be covered in DC_1A_n3A-n78A - The 5th IMD will be covered in DC_1A-18A_n78A |
|  | DC_18A_n3A | 2nd harmonic from n3 into n78 4th harmonic from B18 into n78 |  | 3rd & 5th IMDs into n78 |  | - |  |  | - These harmonic problem already specified in in Table 7.3B.2.3.1-1 in TS38.101-3 - These IMD issues will be covered in DC_18A_n3A-n78A |
|  | DC_18A_n78A | - |  | 3rd IMD into n3 3rd IMD into B1 |  | - |  |  | - 3rd IMD issue will be covered in DC_18A_n3A-n78A - 3rd IMD issue will be covered in DC_1A-18A_n78A |
| DC_1-28_n3-n78 | DC_1A_n3A | 2nd harmonic from 3 into n78 |  | 2nd & 4th IMDs into n78 |  | Yes |  |  | - Harmonic problems was already covered in DC_3A_n78A. - 2nd &4th IMDs problems will be covered in DC_1A_n3A-n78A. |
|  | DC_1A_n78A | - |  | 2nd IMD into n3 5th IMD into B28 |  | Yes |  |  | - The 2nd IMD will be covered in DC_1A_n3A-n78A - The 5th IMD will be covered in DC_1A-28A_n78A |
|  | DC_28A_n3A | 2nd harmonic from n3 into n78 5th harmonic from B28 into n78 3rd harmonic from B28 into B1 |  | 4th IMD into B1 5th IMD into n78 |  | - |  |  | - These harmonic issue already covered in CA_n3A-n78A or DC_28A_n78A. or CA_1A-28A - 4th IMD problem will be covered in DC_1A-28A_n3A - 5th IMD problem same as DC_3A_n28A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
|  | DC_28A_n78A | 3rd harmonic from B28 into B1 |  | 3rd IMD into B1 3rd IMD into n3 |  | - |  |  | - 3rd harmonic will be covered in CA_1A-28A - 3rd IMD issue will be covered in DC_1A-28A_n78A - 3rd IMD will be covered in DC_28A_n3A-n78A |
| DC_1-8_n3-n28 | DC_1A_n3A | - |  | - |  | - |  |  | No issue |
|  | DC_1A_n28A | - |  | 5th IMD into n3 5th IMD into B8 |  | Yes |  |  | - 5th IMD problem same as DC_1A-3A_n28A in Table 7.3B.2.3.5.2-1 in TS38.101-3 - 5th IMD issue will be covered in DC_1A-8A_n28A |
|  | DC_8A_n3A | - |  | 2nd & 5th IMDs into n28 |  | - |  |  | - These IMD issues will be covered in DC_8A_n3A-n28A |
|  | DC_8A_n28A | 2nd harmonic from B8 into n3 3rd harmonic from n28 into B1 |  | 4th IMD into B1 |  | - |  |  | These harmonic problem will be covered in DC_8A_n3A or DC_1A_n28A 4th IMD issue will be covered in DC_1A-8A_n28A |
| DC_3-8_n1-n78 DC_3-3-8_n1-n78 | DC_3A_n1A | 2nd harmonic from 3 into n78 |  | 2nd & 4th IMDs into n78 |  | - |  |  | - 2nd harmonic will be covered in DC_3A_n78A The MSD issue will be covered in 3DL_3A_n1A-n78A |
|  | DC_3A_n78A | - |  | 5th IMD into n1 |  | - |  |  | - The 5th IMD issue will be covered in DC_3A_n1A-n78A |
|  | DC_8A_n1A | 2nd harmonic from B8 into B3 4th harmonic from B8 into n78 |  | 3rd IMD into n78 |  | - |  |  | - The harmonic problem already covered in CA_3A-8A or DC_8A_n78A - 3rd IMD issue will be covered in DC_8A_n1A-n78A |
|  | DC_8A_n78A | 2nd harmonic from B8 into B3 |  | 3rd IMD into B3 |  | - |  |  | - Harmonic problem already be solved in CA_3A_8A - 3rd IMD already covered in DC_3A-8A_n78A |
| DC_7-8_n1-n78 DC_7-7-8_n1-n78 | DC_7A_n1A | - |  | 4th & 5th IMDs into n78 5th IMD into B8 |  | Yes |  |  | - These IMD problems were already covered in DC_7A_n1A-n78A with 2UL_DC_7A_n1A - 5th IMD issue will be covered in DC_7A-8A_n1A |
|  | DC_7A_n78A |  |  | 2nd & 5th IMDs into B8 4th IMD into n1 |  | Yes |  |  | - 2nd & 5th IMDs will be covered in DC_7A-8A_n78A - 4th IMD will be covered in DC_7A_n1A-n78A |
|  | DC_8A_n1A | 3rd harmonic from B8 into B7 4th harmonic from B8 into n78 |  | 3rd IMD into n78 |  | - |  |  | - The harmonic problem already covered in CA_7A-8A or DC_8A_n78A - 3rd IMD issue will be covered in DC_8A_n1A-n78A |
|  | DC_8A_n78A | 3rd harmonic from B8 into B7 |  | 2nd IMD into B7 |  | Yes |  |  | - The harmonic problem already covered in CA_7A-8A - 2nd IMD issue will be covered in DC_7A-8A_n78A |
| DC_1-3_n38-n78 | DC_3A_n38A | 2nd harmonic from B3 into n78 |  | 3rd IMD into n78 |  | - |  |  | - The harmonic problem already covered in DC_3A_n78A - 3rd IMD issue will be covered in DC_3A_n38A-n78A |
|  | DC_3A_n78A | - |  | 5th IMD into B1 |  | - |  |  | - The 5th IMD issue will be covered in DC_1A-3A_n78A |
| DC_3-20_n38-n78 | DC_3A_n38A | 2nd harmonic from B3 into n78 |  | 2nd & 3rd IMDs into B20 3rd IMD into n78 |  | - |  |  | - The harmonic problem already covered in DC_3A_n78A - 2nd & 3rd IMDs issue will be covered in DC_3A-20A_38A - 3rd IMD issue will be covered in DC_3A_n38A-n78A |
|  | DC_3A_n78A | - |  | - |  | - |  |  | No issue |
| DC_1-20_n3-n78 | DC_1A_n3A | 2nd harmonic from 3 into n78 |  | 2nd & 4th IMDs into n78 |  | Yes |  |  | - Harmonic problems was already covered in DC_3A_n78A. - 2nd &4th IMDs problems will be covered in DC_1A_n3A-n78A. |
|  | DC_20A_n3A | - 2nd harmonic from n3 into n78 - 4th harmonic from B20 into n78 |  | 3rd & 5th IMDs into n78 |  | - |  |  | - Harmonic problems was already covered in DC_3A_n78A or DC_20A_n78A. - 3rd &5th IMDs problems will be covered in DC_20A_n3A-n78A. |
| DC_1-20_n3-n38 | DC_1A_n3A | - |  | - |  | - |  |  | No issue |
|  | DC_20A_n3A | 3rd harmonic from B20 into n38 |  | 2nd & 3rd IMDs into n38 |  | - |  |  | - Harmonic problems was already covered in DC_20A_n38A. - These IMDs problems will be covered in DC_20A_n3A-n38A. |
| DC_7-20_n3-n78 | DC_7A_n3A | 2nd harmonic from n3 into n78 |  | 2nd IMD into B20 3rd IMD into n78 |  | - |  |  | - Harmonic problem already solved in CA_n3A-n78A - 2nd IMD problem will be covered in DC_7A-20A_n3A - 3rd IMD problem already covered in DC_7A_n3A-n78A |
|  | DC_20A_n3A | - 2nd harmonic from n3 into n78 - 4th harmonic from B20 into n78 |  | 2nd & 3rd IMDs into nB7 3rd & 5th IMDs into n78 |  | - |  |  | - Harmonic problems was already covered in DC_3A_n78A or DC_20A_n78A. - 2nd & 3rd IMDs problems will be covered in DC_7A-20A_n3A. - 3rd &5th IMDs problems will be covered in DC_20A_n3A-n78A. |
| DC_2-66_n66-n78 | DC_2A_n66A | 2nd harmonic from B2 into n78 2nd harmonic from n66 inton78 |  | 2nd & 4th IMDs into n78 5th IMD into B66 |  | - |  |  | - These harmonic problems already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - 2nd & 4th IMDs will be covered in DC_2A_n66A-n78A - 5th IMD will be covered in DC_2A-66A_n66A |
|  | DC_2A_n78A | - |  | 4th IMD into B66/n66 |  | - |  |  | 4th IMD problem same as DC_2A-66A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
|  | DC_66A_n66A1 | 2nd harmonic from B66 into n78 |  | 2nd & 4th IMDs into n78 |  |  |  |  | - Harmonic problem already covered in DC_66A_n78A - 2nd & 4th IMDs issues will be covered in DC_66A_n66A-n78A |
|  | DC_66A_n78A | - |  | 2nd , 4th & 5th IMDs into B2 5th IMD into n66 |  |  |  |  | - 2nd , 4th & 5th IMDs issue will be covered in DC_2A-66A_n78A - 5th IMD issue will be covered in DC_66A_n66A-n78A |
| DC_7-66_n66-n78 | DC_7A_n66A | 2nd harmonic from n66 into n78 |  | 3rd IMD into n78 |  | - |  |  | The harmonic problem already covered in Table 7.3B.2.3.1-1 in TS38.101-3 3rd IMD issue will be covered in DC_7A_n66A-n78A |
|  | DC_7A_n78A | - |  | 4th IMD into B66/n66 |  | Yes |  |  | - The 4th IMD problem same as DC_7A-66A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
|  | DC_66A_n66A1 | 2nd harmonic from B66 into n78 |  | 2nd & 4th IMDs into n78 |  |  |  |  | - Harmonic problem already covered in DC_66A_n78A - 2nd & 4th IMDs issues will be covered in DC_66A_n66A-n78A |
|  | DC_66A_n78A |  |  | 5th IMD into n66 |  |  |  |  | - 5th IMD issue will be covered in DC_66A_n66A-n78A |
| DC_2-7_n66-n78 DC_2-7-7_n66-n78 | DC_2A_n66A | 2nd harmonic from B2 into n78 2nd harmonic from n66 inton78 |  | 2nd & 4th IMDs into n78 |  | - |  |  | - These harmonic problems already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - 2nd & 4th IMDs will be covered in DC_2A_n66A-n78A |
|  | DC_2A_n78A | - |  | 4th IMD into n66 |  | - |  |  | 4th IMD problem same as DC_2A-66A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
|  | DC_7A_n66A | 2nd harmonic from n66 into n78 |  | 3rd IMD into n78 |  | - |  |  | The harmonic problem already covered in Table 7.3B.2.3.1-1 in TS38.101-3 3rd IMD issue will be covered in DC_7A_n66A-n78A |
|  | DC_7A_n78A | - |  | 4th IMD into n66 4th IMD into B2 |  | Yes |  |  | The 4th IMD problem same as DC_7A-66A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 4th IMD issue will be covered in DC_2A-7A_n78A |
| DC_7-7-66-n66-n78 | DC_7A_n66A | 2nd harmonic from n66 into n78 |  | 3rd IMD into n78 4th IMD into B7 |  | - |  |  | The harmonic problem already covered in Table 7.3B.2.3.1-1 in TS38.101-3 3rd IMD issue will be covered in DC_7A_n66A-n78A 4th IMD issue will be covered in DC_7A-7A_n66A |
|  | DC_7A_n78A | - |  | 4th IMD into B66/n66 |  | Yes |  |  | The 4th IMD problem same as DC_7A-66A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 4th IMD issue will be covered in DC_2A-7A_n78A |
|  | DC_66A_n66A1 | 2nd harmonic from B66 into n78 |  | 2nd & 4th IMDs into n78 |  |  |  |  | - Harmonic problem already covered in DC_66A_n78A - 2nd & 4th IMDs issues will be covered in DC_66A_n66A-n78A |
|  | DC_66A_n78A |  |  | 5th IMD into n66 |  |  |  |  | - 5th IMD issue will be covered in DC_66A_n66A-n78A |
| DC_1-18_n3-n257 | DC_1A_n3A | High order harmonic into n257 |  | - |  | -- |  |  | No harmonic impact to n257 |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | - |  | - |  | - |  |  | No issue |
|  | DC_18A_n3A | High order harmonics into n257 |  | - |  | - |  |  | No harmonic problem to n257 |
|  | DC_18A_n257A, DC_18A_n257G, DC_18A_n257H, DC_18A_n257I | - |  | - |  | - |  |  | No issue |
| DC_1-28_n3-n257 | DC_1A_n3A | High order harmonic into n257 |  | - |  | -- |  |  | No harmonic impact to n257 |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | - |  | - |  | - |  |  | No issue |
|  | DC_28A_n3A | 3rd harmonic from B28 into B1 |  | 4th IMD into B1 |  | - |  |  | - These harmonic issue already covered in CA_n3A-n78A or DC_28A_n78A. or CA_1A-28A - 4th IMD problem will be covered in DC_1A-28A_n3A |
|  | DC_28A_n257A, DC_28A_n257G, DC_28A_n257H, DC_28A_n257I | - |  | - |  | - |  |  | No issue |
| DC_1-41_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 |  | 4th IMD into B41 |  | - |  |  | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 4th IMD issue will be covered in DC_1A-41A_n78A |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | 2nd harmonic from B1 into n78 |  | - |  | - |  |  | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_41A_n78A | 7th & 8th harmonics from n78 into n257 |  | 4th IMD into B1 |  | - |  |  | - No harmonic impact to n257 - 4th IMD issue will be covered in DC_1A-41A_n78A |
|  | DC_41A_n257A, DC_41A_n257G, DC_41A_n257H, DC_41A_n257I | - |  | - |  | - |  |  | No issue |
| DC_1-42_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 |  | 4th IMD into B42 |  | - |  |  | - No harmonic problems by 7th & 8th order between FR1 and FR2 - B42 and n78 will be synchronous operation. |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | - |  | - |  | - |  |  | No issue |
|  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - |  | - |  | - |  |  | No issue |
| DC_3-41_n78-n257 | DC_3A_n78A | 7th & 8th harmonics from n78 into n257 |  | - |  | - |  |  | - Up to 7th harmonic order will be not any impact to n257 |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 |  | - |  | - |  |  | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_41A_n78A | 7th & 8th harmonics from n78 into n257 |  | 3rd & 4th IMDs into B3 |  | - |  |  | - No harmonic impact to n257 - 3rd & 4th IMDs issue will be covered in DC_3A-41A_n78A |
|  | DC_41A_n257A, DC_41A_n257G, DC_41A_n257H, DC_41A_n257I | - |  | - |  | - |  |  | No issue |
| DC_3-42_n78-n257 | DC_3A_n78A | 2nd harmonic from B3 into B42 7th & 8th harmonics from n78 into n257 |  | 4th & 5th IMDs into B42 - |  | - |  |  | - 2nd harmonic already covered in CA_3A-42A and Up to 7th harmonic order will be not any impact to n257 - These MSD issues are already covered in 3DL_DC_3A-42A_n78A with 2UL_DC_3A_n78A |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 |  | - |  | - |  |  | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - |  | - |  | - |  |  | No issue |
| DC_41-42_n78-n257 | DC_41A_n78A | 7th & 8th harmonics from n78 into n257 |  | - |  | - |  |  | - No harmonic impact to n257 |
|  | DC_41A_n257A, DC_41A_n257G, DC_41A_n257H, DC_41A_n257I | - |  | - |  | - |  |  | No issue |
|  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - |  | - |  | - |  |  | No issue |
| DC_1-18_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 |  | 5th IMD into B18 |  | - |  |  | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 5th IMD issue will be covered in DC_1A-18A_n78A |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | - |  | - |  | - |  |  | No issue |
|  | DC_18A_n78A | 7th & 8th harmonics from n78 into n257 |  | 3rd IMD into B1 |  | - |  |  | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 3rd IMD issue will be covered in DC_1A-18A_n78A |
|  | DC_18A_n257A, DC_18A_n257G, DC_18A_n257H, DC_18A_n257I | - |  | - |  | - |  |  | No issue |
| DC_3-18_n78-n257 | DC_3A_n78A | 7th & 8th harmonics from n78 into n257 |  |  |  |  |  |  | Up to 7th harmonic order will be not any impact to n257 |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 |  | - |  | - |  |  | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_18A_n78A | 7th & 8th harmonics from n78 into n257 |  | 3rd IMD into B3 |  | - |  |  | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 3rd IMD issue will be covered in DC_3A-18A_n78A |
|  | DC_18A_n257A, DC_18A_n257G, DC_18A_n257H, DC_18A_n257I | - |  | - |  | - |  |  | No issue |
| DC_18-42_n78-n257 | DC_18A_n78A | 7th & 8th harmonics from n78 into n257 |  |  |  | - |  |  | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  | DC_18A_n257A, DC_18A_n257G, DC_18A_n257H, DC_18A_n257I | - |  | - |  | - |  |  | No issue |
|  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - |  | - |  | - |  |  | No issue |
| DC_3-28_n78-n257 | DC_3A_n78A | 7th & 8th harmonics from n78 into n257 |  |  |  |  |  |  | Up to 7th harmonic order will be not any impact to n257 |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 |  | - |  | - |  |  | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_28A_n78A | 7th & 8th harmonics from n78 into n257 |  | 3rd IMD into n3 |  | - |  |  | Up to 7th harmonic order will be not any impact to n257 - 3rd IMD will be covered in DC_28A_n3A-n78A |
|  | DC_28A_n257A, DC_28A_n257G, DC_28A_n257H, DC_28A_n257I | - |  | - |  | - |  |  | No issue |
| DC_28-41_n78-n257 | DC_28A_n78A | 7th & 8th harmonics from n78 into n257 |  | 2nd IMD into B41 |  | - |  |  | Up to 7th harmonic order will be not any impact to n257 - 2nd IMD issue will be covered in DC_28A-41A_n78A |
|  | DC_28A_n257A, DC_28A_n257G, DC_28A_n257H, DC_28A_n257I | - |  | - |  | - |  |  | No issue |
|  | DC_41A_n78A | 7th & 8th harmonics from n78 into n257 |  | 2nd & 5th IMDs into B28 |  | - |  |  | - No harmonic impact to n257 - 2nd & 5th IMDs will be covered in DC_28A-41A_n78A |
|  | DC_41A_n257A, DC_41A_n257G, DC_41A_n257H, DC_41A_n257I | - |  | - |  | - |  |  | No issue |
| DC_28-42_n78-n257 | DC_28A_n78A | 7th & 8th harmonics from n78 into n257 |  | - |  | - |  |  | Up to 7th harmonic order will be not any impact to n257 |
|  | DC_28A_n257A, DC_28A_n257G, DC_28A_n257H, DC_28A_n257I | - |  | - |  | - |  |  | No issue |
|  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - |  | - |  | - |  |  | No issue |
| DC_1-28_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 |  | 5th IMD into B28 |  | - |  |  | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 5th IMD issue will be covered in DC_1A-28A_n78A |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | - |  | - |  | - |  |  | No issue |
|  | DC_28A_n78A | 3rd harmonic from B28 into B1 7th & 8th harmonics from n78 into n257 |  | -3rd IMD into B1 |  | - |  |  | - 3rd harmonic already covered in CA_1A-28A Up to 7th harmonic order will be not any impact to n257 - 3rd IMD issue will be covered in DC_1A-28A_n78A |
|  | DC_28A_n257A, DC_28A_n257G, DC_28A_n257H, DC_28A_n257I | - |  | - |  | - |  |  | No issue |
| DC_1-11_n77-n257 | DC_1A_n77A | 7th & 8th Harmonics from n77 into n257 |  | 2nd & 5th IMDs into B11 |  | Yes |  |  | - No harmonic problems by 7th & 8th order between FR1 and FR2 - These IMDs issue will be covered in DC_1A-11A_n77A |
|  | DC_1A_n257A | 2nd harmonic from B1 into n77 |  | - |  | - |  |  | - Harmonic problem already discussed and will be solved in DC_1A_n77A |
|  | DC_11A_n77A | 7th & 8th harmonics from n77 into n257 |  | 2nd & 5th IMDs into B1 |  | - |  |  | No harmonic problem to n257 - These IMDs issue will be covered in DC_1A-11A_n77A |
|  | DC_11A_n257A | - |  | - |  | - |  |  | No issue |
| DC_8-11_n77-n257 | DC_8A_n77A | 7th & 8th Harmonics from n77 into n257 |  | 3rd & 4th IMDs into B11 |  | - |  |  | - No harmonic problems by 7th & 8th order between FR1 and FR2 The MSD issue will be covered in 3DL_DC_8A-11A_n77A |
|  | DC_8A_n257A | 4th harmonic from B8 into n77 |  | - |  | - |  |  | - Harmonic problem will be solved in DC_8A_n77A |
|  | DC_11A_n77A | 7th & 8th harmonics from n77 into n257 |  | 3rd & 4th IMDs into B8 |  | - |  |  | No harmonic problem to n257 - These IMDs issue will be covered in DC_8A-11A_n77A |
|  | DC_11A_n257A | - |  | - |  | - |  |  | No issue |
| DC_46-66_n25-n41 | DC_66A_n25A | 3rd harmonics into B46 |  | 3rd & 5th IMDs into B46 |  | - |  |  | - 3rd harmonics were covered in CA_46A-66A and DC_46A_n25A - 3rd & 5th IMDs will be covered in DC_46A-66A_n25A |
|  | DC_66A_n41A | 2nd harmonic from n41 into B46 |  | 4th IMD into n25 3rd & 4th IMDs into B46 |  | - |  |  | The harmonic was covered in DC_46A_n41A 4th IMD into n25 was covered in DC_66A_n25A-n41A 3rd & 4th IMDs into B46 were covered in DC_46A-66A_n41A |
| DC_46-66_n41-n71 | DC_66A_n41A | 2nd harmonic from n41 into B46 |  | 3rd & 4th IMDs into B46 |  | - |  |  | The harmonic was covered in DC_46A_n41A 3rd & 4th IMDs into B46 were covered in DC_46A-66A_n41A |
|  | DC_66A_n71A | 4th harmonic from n71 into n41 3rd harmonic from B66 into B46 |  | 4th & 5th IMDs into B46 |  | - |  |  | These harmonic problem will be treated CA_n41A_n71A or CA_46A-66A The 4th & 5th IMDs were covered in DC_46A-66A_n71A |
| DC_2-46_n41-n71 | DC_2A_n41A | 2nd harmonic from n41 into B46 3rd harmonic from B2 into B46 |  | 2nd & 5th IMDs into n71 4th IMD into B46 |  | - |  |  | - These harmonic problem were covered in DC_46A_n41A or CA_2A-46A - These IMD issue will be covered in DC_2A_n41A-n71A or DC_2A-46A_n41A |
|  | DC_2A_n71A | 4th harmonic from n71 into n41 3rd harmonic from B2 into B46 |  | 2nd IMD into n41 4th & 5th IMDs into B46 |  | - |  |  | - The harmonic problem will be treated CA_n41A_n71A or CA_2A-46A - 2nd IMD issue will be covered in DC_2A_n41A-n71A - 4th & 5th IMDs will be covered in DC_2A-46A_n71A |
| DC_2-46_n41-n66 | DC_2A_n41A | 2nd harmonic from n41 into B46 3rd harmonic from B2 into B46 |  | 4th IMD into B46 |  | - |  |  | - These harmonic problem were covered in DC_46A_n41A or CA_2A-46A - These IMD issue will be covered in DC_2A-46A_n41A |
|  | DC_2A_n66A | 3rd harmonic from B2 into B46 |  | 3rd & 5th IMDs into B46 |  | - |  |  | - The harmonic problem was covered in CA_2A-46A - These IMDs issue will be covered in DC_2A-46A_n66A |
| DC_13-66_n5-n48 | DC_13A_n48A | - |  | 3rd IMD into B66 |  | - |  |  | - The IMD problem will be covered in DC_13A-66A_n48A |
|  | DC_66A_n5A | 2nd harmonic from n66 into n48 |  | 5th IMD into n48 4th IMD into B13 |  | - |  |  | - 2nd harmonic issue will be solved in CA_n48-n66 - These IMDs problem will be covered in DC_66A_n5A-n48A or DC_13A-66A_n5A |
|  | DC_66A_n48A | - |  | - |  | - |  |  | No issue |
| DC_7-28_n3-n78 | DC_7A_n3A DC_7C_n3A | 2nd harmonic from n3 into n78 |  | 2nd IMD into B28 3rd IMD into n78 |  | - |  |  | - Harmonic problem already solved in CA_n3A-n78A - 2nd IMD problem will be covered in DC_7A-28A_n3A - 3rd IMD problem already covered in DC_7A_n3A-n78A |
|  | DC_7A_n78A DC_7C_n78A | - |  | 3rd & 4th IMDs into n3 2nd & 5th IMDs into B28 |  | - |  |  | - The 3rd & 4th IMDs will be covered in DC_3A-7A_n78A - The 2nd & 5th IMDs will be covered in DC_7A-28A_n78A |
|  | DC_28A_n3A | 2nd harmonic from n3 into n78 5th harmonic from B28 into n78 |  | 3rd IMD into B7 5th IMD into n78 |  | - |  |  | - These harmonic issue already covered in CA_n3A-n78A or DC_28A_n78A. - 3rd IMD problem was covered in DC_7A-28A_n3A - 5th IMD problem same as DC_3A_n28A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
|  | DC_28A_n78A | - |  | 2nd IMD into B7 3rd IMD into n3 |  | - |  |  | - 2nd IMD will be covered in DC_7A-28A_n78A - 3rd IMD will be covered in DC_28A_n3A-n78A |
| DC_1-7_n7-n78 | DC_1A_n7A | - |  | 4th & 5th IMDs into n78 |  | - |  |  | These IMD problems were already covered in DC_7_n1-n78 with 2UL_DC_7A_n1A |
|  | DC_1A_n78A | - |  | 4th IMD into B7/n7 |  | - |  |  | The 4th IMD will be covered in DC_1A-7A_n78A |
|  | DC_7A_n7A1 | - |  | - |  | - |  |  | No issue |
|  | DC_7A_n78A | - |  | 4th IMD into B1 |  | - |  |  | - 4th IMD will be covered in DC_7A_n1A-n78A |
| DC_3-7_n7-n78 DC_3-3-7_n7-n78 | DC_3A_n7A DC_3C_n7A | 2nd harmonic from 3 into n78 |  | 3rd IMD into B7 3rd IMD into n78 |  | - |  |  | - Harmonic problem was solved in DC_3A_n78A - 3rd IMD problem will be covered in DC_3A-7A_n7A. - 3rd IMD problem already covered in DC_3A_n7A-n78A. |
|  | DC_3A_n78A DC_3C_n78A | - |  | - |  | - |  |  | No issue |
|  | DC_7A_n7A1 | - |  | - |  | - |  |  | No issue |
|  | DC_7A_n78A | - |  | 3rd & 4th IMDs into B3 |  | - |  |  | - The 3rd & 4th IMDs will be covered in DC_3A-7A_n78A |
| DC_7-28_n7-n78 | DC_7A_n7A1 | - |  | - |  | - |  |  | No issue |
|  | DC_7A_n78A | - |  | 2nd & 5th IMDs into B28 |  | - |  |  | - The 2nd & 5th IMDs will be covered in DC_7A-28A_n78A |
|  | DC_28A_n7A | 5th harmonic from B28 into n78 |  | 2nd & 4th IMDs into n78 |  | - |  |  | - 5th harmonic issue will be covered in DC_28-n78 - 2nd & 4th IMDs will be covered in DC_28A_n7A-n78A |
|  | DC_28A_n78A | - |  | 2nd IMD into B7/n7 |  | - |  |  | - 2nd IMD will be covered in DC_7A-28A_n78A |
| DC_3-28_n7-n78 DC_3-3-28_n7-n78 | DC_3A_n7A DC_3A_n7B DC_3C_n7A DC_3C_n7B | 2nd harmonic from 3 into n78 |  | 2nd IMD into B28 3rd IMD into n78 |  | - |  |  | - Harmonic problem was solved in DC_3A_n78A - 2nd IMD problem will be covered in DC_3A-28A_n7A. - 3rd IMD problem already covered in DC_3A_n7A-n78A. |
|  | DC_3A_n78A DC_3C_n78A | - |  | - |  | - |  |  | No issue |
|  | DC_28A_n7A DC_28A_n7B | 5th harmonic from B28 into n78 |  | 2nd & 4th IMDs into n78 |  | - |  |  | - 5th harmonic issue will be covered in DC_28-n78 - 2nd & 4th IMDs will be covered in DC_28A_n7A-n78A |
|  | DC_28A_n78A | - |  | 3rd IMD into B3 2nd IMD into n7 |  | - |  |  | - 3rd IMD will be covered in DC_3A-28A_n78A - 2nd IMD will be covered in DC_7A-28A_n78A |
| DC_1-28_n7-n78 | DC_1A_n7A DC_1A_n7B | - |  | 4th & 5th IMDs into n78 5th IMD into B28 |  | - |  |  | - 4th & 5th IMDs problems were already covered in DC_7_n1-n78 with 2UL_DC_7A_n1A - 5th IMD issue will be covered in DC_1A-28A_n7A |
|  | DC_1A_n78A | - |  | 4th IMD into n7 5th IMD into B28 |  | - |  |  | The 4th IMD will be covered in DC_1A-7A_n78A 5th IMD will be covered in DC_1A-28A_n78A |
|  | DC_28A_n7A DC_28A_n7B | 3rd harmonic from B28 into B1 5th harmonic from B28 into n78 |  | 2nd & 4th IMDs into n78 |  | - |  |  | - 3rd harmonic issue will be covered in CA_1A-28A - 5th harmonic issue will be covered in DC_28-n78 - 2nd & 4th IMDs will be covered in DC_28A_n7A-n78A |
|  | DC_28A_n78A | 3rd harmonic from B28 into B1 |  | - 2nd IMD into n7 - 3rd IMD into B1 |  | - |  |  | - 3rd harmonic already covered in CA_1A-28A - 2nd IMD will be covered in DC_7A-28A_n78A - 3rd IMD issue will be covered in DC_1A-28A_n78A |
| DC_1-11_n3-n28 | DC_1A_n3A | - |  | 3rd & 5th IMDs into B11 |  | - |  |  | - 3rd & 5th IMDs issue will be covered in DC_1A-11A_n3A |
|  | DC_1A_n28A | 2nd harmonic from n28 into B11 |  | 5th IMD into n3 |  | Yes |  |  | - 2nd harmonic problem will be covered in DC_11A_n28A - 5th IMD problem same as DC_1A-3A_n28A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
|  | DC_11A_n3A | - |  | 3rd IMD into B1 5th IMD into n28 |  |  |  |  | - 3rd IMD issue will be covered in DC_1A-11A_n3A - 5th IMD issue will be covered in DC_11A_n3A-n28A |
|  | DC_11A_n28A | 3rd harmonic from n28 into B1 |  | 2nd & 3rd IMDs into B1 |  | - |  |  | 3rd harmonic will be covered in DC_1A_n28A 2nd & 3rd IMDs issue will be covered in DC_1A-11A_n28A |
| DC_8-11_n3-n28 | DC_8A_n3A | - |  | 2nd & 5th IMDs into n28 |  | - |  |  | - These IMD issues will be covered in DC_8A_n3A-n28A |
|  | DC_8A_n28A | 2nd harmonic from B8 into n3 2nd harmonic from n28 into B11 |  | - |  | - |  |  | The 2nd harmonic problem will be covered in DC_8A_n3A The 2nd harmonic problem will be covered in DC_11A_n28A |
|  | DC_11A_n3A | - |  | 5th IMD into n28 |  |  |  |  | - 5th IMD issue will be covered in DC_11A_n3A-n28A |
|  | DC_11A_n28A | - |  | - |  | - |  |  | No issue |
| Note1: These band combinations should be completed the dual uplink EN-DC firstly |  |  |  |  |  |  |  |  |  |

Table 5.3-3 summarizes the EN-DC band combinations with self-interference problems for 5DL/2UL DC operation.

Table 5.3-3: Summary of Self-interference analysis for LTE 3 bands & NR 2 bands DL and 2 bands UL EN-DC operation

| Downlink band configuration | Uplink DC Configuration | Harmonic relation issues | intermodulation to own rx band | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| DC_1-3-5_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 2nd IMD into B3 5th IMD into B5 | Yes | - Up to 7th harmonic order will be not any impact to n257 - These IMDs problem will be covered in DC_1A-3A_n78A and DC_1A-5A_n78A - Proximity also follow LTE_CA_1A-3A |
|  | DC_1A_n257A | 2nd harmonic from B1 into n78 | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_3A_n78A | 7th & 8th harmonics from n78 into n257 | 5th IMD into B1 | - | - Up to 7th harmonic order will be not any impact to n257 - 5th IMD problems will be covered in DC_1A-3A_n78A. |
|  | DC_3A_n257A | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_5A_n78A | 7th & 8th harmonics from n78 into n257 | 3rd IMD into B1 3rd IMD into B3 | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 3rd IMD problems will be covered in DC_1A-5A_n78A and DC_3A-5A_n78A. |
|  | DC_5A_n257A | 4th harmonic from B5 into n78 | - | - | - Harmonic problem will be solved in DC_5A_n78A |
| DC_1-3-7_n28-n78 | DC_1A_n28A | 3rd harmonic into B1 5th harmonic into n78 | 2nd IMD into B7 3rd IMD into n78 | Yes | - Harmonic problems already solved in DC_1A_n78A or DC_n28A-n78A - 2nd IMD problem was solved in 3DL_DC_1A-7A_n28A - 3rd IMD problem was solved in 3DL_DC_1A_n28A-n78A |
|  | DC_1A_n78A | - | 2nd IMD into B3 4th IMD into B1 4th IMD into n78 5th IMD into n28 | Yes | - 2nd IMD problem was solved in 3DL_DC_1A-3A_n78A - 4th IMD problem was solved in DC_1A-n78A - 4th IMD problem is no issue for TDD band - 5th IMD problem was solved in 3DL_DC_1A_n28A-n78A |
|  | DC_3A_n28A | 2nd harmonic into n78 3rd harmonic into B1 5th harmonic into n78 | 3rd IMD into B7 4th IMD into B1 5th IMD into n78 |  | - Harmonic problems already solved in DC_3A_n78A or DC_1A_n28A or DC_28A-n78A - 3rd IMD problem was solved in 3DL_DC_3A-7A_n28A - 4th IMD problem was solved in 3DL_DC_1A-3A_n28A - 5th IMD problem was solved in 3DL_DC_3A_n28A-n78A |
|  | DC_3A_n78A | 2nd harmonic into n78 | 2nd,4th & 5th IMDs into B3 4th & 5th IMDs into n78 5th IMD into B1 |  | - 2nd harmonic problem already solved in DC_3A-n78A - 2nd,4th & 5th IMDs problems already solved in DC_3A-n78A - 4th & 5th IMDs problems are no issue for TDD band -5th IMD problem was solved in 3DL_DC_1A-3A_n78A |
|  | DC_7A_n28A | 3rd harmonic into B1 5th harmonic into n78 | 2nd IMD into B3 2nd,4th IMDs into n78 |  | - Harmonic problem was solved in DC_1A_n28A or DC_n28A-n78A - 2nd IMD problem was solved in 3DL_DC_3A-7A_n28A - 2nd & 4th IMDs problems were solved in 3DL_DC_7A_n28A-n78A |
|  | DC_7A_n78A | - | 2nd & 5th IMDs into n28 3rd & 4th IMDs into B3 4th IMD into n78 4th IMD into B1 |  | - 2nd & 5th IMDs problems were solved in 3DL_DC_7A-28A_n78A - 3rd & 4th IMDs problem were solved in 3DL_DC_3A-7A_n78A - 4th IMD problem is no issue for TDD band - 4th IMD problem was solved in 3DL_DC_1A-7A_n78A |
| DC_1-3-7-7_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 2nd IMD into B3 4th IMD into B7 | Yes | - Up to 7th harmonic order will be not any impact to n257 - These IMDs problem will be covered in DC_1A-3A_n78A and DC_1A-7A_n78A - Proximity also follow LTE_CA_1A-3A |
|  | DC_1A_n257A | 2nd harmonic from B1 into n78 | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_3A_n78A | 7th & 8th harmonics from n78 into n257 | 5th IMD into B1 | - | - Up to 7th harmonic order will be not any impact to n257 - 5th IMD problems will be covered in DC_1A-3A_n78A. |
|  | DC_3A_n257A | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_7A_n78A | 7th harmonic from B7 into n257 | 3rd & 4th IMDs into B3 4th IMD into B1 | - | - Up to 7th harmonic order will be not any impact to n257 - The IMD problem will be covered in DC_3A-7A_n78A and DC_1A-7A_n78A |
|  | DC_7A_n257A | - | - | - | No issues |
| DC_1-3-7_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 2nd IMD into B3 4th IMD into B7 | Yes | - Up to 7th harmonic order will be not any impact to n257 - These IMDs problem will be covered in DC_1A-3A_n78A and DC_1A-7A_n78A - Proximity also follow LTE_CA_1A-3A |
|  | DC_1A_n257A | 2nd harmonic from B1 into n78 | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_3A_n78A | 7th & 8th harmonics from n78 into n257 | 5th IMD into B1 | - | - Up to 7th harmonic order will be not any impact to n257 - 5th IMD problems will be covered in DC_1A-3A_n78A. |
|  | DC_3A_n257A | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_7A_n78A | 7th harmonic from B7 into n257 | 3rd & 4th IMDs into B3 4th IMD into B1 | - | - Up to 7th harmonic order will be not any impact to n257 - The IMD problem will be covered in DC_3A-7A_n78A and DC_1A-7A_n78A |
|  | DC_7A_n257A | - | - | - | No issues |
| DC_1-3-8_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 2nd IMD into B3 5th IMD into B8 | Yes | - Up to 7th harmonic order will be not any impact to n257 - These IMDs problem will be covered in DC_1A-3A_n78A and DC_1A-8A_n78A - Proximity also follow LTE_CA_1A-3A |
|  | DC_1A_n257A | 2nd harmonic from B1 into n78 | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_3A_n78A | 7th & 8th harmonics from n78 into n257 | 5th IMD into B1 | - | - Up to 7th harmonic order will be not any impact to n257 - 5th IMD problems will be covered in DC_1A-3A_n78A. |
|  | DC_3A_n257A | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_8A_n78A | 2nd harmonic from B8 into B3 | 3rd IMD into B3 | - | - Harmonic problem already be solved in CA_3A_8A - 3rd IMD already covered in DC_3A-8A_n78A |
|  | DC_8A_n257A | 4th harmonic from B8 into n78 | - | - | - Harmonic problem will be solved in DC_8A_n78A |
| DC_1-5-7-7_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 4th IMD into B7 5th IMD into B5 | Yes | - Up to 7th harmonic order will be not any impact to n257 - These IMDs problem will be covered in DC_1A-7A_n78A and DC_1A-5A_n78A - Proximity also follow LTE_CA_1A-3A |
|  | DC_1A_n257A | 2nd harmonic from B1 into n78 | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_5A_n78A | 7th & 8th harmonics from n78 into n257 | 2nd IMD into B7 3rd IMD into B1 | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - These IMDs problems will be covered in DC_5A-7A_n78A and DC_1A-5A_n78A. |
|  | DC_5A_n257A | 4th harmonic from B5 into n78 | - | - | - Harmonic problem will be solved in DC_5A_n78A |
|  | DC_7A_n78A | 7th harmonic from B7 into n257 | 2nd & 5th IMDs into B5 4th IMD into B1 | - | - Up to 7th harmonic order will be not any impact to n257 - These IMDs problems will be covered in DC_5A-7A_n78A and DC_1A-7A_n78A |
|  | DC_7A_n257A | - | - | - | No issues |
| DC_1-5-7_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 4th IMD into B7 5th IMD into B5 | Yes | - Up to 7th harmonic order will be not any impact to n257 - These IMDs problem will be covered in DC_1A-7A_n78A and DC_1A-5A_n78A - Proximity also follow LTE_CA_1A-3A |
|  | DC_1A_n257A | 2nd harmonic from B1 into n78 | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_5A_n78A | 7th & 8th harmonics from n78 into n257 | 2nd IMD into B7 3rd IMD into B1 | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - These IMDs problems will be covered in DC_5A-7A_n78A and DC_1A-5A_n78A. |
|  | DC_5A_n257A | 4th harmonic from B5 into n78 | - | - | - Harmonic problem will be solved in DC_5A_n78A |
|  | DC_7A_n78A | 7th harmonic from B7 into n257 | 2nd & 5th IMDs into B5 4th IMD into B1 | - | - Up to 7th harmonic order will be not any impact to n257 - These IMDs problems will be covered in DC_5A-7A_n78A and DC_1A-7A_n78A |
|  | DC_7A_n257A | - | - | - | No issues |
| DC_3-5-7-7_n78-n257 | DC_3A_n78A | 7th & 8th harmonics from n78 into n257 | - | - | - Up to 7th harmonic order will be not any impact to n257 |
|  | DC_3A_n257A | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_5A_n78A | 7th & 8th harmonics from n78 into n257 | 3rd IMD into B3 2nd IMD into B7 | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 3rd IMD problems will be covered in DC_3A-5A_n78A and DC_5A-7A_n78A. |
|  | DC_5A_n257A | 4th harmonic from B5 into n78 | - | - | - Harmonic problem will be solved in DC_5A_n78A |
|  | DC_7A_n78A | 7th harmonic from B7 into n257 | 2nd & 5th IMDs into B5 3rd & 4th IMDs into B3 | - | - Up to 7th harmonic order will be not any impact to n257 - These IMDs problems will be covered in DC_5A-7A_n78A and DC_3A-7A_n78A |
|  | DC_7A_n257A | - | - | - | No issues |
| DC_3-5-7_n78-n257 | DC_3A_n78A | 7th & 8th harmonics from n78 into n257 | - | - | - Up to 7th harmonic order will be not any impact to n257 |
|  | DC_3A_n257A | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_5A_n78A | 7th & 8th harmonics from n78 into n257 | 3rd IMD into B3 2nd IMD into B7 | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 3rd IMD problems will be covered in DC_3A-5A_n78A and DC_5A-7A_n78A. |
|  | DC_5A_n257A | 4th harmonic from B5 into n78 | - | - | - Harmonic problem will be solved in DC_5A_n78A |
|  | DC_7A_n78A | 7th harmonic from B7 into n257 | 2nd & 5th IMDs into B5 3rd & 4th IMDs into B3 | - | - Up to 7th harmonic order will be not any impact to n257 - These IMDs problems will be covered in DC_5A-7A_n78A and DC_3A-7A_n78A |
|  | DC_7A_n257A | - | - | - | No issues |
| DC_1-3-21_n77-n78 | DC_3A_n77A | 2nd harmonic from B3 into n78 | 2nd & 5th IMDs into B1 4th & 5th IMDs into B21 4th & 5th IMDs into n78 | - | 2nd harmonic issue already covered in DC_3A_n78A These MSD issues are already covered in 3DL_DC_1A-3A_n77A and 3DL_DC_3A-21A_n77A and 3DL_DC_3A_n77A-n78A with 2UL_DC_3A_n77A |
|  | DC_3A_n78A | 2nd harmonic from B3 into n77 | 5th IMD into B1 4th & 5th IMDs into B21 4th & 5th IMDs into n77 | - | 2nd harmonic issue already covered in DC_3A_n77A These MSD issues are already covered in 3DL_DC_1A-3A_n78A and 3DL_DC_3A-21A_n78A and 3DL_DC_3A_n77A-n78A with 2UL_DC_3A_n78A |
| DC_1-3-21_n77-n79 | DC_3A_n77A | - | 2nd & 5th IMDs into B1 4th & 5th IMDs into B21 3rd & 4th IMDs into n79 | - | These MSD issues are already covered in 3DL_DC_1A-3A_n77A and 3DL_DC_3A-21A_n77A and 3DL_DC_3A_n77A-n79A with 2UL_DC_3A_n77A |
|  | DC_3A_n79A | 2nd harmonic from B3 into n77 | 5th IMD into B1 3rd IMD into B21 5th IMD into n77 | - | 2nd harmonic issue already covered in DC_3A_n77A These MSD issues are already covered in 3DL_DC_1A-3A_n79A and 3DL_DC_3A-21A_n79A and 3DL_DC_3A_n77A-n79A with 2UL_DC_3A_n79A |
|  | DC_1A_n77A | - | 2nd, 4th & 5th IMDs into B3 2nd & 5th IMDs into B21 3rd, 4th & 5th IMD into n79 | Yes | - These IMDs issue will be covered in DC_1A-3A_n77A - 2nd & 5th IMDs will be covered in DC_1A-21A_n77A - 3rd, 4th & 5th IMDs will be covered in DC_1A_n77A-n79A |
|  | DC_1A_n79A | 2nd harmonic from B1 into n77 | 4th IMD into B21 5th IMD into n77 | - | 2nd harmonic issue already covered in DC_1A_n77A The MSD issue are already covered in 3DL_DC_1A-21A_n79A The MSD issue are already covered in 3DL_DC_1A_n77A-n79A |
|  | DC_21A_n77A |  | 2nd & 5th IMDs into B1 2nd & 5th IMDs into B3 2nd & 4th IMDs into n79 | - | These MSD issues are already covered in 3DL_DC_1A-21A_n77A These MSD issues are already covered in 3DL_DC_3A-21A_n77A These MSD issues are already covered in 3DL_21A_n77A-n79A |
|  | DC_21A_n79A |  | 3rd IMD into B3 2nd IMD into n77 |  | These MSD issues are already covered in DC_3A-21A_n79A These MSD issues are already covered in DC_21A_n77A-n79A |
| DC_1-3-21_n78-n79 | DC_3A_n78A | - | 5th IMD into B1 4th & 5th IMDs into B21 3rd IMD into n79 | - | These MSD issues are already covered in 3DL_DC_1A-3A_n78A and 3DL_DC_3A-21A_n78A and 3DL_DC_3A_n78A-n79A with 2UL_DC_3A_n78A |
|  | DC_3A_n79A | 2nd harmonic from B3 into n78 | 5th IMD into B1 3rd IMD into B21 5th IMD into n78 | - | 2nd harmonic issue already covered in DC_3A_n78A These MSD issues are already covered in 3DL_DC_1A-3A_n79A and 3DL_DC_3A-21A_n79A and 3DL_DC_3A_n78A-n79A with 2UL_DC_3A_n79A |
|  | DC_1A_n78A | - | 2nd IMD into B3 2nd & 5th IMDs into B21 3rd & 5th IMD into n79 | Yes | These MSD issues are already covered in 3DL_DC_1A-3A_n78A. These MSD issues are already covered in 3DL_DC_1A-21A_n78A. These MSD issues are already covered in 3DL_DC_1A_n78A-n79A |
|  | DC_1A_n79A | - | 4th IMD into B21 5th IMD into n78 | - | The MSD issue are already covered in 3DL_DC_1A-21A_n79A The MSD issue are already covered in 3DL_DC_1A_n78A-n79A |
|  | DC_21A_n78A | - | 2nd & 5th IMDs into B1 2nd IMDs into B3 2nd & 4th IMDs into n79 | - | These MSD issue is already covered in 3DL_DC_1A-21A_n78A These MSD issue is already covered in 3DL_DC_3A-21A_n78A. These MSD issue is already covered in 3DL_DC_21A_n78A-n79A |
|  | DC_21A_n79A | - | 3rd IMD into B3 2nd IMD into n78 | - | These MSD issues are already covered in DC_3A-21A_n79A These MSD issues are already covered in DC_21A_n78A-n79A |
| DC_1-3-28_n28-n78 | DC_1A_n28A | 5th harmonic from n28 into n78 | 3rd IMD into n78 | Yes | Harmonic issue already covered in CA_n28A-n78A These MSD issues are already covered in 3DL_DC_1A_n28A-n78A with 2UL_DC_1A_n28A |
|  | DC_3A_n28A DC_3C_n28A | 2nd harmonic from B3 into n78 3rd harmonic from n28 into B1 5th harmonic from n28 into n78 | 4th IMD into B1 5th IMD into n78 |  | Harmonic issues are already covered in DC_3A_n78A, DC_1A_n28A and CA_n28A-n78A. These MSD issues are already covered in 3DL_DC_1A-3A_n28A with 2UL_DC_3A_n28A and 3DL_DC_3A_n28A-n78A with 2UL_DC_3A_n28A |
|  | DC_28A_n28A | 3rd harmonic from B28 into B1 | 3rd & 5th IMDs into B1 5th IMD into n78 | - | Harmonic issue already covered in DC_1A_n28A The MSD issue will be covered in 3DL_1A-28A_n28A with 2UL_DC_28A_n28A and 3DL_28A_n28A-n78A with 2UL_DC_28A_n28A |
|  | DC_1A_n78A | - | 2nd IMD into B3 5th IMD into n28 | Yes | The MSD issue will be covered in 3DL_1A-3A_n78A with 2UL_DC_1A_n78A and 3DL_1A_n28A-n78A with 2UL_DC_1A_n78A |
|  | DC_3A_n78A DC_3C_n78A | - | 5th IMD into B1 | - | The MSD issue will be covered in 3DL_1A-3A_n78A with 2UL_DC_3A_n78A |
|  | DC_28A_n78A | 3rd harmonic from B28 into B1 | 3rd IMD into B1 3rd IMD into B3 5th IMD into n28 | - | Harmonic issue already covered in DC_1A_n28A The MSD issue will be covered in 3DL_DC_1A-28A_n78A and 3DL_DC_3A-28A_n28A-n78A and 3DL_DC_28A_n28A-n78A with 2UL_DC_28A_n78A |
| DC_1-7-28_n28-n78 | DC_1A_n28A | 5th harmonic from n28 into n78 | 2nd IMD into B7 3rd IMD into n78 | - | Harmonic issue already covered in CA_n28A-n78A These MSD issues are already covered in 3DL_DC_1A-7A_n28A and 3DL_DC_1A_n28A-n78A with 2UL_DC_1A_n28A |
|  | DC_7A_n28A DC_7C_n28A | 3rd harmonic into B1 5th harmonic into n78 | 2nd,4th IMDs into n78 | - | Harmonic issue already covered in DC_1A_n28A or CA_n28A-n78A These MSD issues are already covered in 3DL_DC_7A_n28A-n78A with 2UL_DC_7A_n28A |
|  | DC_28A_n28A | 3rd harmonic from B28 into B1 | 3rd & 5th IMDs into B1 5th IMD into n78 | - | Harmonic issue already covered in DC_1A_n28A The MSD issue will be covered in 3DL_1A-28A_n28A with 2UL_DC_28A_n28A and 3DL_28A_n28A-n78A with 2UL_DC_28A_n28A |
|  | DC_1A_n78A | - | 4th IMD into B7 5th IMD into B28 or n28 | - | The MSD issue will be covered in 3DL_DC_1A-7A_n78A with 2UL_DC_1A_n78A and 3DL_DC_1A_n28A-n78A with 2UL_DC_1A_n78A |
|  | DC_7A_n78A DC_7C_n78A | - | 4th IMD into B1 2nd & 5th IMDs into B28 or n28 | - | The MSD issue will be covered in 3DL_DC_1A-7A_n78A with 2UL_DC_7A_n78A and 3DL_DC_7A_n28A-n78A with 2UL_DC_7A_n78A |
|  | DC_28A_n78A | 3rd harmonic from B28 into B1 | 3rd IMD into B1 2nd IMD into B7 5th IMD into n28 |  | Harmonic issue already covered in DC_1A_n28A The MSD issue will be covered in 3DL_DC_1A-28A_n78A and 3DL_DC_7A-28A_n78A and 3DL_DC_28A_n28A-n78A with 2UL_DC_28A_n78A |
| DC_1-19-42_n77-n79 | DC_19A_n77A | 6th harmonic from B19 into n79 | 3rd IMD into B1 2nd, 3rd , 4th & 5th IMDs into n79 | - | 6th harmonic problem already covered in DC_19A_n79A These MSD issues are already covered in 3DL_DC_1A-19A_n77A and 3DL_DC_19A_n77A-n79A with 2UL_DC_19A_n77A. |
|  | DC_19A_n79A | 4th harmonic from B19 into n77 | 4th IMD into B1 2nd IMD into B42 2nd & 3rd IMDs into n77 | - | 4th harmonic will be covered in DC_19A_n77A The MSD issue will be covered in 3DL_DC_1A-19A_n79A and 3DL_DC_19A-42A_n79A and 3DL_DC_19A_n77A-n79A with 2UL_DC_19A_n79A |
|  | DC_1A_n77A |  | 5th IMD into B19 4th & 5th IMD into B42 3rd, 4th & 5th IMD into n79 | - | These MSD issues are already covered in 3DL_DC_1A-19A_n77A. These MSD issues are already covered in 3DL_DC_1A-42A_n77A. These MSD issues are already covered in 3DL_DC_1A_n77A-n79A. |
|  | DC_1A_n79A | 2nd harmonic from B1 into n77 | 3rd & 4th IMDs into B19 5th IMD into B42 5th IMD into n77 | - | 2nd harmonic issue already covered in DC_1A_n77A The MSD issue are already covered in 3DL_DC_1A-19A_n79A The MSD issue are already covered in 3DL_DC_1A-42A_n79A The MSD issue are already covered in 3DL_DC_1A_n77A-n79A |
| DC_1-19-42_n78-n79 | DC_19A_n78A | 6th harmonic from B19 into n79 | 3rd IMD into B1 2nd, 3rd, 4th & 5th IMD into n79 | - | 6th harmonic problem already covered in DC_19A_n79A These MSD issues are already covered in 3DL_DC_1A-19A_n78A and 3DL_DC_19A_n78A-n79A with 2UL_DC_19A_n78A. |
|  | DC_19A_n79A | 4th harmonic from B19 into n78 | 4th IMD into B1 2nd IMD into B42 2nd & 3rd IMDs into n78 | - | 4th harmonic will be covered in DC_19A_n78A The MSD issue will be covered in 3DL_DC_1A-19A_n79A and 3DL_DC_19A-42A_n79A and 3DL_DC_19A_n78A-n79A with 2UL_DC_19A_n79A |
|  | DC_1A_n78A | - | 5th IMD into B19 4th IMD into B42 3rd & 5th IMD into n79 | - | The MSD issue will be covered in 3DL_DC_1A-19A_n78A. The MSD issue will be covered in 3DL_DC_1A-42A_n78A. These MSD issue will be covered in 3DL_DC_1A_n78A-n79A. |
|  | DC_1A_n79A | - | 3rd & 4th IMDs into B19 5th IMD into B42 5th IMD into n78 | - | 2nd harmonic issue already covered in DC_1A_n77A The MSD issue are already covered in 3DL_DC_1A-19A_n79A The MSD issue are already covered in 3DL_DC_1A-42A_n79A The MSD issue are already covered in 3DL_DC_1A_n78A-n79A |
| DC_1-21-42_n77-n78 | DC_1A_n77A | - | 2nd & 5th IMDs into B21 4th & 5th IMD into B42 4th & 5th IMD into n78 | - | These MSD issues are already covered in 3DL_DC_1A-21A_n77A and 3DL_DC_1A-42A_n77A and 3DL_DC_1A_n77A-n78A with 2UL_DC_1A_n77A |
|  | DC_1A_n78A | 2nd harmonic from B1 into n77 | 2nd & 5th IMDs into B21 4th IMD into B42 4th & 5th IMD into n77 | - | The harmonic already covered in DC_1A_n77A These MSD issues are already covered in 3DL_DC_1A-21A_n78A and 3DL_DC_1A-42A_n78A and 3DL_DC_1A_n77A-n78A with 2UL_DC_1A_n78A. |
| DC_1-21-42_n77-n79 | DC_1A_n77A | - | 2nd & 5th IMDs into B21 4th & 5th IMD into B42 3rd, 4th & 5th IMD into n79 | - | These MSD issues are already covered in 3DL_DC_1A-21A_n77A and 3DL_DC_1A-42A_n77A and 3DL_DC_1A_n77A-n79A with 2UL_DC_1A_n77A |
|  | DC_1A_n79A | 2nd harmonic from B1 into n77 | 4th IMD into B21 5th IMD into B42 5th IMD into n77 | - | 2nd harmonic issue already covered in DC_1A_n77A The MSD issue are already covered in 3DL_DC_1A-21A_n79A and 3DL_DC_1A-42A_n79A and 3DL_DC_1A_n77A-n79A with 2UL_DC_1A_n79A |
|  | DC_21A_n77A | - | 2nd & 5th IMDs into B1 5th IMD into B42 2nd & 4th IMDs into n79 | - | These MSD issues are already covered in 3DL_DC_1A-21A_n77A These MSD issues are already covered in 3DL_DC_21A-42A_n77A These MSD issues are already covered in 3DL_21A_n77A-n79A |
|  | DC_21A_n79A | - | 2nd IMD into B42 2nd IMD into n77 | - | These MSD issues are already covered in DC_21A-42A_n79A These MSD issues are already covered in DC_21A_n77A-n79A |
| DC_1-21-42_n78-n79 | DC_1A_n78A | - | 2nd & 5th IMDs into B21 4th IMD into B42 3rd & 5th IMD into n79 | - | These MSD issues are already covered in 3DL_DC_1A-21A_n78A and 3DL_DC_1A-42A_n78A and 3DL_DC_1A_n78A-n79A with 2UL_DC_1A_n78A. |
|  | DC_1A_n79A | - | 4th IMD into B21 5th IMD into B42 5th IMD into n78 | - | The MSD issue are already covered in 3DL_DC_1A-21A_n79A and 3DL_DC_1A-42A_n79A and 3DL_DC_1A_n78A-n79A with 2UL_DC_1A_n79A |
|  | DC_21A_n78A | - | 2nd & 5th IMDs into B1 2nd & 4th IMDs into n79 | - | These MSD issue is already covered in 3DL_DC_1A-21A_n78A. These MSD issue is already covered in 3DL_DC_21A_n78A-n79A |
|  | DC_21A_n79A | - | 2nd IMD into B42 2nd IMD into n78 | - | These MSD issues are already covered in DC_21A-42A_n79A These MSD issues are already covered in DC_21A_n78A-n79A |
| DC_3-7-28_n28-n78 | DC_3A_n28A DC_3C_n28A | 2nd harmonic from B3 into n78 5th harmonic from n28 into n78 | 3rd IMD into B7 5th IMD into n78 |  | Harmonic issues are already covered in DC_3A_n78A or CA_n28A-n78A. These MSD issues are already covered in 3DL_DC_3A-7A_n28A with 2UL_DC_3A_n28A and 3DL_DC_3A_n28A-n78A with 2UL_DC_3A_n28A |
|  | DC_7A_n28A DC_7C_n28A | 5th harmonic from n28 into n78 | 2nd IMD into B3 2nd,4th IMDs into n78 | - | Harmonic issue already covered in CA_n28A-n78A These MSD issues are already covered in 3DL_DC_3A-7A_n28A with 2UL_DC_7A_n28A and 3DL_DC_7A_n28A-n78A with 2UL_DC_7A_n28A |
|  | DC_28A_n28A | - | 5th IMD into n78 | - | The MSD issue will be covered in 3DL_DC_28A_n28A-n78A with 2UL_DC_28A_n28A |
|  | DC_3A_n78A DC_3C_n78A | - | - | - | No issue |
|  | DC_7A_n78A DC_7C_n78A | - | 3rd & 4th IMDs into B3 2nd & 5th IMDs into B28 or n28 | - | The MSD issue will be covered in 3DL_DC_3A-7A_n78A with 2UL_DC_7A_n78A and 3DL_DC_7A_n28A-n78A with 2UL_DC_7A_n78A |
|  | DC_28A_n78A | - | 3rd IMD into B3 2nd IMD into B7 5th IMD into n28 |  | The MSD issue will be covered in 3DL_DC_3A-28A_n78A and 3DL_DC_7A-28A_n78A and 3DL_DC_28A_n28A-n78A with 2UL_DC_28A_n78A |
| DC_19-21-42_n77-n78 | DC_19A_n77A | 4th harmonic from B19 into n78 | 4th IMD into B21 2nd IMD into n78 | - | 4th harmonic problem already covered in DC_19A_n78A These MSD issues are already covered in 3DL_DC_19A-21A_n77A and 3DL_DC_19A_n77A-n78A with 2UL_DC_19A_n77A. |
|  | DC_19A_n78A | 4th harmonic from B19 into n77 | 2nd & 5th IMDs into n77 | - | 4th harmonic problem already covered in DC_19A_n77A These MSD issues are already covered in 3DL_DC_19A_n77A-n78A with 2UL_DC_19A_n78A. |
| DC_19-21-42_n77-n79 | DC_19A_n77A | 6th harmonic from B19 into n79 | 4th IMD into B21 2nd, 3rd , 4th & 5th IMDs into n79 | - | 6th harmonic problem already covered in DC_19A_n79A These MSD issues are already covered in 3DL_DC_19A-21A_n77A and 3DL_DC_19A_n77A-n79A with 2UL_DC_19A_n77A. |
|  | DC_19A_n79A | 4th harmonic from B19 into n77 | 5th IMD into B21 2nd IMD into B42 2nd & 3rd IMDs into n77 | - | 4th harmonic will be covered in DC_19A_n77A The MSD issue will be covered in 3DL_DC_19A-21A_n79A and 3DL_DC_19A-42A_n79A and 3DL_DC_19A_n77A-n79A with 2UL_DC_19A_n79A |
|  | DC_21A_n77A | - | 3rd & 4th IMDs into B19 5th IMD into B42 2nd & 4th IMDs into n79 | - | These MSD issues are already covered in 3DL_DC_19A-21A_n77A These MSD issues are already covered in 3DL_DC_21A-42A_n77A These MSD issues are already covered in 3DL_21A_n77A-n79A |
|  | DC_21A_n79A | - | 5th IMD into B19 2nd IMD into B42 2nd IMD into n77 | - | These MSD issues are already covered in DC_19A-21A_n79A These MSD issues are already covered in DC_21A-42A_n79A These MSD issues are already covered in DC_21A_n77A-n79A |
| DC_19-21-42_n78-n79 | DC_19A_n78A | 6th harmonic from B19 into n79 | 2nd, 3rd, 4th & 5th IMD into n79 | - | 6th harmonic problem already covered in DC_19A_n79A These MSD issues are already covered in 3DL_DC_19A_n78A-n79A with 2UL_DC_19A_n78A. |
|  | DC_19A_n79A | 4th harmonic from B19 into n78 | 5th IMD into B21 2nd IMD into B42 2nd & 3rd IMDs into n78 | - | 4th harmonic will be covered in DC_19A_n78A The MSD issue will be covered in 3DL_DC_19A-21A_n79A and 3DL_DC_19A-42A_n79A and 3DL_DC_19A_n78A-n79A with 2UL_DC_19A_n79A |
|  | DC_21A_n78A | - | 3rd & 4th IMDs into B19 2nd & 4th IMDs into n79 | - | These MSD issue is already covered in DC_19A-21A_n78A. These MSD issue is already covered in DC_21A_n78A-n79A |
|  | DC_21A_n79A | - | 5th IMD into B19 2nd IMD into B42 2nd IMD into n78 | - | These MSD issues are already covered in DC_19A-21A_n79A These MSD issues are already covered in DC_21A-42A_n79A These MSD issues are already covered in DC_21A_n78A-n79A |
| DC_1-3-7_n5-n78 | DC_1A_n5A | 4th harmonic from n5 into n78 | 3rd IMD into n78 | - | - 4th harmonic already covered in Table 7.3B.2.3.1-1 in TS38.101-3 - The 3rd IMD issue will be covered in DC_1_n5A-n78A |
|  | DC_3A_n5A, DC_3C_n5A | 2nd harmonic from B3 into n78 | 2nd & 3rd IMDs into B7 3rd & 5th IMDs into n78 | - | - 2nd harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - These IMD issue will be covered in DC_3A-7A_n5A These IMD issue will be covered in DC_3A_n5A-n78A |
|  | DC_7A_n5A, DC_7C_n5A | - | 2nd & 4th IMDs into n78 | - | These IMDs problem will be covered in DC_7A_n5A-n78A |
|  | DC_1A_n78A | - | 2nd IMD into B3 5th IMD into n5 4th IMD into B7 | - | The MSD issue will be covered in 3DL_DC_1A-3A_n78A. The MSD issue will be covered in 3DL_DC_1A_n5A-n78A. The MSD issue will be covered in 3DL_DC_1A-7A_n78A. |
|  | DC_3A_n78A, DC_3C_n78A | - | 5th IMD into B1 | - | The MSD issue will be covered in 3DL_1A-3A_n78A. |
|  | DC_7A_n78A, DC_7C_n78A |  | 4th IMD into B1 3rd & 4th IMDs into B3 2nd & 5th IMDs into n5 | - | The MSD issue will be covered in DC_1A-7A_n78A. The MSD issue will be covered in DC_3A-7A_n78A. The MSD issue will be covered in DC_7A_n5A-n78A. |
| DC_1-3-28_n5-n78 | DC_1A_n5A | 4th harmonic from n5 into n78 | 3rd IMD into n78 | - | - 4th harmonic already covered in Table 7.3B.2.3.1-1 in TS38.101-3 - The 3rd IMD issue will be covered in DC_1_n5A-n78A |
|  | DC_3A_n5A, DC_3C_n5A | 2nd harmonic from B3 into n78 | 4th IMD into B28 3rd & 5th IMDs into n78 | - | - 2nd harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - The IMD issue will be covered in DC_3A-28A_n5A These IMD issue will be covered in DC_3A_n5A-n78A |
|  | DC_28A_n5A | 3rd harmonic from B28 into B1 4th harmonic from n5 into n78 5th harmonic from B28 into n78 | 5th IMD into B1 4th IMD into B3 |  | - These harmonic problems already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - The 5th IMD will be covered in DC_1A-28A_n5A - The 4th IMD will be covered in DC_3A-28A_n5A |
|  | DC_1A_n78A | - | 2nd IMD into B3 5th IMD into n5 5th IMD into B28 | - | The MSD issue will be covered in 3DL_DC_1A-3A_n78A. The MSD issue will be covered in 3DL_DC_1A_n5A-n78A. The MSD issue will be covered in 3DL_DC_1A-28A_n78A. |
|  | DC_3A_n78A, DC_3C_n78A | - | 5th IMD into B1 | - | The MSD issue will be covered in 3DL_1A-3A_n78A. |
|  | DC_28A_n78A | 3rd harmonic from B28 into B1 | 3rd IMD into B1 3rd IMD into B3 5th IMD into n5 | - | Harmonic issue already covered in DC_1A_n28A The MSD issue will be covered in DC_1A-28A_n78A. The MSD issue will be covered in DC_3A-28A_n78A. The MSD issue will be covered in DC_28A_n5A-n78A. |
| DC_1-7-28_n5-n78 | DC_1A_n5A | 4th harmonic from n5 into n78 | 3rd IMD into n78 | - | - 4th harmonic already covered in Table 7.3B.2.3.1-1 in TS38.101-3 - The 3rd IMD issue will be covered in DC_1_n5A-n78A |
|  | DC_7A_n5A, DC_7C_n5A | - | 3rd & 5th IMDs into B28 2nd & 4th IMDs into n78 | - | These IMDs problem will be covered in DC_7A-28A_n5A These IMDs problem will be covered in DC_7A_n5A-n78A |
|  | DC_28A_n5A | 3rd harmonic from B28 into B1 4th harmonic from n5 into n78 5th harmonic from B28 into n78 | 5th IMD into B1 5th IMD into B7 |  | - These harmonic problems already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - The 5th IMD will be covered in DC_1A-28A_n5A - The 5th IMD will be covered in DC_7A-28A_n5A |
|  | DC_1A_n78A | - | 5th IMD into n5 4th IMD into B7 5th IMD into B28 | - | The MSD issue will be covered in DC_1A_n5A-n78A. The MSD issue will be covered in DC_1A-7A_n78A The MSD issue will be covered in DC_1A-28A_n78A.. |
|  | DC_7A_n78A, DC_7C_n78A |  | 4th IMD into B1 2nd & 5th IMDs into n5 2nd & 5th IMDs into B28 | - | The MSD issue will be covered in DC_1A-7A_n78A. The MSD issue will be covered in DC_7A_n5A-n78A. The MSD issue will be covered in DC_7A-28A_n78A. |
|  | DC_28A_n78A | 3rd harmonic from B28 into B1 | 3rd IMD into B1 5th IMD into n5 2nd IMD into B7 | - | Harmonic issue already covered in DC_1A_n28A The MSD issue will be covered in DC_1A-28A_n78A. The MSD issue will be covered in DC_28A_n5A-n78A. The MSD issue will be covered in DC_7A-28A_n78A. |
| DC_3-7-28_n5-n78 | DC_3A_n5A, DC_3C_n5A | 2nd harmonic from B3 into n78 | 2nd & 3rd IMDs into B7 4th IMD into B28 3rd & 5th IMDs into n78 | - | - 2nd harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - The IMD issue will be covered in DC_3A-7A_n5A - The IMD issue will be covered in DC_3A-28A_n5A These IMD issue will be covered in DC_3A_n5A-n78A |
|  | DC_7A_n5A | - | 3rd & 5th IMDs into B28 2nd & 4th IMDs into n78 | - | These IMDs problem will be covered in DC_7A-28A_n5A These IMDs problem will be covered in DC_7A_n5A-n78A |
|  | DC_28A_n5A | 4th harmonic from n5 into n78 5th harmonic from B28 into n78 | 4th IMD into B3 5th IMD into B7 |  | - These harmonic problems already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - The 4th IMD will be covered in DC_3A-28A_n5A - The 5th IMD will be covered in DC_7A-28A_n5A |
|  | DC_3A_n78A, DC_3C_n78A | - | - | - | No issue |
|  | DC_7A_n78A |  | 3rd & 4th IMDs into B3 2nd & 5th IMDs into n5 2nd & 5th IMDs into B28 | - | The MSD issue will be covered in DC_3A-7A_n78A. The MSD issue will be covered in DC_7A_n5A-n78A. The MSD issue will be covered in DC_7A-28A_n78A. |
|  | DC_28A_n78A | - | 3rd IMD into B3 5th IMD into n5 2nd IMD into B7 | - | The MSD issue will be covered in DC_3A-28A_n78A. The MSD issue will be covered in DC_28A_n5A-n78A. The MSD issue will be covered in DC_7A-28A_n78A. |
| DC_2-7-66-n66-n78 DC_2-7-7-66-n66-n78 | DC_2A_n66A | 2nd harmonic from B2 into n78 2nd harmonic from n66 inton78 | 5th IMD into B66 2nd & 4th IMDs into n78 | - | - These harmonic problems already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - 5th IMD will be covered in DC_2A-66A_n66A - 2nd & 4th IMDs will be covered in DC_2A_n66A-n78A |
|  | DC_2A_n78A | - | 4th IMD into B66/n66 | - | 4th IMD problem same as DC_2A-66A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
|  | DC_7A_n66A | 2nd harmonic from n66 into n78 | 3rd IMD into n78 | - | The harmonic problem already covered in Table 7.3B.2.3.1-1 in TS38.101-3 3rd IMD issue will be covered in DC_7A_n66A-n78A |
|  | DC_7A_n78A | - | 4th IMD into n66 4th IMD into B2 | Yes | The 4th IMD problem same as DC_7A-66A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 4th IMD issue will be covered in DC_2A-7A_n78A |
|  | DC_66A_n66A | 2nd harmonic from B66 into n78 | 2nd & 4th IMDs into n78 |  | - Harmonic problem already covered in DC_66A_n78A - 2nd & 4th IMDs issues will be covered in DC_66A_n66A-n78A |
|  | DC_66A_n78A | - | 2nd , 4th & 5th IMDs into B2 5th IMD into n66 |  | - 2nd , 4th & 5th IMDs issue will be covered in DC_2A-66A_n78A - 5th IMD issue will be covered in DC_66A_n66A-n78A |
| DC_1-3-20_n38-n78 | DC_3A_n78A | - | 5th IMD into B1 | - | The MSD issue will be covered in 3DL_1A-3A_n78A. |
|  | DC_20A_n78A | 3rd harmonic from B20 into n38 | 3rd IMD into B1 3rd IMD into B3 2nd IMD into n38 | - | The 3rd harmonic issue will be covered in DC_20A_n38A - 3rd IMD issue will be covered in DC_1A-20A_n78A - 3rd IMD issue will be covered in DC_3A-20A_n78A - 2nd IMD issue will be covered in DC_20A_n38A-n78A |
|  | DC_3A_n38A | 2nd harmonic from B3 into n78 | 2nd & 3rd IMDs into B20 3rd IMD into n78 | - | - The harmonic problem already covered in DC_3A_n78A - 2nd & 3rd IMDs issue will be covered in DC_3A-20A_38A - 3rd IMD issue will be covered in DC_3A_n38A-n78A |
|  | DC_20A_n38A | 4th harmonic from B20 into n78 | 2nd & 4th IMDs into n78 |  | - The harmonic problem already covered in DC_20A_n78A - 2nd & 4th IMDs issue will be covered in DC_20A_n38A-n78A |
| DC_1-20-38_n3-n78 | DC_1A_n3A | 2nd harmonic from 3 into n78 | 2nd & 4th IMDs into n78 | Yes | - Harmonic problems was already covered in DC_3A_n78A. - 2nd &4th IMDs problems will be covered in DC_1A_n3A-n78A. |
|  | DC_20A_n3A | 3rd harmonic from B20 into n38 - 2nd harmonic from n3 into n78 - 4th harmonic from B20 into n78 | 2nd & 3rd IMDs into n38 3rd & 5th IMDs into n78 | - | - Harmonic problems was already covered in DC_20A_n38A or DC_20A_n78A or CA_n3A-n78A. - These IMDs problems will be covered in DC_20A_n3A-n38A - These IMDs problems will be covered in DC_20A_n3A-n78A. |
|  | DC_1A_n78A | - | 2nd IMD into n3 5th IMD into B20 4th IMD into B38 | Yes | The MSD issue will be covered in DC_1A_n3A-n78A. The MSD issue will be covered in DC_1A-20A_n78A. The MSD issue will be covered in DC_1A-38A_n78A |
|  | DC_20A_n78A | 3rd harmonic from B20 into B38 | 3rd IMD into B1 3rd IMD into n3 2nd IMD into B38 | - | The 3rd harmonic issue will be covered in CA_20A-38A - 3rd IMD issue will be covered in DC_1A-20A_n78A - 3rd IMD issue will be covered in DC_20A_n3A-n78A - 2nd IMD issue will be covered in DC_20A-38A_n78A |
| DC_1-7-20_n3-n78 | DC_1A_n3A | 2nd harmonic from 3 into n78 | 2nd & 4th IMDs into n78 | Yes | - Harmonic problems was already covered in DC_3A_n78A. - 2nd &4th IMDs problems will be covered in DC_1A_n3A-n78A. |
| DC_3-7-8_n1-n78 DC_3-3-7-8_n1-n78 DC_3-7-7-8_n1-n78 DC_3-3-7-7-8_n1-n78 | DC_3A_n1A | 2nd harmonic from B3 into n78 | 2nd & 4th IMDs into n78 | Yes | - Harmonic problems was already covered in DC_3A_n78A. - 2nd &4th IMDs problems will be covered in DC_3A_n1A-n78A. |
|  | DC_3A_n78A | - | 5th IMD into n1 | - | The MSD issue will be covered in 3DL_3A_n1A-78A. |
|  | DC_7A_n1A | - | 4th & 5th IMDs into n78 5th IMD into B8 | Yes | - These IMD problems were already covered in DC_7A_n1A-n78A with 2UL_DC_7A_n1A - 5th IMD issue will be covered in DC_7A-8A_n1A |
|  | DC_7A_n78A | - | 4th IMD into n1 3rd & 4th IMDs into B3 2nd & 5th IMDs into B8 | Yes | - Up to 7th harmonic order will be not any impact to n257 - 4th IMD problem will be covered in DC_7A_n1A-n78A 3rd & 4th IMDs issue will be covered in DC_3A-7A_n78A 2nd & 5th IMDs issue will be covered in DC_7A-8A_n78A |
|  | DC_8A_n1A | 2nd harmonic from B8 into B3 3rd harmonic from B8 into B7 4th harmonic from B8 into n78 | 3rd IMD into n78 | - | - The harmonic problem already covered in CA_3A-8A or CA_7A-8A or DC_8A_n78A - 3rd IMD issue will be covered in DC_8A_n1A-n78A |
|  | DC_8A_n78A | 2nd harmonic from B8 into B3 3rd harmonic from B8 into B7 | 3rd IMD into B3 2nd IMD into B7 | -Yes | - Harmonic problem already be solved in CA_3A_8A or CA_7A-8A - 3rd IMD already covered in DC_3A-8A_n78A - 2nd IMD issue will be covered in DC_7A-8A_n78A |
| DC_1-3-41_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 2nd IMD into B3 4th IMD into B41 | Yes | - Up to 7th harmonic order will be not any impact to n257 - The IMD problem will be covered in DC_1A-3A_n78A - The IMD problem will be covered in DC_1A-41A_n78A |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | 2nd harmonic from B1 into n78 | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_3A_n78A | 7th & 8th harmonics from n78 into n257 | 5th IMD into B1 | - | - Up to 7th harmonic order will be not any impact to n257 - 5th IMD problems will be covered in DC_1A-3A_n78A. |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_41A_n78A | 7th & 8th harmonics from n78 into n257 | 4th IMD into B1 3rd & 4th IMDs into B3 | - | - No harmonic impact to n257 - 4th IMD issue will be covered in DC_1A-41A_n78A - 3rd & 4th IMDs issue will be covered in DC_3A-41A_n78A |
|  | DC_41A_n257A, DC_41A_n257G, DC_41A_n257H, DC_41A_n257I | - | - | - | No issue |
| DC_1-3-42_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 2nd IMD into B3 4th IMD into B42 | Yes | - Up to 7th harmonic order will be not any impact to n257 - The IMD problem will be covered in DC_1A-3A_n78A - The IMD problem will be covered in DC_1A-42A_n78A |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | 2nd harmonic from B1 into n78 | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_3A_n78A | 2nd harmonic from B3 into B42 7th & 8th harmonics from n78 into n257 | 5th IMD into B1 4th & 5th IMDs into B42 | - | - 2nd harmonic already covered in CA_3A-42A and Up to 7th harmonic order will be not any impact to n257 - 5th IMD issue will be covered in DC_1A-3A_n78A - These MSD issues are already covered in 3DL_DC_3A-42A_n78A |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - | - | - | No issue |
| DC_1-41-42_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 4th IMD into B41 4th IMD into B42 | Yes | - Up to 7th harmonic order will be not any impact to n257 - The IMD problem will be covered in DC_1A-41A_n78A - The IMD problem will be covered in DC_1A-42A_n78A |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | 2nd harmonic from B1 into n78 | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_41A_n78A | 7th & 8th harmonics from n78 into n257 | 4th IMD into B1 | - | - No harmonic impact to n257 - 4th IMD issue will be covered in DC_1A-41A_n78A |
|  | DC_41A_n257A, DC_41A_n257G, DC_41A_n257H, DC_41A_n257I | - | - | - | No issue |
|  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - | - | - | No issue |
| DC_3-41-42_n78-n257 | DC_3A_n78A | 2nd harmonic from B3 into B42 7th & 8th harmonics from n78 into n257 | 4th & 5th IMDs into B42 | - | - 2nd harmonic already covered in CA_3A-42A and Up to 7th harmonic order will be not any impact to n257 - These MSD issues are already covered in 3DL_DC_3A-42A_n78A |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_41A_n78A | 7th & 8th harmonics from n78 into n257 | 3rd & 4th IMDs into B3 | - | - No harmonic impact to n257 - 3rd & 4th IMDs issue will be covered in DC_3A-41A_n78A |
|  | DC_41A_n257A, DC_41A_n257G, DC_41A_n257H, DC_41A_n257I | - | - | - | No issue |
|  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - | - | - | No issue |
| DC_1-3-18_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 2nd IMD into B3 5th IMD into B18 | Yes | - Up to 7th harmonic order will be not any impact to n257 - The IMD problem will be covered in DC_1A-3A_n78A - The IMD problem will be covered in DC_1A-18A_n78A |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | 2nd harmonic from B1 into n78 | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_3A_n78A | 7th & 8th harmonics from n78 into n257 | 5th IMD into B1 | - | - Up to 7th harmonic order will be not any impact to n257 - 5th IMD issue will be covered in DC_1A-3A_n78A |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_18A_n78A | 7th & 8th harmonics from n78 into n257 | 3rd IMD into B1 3rd IMD into B3 | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 3rd IMD issue will be covered in DC_1A-18A_n78A - 3rd IMD issue will be covered in DC_3A-18A_n78A |
|  | DC_18A_n257A, DC_18A_n257G, DC_18A_n257H, DC_18A_n257I | - | - | - | No issue |
| DC_1-18-42_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 5th IMD into B18 4th IMD into B42 | Yes | - Up to 7th harmonic order will be not any impact to n257 - The IMD problem will be covered in DC_1A-18A_n78A - The IMD problem will be covered in DC_1A-42A_n78A |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | 2nd harmonic from B1 into n78 | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_18A_n78A | 7th & 8th harmonics from n78 into n257 | 3rd IMD into B1 | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 3rd IMD issue will be covered in DC_1A-18A_n78A |
|  | DC_18A_n257A, DC_18A_n257G, DC_18A_n257H, DC_18A_n257I | - | - | - | No issue |
|  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - | - | - | No issue |
| DC_3-18-42_n78-n257 | DC_3A_n78A | 2nd harmonic from B3 into B42 7th & 8th harmonics from n78 into n257 | 4th & 5th IMDs into B42 | - | - 2nd harmonic already covered in CA_3A-42A and Up to 7th harmonic order will be not any impact to n257 - These IMDs issue will be covered in DC_3A-42A_n78A |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_18A_n78A | 7th & 8th harmonics from n78 into n257 | 3rd IMD into B3 | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 3rd IMD issue will be covered in DC_3A-18A_n78A |
|  | DC_18A_n257A, DC_18A_n257G, DC_18A_n257H, DC_18A_n257I | - | - | - | No issue |
|  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - | - | - | No issue |
| DC_3-28-41_n78-n257 | DC_3A_n78A | 7th & 8th harmonics from n78 into n257 | - | - | - Up to 7th harmonic order will be not any impact to n257 |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_28A_n78A | 7th & 8th harmonics from n78 into n257 | 3rd IMD into B3 2nd IMD into B41 | - | Up to 7th harmonic order will be not any impact to n257 - 3rd IMD issue will be covered in DC_3A-28A_n78A - 2nd IMD issue will be covered in DC_28A-41A_n78A |
|  | DC_28A_n257A, DC_28A_n257G, DC_28A_n257H, DC_28A_n257I | - | - | - | No issue |
|  | DC_41A_n78A | 7th & 8th harmonics from n78 into n257 | 3rd & 4th IMDs into B3 2nd & 5th IMDs into B28 | - | - No harmonic impact to n257 - 3rd & 4th IMDs issue will be covered in DC_3A-41A_n78A - 2nd & 5th IMDs issue will be covered in DC_28A-41A_n78A |
|  | DC_41A_n257A, DC_41A_n257G, DC_41A_n257H, DC_41A_n257I | - | - | - | No issue |
| DC_3-28-42_n78-n257 | DC_3A_n78A | 2nd harmonic from B3 into B42 7th & 8th harmonics from n78 into n257 | 4th & 5th IMDs into B42 | - | - 2nd harmonic already covered in CA_3A-42A and Up to 7th harmonic order will be not any impact to n257 - These IMDs issue will be covered in DC_3A-42A_n78A |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_28A_n78A | 7th & 8th harmonics from n78 into n257 | 3rd IMD into B3 | - | Up to 7th harmonic order will be not any impact to n257 - 3rd IMD issue will be covered in DC_3A-28A_n78A |
|  | DC_28A_n257A, DC_28A_n257G, DC_28A_n257H, DC_28A_n257I | - | - | - | No issue |
|  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - | - | - | No issue |
| DC_28-41-42_n78-n257 | DC_28A_n78A | 7th & 8th harmonics from n78 into n257 | 2nd IMD into B41 | - | Up to 7th harmonic order will be not any impact to n257 - 3rd IMD issue will be covered in DC_28A-41A_n78A |
|  | DC_28A_n257A, DC_28A_n257G, DC_28A_n257H, DC_28A_n257I | - | - | - | No issue |
|  | DC_41A_n78A | 7th & 8th harmonics from n78 into n257 | 2nd & 5th IMDs into B28 | - | - No harmonic impact to n257 - 2nd & 5th IMDs issue will be covered in DC_28A-41A_n78A |
|  | DC_41A_n257A, DC_41A_n257G, DC_41A_n257H, DC_41A_n257I | - | - | - | No issue |
|  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - | - | - | No issue |
| DC_1-3-28_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 2nd IMD into B3 5th IMD into B28 | Yes | - Up to 7th harmonic order will be not any impact to n257 - The IMD problem will be covered in DC_1A-3A_n78A - The IMD problem will be covered in DC_1A-28A_n78A |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | 2nd harmonic from B1 into n78 | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_3A_n78A | 7th & 8th harmonics from n78 into n257 | 5th IMD into B1 | - | - Up to 7th harmonic order will be not any impact to n257 - The IMD issue will be covered in DC_1A-3A_n78A |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_28A_n78A | 3rd harmonic from B28 into B1 7th & 8th harmonics from n78 into n257 | 3rd IMD into B1 3rd IMD into B3 | - | - 3rd harmonic already covered in CA_1A-28A and Up to 7th harmonic order will be not any impact to n257 - 3rd IMD issue will be covered in DC_1A-28A_n78A - 3rd IMD issue will be covered in DC_3A-28A_n78A |
|  | DC_28A_n257A, DC_28A_n257G, DC_28A_n257H, DC_28A_n257I | - | - | - | No issue |
| DC_1-28-42_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 5th IMD into B28 4th IMD into B42 | - | - Up to 7th harmonic order will be not any impact to n257 - The IMD problem will be covered in DC_1A-28A_n78A - The IMD problem was not impact to B42 due to synchronous TDD operation |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | 2nd harmonic from B1 into n78 | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_28A_n78A | 3rd harmonic from B28 into B1 7th & 8th harmonics from n78 into n257 | 3rd IMD into B1 | - | - 3rd harmonic already covered in CA_1A-28A and Up to 7th harmonic order will be not any impact to n257 - 3rd IMD issue will be covered in DC_1A-28A_n78A |
|  | DC_28A_n257A, DC_28A_n257G, DC_28A_n257H, DC_28A_n257I | - | - | - | No issue |
|  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - | - | - | No issue |
| DC_1-8-11_n77-n257 | DC_1A_n77A | 7th & 8th Harmonics from n77 into n257 | 5th IMD into B8 2nd & 5th IMDs into B11 | Yes | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 5th IMDs issue will be covered in DC_1A-8A_n77A - These IMDs issue will be covered in DC_1A-11A_n77A |
|  | DC_1A_n257A | 2nd harmonic from B1 into n77 | - | - | - Harmonic problem already discussed and will be solved in DC_1A_n77A |
|  | DC_8A_n77A | 7th & 8th Harmonics from n77 into n257 | 3rd IMD into B1 3rd & 4th IMDs into B11 | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - The MSD issue will be covered in 3DL_DC_1A-8A_n77A - The MSD issue will be covered in 3DL_DC_8A-11A_n77A |
|  | DC_8A_n257A | 4th harmonic from B8 into n77 | - | - | - Harmonic problem will be solved in DC_8A_n77A |
|  | DC_11A_n77A | 7th & 8th harmonics from n77 into n257 | 2nd & 5th IMDs into B1 3rd & 4th IMDs into B8 | - | No harmonic problem to n257 - These IMDs issue will be covered in DC_1A-11A_n77A - These IMDs issue will be covered in DC_8A-11A_n77A |
|  | DC_11A_n257A | - | - | - | No issue |
| DC_2-46-66_n41-n71 | DC_2A_n41A | 2nd harmonic from n41 into B46 3rd harmonic from B2 into B46 | 4th IMD into B46 2nd & 5th IMDs into n71 | - | - These harmonic problem were covered in DC_46A_n41A or CA_2A-46A - 4th IMD issue will be covered in DC_2A-46A_n41A - 2nd & 5th IMDs issue will be covered in DC_2A_n41A-n71A |
|  | DC_2A_n71A | 4th harmonic from n71 into n41 3rd harmonic from B2 into B46 | 2nd IMD into n41 4th & 5th IMDs into B46 | - | - The harmonic problem will be treated CA_n41A_n71A or CA_2A-46A - 2nd IMD issue will be covered in DC_2A_n41A-n71A - 4th & 5th IMDs will be covered in DC_2A-46A_n71A |
|  | DC_66A_n41A | 2nd harmonic from n41 into B46 | 3rd & 4th IMDs into B46 4th IMD into B2 | - | - The harmonic was covered in DC_46A_n41A - 3rd & 4th IMDs into B46 were covered in DC_46A-66A_n41A - 4th IMD will be covered in DC_2A-66A_n41A |
|  | DC_66A_n71A | 4th harmonic from n71 into n41 3rd harmonic from B66 into B46 | 4th & 5th IMDs into B46 | - | These harmonic problem will be treated CA_n41A_n71A or CA_46A-66A The 4th & 5th IMDs were covered in DC_46A-66A_n71A |
| DC_3-7-28_n7-n78 | DC_3A_n7A | 2nd harmonic from 3 into n78 | 2nd IMD into B28 3rd IMD into n78 3rd IMD into B7 | - | - Harmonic problem was solved in DC_3A_n78A - 2nd IMD problem will be covered in DC_3A-28A_n7A. - 3rd IMD problem already covered in DC_3A_n7A-n78A. - 3rd IMD problem already covered in DC_3A_n7A. |
|  | DC_3A_n78A | - | - | - | No issue |
|  | DC_7A_n7A1 | - | - | - | No issue |
|  | DC_7A_n78A | - | 3rd & 4th IMDs into B3 2nd & 5th IMDs into B28 | - | The MSD issue will be covered in DC_3A-7A_n78A. The MSD issue will be covered in DC_7A-28A_n78A. |
|  | DC_28A_n7A | 5th harmonic from B28 into n78 | 2nd & 4th IMDs into n78 | - | - 5th harmonic issue will be covered in DC_28-n78 - 2nd & 4th IMDs will be covered in DC_28A_n7A-n78A |
|  | DC_28A_n78A | - | 3rd IMD into B3 2nd IMD into B7/n7 | - | The MSD issue will be covered in DC_3A-28A_n78A. The MSD issue will be covered in DC_7A-28A_n78A. |
| DC_1-3-28_n7-n78 | DC_1A_n7A DC_1A_n7B | - | 4th & 5th IMDs into n78 5th IMD into B28 | - | - 4th & 5th IMDs problems were already covered in DC_7_n1-n78 with 2UL_DC_7A_n1A - 5th IMD issue will be covered in DC_1A-28A_n7A |
|  | DC_1A_n78A |  | 2nd IMD into B3 5th IMD into B28 4th IMD into n7 | Yes | - 2nd IMD problem will be covered in DC_1A-3A_n78A - 5th IMD problem will be covered in DC_1A-28A_n78A - 4th IMD problem will be covered in DC_1A_n7A-n78A |
|  | DC_3A_n7A DC_3A_n7B DC_3C_n7A DC_3C_n7B | 2nd harmonic from 3 into n78 | 2nd IMD into B28 3rd IMD into n78 | - | - Harmonic problem was solved in DC_3A_n78A - 2nd IMD problem will be covered in DC_3A-28A_n7A. - 3rd IMD problem already covered in DC_3A_n7A-n78A. |
|  | DC_3A_n78A DC_3C_n78A | - | 5th IMD into B1 | - | - The IMD issue will be covered in DC_1A-3A_n78A |
|  | DC_28A_n7A DC_28A_n7B | 3rd harmonic from B28 into B1 5th harmonic from B28 into n78 | 2nd & 4th IMDs into n78 | - | - 3rd harmonic issue will be covered in CA_1A-28A - 5th harmonic issue will be covered in DC_28-n78 - 2nd & 4th IMDs will be covered in DC_28A_n7A-n78A |
|  | DC_28A_n78A | - | 3rd IMD into B3 2nd IMD into n7 | - | The MSD issue will be covered in DC_3A-28A_n78A. The MSD issue will be covered in DC_7A-28A_n78A. |
| DC_1-3-7_n7-n78 | DC_1A_n7A | - | 4th & 5th IMDs into n78 | - | - 4th & 5th IMDs problems were already covered in DC_7_n1-n78 with 2UL_DC_7A_n1A |
|  | DC_1A_n78A |  | 2nd IMD into B3 4th IMD into B7/n7 | Yes | - 2nd IMD problem will be covered in DC_1A-3A_n78A - 4th IMD problem will be covered in DC_1A_n7A-n78A |
|  | DC_3A_n7A DC_3C_n7A | 2nd harmonic from 3 into n78 | 3rd IMD into B7 3rd IMD into n78 | - | - Harmonic problem was solved in DC_3A_n78A - 3rd IMD problem will be covered in DC_3A-7A_n7A. - 3rd IMD problem already covered in DC_3A_n7A-n78A. |
|  | DC_3A_n78A DC_3C_n78A | - | 5th IMD into B1 | - | - The IMD issue will be covered in DC_1A-3A_n78A |
|  | DC_7A_n7A1 | - | - | - | No issue |
|  | DC_7A_n78A | - | 4th IMD into B1 3rd & 4th IMDs into B3 | Yes | - 4th IMD problem will be covered in DC_7A_n1A-n78A 3rd & 4th IMDs issue will be covered in DC_3A-7A_n78A |
| DC_1-7-28_n7-n78 | DC_1A_n7A | - | 4th & 5th IMDs into n78 5th IMD into B28 | - | - 4th & 5th IMDs problems were already covered in DC_7_n1-n78 with 2UL_DC_7A_n1A - 5th IMD issue will be covered in DC_1A-28A_n7A |
|  | DC_1A_n78A |  | 5th IMD into B28 4th IMD into B7/n7 | Yes | - 5th IMD problem will be covered in DC_1A-28A_n78A - 4th IMD problem will be covered in DC_1A_n7A-n78A |
|  | DC_7A_n7A1 | - | - | - | No issue |
|  | DC_7A_n78A | - | 4th IMD into B1 2nd & 5th IMDs into B28 | Yes | - 4th IMD problem will be covered in DC_7A_n1A-n78A - 2nd & 5th IMDs problem will be covered in DC_7A-28A_n78A |
|  | DC_28A_n7A | 3rd harmonic from B28 into B1 5th harmonic from B28 into n78 | 2nd & 4th IMDs into n78 | - | - 3rd harmonic issue will be covered in CA_1A-28A - 5th harmonic issue will be covered in DC_28-n78 - 2nd & 4th IMDs will be covered in DC_28A_n7A-n78A |
|  | DC_28A_n78A | - | 2nd IMD into n7 | - | The MSD issue will be covered in DC_7A-28A_n78A. |
| Note1: These band combinations should be completed the dual uplink EN-DC firstly |  |  |  |  |  |

Table 5.3-4 summarizes the EN-DC band combinations with self-interference problems for 6DL/2UL DC operation.

Table 5.3-4: Summary of Self-interference analysis for LTE 4 bands & NR 2 bands DL and 2 bands UL EN-DC operation

| Downlink band configuration | Uplink DC Configuration | Harmonic relation issues |  | intermodulation to own rx band | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- | --- |
| DC_1-3-5-7_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 |  | 2nd IMD into B3 4th IMD into B7 5th IMD into B5 | Yes | - Up to 7th harmonic order will be not any impact to n257 - These IMDs problem will be covered in DC_1A-3A_n78A, DC_1A-7A_n78A and DC_1A-5A_n78A - Proximity also follow LTE_CA_1A-3A |
|  | DC_1A_n257A | 2nd harmonic from B1 into n78 |  | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_3A_n78A | 7th & 8th harmonics from n78 into n257 |  | 5th IMD into B1 | - | - Up to 7th harmonic order will be not any impact to n257 - 5th IMD problems will be covered in DC_1A-3A_n78A. |
|  | DC_3A_n257A | 2nd harmonic from B3 into n78 |  | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_5A_n78A | 7th & 8th harmonics from n78 into n257 |  | 3rd IMD into B1 3rd IMD into B3 2nd IMD into B7 | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 3rd IMD problems will be covered in DC_1A-5A_n78A, DC_3A-5A_n78A and DC_5A-7A_n78A. |
|  | DC_5A_n257A | 4th harmonic from B5 into n78 |  | - | - | - Harmonic problem will be solved in DC_5A_n78A |
|  | DC_7A_n78A | 7th harmonic from B7 into n257 |  | 3rd & 4th IMDs into B3 4th IMD into B1 2nd & 5th IMDs into B5 | - | - Up to 7th harmonic order will be not any impact to n257 - The IMD problem will be covered in DC_3A-7A_n78A, DC_1A-7A_n78A and DC_5A-7A_n78A |
|  | DC_7A_n257A | - |  | - | - | No issues |
| DC_1-3-7-28_n28-n78 | DC_1A_n28A | 5th harmonic from n28 into n78 |  | 2nd IMD into B7 3rd IMD into n78 | Yes | All desense problems are covered in lower fallback mode. |
|  | DC_3A_n28A DC_3C_n28A | 2nd harmonic from B3 into n78 3rd harmonic from n28 into B1 5th harmonic from n28 into n78 |  | 4th IMD into B1 3rd IMD into B7 5th IMD into n78 | - | All desense problems are covered in lower fallback mode. |
|  | DC_7A_n28A DC_7C_n28A | 3rd harmonic from n28 into B1 5th harmonic from n28 into n78 |  | 2nd IMD into B3 2nd,4th IMDs into n78 | - | All desense problems are covered in lower fallback mode. |
|  | DC_28A_n28A | 3rd harmonic from n28 into B1 |  | 3rd & 5th IMDs into B1 5th IMD into n78 | - | All desense problems are covered in lower fallback mode. |
|  | DC_1A_n78A | - |  | 2nd IMD into B3 4th IMD into B7 5th IMD into B28 or n28 | Yes | All desense problems are covered in lower fallback mode. |
|  | DC_3A_n78A DC_3C_n78A | - |  | 5th IMD into B1 | - | All desense problems are covered in lower fallback mode. |
|  | DC_7A_n78A DC_7C_n78A | - |  | 4th IMD into B1 3rd & 4th IMDs into B3 2nd & 5th IMDs into B28 or n28 | - | All desense problems are covered in lower fallback mode. |
|  | DC_28A_n78A | 3rd harmonic from B28 into B1 |  | 3rd IMD into B1 3rd IMD into B3 2nd IMD into B7 5th IMD into n28 | - | All desense problems are covered in lower fallback mode. |
| DC_1-3-7-28_n5-n78 | DC_1A_n5A | 4th harmonic from n5 into n78 | 3rd IMD into n78 |  | - | - 4th harmonic already covered in Table 7.3B.2.3.1-1 in TS38.101-3 - The 3rd IMD issue will be covered in DC_1_n5A-n78A |
|  | DC_3A_n5A, DC_3C_n5A | 2nd harmonic from B3 into n78 | 2nd & 3rd IMDs into B7 4th IMD into B28 3rd & 5th IMDs into n78 |  | - | - 2nd harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - The IMD issue will be covered in DC_3A-7A_n5A - The IMD issue will be covered in DC_3A-28A_n5A These IMD issue will be covered in DC_3A_n5A-n78A |
|  | DC_7A_n5A, DC_7C_n5A | - | 3rd & 5th IMDs into B28 2nd & 4th IMDs into n78 |  | - | These IMDs problem will be covered in DC_7A-28A_n5A These IMDs problem will be covered in DC_7A_n5A-n78A |
|  | DC_28A_n5A | 3rd harmonic from B28 into B1 4th harmonic from n5 into n78 5th harmonic from B28 into n78 | 5th IMD into B1 4th IMD into B3 5th IMD into B7 |  |  | - These harmonic problems already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - The 5th IMD will be covered in DC_1A-28A_n5A - The 4th IMD will be covered in DC_3A-28A_n5A - The 5th IMD will be covered in DC_7A-28A_n5A |
|  | DC_1A_n78A | - | 2nd IMD into B3 5th IMD into n5 4th IMD into B7 5th IMD into B28 |  | Yes | All desense problems are covered in lower fallback mode. |
|  | DC_3A_n78A, DC_3C_n78A | - | 5th IMD into B1 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_7A_n78A, DC_7C_n78A | - | 4th IMD into B1 3rd & 4th IMDs into B3 2nd & 5th IMDs into n5 2nd & 5th IMDs into B28 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_28A_n78A | 3rd harmonic from B28 into B1 | 3rd IMD into B1 3rd IMD into B3 5th IMD into n5 2nd IMD into B7 |  | - | All desense problems are covered in lower fallback mode. |
| DC_1-3-41-42_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 2nd IMD into B3 4th IMD into B41 4th IMD into B42 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | 2nd harmonic from B1 into n78 | - |  | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_3A_n78A | 2nd harmonic from B3 into B42 7th & 8th harmonics from n78 into n257 | 5th IMD into B1 4th & 5th IMDs into B42 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 | - |  | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_41A_n78A | 7th & 8th harmonics from n78 into n257 | 4th IMD into B1 3rd & 4th IMDs into B3 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I | - | - |  | - | No issue |
|  | DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | - | - |  | - | No issue |
| DC_1-3-18-42_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 2nd IMD into B3 5th IMD into B18 4th IMD into B42 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | 2nd harmonic from B1 into n78 | - |  | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_3A_n78A | 2nd harmonic from B3 into B42 7th & 8th harmonics from n78 into n257 | 5th IMD into B1 4th & 5th IMDs into B42 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 | - |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_18A_n78A | 7th & 8th harmonics from n78 into n257 | 3rd IMD into B1 3rd IMD into B3 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I | - | - |  | - | No issue |
|  | DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | - | - |  | - | No issue |
| DC_1-3-28-42_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 2nd IMD into B3 5th IMD into B28 4th IMD into B42 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | 2nd harmonic from B1 into n78 | - |  | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_3A_n78A | 2nd harmonic from B3 into B42 7th & 8th harmonics from n78 into n257 | 5th IMD into B1 4th & 5th IMDs into B42 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 | - |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_28A_n78A | 3rd harmonic from B28 into B1 7th & 8th harmonics from n78 into n257 | 3rd IMD into B1 3rd IMD into B3 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I | - | - |  | - | No issue |
|  | DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | - | - |  | - | No issue |
| DC_3-28-41-42_n78-n257 | DC_3A_n78A | 2nd harmonic from B3 into B42 7th & 8th harmonics from n78 into n257 | 4th & 5th IMDs into B42 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 | - |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_28A_n78A | 7th & 8th harmonics from n78 into n257 | 3rd IMD into B3 2nd IMD into B41 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I | - | - |  | - | No issue |
|  | DC_41A_n78A | 7th & 8th harmonics from n78 into n257 | 3rd & 4th IMDs into B3 2nd & 5th IMDs into B28 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I | - | - |  | - | No issue |
|  | DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | - | - |  | - | No issue |
| DC_1-3-7-28_n7-n78 | DC_1A_n7A | - | 4th & 5th IMDs into n78 5th IMD into B28 |  | Yes | - 4th & 5th IMDs problems were already covered in DC_7_n1-n78 with 2UL_DC_7A_n1A - 5th IMD issue will be covered in DC_1A-28A_n7A |
|  | DC_1A_n78A | - | 2nd IMD into B3 4th IMD into B7 5th IMD into B28 |  | Yes | All desense problems are covered in lower fallback mode. |
|  | DC_3A_n7A | 2nd harmonic from 3 into n78 | 3rd IMD into B7 2nd IMD into B28 3rd IMD into n78 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_3A_n78A | - | 5th IMD into B1 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_7A_n7A1 | - | - |  | - | No issue |
|  | DC_7A_n78A | - | 4th IMD into B1 3rd & 4th IMDs into B3 2nd & 5th IMDs into B28 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_28A_n7A | 3rd harmonic from B28 into B1 5th harmonic from B28 into n78 | 2nd & 4th IMDs into n78 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_28A_n78A | 3rd harmonic from B28 into B1 | 3rd IMD into B1 3rd IMD into B3 2nd IMD into B7 |  | - | All desense problems are covered in lower fallback mode. |
| Note1: These band combinations should be completed the dual uplink EN-DC firstly |  |  |  |  |  |  |

Based on these assumptions, we proposed the MSD test configuration and required MSD levels by self-interference as below.

Table 5.3-5: Proposed MSD test configuration for 3DL/2UL DC UE

| DC bands | UL DC | IMD |  | UL Fc (MHz) |  | UL BW (MHz) |  | UL RB # |  | DL Fc (MHz) |  | DL BW (MHz) |  | CF (dB) |  | MSD (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n3A-n78A | 1 | IMD2 | ¦fB1 +fn3¦ | 1950 | 5 |  |  | 25 |  | 2140 |  | 5 |  | 2.1 |  | N/A |  |
|  | n3 |  |  | 1750 |  | 5 |  | 25 |  | 1845 |  | 5 |  |  |  |  |  |
|  | n78 |  |  | 3700 |  | 10 |  | 52 |  | 3700 |  | 10 |  |  |  | 28.4 |  |
|  | 1 | IMD4 | ¦3*fn3 -fB1¦ | 1950 |  | 5 |  | 25 |  | 2140 |  | 5 |  | 1.2 |  | N/A |  |
|  | n3 |  |  | 1770 |  | 5 |  | 25 |  | 1865 |  | 5 |  |  |  |  |  |
|  | n78 |  |  | 3360 |  | 10 |  | 52 |  | 3360 |  | 10 |  |  |  | 11.2 |  |
|  | 1 | IMD2 | ¦fn78 -fB1¦ | 1950 |  | 5 |  | 25 |  | 2140 |  | 5 |  | 1.9 |  | N/A |  |
|  | n78 |  |  | 3780 |  | 10 |  | 52 |  | 3780 |  | 10 |  |  |  |  |  |
|  | n3 |  |  | 1735 |  | 5 |  | 25 |  | 1830 |  | 5 |  |  |  | 27.9 |  |
| DC_1A-7A_n78A | 1 | IMD4 | ¦fB7 -3*fB1¦ | 1970 |  | 5 |  | 25 |  | 2160 |  | 5 |  | 1.2 |  | N/A |  |
|  | 7 |  |  | 2520 |  | 5 |  | 25 |  | 2640 |  | 5 |  |  |  |  |  |
|  | n78 |  |  | 3390 |  | 10 |  | 52 |  | 3390 |  | 10 |  |  |  | 10.1 |  |
|  | 1 | IMD5 | ¦3*fB7 -2*fB1¦ | 1970 |  | 5 |  | 25 |  | 2160 |  | 5 |  | 0.3 |  | N/A |  |
|  | 7 |  |  | 2520 |  | 5 |  | 25 |  | 2640 |  | 5 |  |  |  |  |  |
|  | n78 |  |  | 3620 |  | 10 |  | 52 |  | 3620 |  | 10 |  |  |  | 3.8 |  |
|  | 1 | IMD4 | ¦fn78 -3*fB1¦ | 1977.5 |  | 5 |  | 25 |  | 2167.5 |  | 5 |  | 0.9 |  | N/A |  |
|  | n78 |  |  | 3305 |  | 10 |  | 52 |  | 3305 |  | 10 |  |  |  |  |  |
|  | 7 |  |  | 2507.5 |  | 5 |  | 25 |  | 2627.5 |  | 5 |  |  |  | 9.1 |  |
| DC_3A_n1A-n78A, DC_3C_n1A-n78A and DC_3A_n1A-n77A | n1 | IMD2 | ¦fn1 +fB3¦ | 1950 |  | 5 |  | 25 |  | 2140 |  | 5 |  | 2.1 |  | N/A |  |
|  | 3 |  |  | 1750 |  | 5 |  | 25 |  | 1845 |  | 5 |  |  |  |  |  |
|  | n78/n77 |  |  | 3700 |  | 10 |  | 52 |  | 3700 |  | 10 |  |  |  | 28.4 |  |
|  | n1 | IMD4 | ¦3*fB3 -fn1¦ | 1950 |  | 5 |  | 25 |  | 2140 |  | 5 |  | 1.2 |  | N/A |  |
|  | 3 |  |  | 1770 |  | 5 |  | 25 |  | 1865 |  | 5 |  |  |  |  |  |
|  | n78/n77 |  |  | 3360 |  | 10 |  | 52 |  | 3360 |  | 10 |  |  |  | 11.2 |  |
|  | 3 | IMD2 | ¦fn77 –fB3¦ | 1775 |  | 5 |  | 25 |  | 1870 |  | 5 |  | 2.4 |  | N/A |  |
|  | n77 |  |  | 3915 |  | 10 |  | 52 |  | 3915 |  | 10 |  |  |  |  |  |
|  | n1 |  |  | 1950 |  | 5 |  | 25 |  | 2140 |  | 5 |  |  |  | 31.0 |  |
|  | 3 | IMD5 | ¦2*fn78 -3*fB3¦ | 1770 |  | 5 |  | 25 |  | 1865 |  | 5 |  | 0.3 |  | N/A |  |
|  | n78/n77 |  |  | 3720 |  | 10 |  | 52 |  | 3720 |  | 10 |  |  |  |  |  |
|  | n1 |  |  | 1940 |  | 5 |  | 25 |  | 2130 |  | 5 |  |  |  | 3.5 |  |
| DC_3A_n7A-n78A | 3 | IMD3 | ¦2*fn7-fB3¦ | 1730 |  | 5 |  | 25 |  | 1825 |  | 5 |  | 1.5 |  | N/A |  |
|  | n7 |  |  | 2560 |  | 5 |  | 25 |  | 2680 |  | 5 |  |  |  |  |  |
|  | n78 |  |  | 3390 |  | 10 |  | 52 |  | 3390 |  | 10 |  |  |  | 16.1 |  |
| DC_3A_n20A-n78A | 3 | IMD3 | ¦fB3+2*fn20¦ | 1730 |  | 5 |  | 25 |  | 1825 |  | 5 |  | 1.5 |  | N/A |  |
|  | n20 |  |  | 845 |  | 5 |  | 25 |  | 804 |  | 5 |  |  |  |  |  |
|  | n78 |  |  | 3420 |  | 10 |  | 52 |  | 3420 |  | 10 |  |  |  | 16.1 |  |
|  | 3 | IMD5 | ¦3*fB3 -2*fn20¦ | 1730 |  | 5 |  | 25 |  | 1825 |  | 5 |  | 0.4 |  | N/A |  |
|  | n20 |  |  | 845 |  | 5 |  | 25 |  | 804 |  | 5 |  |  |  |  |  |
|  | n78 |  |  | 3500 |  | 10 |  | 52 |  | 3500 |  | 10 |  |  |  | 4.5 |  |
| DC_3A_n28A-n78A | 3 | IMD5 | ¦3*fB3 -2*fn28¦ | 1750 |  | 5 |  | 25 |  | 1845 |  | 5 |  | 0.8 |  | N/A |  |
|  | n28 |  |  | 743 |  | 5 |  | 25 |  | 798 |  | 5 |  |  |  |  |  |
|  | n78 |  |  | 3764 |  | 10 |  | 52 |  | 3764 |  | 10 |  |  |  | 4.5 |  |
| DC_7A-n28A-n78A | 7 | IMD2 | ¦fB7 +fB28¦ | 2565 |  | 5 |  | 25 |  | 2685 |  | 5 |  | 2.1 |  | N/A |  |
|  | n28 |  |  | 745 |  | 5 |  | 25 |  | 800 |  | 5 |  |  |  |  |  |
|  | n78 |  |  | 3310 |  | 10 |  | 52 |  | 3310 |  | 10 |  |  |  | 29.7 |  |
|  | 7 | IMD4 | ¦2*fB7 -2*fB28¦ | 2565 |  | 5 |  | 25 |  | 2685 |  | 5 |  | 1.1 |  | N/A |  |
|  | n28 |  |  | 745 |  | 5 |  | 25 |  | 800 |  | 5 |  |  |  |  |  |
|  | n78 |  |  | 3640 |  | 10 |  | 52 |  | 3640 |  | 10 |  |  |  | 10.3 |  |
|  | 7 | IMD2 | ¦fB78 -fB7¦ | 2565 |  | 5 |  | 25 |  | 2685 |  | 5 |  | 2.0 |  | N/A |  |
|  | n78 |  |  | 3365 |  | 10 |  | 52 |  | 3365 |  | 10 |  |  |  |  |  |
|  | n28 |  |  | 745 |  | 5 |  | 25 |  | 800 |  | 5 |  |  |  | 28.8 |  |
|  | 7 | IMD5 | ¦2*fB78 -3*fB7¦ | 2520 |  | 5 |  | 25 |  | 2640 |  | 5 |  | 0.7 |  | N/A |  |
|  | n78 |  |  | 3387 |  | 10 |  | 52 |  | 3387 |  | 10 |  |  |  |  |  |
|  | n28 |  |  | 731 |  | 5 |  | 25 |  | 786 |  | 5 |  |  |  | 4.0 |  |
| DC_28A-n8A-n78A | 28 | IMD4 | ¦3*fB8 +fB28¦ | 728 |  | 5 |  | 25 |  | 783 |  | 5 |  | 1.1 |  | N/A |  |
|  | n8 |  |  | 910 |  | 5 |  | 25 |  | 955 |  | 5 |  |  |  |  |  |
|  | n78 |  |  | 3458 |  | 10 |  | 52 |  | 3458 |  | 10 |  |  |  | 9.1 |  |
|  | 28 | IMD5 | ¦fB8 +4*fB28¦ | 713 |  | 5 |  | 25 |  | 768 |  | 5 |  | 0.7 |  | N/A |  |
|  | n8 |  |  | 910 |  | 5 |  | 25 |  | 955 |  | 5 |  |  |  |  |  |
|  | n78 |  |  | 3762 |  | 10 |  | 52 |  | 3762 |  | 10 |  |  |  | 3.7 |  |
|  | 28 | IMD5 | ¦fB78 -4*fB28¦ | 713 |  | 5 |  | 25 |  | 768 |  | 5 |  | 0.5 |  | N/A |  |
|  | n78 |  |  | 3787 |  | 10 |  | 52 |  | 3787 |  | 10 |  |  |  |  |  |
|  | n8 |  |  | 890 |  | 5 |  | 25 |  | 935 |  | 5 |  |  |  | 4.3 |  |
| DC_7A_n1A-n78A, DC_7C_n1A-n78A | n1 | IMD4 | ¦fB7 -3*fn1¦ | 1970 |  | 5 |  | 25 |  | 2160 |  | 5 |  | 1.2 |  | N/A |  |
|  | 7 |  |  | 2520 |  | 5 |  | 25 |  | 2640 |  | 5 |  |  |  |  |  |
|  | n78 |  |  | 3390 |  | 10 |  | 52 |  | 3390 |  | 10 |  |  |  | 10.1 |  |
|  | n1 | IMD5 | ¦3*fB7 -2*fn1¦ | 1970 |  | 5 |  | 25 |  | 2160 |  | 5 |  | 0.3 |  | N/A |  |
|  | 7 |  |  | 2520 |  | 5 |  | 25 |  | 2640 |  | 5 |  |  |  |  |  |
|  | n78 |  |  | 3620 |  | 10 |  | 52 |  | 3620 |  | 10 |  |  |  | 3.8 |  |
|  | 7 | IMD4 | ¦2*fn78 -2*fB7¦ | 2530 |  | 5 |  | 25 |  | 2650 |  | 5 |  | 0.9 |  | N/A |  |
|  | n78 |  |  | 3610 |  | 10 |  | 52 |  | 3610 |  | 10 |  |  |  |  |  |
|  | n1 |  |  | 1970 |  | 5 |  | 25 |  | 2160 |  | 5 |  |  |  | 9.0 |  |
| DC_7A_n3A-n78A | 7 | IMD3 | ¦2*fB7-fn3¦ | 2560 |  | 5 |  | 25 |  | 2680 |  | 5 |  | 1.5 |  | N/A |  |
|  | n3 |  |  | 1730 |  | 5 |  | 25 |  | 1825 |  | 5 |  |  |  |  |  |
|  | n78 |  |  | 3390 |  | 10 |  | 52 |  | 3390 |  | 10 |  |  |  | 16.1 |  |
|  | 7 | IMD3 | ¦2*fB7 -fn78¦ | 2565 |  | 5 |  | 25 |  | 2685 |  | 5 |  | 1.4 |  | N/A |  |
|  | n78 |  |  | 3310 |  | 10 |  | 52 |  | 3310 |  | 10 |  |  |  |  |  |
|  | n3 |  |  | 1725 |  | 5 |  | 25 |  | 1820 |  | 5 |  |  |  | 15.6 |  |
|  | 7 | IMD4 | ¦2*fn78 -2*fB7¦ | 2565 |  | 5 |  | 25 |  | 2685 |  | 5 |  | 0.8 |  | N/A |  |
|  | n78 |  |  | 3480 |  | 10 |  | 52 |  | 3480 |  | 10 |  |  |  |  |  |
|  | n3 |  |  | 1735 |  | 5 |  | 25 |  | 1830 |  | 5 |  |  |  | 9.2 |  |
| DC_20A_n1A-n78A | 20 | IMD3 | ¦fn1 +2*fB20¦ | 845 |  | 5 |  | 25 |  | 804 |  | 5 |  | 1.5 |  | N/A |  |
|  | n1 |  |  | 1940 |  | 5 |  | 25 |  | 2130 |  | 5 |  |  |  |  |  |
|  | n78 |  |  | 3630 |  | 10 |  | 52 |  | 3630 |  | 10 |  |  |  | 16.0 |  |
|  | 20 | IMD3 | ¦fn78 -2*fB20¦ | 835 |  | 5 |  | 25 |  | 794 |  | 5 |  | 1.3 |  | N/A |  |
|  | n78 |  |  | 3790 |  | 10 |  | 52 |  | 3790 |  | 10 |  |  |  |  |  |
|  | n1 |  |  | 1930 |  | 5 |  | 25 |  | 2120 |  | 5 |  |  |  | 15.3 |  |
| DC_20A_n3A-n78A | 20 | IMD3 | ¦2*fB20+fn3¦ | 845 |  | 5 |  | 25 |  | 804 |  | 5 |  | 1.5 |  | N/A |  |
|  | n3 |  |  | 1730 |  | 5 |  | 25 |  | 1825 |  | 5 |  |  |  |  |  |
|  | n78 |  |  | 3420 |  | 10 |  | 52 |  | 3420 |  | 10 |  |  |  | 16.1 |  |
|  | 20 | IMD5 | ¦2*fB20 -3*fn3¦ | 845 |  | 5 |  | 25 |  | 804 |  | 5 |  | 0.4 |  | N/A |  |
|  | n3 |  |  | 1730 |  | 5 |  | 25 |  | 1825 |  | 5 |  |  |  |  |  |
|  | n78 |  |  | 3500 |  | 10 |  | 52 |  | 3500 |  | 10 |  |  |  | 4.5 |  |
|  | 20 | IMD3 | ¦2*fB20-fn78¦ | 845 |  | 5 |  | 25 |  | 804 |  | 5 |  | 1.3 |  | N/A |  |
|  | n78 |  |  | 3550 |  | 10 |  | 52 |  | 3550 |  | 10 |  |  |  |  |  |
|  | n3 |  |  | 1765 |  | 5 |  | 25 |  | 1860 |  | 5 |  |  |  | 15.7 |  |
| DC_1A_n40A-n78A | 1 | IMD4 | ¦3*fB1-fn40¦ | 1930 |  |  | 5 |  | 25 |  | 2120 |  | 5 | 1.0 | N/A |  |
|  | n40 |  |  | 2340 |  |  | 5 |  | 25 |  | 2340 |  | 5 |  |  |  |
|  | n78 |  |  | 3450 |  |  | 10 |  | 50 |  | 3450 |  | 10 |  | 9.8 |  |
|  | 1 | IMD5 | ¦2*fB1 -3*fn40¦ | 1925 |  |  | 5 |  | 25 |  | 2115 |  | 5 | 0.4 | N/A |  |
|  | n40 |  |  | 2390 |  |  | 5 |  | 25 |  | 2390 |  | 5 |  |  |  |
|  | n78 |  |  | 3320 |  |  | 10 |  | 50 |  | 3320 |  | 10 |  | 4.2 |  |
|  | 1 | IMD4 | ¦3*fB1 -fn78¦ | 1960 |  |  | 5 |  | 25 |  | 2150 |  | 5 | 0.8 | N/A |  |
|  | n78 |  |  | 3520 |  |  | 10 |  | 50 |  | 3520 |  | 10 |  |  |  |
|  | n40 |  |  | 2360 |  |  | 5 |  | 25 |  | 2360 |  | 5 |  | 10.6 |  |
| DC_3A_n1A-n79A | 3 | IMD5 | ¦4*fB3-fn1¦ | 1720 |  |  | 5 |  | 25 |  | 1815 |  | 5 | 0.5 | N/A |  |
|  | n1 |  |  | 1930 |  |  | 5 |  | 25 |  | 2120 |  | 5 |  |  |  |
|  | n79 |  |  | 4950 |  |  | 40 |  | 216 |  | 4950 |  | 40 |  | 4.7 |  |
|  | 3 | IMD5 | ¦4*fB3 -fn79¦ | 1750 |  |  | 5 |  | 25 |  | 1845 |  | 5 | 0.5 | N/A |  |
|  | n79 |  |  | 4860 |  |  | 40 |  | 216 |  | 4860 |  | 10 |  |  |  |
|  | n1 |  |  | 1950 |  |  | 5 |  | 25 |  | 2140 |  | 5 |  | 3.6 |  |
| DC_3A_n40A-n78A | 3 | IMD5 | ¦2*fB3-3*fn40¦ | 1730 |  |  | 5 |  | 25 |  | 1825 |  | 5 | 0.5 | N/A |  |
|  | n40 |  |  | 2360 |  |  | 5 |  | 25 |  | 2360 |  | 5 |  |  |  |
|  | n78 |  |  | 3620 |  |  | 10 |  | 50 |  | 3620 |  | 10 |  | 4.8 |  |
|  | 3 | IMD5 | ¦3*fB3 -2*fn78¦ | 1720 |  |  | 5 |  | 25 |  | 1815 |  | 5 | 0.5 | N/A |  |
|  | n78 |  |  | 3760 |  |  | 10 |  | 50 |  | 3760 |  | 10 |  |  |  |
|  | n40 |  |  | 2360 |  |  | 5 |  | 25 |  | 2360 |  | 5 |  | 4.4 |  |
| DC_66A_n25A-n41A | 66 | IMD4 | ¦2*fB66 -2*fn41¦ | 1715 |  |  | 5 |  | 25 |  | 2115 |  | 5 | 1.1 | N/A |  |
|  | n41 |  |  | 2685 |  |  | 10 |  | 50 |  | 2685 |  | 10 |  |  |  |
|  | n25 |  |  | 1860 |  |  | 5 |  | 25 |  | 1940 |  | 5 |  | 11.0 |  |
| DC_40A_n41A-n79A | 40 | IMD2 | ¦fB40+fn41¦ | 2340 |  |  | 5 |  | 25 |  | 2340 |  | 5 |  |  | N/A |  |
|  | n41 |  |  | 2600 |  |  | 10 |  | 50 |  | 2600 |  | 10 |  |  |  |  |
|  | n79 |  |  | 4940 |  |  | 40 |  | 216 |  | 4940 |  | 40 |  |  | 30.5 |  |
| DC_8A_n3A-n28A | 8 | IMD2 | ¦fB8-fn3¦ | 912.5 |  |  | 5 |  | 25 |  | 957.5 |  | 5 |  |  | N/A |  |
|  | n3 |  |  | 1712.5 |  |  | 5 |  | 25 |  | 1807.5 |  | 5 |  |  |  |  |
|  | n28 |  |  | 745 |  |  | 5 |  | 25 |  | 800 |  | 5 |  |  | 30.4 |  |
|  | 8 | IMD5 | ¦3*fB8 -2*fn3¦ | 910 |  |  | 5 |  | 25 |  | 955 |  | 5 |  |  | N/A |  |
|  | n3 |  |  | 1750 |  |  | 5 |  | 25 |  | 1845 |  | 5 |  |  |  |  |
|  | n28 |  |  | 715 |  |  | 5 |  | 25 |  | 770 |  | 5 |  |  | 2.6 |  |
| DC_8A_n1A-n78A | 8 | IMD3 | ¦2*fB8+fn1¦ | 900 |  |  | 5 |  | 25 |  | 945 |  | 5 |  |  | N/A |  |
|  | n1 |  |  | 1945 |  |  | 5 |  | 25 |  | 2135 |  | 5 |  |  |  |  |
|  | n78 |  |  | 3745 |  |  | 10 |  | 50 |  | 3745 |  | 10 |  |  | 14.9 |  |
| DC_2A_n41A-n71A | 2 | IMD2 | ¦fB2-fn41¦ | 1900 |  |  | 5 |  | 25 |  | 1980 |  | 5 |  |  | N/A |  |
|  | n41 |  |  | 2530 |  |  | 10 |  | 50 |  | 2530 |  | 10 |  |  |  |  |
|  | n71 |  |  | 676 |  |  | 5 |  | 50 |  | 630 |  | 5 |  |  | 28.7 |  |
|  | 2 | IMD5 | ¦3*fB2 -2*fn41¦ | 1900 |  |  | 5 |  | 25 |  | 1980 |  | 5 |  |  | N/A |  |
|  | n41 |  |  | 2530 |  |  | 10 |  | 50 |  | 2530 |  | 10 |  |  |  |  |
|  | n71 |  |  | 686 |  |  | 5 |  | 50 |  | 640 |  | 5 |  |  | 3.8 |  |
|  | 2 | IMD2 | ¦fB2+fn71¦ | 1900 |  |  | 5 |  | 25 |  | 1980 |  | 5 |  |  | N/A |  |
|  | n41 |  |  | 2586 |  |  | 10 |  | 50 |  | 2586 |  | 10 |  |  | 29.2 |  |
|  | n71 |  |  | 686 |  |  | 5 |  | 50 |  | 640 |  | 5 |  |  | N/A |  |
| DC_1A-n3A_n28A | 1 | IMD5 | ¦2*fB1 -3*fn28¦ | 1975 |  |  | 5 |  | 25 |  | 2165 |  | 5 |  |  | N/A |  |
|  | n28 |  |  | 710.5 |  |  | 5 |  | 25 |  | 765.5 |  | 5 |  |  |  |  |
|  | n3 |  |  | 1723.5 |  |  | 5 |  | 25 |  | 1818.5 |  | 5 |  |  | 4.0 |  |
| DC_39A_n41A-n79A | 39 | IMD2 | ¦fB39+fn41¦ | 1900 |  |  | 5 |  | 25 |  | 1900 |  | 5 |  |  | N/A |  |
|  | n41 |  |  | 2620 |  |  | 10 |  | 50 |  | 2620 |  | 10 |  |  |  |  |
|  | n79 |  |  | 4520 |  |  | 40 |  | 216 |  | 4520 |  | 40 |  |  | 29.8 |  |
|  | 39 | IMD5 | ¦4*fB39 -fn41¦ | 1885 |  |  | 5 |  | 25 |  | 1885 |  | 5 |  |  | N/A |  |
|  | n41 |  |  | 2660 |  |  | 10 |  | 50 |  | 2660 |  | 10 |  |  |  |  |
|  | n79 |  |  | 4880 |  |  | 40 |  | 216 |  | 4880 |  | 40 |  |  | 4.4 |  |
|  | 39 | IMD2 | ¦fB39 -fn79¦ | 1900 |  |  | 5 |  | 25 |  | 1900 |  | 5 |  |  | N/A |  |
|  | n79 |  |  | 4520 |  |  | 40 |  | 216 |  | 4520 |  | 40 |  |  |  |  |
|  | n41 |  |  | 2620 |  |  | 10 |  | 50 |  | 2620 |  | 10 |  |  | 30.2 |  |
|  | 39 | IMD5 | ¦4*fB39 -fn79¦ | 1885 |  |  | 5 |  | 25 |  | 1885 |  | 5 |  |  | N/A |  |
|  | n79 |  |  | 4880 |  |  | 40 |  | 216 |  | 4880 |  | 40 |  |  |  |  |
|  | n41 |  |  | 2660 |  |  | 10 |  | 50 |  | 2660 |  | 10 |  |  | 4.3 |  |
| DC_12A_n7A-n78A | 12 | IMD4 | ¦2*fB12 -2*fn7¦ | 708 |  |  | 5 |  | 25 |  | 738 |  | 5 |  |  | N/A |  |
|  | n7 |  |  | 2520 |  |  | 5 |  | 25 |  | 2640 |  | 5 |  |  |  |  |
|  | n78 |  |  | 3624 |  |  | 10 |  | 50 |  | 3624 |  | 10 |  |  | 8.4 |  |
|  | 12 | IMD2 | ¦fB12 -fn78¦ | 708 |  |  | 5 |  | 25 |  | 738 |  | 5 |  |  | N/A |  |
|  | n78 |  |  | 3360 |  |  | 10 |  | 50 |  | 3360 |  | 10 |  |  |  |  |
|  | n7 |  |  | 2542 |  |  | 5 |  | 25 |  | 2662 |  | 5 |  |  | 28.7 |  |
| DC_2A_n66A-n78A | 2 | IMD2 | ¦fB2+fn66¦ | 1880 |  |  | 5 |  | 25 |  | 1960 |  | 5 |  |  | N/A |  |
|  | n66 |  |  | 1740 |  |  | 5 |  | 25 |  | 2140 |  | 5 |  |  |  |  |
|  | n78 |  |  | 3620 |  |  | 10 |  | 50 |  | 3620 |  | 10 |  |  | 29.4 |  |
|  | 2 | IMD4 | ¦fB2 -3*fn66¦ | 1880 |  |  | 5 |  | 25 |  | 1960 |  | 5 |  |  | N/A |  |
|  | n66 |  |  | 1740 |  |  | 5 |  | 25 |  | 2140 |  | 5 |  |  |  |  |
|  | n78 |  |  | 3340 |  |  | 10 |  | 50 |  | 3340 |  | 10 |  |  | 8.9 |  |
| DC_12A_n66A-n78A | 12 | IMD5 | ¦2*fB12-3*fn66¦ | 703 |  |  | 5 |  | 25 |  | 733 |  | 5 |  |  | N/A |  |
|  | n66 |  |  | 1720 |  |  | 5 |  | 25 |  | 2120 |  | 5 |  |  |  |  |
|  | n78 |  |  | 3754 |  |  | 10 |  | 50 |  | 3754 |  | 10 |  |  | 4.1 |  |
|  | 12 | IMD3 | ¦2*fB12 -fn78¦ | 703 |  |  | 5 |  | 25 |  | 733 |  | 5 |  |  | N/A |  |
|  | n78 |  |  | 3546 |  |  | 10 |  | 50 |  | 3546 |  | 10 |  |  |  |  |
|  | n66 |  |  | 1740 |  |  | 5 |  | 25 |  | 2140 |  | 5 |  |  | 16.5 |  |
| DC_7A_n66A-n78A | 7 | IMD3 | ¦2*fB7-fn66¦ | 2542 |  |  | 5 |  | 25 |  | 2662 |  | 5 |  |  | N/A |  |
|  | n66 |  |  | 1740 |  |  | 5 |  | 25 |  | 2140 |  | 5 |  |  |  |  |
|  | n78 |  |  | 3344 |  |  | 10 |  | 50 |  | 3344 |  | 10 |  |  | 16.0 |  |
| DC_20A_n7A-n28A | 20 | IMD3 | ¦2*fB20-fn7¦ | 857 |  |  | 5 |  | 25 |  | 816 |  | 5 |  |  | N/A |  |
|  | n7 |  |  | 2512 |  |  | 5 |  | 25 |  | 2632 |  | 5 |  |  |  |  |
|  | n28 |  |  | 743 |  |  | 5 |  | 25 |  | 798 |  | 5 |  |  | 15.7 |  |
|  | 20 | IMD5 | ¦4*fB20-fn7¦ | 837 |  |  | 5 |  | 25 |  | 796 |  | 5 |  |  | N/A |  |
|  | n7 |  |  | 2555 |  |  | 5 |  | 25 |  | 2675 |  | 5 |  |  |  |  |
|  | n28 |  |  | 738 |  |  | 5 |  | 25 |  | 793 |  | 5 |  |  | 2.8 |  |
| DC_20A_n7A-n78A | 20 | IMD2 | ¦fB20+fn7¦ | 837 |  |  | 5 |  | 25 |  | 796 |  | 5 |  |  | N/A |  |
|  | n7 |  |  | 2555 |  |  | 5 |  | 25 |  | 2675 |  | 5 |  |  |  |  |
|  | n78 |  |  | 3392 |  |  | 10 |  | 50 |  | 3392 |  | 10 |  |  | 28.8 |  |
|  | 20 | IMD4 | ¦2*fB20-2*fn7¦ | 837 |  |  | 5 |  | 25 |  | 796 |  | 5 |  |  | N/A |  |
|  | n7 |  |  | 2555 |  |  | 5 |  | 25 |  | 2675 |  | 5 |  |  |  |  |
|  | n78 |  |  | 3436 |  |  | 10 |  | 50 |  | 3436 |  | 10 |  |  | 8.5 |  |
| DC_3A_n7A-n28A | 3 | IMD2 | ¦fB3-fn7¦ | 1730 |  |  | 5 |  | 25 |  | 1825 |  | 5 |  |  | N/A |  |
|  | n7 |  |  | 2518 |  |  | 5 |  | 25 |  | 2638 |  | 5 |  |  |  |  |
|  | n28 |  |  | 733 |  |  | 5 |  | 25 |  | 788 |  | 5 |  |  | 29.0 |  |
| DC_18A_n3A-n78A | 18 | IMD3 | ¦2*fB18+fn3¦ | 820 |  |  | 5 |  | 25 |  | 865 |  | 5 |  |  | N/A |  |
|  | n3 |  |  | 1750 |  |  | 5 |  | 25 |  | 1845 |  | 5 |  |  |  |  |
|  | n78 |  |  | 3390 |  |  | 10 |  | 50 |  | 3390 |  | 10 |  |  | 15.2 |  |
|  | 18 | IMD5 | ¦2*fB18 -3*fn3¦ | 820 |  |  | 5 |  | 25 |  | 865 |  | 5 |  |  | N/A |  |
|  | n3 |  |  | 1750 |  |  | 5 |  | 25 |  | 1845 |  | 5 |  |  |  |  |
|  | n78 |  |  | 3610 |  |  | 10 |  | 50 |  | 3610 |  | 10 |  |  | 4.0 |  |
| DC_3A_n40A-n79A | 3 | IMD5 | ¦4*fB3-fn40¦ | 1720 |  |  | 5 |  | 25 |  | 1815 |  | 5 |  |  | N/A |  |
|  | n40 |  |  | 2330 |  |  | 5 |  | 25 |  | 2330 |  | 5 |  |  |  |  |
|  | n79 |  |  | 4550 |  |  | 40 |  | 216 |  | 4550 |  | 40 |  |  | 4.7 |  |
|  | 3 | IMD5 | ¦4*fB3-fn79¦ | 1720 |  |  | 5 |  | 25 |  | 1815 |  | 5 |  |  | N/A |  |
|  | n79 |  |  | 4550 |  |  | 40 |  | 216 |  | 4550 |  | 40 |  |  |  |  |
|  | n40 |  |  | 2330 |  |  | 5 |  | 25 |  | 2330 |  | 5 |  |  | 3.2 |  |
| DC_3A_n41A-n79A | 3 | IMD2 | ¦fB3+fn41¦ | 1770 |  |  | 5 |  | 25 |  | 1865 |  | 5 |  |  | N/A |  |
|  | n41 |  |  | 2670 |  |  | 10 |  | 50 |  | 2670 |  | 10 |  |  |  |  |
|  | n79 |  |  | 4440 |  |  | 40 |  | 216 |  | 4440 |  | 40 |  |  | 30.8 |  |
|  | 3 | IMD5 | ¦4*fB3-fn41¦ | 1770 |  |  | 5 |  | 25 |  | 1865 |  | 5 |  |  | N/A |  |
|  | n41 |  |  | 2550 |  |  | 10 |  | 50 |  | 2550 |  | 10 |  |  |  |  |
|  | n79 |  |  | 4530 |  |  | 40 |  | 216 |  | 4530 |  | 40 |  |  | 5.2 |  |
| DC_8A_n40A-n79A | 8 | IMD4 | ¦3*fB8+fn40¦ | 885 |  |  | 5 |  | 25 |  | 930 |  | 5 |  |  | N/A |  |
|  | n40 |  |  | 2305 |  |  | 5 |  | 25 |  | 2305 |  | 5 |  |  |  |  |
|  | n79 |  |  | 4960 |  |  | 40 |  | 216 |  | 4960 |  | 40 |  |  | 10.7 |  |
|  | 8 | IMD4 | ¦3*fB8-fn79¦ | 885 |  |  | 5 |  | 25 |  | 930 |  | 5 |  |  | N/A |  |
|  | n79 |  |  | 4960 |  |  | 40 |  | 216 |  | 4960 |  | 40 |  |  |  |  |
|  | n40 |  |  | 2305 |  |  | 5 |  | 25 |  | 2305 |  | 5 |  |  | 9.2 |  |
| DC_8A_n41A-n79A | 8 | IMD3 | ¦2*fB8+fn41¦ | 910 |  |  | 5 |  | 25 |  | 955 |  | 5 |  |  | N/A |  |
|  | n41 |  |  | 2650 |  |  | 10 |  | 50 |  | 2650 |  | 10 |  |  |  |  |
|  | n79 |  |  | 4470 |  |  | 40 |  | 216 |  | 4470 |  | 40 |  |  | 16.3 |  |
|  | 8 | IMD3 | ¦2*fB8-fn79¦ | 910 |  |  | 5 |  | 25 |  | 955 |  | 5 |  |  | N/A |  |
|  | n79 |  |  | 4470 |  |  | 40 |  | 216 |  | 4470 |  | 40 |  |  |  |  |
|  | n41 |  |  | 2650 |  |  | 10 |  | 50 |  | 2650 |  | 10 |  |  | 15.5 |  |
| DC_39A_n40A-n79A | 39 | IMD4 | ¦fB39-3*fn40¦ | 1917.5 |  |  | 5 |  | 25 |  | 1917.5 |  | 5 |  |  | N/A |  |
|  | n40 |  |  | 2302.5 |  |  | 5 |  | 25 |  | 2302.5 |  | 5 |  |  |  |  |
|  | n79 |  |  | 4980 |  |  | 40 |  | 216 |  | 4980 |  | 40 |  |  | 5.8 |  |
| DC_7A-7A_n66A-n78A | 7 | IMD3 | ¦2*fB7 -fn66¦ | 2555 |  |  | 5 |  | 25 |  | 2675 |  | 5 |  |  | N/A |  |
|  | n66 |  |  | 1740 |  |  | 5 |  | 25 |  | 2140 |  | 5 |  |  |  |  |
|  | n78 |  |  | 3370 |  |  | 10 |  | 50 |  | 3370 |  | 10 |  |  | 15.1 |  |
| DC_13A_n48A-n66A | 13 | IMD3 | ¦2*fB13 -fn48¦ | 782 |  |  | 5 |  | 25 |  | 751 |  | 5 |  |  | N/A |  |
|  | n48 |  |  | 3680 |  |  | 5 |  | 25 |  | 3680 |  | 5 |  |  |  |  |
|  | n66 |  |  | 1716 |  |  | 5 |  | 25 |  | 2116 |  | 5 |  |  | TBD |  |
|  | 13 | IMD5 | ¦2*fB13 -3*fn66¦ | 782 |  |  | 5 |  | 25 |  | 751 |  | 5 |  |  | N/A |  |
|  | n66 |  |  | 1716 |  |  | 5 |  | 25 |  | 2116 |  | 5 |  |  |  |  |
|  | n48 |  |  | 3584 |  |  | 5 |  | 25 |  | 3584 |  | 5 |  |  | TBD |  |
| DC_66A_n5A-n48A | 66 | IMD5 | ¦3*fB66 -2*fn5¦ | 1750 |  |  | 5 |  | 25 |  | 2150 |  | 5 |  |  | N/A |  |
|  | n5 |  |  | 834 |  |  | 5 |  | 25 |  | 879 |  | 5 |  |  |  |  |
|  | n48 |  |  | 3582 |  |  | 5 |  | 25 |  | 3582 |  | 5 |  |  | TBD |  |
| DC_11A_n3A-n28A | 11 | IMD5 | ¦3*fB11 -2*fn3¦ | 1438 |  |  | 5 |  | 25 |  | 1486 |  | 5 |  |  | N/A |  |
|  | n3 |  |  | 1758 |  |  | 5 |  | 25 |  | 1853 |  | 5 |  |  |  |  |
|  | n28 |  |  | 743 |  |  | 5 |  | 25 |  | 798 |  | 5 |  |  | TBD |  |
| DC_1A_n8A-n40A | 1 | IMD4 | ¦2*fB1 -2*fn40¦ | 1930 |  |  | 5 |  | 25 |  | 2120 |  | 5 |  |  | N/A |  |
|  | n40 |  |  | 2395 |  |  | 5 |  | 25 |  | 2395 |  | 5 |  |  |  |  |
|  | n8 |  |  | 885 |  |  | 5 |  | 25 |  | 930 |  | 5 |  |  | TBD |  |
| DC_1A_n28A-n40A | 1 | IMD4 | ¦2*fB1 -2*fn28¦ | 1930 |  |  | 5 |  | 25 |  | 2120 |  | 5 |  |  | N/A |  |
|  | n28 |  |  | 743 |  |  | 5 |  | 25 |  | 798 |  | 5 |  |  |  |  |
|  | n40 |  |  | 2374 |  |  | 5 |  | 25 |  | 2374 |  | 5 |  |  | TBD |  |
|  | 1 | IMD4 | ¦2*fB1 -2*fn40¦ | 1930 |  |  | 5 |  | 25 |  | 2120 |  | 5 |  |  | N/A |  |
|  | n40 |  |  | 2314 |  |  | 5 |  | 25 |  | 2314 |  | 5 |  |  |  |  |
|  | n28 |  |  | 713 |  |  | 5 |  | 25 |  | 768 |  | 5 |  |  | TBD |  |
| DC_3A_n1A-n40A | 3 | IMD5 | ¦2*fB3 -3*fn1¦ | 1740 |  |  | 5 |  | 25 |  | 1835 |  | 5 |  |  | N/A |  |
|  | n1 |  |  | 1940 |  |  | 5 |  | 25 |  | 2130 |  | 5 |  |  |  |  |
|  | n40 |  |  | 2340 |  |  | 5 |  | 25 |  | 2340 |  | 5 |  |  | TBD |  |
| DC_3A_n8A-n78A | 3 | IMD3 | ¦fB3 +2*fn8¦ | 1740 |  |  | 5 |  | 25 |  | 1835 |  | 5 |  |  | N/A |  |
|  | n8 |  |  | 900 |  |  | 5 |  | 25 |  | 945 |  | 5 |  |  |  |  |
|  | n78 |  |  | 3540 |  |  | 10 |  | 50 |  | 3540 |  | 10 |  |  | TBD |  |
| DC_7A_n1A-n40A | 7 | IMD3 | ¦fB7 -2*fn40¦ | 2540 |  |  | 5 |  | 25 |  | 2660 |  | 5 |  |  | N/A |  |
|  | n40 |  |  | 2335 |  |  | 5 |  | 25 |  | 2335 |  | 5 |  |  |  |  |
|  | n1 |  |  | 1940 |  |  | 5 |  | 25 |  | 2130 |  | 5 |  |  | TBD |  |
| DC_7A_n8A-n78A | 7 | IMD2 | ¦fB7 +fn8¦ | 2555 |  |  | 5 |  | 25 |  | 2675 |  | 5 |  |  | N/A |  |
|  | n8 |  |  | 900 |  |  | 5 |  | 25 |  | 945 |  | 5 |  |  |  |  |
|  | n78 |  |  | 3455 |  |  | 10 |  | 50 |  | 3455 |  | 10 |  |  | TBD |  |
|  | 7 | IMD2 | ¦fB7 -fn78¦ | 2555 |  |  | 5 |  | 25 |  | 2675 |  | 5 |  |  | N/A |  |
|  | n78 |  |  | 3500 |  |  | 10 |  | 50 |  | 3500 |  | 10 |  |  |  |  |
|  | n8 |  |  | 900 |  |  | 5 |  | 25 |  | 945 |  | 5 |  |  | TBD |  |
| DC_2A_n7A-n78A | 2 | IMD5 | ¦2*fB2 -3*fn7¦ | 1900 |  |  | 5 |  | 25 |  | 1980 |  | 5 |  |  | N/A |  |
|  | n7 |  |  | 2525 |  |  | 5 |  | 25 |  | 2645 |  | 5 |  |  |  |  |
|  | n78 |  |  | 3775 |  |  | 10 |  | 50 |  | 3775 |  | 10 |  |  | TBD |  |
| DC_13A_n7A-n78A | 13 | IMD2 | ¦fB13 +fn7¦ | 782 |  |  | 5 |  | 25 |  | 751 |  | 5 |  |  | N/A |  |
|  | n7 |  |  | 2530 |  |  | 5 |  | 25 |  | 2650 |  | 5 |  |  |  |  |
|  | n78 |  |  | 3312 |  |  | 10 |  | 50 |  | 3312 |  | 10 |  |  | TBD |  |
|  | 13 | IMD2 | ¦fB13 -fn78¦ | 782 |  |  | 5 |  | 25 |  | 751 |  | 5 |  |  | N/A |  |
|  | n78 |  |  | 3432 |  |  | 10 |  | 50 |  | 3432 |  | 10 |  |  |  |  |
|  | n7 |  |  | 2530 |  |  | 5 |  | 25 |  | 2650 |  | 5 |  |  | TBD |  |

<End of TP>