3GPP TSG-RAN WG4 #94	R4-2001179

E-meeting, 24th February – 6th March 2020

Source:	Ericsson

Title:	Proposals for BS slot aggregation requirement

Agenda Item:	8.9.1.3

Document for:	Approval

# Introduction

At RAN4#93, a way forward on NR URLLC demodulation and CSI requirements was defined in [1].

One of the BS requirements for high reliability in [1] is to define a test cases with higher BLER and/or lower confidence level for slot aggregation feature.

In this document the simulation parameters for slot aggregation for BS are proposed.

# Discussion

In order to assess the feasibility of testing the slot aggregation with higher BLER and targeting the confidence level of 95%, we performed some different link level simulations. The proposed demodulation requirement is 10% BLER for URLLC; we think that 10% BLER is is adequate for assessing algorithm performance and does not imply a long test time. Link level simulation parameters assumptions for BS will be discussed below.

## Link level simulation parameters assumptions for BS slot aggregation

The different preliminary link level simulations performed for PUSCH slot aggregation feature. Figure 1 shows the BLER plot for PUSCH channel for 2, 4 and 8 slots aggregation vs SNR between -10 dB to +10dB, at 4GHz frequency with TDL-c channel model, 3nsec Channel delay spread, 11Hz Channel doppler, MCS was chosen as 5 where the target code rate is 379/1024, one additional DMRS is proposed and 106 slots were used in the simulations. It is clear from the simulations results in figure 1, that the higher the number of aggregated slots the better the reliability performance.

Figure 1 BLER for PUSCH channel for 2, 4 and 8 slots aggregation vs SNR

For simulation assumption parameters we propose the following, in order to cover a wide range of slot aggregation variety we propose 3 types of slots aggregation 2, 4 or potentially 8 (for FDD duplex mode). For TDD mode the URLLC coverage slot aggregation demodulation requirements will be different from the FDD mode, because with the current TDD patterns in [2] up to 4 PUSCH none consecutive slot aggregation is possible. (In fact only 2 consecutive PUSCH slots is possible, however it is possible use 2 groups of 2 consecutive slots). We propose 2 aggregated slots in the simulation assumption for TDD mode. In case RAN4 considers that a requirement for 4 or 8 slot aggregation is needed, we propose to consider in this case to set an additional requirement on 8 slot aggregation assuming FDD (as well as the TDD requirement for 2). Alternatively, RAN4 could to discuss if a different TDD format more PUSCH aggregated slots are need for the demodulation requirements in TDD. Or a third alternative is not to set requirements for more than aggregation 2. We assume one Tx antenna and two Rx antennas. We think that in practice the URLLC channel will have at least some fading and therefore we see no point to run simulations with AWGN channel but instead proposing TDL channels as for the existing PUSCH requirements. We also proposing to use one additional DMRS and the HARQ is switched OFF (since it does not make sense to configure both repetitions and HARQ). We assume that if the UE is in a coverage limited situation that needs resources to be spent on repetitions then it will not be scheduled high data rates (in that case, it would be better to schedule reduced MCS without repetition to achieve the desired coverage), hence we think it would be enough to propose a QPSK MCS. From a link budget perspective, it is unreasonable to allocate a very high number of PRBs, according to [4] the applicable data packet payload size could vary between 32 bytes and 200 bytes for different URLLC services, assuming a maximum payload size of 169 bytes and keeping the reference channel for the current RAN4 performance requirements of QPSK and code rate 193/1024 in [3], then 25 PRBs are need to meet the intermediate TBS of 169 Bytes according to [2], hence FRC G-FR1-A3-8 can be re-used with 1352 bits of payload size as in [3], consequently we proposed 25 PRBs in the simulation’s assumptions. For TDD FR2, the smallest payload size in [3] is 2664 bits (333 bytes) defined in G-FR2-A3-1 in Table A.3-7 in [3],  the 333 bytes payload size seems unnecessary large for the maximum of approximately 200 bytes payload size for URLLC services, this will necessitate development of new FRCs. All proposed simulations assumptions are listed in tables 1-4 below.

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Subcarrier spacing |  | 15kHz, 30kHz |
| Carrier Frequency |  | 3.5 GHz |
| Channel model |  | TDLB100-400 Low |
| Default TDD UL-DL pattern |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| № of Tx antennas |  | 1 |
| № of Rx antennas |  | 2 |
| No of users |  | Single user |
| № of UL scheduled layers |  | 1 |
| MCS |  | G-FR1-A3-8 |
| № of PRBs |  | 25 |
| HARQ | Maximum number of HARQ transmissions | 1 |
|  | RV sequence | 0 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0 |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | 25 RBs |
|  | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| No of slots used in simulations |  | 1 000 000 |
| № of slot aggregation for TDD |  | 2 |
| Testing metric |  | 10% BLER Target |
| Confidence Level |  | 95% |
| Geometry (BLER/SNR) |  | -10: 2: +10 dB |

Table 1: Link level simulation assumptions setup for TDD FR1 slot aggregation PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Subcarrier spacing |  | 60kHz, 120kHz |
| Carrier frequency |  | 28GHz |
| Channel model |  | TDLA30-300 Low |
| Default TDD UL-DL pattern |  | 60 kHz and 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| № of Tx antennas |  | 1 |
| № of Rx antennas |  | 2 |
| No of users |  | Single user |
| № of UL scheduled layers |  | 1 |
| MCS |  | Target code Rate 193/1024 |
| № of PRBs |  | 25 |
| HARQ | Maximum number of HARQ transmissions | 1 |
|  | RV sequence | 0 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0 |
| PT-RS |  | OFF |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | 25 RBs |
|  | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| No of slots used in simulations |  | 1 000 000 |
| № of slot aggregation for TDD |  | 2 |
| Testing metric |  | 10% BLER Target |
| Confidence Level |  | 95% |
| Geometry (BLER/SNR) |  | -10: 2: +10 dB |

Table 2: Link level simulation assumptions setup for TDD FR2 slot aggregation PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Duplex mode |  | FR1 FDD |
| Subcarrier spacing |  | 15 kHz |
| Carrier frequency |  | 2GHz |
| Channel type |  | TDLB100-400 Low |
| № of Tx antennas |  | 1 |
| № of Rx antennas |  | 2 |
| No of users |  | Single user |
| № of UL scheduled layers |  | 1 |
| MCS |  | G-FR1-A3-8 |
| № of PRBs |  | 25 |
| HARQ | Maximum number of HARQ transmissions | 1 |
|  | RV sequence | 0 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0 |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | 25 RBs |
|  | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| No of slots used in simulations |  | 1 000 000 |
| № of slot aggregation for FDD |  | 4, 8 |
| Testing metric |  | 10% BLER Target |
| Confidence Level |  | 95% |
| Geometry (BLER/SNR) |  | -10: 2: +10 dB |

Table 3: Link level simulation assumptions setup for FDD FR1 slot aggregation PUSCH

# Conclusion

We kindly ask RAN4 to agree on the following Link level simulation assumptions setup for slot aggregation PUSCH:

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Subcarrier spacing |  | 15kHz, 30kHz |
| Carrier Frequency |  | 3.5 GHz |
| Channel model |  | TDLB100-400 Low |
| Default TDD UL-DL pattern |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| № of Tx antennas |  | 1 |
| № of Rx antennas |  | 2 |
| No of users |  | Single user |
| № of UL scheduled layers |  | 1 |
| MCS |  | G-FR1-A3-8 |
| № of PRBs |  | 25 |
| HARQ | Maximum number of HARQ transmissions | 1 |
|  | RV sequence | 0 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0 |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | 25 RBs |
|  | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| No of slots used in simulations |  | 1 000 000 |
| № of slot aggregation for TDD |  | 2 |
| Testing metric |  | 10% BLER Target |
| Confidence Level |  | 95% |
| Geometry (BLER/SNR) |  | -10: 2: +10 dB |

Table 1: Link level simulation assumptions setup for TDD FR1 slot aggregation PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Subcarrier spacing |  | 60kHz, 120kHz |
| Carrier frequency |  | 28GHz |
| Channel model |  | TDLA30-300 Low |
| Default TDD UL-DL pattern |  | 60 kHz and 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| № of Tx antennas |  | 1 |
| № of Rx antennas |  | 2 |
| No of users |  | Single user |
| № of UL scheduled layers |  | 1 |
| MCS |  | Target code Rate 193/1024 |
| № of PRBs |  | 25 |
| HARQ | Maximum number of HARQ transmissions | 1 |
|  | RV sequence | 0 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0 |
| PT-RS |  | OFF |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | 25 RBs |
|  | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| No of slots used in simulations |  | 1 000 000 |
| № of slot aggregation for TDD |  | 2 |
| Testing metric |  | 10% BLER Target |
| Confidence Level |  | 95% |
| Geometry (BLER/SNR) |  | -10: 2: +10 dB |

Table 2: Link level simulation assumptions setup for TDD FR2 slot aggregation PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Duplex mode |  | FR1 FDD |
| Subcarrier spacing |  | 15 kHz |
| Carrier frequency |  | 2GHz |
| Channel type |  | TDLB100-400 Low |
| № of Tx antennas |  | 1 |
| № of Rx antennas |  | 2 |
| No of users |  | Single user |
| № of UL scheduled layers |  | 1 |
| MCS |  | G-FR1-A3-8 |
| № of PRBs |  | 25 |
| HARQ | Maximum number of HARQ transmissions | 1 |
|  | RV sequence | 0 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0 |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | 25 RBs |
|  | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| No of slots used in simulations |  | 1 000 000 |
| № of slot aggregation for FDD |  | 4, 8 |
| Testing metric |  | 10% BLER Target |
| Confidence Level |  | 95% |
| Geometry (BLER/SNR) |  | -10: 2: +10 dB |

Table 3: Link level simulation assumptions setup for FDD FR1 slot aggregation PUSCH

# References

Huawei, HiSilicon, “Way forward on NR URLLC demodulation and CSI requirements”, R4-1915867, RAN4#93, November 2019

3GPP TR 38.214, V16.2.0 “Base Station (BS) radio transmission and reception.”

3GPP TR 38.104, V16.2.0 “NR; Base Station (BS) radio transmission and reception”

3GPP TS 38.824. Study on physical layer enhancements for NR ultra-reliable and low latency case (URLLC)