3GPP TSG-RAN WG4 Meeting #94-e                                       R4-2001193

Electronic Meeting, February 24 – March 6, 2020

Agenda item:	9.16.4	Others 	[NR_n259-Core/Perf]

Source: 		NTT DOCOMO, INC.

Title: 	BS conformance requirements for Band n259

Document for:	Approval

- Introduction

In RAN4#93, the need for updates related to BS RF/Conformance for the introduction of band n259 was discussed [1]. However, the decision on conformance testing, such as test tolerance and test requirements, was postponed due to further checks by test equipment vendors. In this contribution, we provide candidate updates related to conformance testing for TS 38.141-2.

2.	Discussion

2.1.	Previous discussion

In [1, 2], the comparison among the frequency range applicable to test requirement, test system uncertainty (MU) and test tolerance (TT) was summarized as below.

Table 2: Summary of the necessity of changes for BS RF conformance specification

| Section (TS 38.141-2) | Requirement | Frequency range applicable to test requirement (NOTE 1) | Frequency range applicable to test system uncertainty (MU) for test requirements (Sec. 4.1.2) (NOTE 1) | Frequency range applicable to test tolerance (TT) for test requirements (Annex C) (NOTE 1) | Necessity to update for introduction of n259 |
| --- | --- | --- | --- | --- | --- |
| Radiated transmitter characteristics |  |  |  |  |  |
| 6.2 | Radiated transmit power | 24.15 GHz < f ≤ 29.5 GHz 37 GHz < f ≤ 40 GHz | 24.25 – 29.5 GHz 37 – 40 GHz | 24.15 GHz < f ≤ 29.5 GHz 37 GHz < f ≤ 40 GHz | Yes |
| 6.3 | OTA base station output power | 24.15 GHz < f ≤ 29.5 GHz 37 GHz < f ≤ 40 GHz | 24.25 – 29.5 GHz 37 – 40 GHz | 24.15 GHz < f ≤ 29.5 GHz 37 GHz < f ≤ 40 GHz | Yes |
| 9.4.3 | OTA total power dynamic range | Not mentioned | Not mentioned | Not mentioned | No |
| 6.5.1 | OTA transmitter OFF power | Refer to OTA transient period | 24.25 – 29.5 GHz 37 – 40 GHz | 24.15 GHz < f ≤ 29.5 GHz 37 GHz < f ≤ 40 GHz | Yes |

| Section (TS 38.141-2) | Requirement | Frequency range of operating band applicable to test requirement (NOTE 1) | Frequency range applicable to test system uncertainty (MU) for test requirements (Sec. 4.1.2) (NOTE 1) | Frequency range applicable to test tolerance (TT) for test requirements (Annex C) (NOTE 1) | Necessity to update for introduction of n259 |
| --- | --- | --- | --- | --- | --- |
| Radiated transmitter characteristics |  |  |  |  |  |
| 6.5.2 | OTA transient period | 24.15 GHz < f ≤ 29.5 GHz 37 GHz < f ≤ 40 GHz | - | - | Yes |
| 6.6.2 | OTA frequency error | Not mentioned | Not mentioned | Not mentioned | No |
| 6.6.3 | OTA modulation quality | Not mentioned | Not mentioned | Not mentioned | No |
| 6.6.4 | OTA time alignment error | Not mentioned | Not mentioned | Not mentioned | No |
| 6.7.1 | General for OTA unwanted emissions | Not mentioned (However the definition of ΔfOBUE is not enough) | - | - | Yes (Need to update the definition of ΔfOBUE) |
| 6.7.2 | OTA occupied bandwidth | Not mentioned | Not mentioned | Not mentioned | No |
| 6.7.3 | OTA Adjacent Channel Leakage Power Ratio (ACLR) | 24.25 – 33.4 GHz 37 – 52.6 GHz | 24.25 – 29.5 GHz 37 – 40 GHz | 24.15 GHz < f ≤ 29.5 GHz 37 GHz < f ≤ 40 GHz | Yes |
| 6.7.4 | OTA operating band unwanted emissions | 24.25 – 33.4 GHz 37 – 52.6 GHz | 24.25 – 29.5 GHz 37 – 40 GHz | 24.15 GHz < f ≤ 29.5 GHz 37 GHz < f ≤ 40 GHz | Yes |
| 6.7.5 | OTA transmitter spurious emissions | Not mentioned (Step frequency for Cat.B does not cover n259) | Not mentioned | Not mentioned | Need to check (Need to add n259 to Table 6.7.5.2.5.2.3-2 if necessary) |
| Radiated receiver characteristics |  |  |  |  |  |
| 7.2 | OTA sensitivity | Not mentioned |  | - | No |
| 7.3 | OTA reference sensitivity level | Not mentioned | 24.15 GHz < f ≤ 29.5 GHz 37 GHz < f ≤ 40 GHz | 24.25 GHz < f ≦ 33.4 GHz 37 GHz < f ≦ 52.6 GHz | Yes |
| 7.5.1 | OTA adjacent channel selectivity | 24.25 – 33.4 GHz 37 – 52.6 GHz | 24.15 GHz < f ≤ 29.5 GHz 37 GHz < f ≤ 40 GHz | Not mentioned | Yes |
| 7.5.2 | OTA in-band blocking | Not mentioned (However the definition of ΔfOOB is not enough) | 24.15 GHz < f ≤ 29.5 GHz 37 GHz < f ≤ 40 GHz | Not mentioned | Yes (Need to update the definition of ΔfOOB) |
| 7.6 | OTA out-of-band blocking | Not mentioned | Not mentioned | Not mentioned | No |
| 7.7 | OTA receiver spurious emissions | Not mentioned | Not mentioned | Not mentioned | No |
| 7.8 | OTA receiver intermodulation | Not mentioned | 24.15 GHz < f ≤ 29.5 GHz 37 GHz < f ≤ 40 GHz | Not mentioned | Yes |
| 7.9 | OTA in-channel selectivity | Not mentioned | 24.15 GHz < f ≤ 29.5 GHz 37 GHz < f ≤ 40 GHz | 24.25 GHz < f ≦ 33.4 GHz 37 GHz < f ≦ 52.6 GHz | Yes |
| NOTE 1: “Not mentioned” imply applicable to 24.25 – 52.6GHz. |  |  |  |  |  |

In [1, 2], we identified the need to updates TS 38.141-2 and proposed to reuse the existing FR2 test requirements, MU and TT for band n259. However, the decision was postponed as test equipment vendors further confirm the feasibility.

If there are no feasibility issues, it is necessary to make the following updates related to conformance testing in order to finalize the introduction of band n259.

2.2.	Test tolerance

Regarding “Radiated transmit power”, “OTA base station output power” and “OTA transmitter OFF power”, the same TT values can be used from 37 – 43.5 GHz as proposed in [2].

Regarding “OTA ACLR”, the common requirement is applicable from 37 – 52.6 GHz according to NOTE 4 in table 6.7.3.5.2-1, TS 38.141-2 as below. This NOTE 4 implies to use the same TT values for frequency range 37 – 52.6 GHz. Hence, the TT value for OTA ACLR shall be updated to apply from 37 – 52.7 GHz.

Table 6.7.3.5.2-1: BS type 2-O ACLR limit

| BS channel bandwidth of lowest/highest NR carrier transmitted BWChannel (MHz) | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | OTA ACLR limit (dB) |
| --- | --- | --- | --- | --- |
| 50, 100, 200, 400 | BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 25.7 (Note 3) 23.4 (Note 4) |
| NOTE 1: BWChannel and BWConfig are the BS channel bandwidth and transmission bandwidth configuration of the lowest/highest NR carrier transmitted on the assigned channel frequency. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: Applicable to bands defined within the frequency spectrum range of 24.25 – 33.4 GHz NOTE 4: Applicable to bands defined within the frequency spectrum range of 37 – 52.6 GHz |  |  |  |  |

Regarding “OTA operating band unwanted emissions”, the common requirement is applicable from 37 – 52.6 GHz according to the heading of table 6.7.4.5.2.2-2, TS 38.141-2 as below. This heading implies to use the same TT values for frequency range 37 – 52.6 GHz. Therefore, the frequency range for TT applicable to OTA operating band unwanted emissions shall be updated to apply from 37 – 52.7 GHz.

Table 6.7.4.5.2.2-2: OBUE limits applicable in the frequency range 37 GHz – 52.6 GHz

| Frequency offset of measurement filter -3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test limit | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1*BWcontiguous | 0.5 MHz f_offset < 0.1* BWcontiguous +0.5 MHz | Min(-2.3 dBm, Max(Prated,t,TRP – 30.3 dB, -9.3 dBm)) | 1 MHz |
| 0.1*BWcontiguous f < fmax | 0.1* BWcontiguous +0.5 MHz f_offset < f_ offsetmax | Min(-13 dBm, Max(Prated,t,TRP – 41 dB, -20 dBm)) | 1 MHz |
| NOTE: For non-contiguous spectrum operation within any operating band the limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. |  |  |  |

From the above, we propose to update the upper frequency range for “Radiated transmit power”, “OTA base station output power”, “OTA transmitter OFF power”, “OTA ACLR” and “OTA operating band unwanted emissions”.

Proposal 1: In order to apply to band n259, update the frequency range in the TT table as below:

- Radiated transmit power:

  - 2.0 dB (37 – 43.5 GHz) for normal condition

  - 3.3 dB (37 – 43.5 GHz) for extreme condition.

- OTA base station output power:

  - 2.4 dB (37 – 43.5 GHz)

- OTA transmitter OFF power:

  - 3.3 dB (37 – 43.5 GHz)

- OTA ACLR

  - 2.6 dB (37 – 52.6 GHz) for relative ACLR

  - 2.7 dB (37 – 52.6 GHz) for absolute ACLR

- OTA operating band unwanted emissions

  - 2.7 dB (37 – 52.6 GHz)

-----------------------------Text example of Annex for test tolerance tables -----------------------------

Table C.1-2: Derivation of test requirements (FR2 OTA transmitter tests)

| Test | Minimum requirement in TS 38.104 [2] | Test Tolerance (TTOTA) | Test requirement in the present document |
| --- | --- | --- | --- |
| 6.2 Radiated transmit power | See TS 38.104 [2], subclause 9.2 | Normal conditions: 1.7 dB, 24.25GHz < f ≦ 29.5GHz 2.0 dB, 37GHz < f ≦ 43.5GHz Extreme conditions: 3.1 dB, 24.25GHz < f ≦ 29.5GHz 3.3 dB, 37GHz < f ≦ 43.5GHz | Formula: Upper limit + TT, Lower limit – TT |
| 6.3 OTA base station output power | See TS 38.104 [2], subclause 9.3 | 2.1 dB, 24.25GHz < f ≦ 29.5GHz 2.4 dB, 37GHz < f ≦ 43.5GHz | Formula: Upper limit + TT, Lower limit – TT |
| 6.4 OTA output power dynamics | See TS 38.104 [2], subclause 9.4 | 0.4 dB | Formula: Total power dynamic range – TT |
| 6.5.1 OTA transmitter OFF power | See TS 38.104 [2], subclause 9.5.2 | 2.9 dB, 24.25GHz < f ≦ 29.5GHz 3.3 dB, 37GHz < f ≦ 43.5GHz | Formula: Minimum Requirement + TT |
| 6.6.2 OTA frequency Error | See TS 38.104 [2], subclause 9.6.1 | 12 Hz | Formula: Frequency Error limit + TT |
| 6.6.3 OTA Modulation quality (EVM) | See TS 38.104 [2], subclause 9.6.2 | 1 % | Formula: EVM limit + TT |
| 6.6.4 OTA time alignment error | See TS 38.104 [2], subclause 9.6.3 | 25 ns |  |
| 6.7.2 OTA occupied bandwidth | See TS 38.104 [2], subclause 9.7.2 | 0 Hz | Formula: Minimum Requirement + TT |
| 6.7.3 OTA Adjacent Channel Leakage Power Ratio (ACLR) | See TS 38.104 [2], subclause 9.7.3 | Relative: 2.3 dB, 24.25GHz < f ≦ 29.5GHz 2.6 dB, 37GHz < f ≦ 52.6GHz Absolute: 2.7 dB, 24.25GHz < f ≦ 29.5GHz 2.7 dB, 37GHz < f ≦ 52.6GHz | Formula: Relative limit - TT Absolute limit +TT |
| 6.7.4 OTA operating band unwanted emissions | See TS 38.104 [2], subclause 9.7.4 | 0 MHz f < 0.1*BWcontiguous 2.7 dB, 24.25GHz < f ≦ 29.5GHz 2.7 dB, 37GHz < f ≦ 52.6GHz 0.1*BWcontiguous f < fmax 0 dB | Formula: Minimum Requirement + TT |
| 6.7.5.2 General transmitter spurious emissions requirements Category A | See TS 38.104 [2], subclause 9.7.5.3.2 | 0 dB | Formula: Minimum Requirement + TT |
| 6.7.5.2 General transmitter spurious emissions requirements Category B | See TS 38.104 [2], subclause 9.7.5.3.2 | 0 dB | Formula: Minimum Requirement + TT |

------------------------------------------------------------------------------------------------------------------------------------

2.3.	Test requirement

As mentioned in section 2.2, the current frequency ranges of the requirements for “radiated transmit power”, “OTA base station output power” and “OTA transmitter transient period” requirements are not sufficient for band n259. If there are no feasibility issues, we propose using the same test requirements as existing.

Proposal 2: Update the frequency range of the test requirements for “radiated transmit power”, “OTA base station output power” and “OTA transmitter transient period” from “37 GHz < f ≤ 40 GHz” to “37 GHz < f ≤ 43.5 GHz”

-----------------------------Text example for radiated transmit power requirement-----------------------------

6.2.5	Test requirement

For each declared conformance beam direction pair, the EIRP measurement results in subclause 6.2.4.2 shall remain within the values provided in table 6.2.5-1, relative to the manufacturer's declared rated beam EIRP (D.11) value:

Table 6.2.5-1: Test requirement for radiated transmit power

|  | Normal test environment | Extreme test environment |
| --- | --- | --- |
| BS type 1-H | f ≤ 3 GHz: ± 3.3 dB | N/A |
|  | 3 GHz < f ≤ 6 GHz: ± 3.5 dB |  |
| BS type 1-O | f ≤ 3 GHz: ± 3.3 dB | f ≤ 3 GHz: ± 5.2 dB |
|  | 3 GHz < f ≤ 6 GHz: ± 3.5 dB | 3 GHz < f ≤ 4.2 GHz: ± 5.3 dB |
|  |  | 4.2 GHz < f ≤ 6 GHz: ± 5.3 dB |
| BS type 2-O | 24.15 GHz < f ≤ 29.5 GHz: ± 5.1 dB 37 GHz < f ≤ 43.5 GHz: ± 5.4 dB … | 24.15 GHz < f ≤ 29.5 GHz: ± 7.6 dB 37 GHz < f ≤ 43.5 GHz: ± 7.8 dB |

-----------------------------Text example for OTA base station output power requirement-----------------------------

6.3.5.2	BS type 2-O

The final TRP measurement result in subclause 6.3.4.2 shall remain:

-	within +5.1 dB and -5.1 dB of the manufacturer's declared rated carrier TRP Prated,c,TRP carrier frequency 24.25 GHz < f ≤ 29.5 GHz.

-	within +5.4 dB and –5.4 dB of the manufacturer's declared rated carrier TRP Prated,c,TRP for carrier frequency 37 GHz < f ≤ 43.5 GHz.

Editor’s note: more frequency divisions for the measuring accuracy may be introduced.

----------------------------Text example for OTA transmitter transient period requirement----------------------------

6.5.2.5.2	BS type 2-O

The measured mean EIRP spectral density according to subclause 6.5.2.4.2 shall be less than -33.1 + Prated,c,EIRP - Prated,c,TRP dBm/MHz for carrier frequency 24.15 GHz < f ≤ 29.5 GHz, where Prated,c,EIRP is the value declared for the reference beam direction pair (D.8) for the beam identifier (D.3) which provides the highest intended EIRP.

The measured mean EIRP spectral density according to subclause 6.5.2.4.2 shall be less than -32.7 + Prated,c,EIRP - Prated,c,TRP dBm/MHz for carrier frequency 37 GHz < f ≤ 43.5 GHz, where Prated,c,EIRP is the value declared for the reference beam direction pair (D.8) for the beam identifier (D.3) which provides the highest intended EIRP.

------------------------------------------------------------------------------------------------------------------------------------

3.	Conclusion

In this contribution, we further analyse the necessity to update TS 38.141-2 for the introduction of n259. The following proposals are obtained.

Proposal 1: In order to apply to band n259, update the frequency range in the TT table as below:

- Radiated transmit power:

  - 2.0 dB (37 – 43.5 GHz) for normal condition

  - 3.3 dB (37 – 43.5 GHz) for extreme condition.

- OTA base station output power:

  - 2.4 dB (37 – 43.5 GHz)

- OTA transmitter OFF power:

  - 3.3 dB (37 – 43.5 GHz)

- OTA ACLR

  - 2.6 dB (37 – 52.6 GHz) for relative ACLR

  - 2.7 dB (37 – 52.6 GHz) for absolute ACLR

- OTA operating band unwanted emissions

  - 2.7 dB (37 – 52.6 GHz)

Proposal 2: Update the frequency range of the test requirements for “radiated transmit power”, “OTA base station output power” and “OTA transmitter transient period” from “37 GHz < f ≤ 40 GHz” to “37 GHz < f ≤ 43.5 GHz”

References

- R4-1912448, “BS RF requirement and test tolerance for n259”, NTT DOCOMO, INC., RAN4 #92bis, October 2019

- R4-1914087, “BS measurement uncertainty and test tolerance for Band n259”, NTT DOCOMO, INC., RAN4 #93, November 2019