3GPP TSG-RAN WG4 Meeting #94-e	R4-2001195

Electronic Meeting, February 24 – March 6, 2020

Agenda item:	8.17.2.2.1	PUSCH requirements	[NR_HST-Perf]

Source: 		NTT DOCOMO, INC.

Title: 	Views on NR PUSCH for high speed

Document for:	Approval

- Introduction

In RAN4 #93, WF for NR BS high speed was approved [1]. In this contribution, we provide views on PUSCH requirements and parameters for HST up to 350km/h and 500km/h.

2.	Discussion

2.1.	Antenna configuration

In the last meeting, antenna configurations for tunnel scenarios were agreed as below:

- Antenna configuration:

  - Tunnel scenario: 1x2, FFS 1x1

    - Test setup for conducted test with 1x1 needs further discussion

    - FFS on OTA test for 1x1

Whether to introduce test for 1x1 needs to consider the testable SNR value

For tunnel scenarios, antenna configuration 1x2 has already been agreed, but 1x1 is still FFS.

In LTE, HST test with a 1x1 antenna configuration was introduced. Especially in limited space, a BS with 1Rx capability has advantage for deployment since it can reduce the size of BS compared to a BS with multiple Rx capability, and such 1Rx capability BSs are now widely deployed. Hence, a 1x1 antenna configuration is one of the typical scenarios for LTE HST. Similarly, in NR HST, a 1x1 antenna configuration should be considered. Furthermore, considering re-forming bands, i.e., the same frequency band as existing LTE, RF components, such as antennas and RF cables, etc., already deployed for LTE are expected to be reusable for NR, especially in tunnel scenarios. In such LTE to NR migration scenarios, the 1x1 requirement can help ensure performance in tunnel scenarios.

Therefore, we propose to introduce a 1x1 conducted HST test for tunnel scenarios. It would be noted that this LTE to NR migration scenario could be considered for non-AAS BSs.

Proposal 1: Introduce conducted PUSCH requirements with 1x1 antenna configuration for HST.

2.2.	Applicability

In the last meeting, it was agreed to introduce applicability rule for HST requirements, but concrete rules are FFS. In Rel.16 NR HST, two different target speeds (350km/h and 500km/h) and two different scenarios (Open space and Tunnel) will be introduced. Basically, if BS supports HST, BS vendors can declare supported speed of either "350km/h HST" or "500km/h HST". When "350km/h HST" is declared, requirements for 350km/h with both open space and tunnel scenarios shall be tested. While, when "500km/h HST" is declared, requirements for both 350km/h and 500km/h with both open space and tunnel scenarios shall be tested.

Table 1: Test cases corresponding to HST support declaration

| HST support to be declared | Requirements to be tested |  |  |  |
| --- | --- | --- | --- | --- |
|  | 350km/h |  | 500km/h |  |
|  | Open space | Tunnel | Open space | Tunnel |
| “350km/h HST” | ✔ | ✔ |  |  |
| “500km/h HST” | [✔] | [✔] | ✔ | ✔ |

If there is no difference in performance between 350km/h and 500km/h HST tests, 350km/h HST tests might be skipped. However, the configuration for 500km/h HST test is not yet stable. Therefore, we need further check for performance differences after the configuration has stabilized.

Proposal 2: Study performance difference between 350km/h and 500km/h HST test, and then whether BS supporting 500km/h HST can skip 350km/h HST test.

3.	Conclusion

In this contribution, we provide parameters for PUSCH HST requirements. The following proposals are obtained.

Proposal 1: Introduce conducted PUSCH requirements with 1x1 antenna configuration for HST.

Proposal 2: Study performance difference between 350km/h and 500km/h HST tests, and then whether BS supporting 500km/h HST can skip 350km/h HST test.

References

- R4-1915886, “Way forward on NR HST PUSCH demodulation requirements”, Huawei, RAN4 #93, November 2019