3GPP TSG-RAN WG4 Meeting #94-e	R4-2001574

Online, 24th February – 6th March, 2020

Title: 	Discussion on conditional PSCell addition/change requirements

Source: 	Huawei, HiSilicon

Agenda item:	8.3.2.3

Document for:	Discussion

Introduction

In RAN2, the conditional PSCell addition change (CPAC) has been introduced, and the following agreements have been achieved:

Agreements

1.	CPAC is defined as the UE having network configuration for initiating access to a candidate PSCell, either to consider the PSCell as suitable for SN addition or SN change including intra-SN change, based on configured condition(s).

2.	Usage of CPAC is decided by the network. The UE evaluates when the condition is valid.

3.	Support configuration of one or more candidate cells for CPAC;

o	FFS how many candidate cells (UE and network impacts should be clarified). FFS whether the number of candidate cells for CPAC different from that of CHO.

5.	 Allow having multiple triggering conditions (using “and”) for CPAC execution of a single candidate cell. Only single RS type per CPAC candidate is supported. At most two triggering quantities (e.g. RSRP and RSRQ, RSRP and SINR, etc.) can be configured simultaneously.  FFS on UE capability

6.	Define an execution condition for conditional PSCell change by the measurement identity which identifies a measurement configuration There is already an agreement for conditional PSCell addition

7.	Cell level quality is used as baseline for Conditional NR PSCell addition/change execution condition;

g.	Only single RS type (SSB or CSI-RS) per candidate PSCell is supported for PSCell change.

h.	At most two triggering quantities (e.g. RSRP and RSRQ, RSRP and SINR, etc.) can be configured simultaneously. FFS on UE capability.

i.	TTT is supported for CPAC execution condition (as per legacy configuration)

8.	No additional optimizations with multi-beam operation are introduced to improve RACH performance for conditional PSCell addition/change completion with multi-beam operation.

9.	For FR1 and FR2, leave it up to UE implementation to select the candidate PSCell if more than one candidate cell meets the triggering condition. UE may consider beam information in this.

10.	UE is not required to continue evaluating the triggering condition of other candidate PSCell(s) during conditional SN execution.

For PSCell addition:

4.	The baseline operation for CPAC procedure assumes the RRC Reconfiguration message contains SCG addition/change triggering condition(s) and the RRC configuration(s) for candidate target PSCells. The UE accesses the prepared PSCell when the relevant condition is met.

a.	Multiple candidate PSCells can be sent in either one or multiple RRC messages.

b.	As part of the CPAC configuration to be sent to the UE, the RRC container is used to carry candidate PSCell configuration, and the MN is not allowed to alter any content of the configuration from the PSCell. moreover, in case of SN change, source SN is not allowed to alter any content of the configuration from the target SN.

c.	Use add/mod list + release list to configure multiple candidate PSCells.

d.	CPAC execution condition and/or candidate PSCell configuration can be updated by modifying the existing CPAC configuration.

e.	Reuse the RRCReconfiguration/RRCConnectionReconfiguration procedure to signal CPAC configuration to UE.

FFS handling of conditional SN addition associated to the SN terminated bearer.

In this contribution, we provide the discussion on the conditional PSCell addition/change requirements.

Discussion

The legacy PSCell addition delay is defined as Tconfig_PSCell, where

Tconfig_PSCell = TRRC_delay + Tprocessing + Tsearch + T∆ + TPSCell_ DU + 2 ms

where

TRRC_delay is the RRC procedure delay as specified in TS 38.331 [2].

Tprocessing is the SW processing time needed by UE, including RF warm up period. Tprocessing = 40 ms.

Tsearch is the time for AGC settling and PSS/SSS detection. If the target cell is known, Tsearch = 0 ms. If the target cell is unknown and the target cell Ês/Iot ≥ -2dB, Tsearch = 24* Trs ms.

T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = 1*Trs ms for a known or unknown PSCell.

TPSCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TPSCell_ DU is up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in Table 8.1-1 of TS 38.213 [3].

Comparing with legacy PSCell addition delay, a measurement time Tmeasure that UE performs measurement on candidate PSCells needs to be considered into conditional PSCell addition delay. The conditional PSCell configuration has been split into 2 steps: decoding the RRC message with conditional PSCells addition execution conditions and applying the target PSCell configuration. Then, the RRC processing time shall be split into two segments TRRC_1 and TRRC_2 for two steps respectively. Since UE will perform cell detection on candidate PSCells during the measurement time, the target PSCell shall be a known cell before UE starts to execute PSCell addition. Then, the time period Tsearch defined for legacy PSCell addition delay shall not be considered. Hence, the conditional PSCell addition delay can be defined as Tconfig_CPAC.

Tconfig_CPAC = TRRC_1 + Tmeasure + TRRC_2 + Tprocessing + T∆ + TPSCell_ DU + 2 ms

For conditional PSCell change delay, the requirements for conditional PSCell addition delay can be apply.

Proposal 1: The conditional PSCell addition/change delay can be defined as Tconfig_CPAC:

Tconfig_CPAC = TRRC_1 + Tmeasure + TRRC_2 + Tprocessing + T∆ + TPSCell_ DU + 2 ms

Where

TRRC_1 is the RRC processing time for RRC reconfiguration message containing conditional PSCells addition execution conditions and candidate PSCell configuration.

Tmeasure is the time period from the time when UE successfully decoded CPAC RRC command until UE detects an event for CPAC.

TRRC_2 is the RRC processing time for applying target PSCell configuration.

Tprocessing, T∆ and TPSCell_ DU are defined same as legacy PSCell addition/change.

For legacy PSCell addition/change, the interruption specified in clause 8.2 is allowed only during the RRC reconfiguration procedure. For conditional PSCell addition/change, the interruption shall be allowed only after UE detects an event for CPAC.

Proposal 2: The interruption at conditional PSCell addition/change shall be allowed only after UE detects an event for CPAC.

Conclusions

This contribution provides discussion on the conditional PSCell addition/change requirements for NR mobility enhancements. The following are provided:

Proposal 1: The conditional PSCell addition/change delay can be defined as Tconfig_CPAC:

Tconfig_CPAC = TRRC_1 + Tmeasure + TRRC_2 + Tprocessing + T∆ + TPSCell_ DU + 2 ms

Where

TRRC_1 is the RRC processing time for RRC reconfiguration message containing conditional PSCells addition execution conditions and candidate PSCell configuration.

Tmeasure is the time period from the time when UE successfully decoded CPAC RRC command until UE detects an event for CPAC.

TRRC_2 is the RRC processing time for applying target PSCell configuration.

Tprocessing, T∆ and TPSCell_ DU are defined same as legacy PSCell addition/change.

Proposal 2: The interruption at conditional PSCell addition/change shall be allowed only after UE detects an event for CPAC.

Reference

- R4-1913436, “Way forward on NR mobility enhancement”, Intel