3GPP TSG-RAN WG4 Meeting #94-e	R4-2001907

Online, 24th February – 6th March 2020

Source:		Ericsson

Title:			Corrections related to Foffset across specifications

Agenda item:	6.8.2.3

Document for:	Agreement

# Introduction

Foffset and Foffset,RAT appear in different circumstances across the specifications. In the following we show paragraphs from different specifications, highlighting the differences.

36.141

Foffset	Frequency offset from FC_high to the upper Base Station RF Bandwidth edge or from F C,block, high to the upper sub-block edge, FC_low to the lower Base Station RF Bandwidth edge or from FC,block, low to the lower sub-block edge.

Table 5.6-2: Definition of Foffset

| Channel Bandwidth of the Lowest or Highest Carrier: BWChannel[MHz] | Foffset[MHz] |
| --- | --- |
| 5, 10, 15, 20 | BWChannel/2 |

NOTE 1:	Foffset  is calculated separately for each Base Station RF Bandwidth edge / sub-block edge.

NOTE 2:	The values of BWChannel_CA /sub-block bandwidth for UE and BS are the same if the channel bandwidths of lowest and the highest component carriers are identical.

Table 5.6-3A:  Foffset for NB-IoT standalone operation

| Lowest or Highest Carrier | Foffset |
| --- | --- |
| Standalone NB-IoT | 200 kHz |

Definition exists, values are also present,  but not the note related to alignment to the channel raster. We need to introduce the note, as in 37 and 38 series specification.

37.141

Foffset, RAT	Frequency offset from the centre frequency of the highest transmitted/received carrier to the upper Base Station RF Bandwidth edge, sub-block edge or Inter RF Bandwidth edge, or from the centre frequency of the lowest transmitted/received carrier to the lower Base Station RF Bandwidth edge, sub-block edge or Inter RF Bandwidth edge for a specific RAT.

We find a note that is not very clear:

## 4.8	MSR test configurations

The test configurations shall be constructed using the methods defined below subject to the parameters declared by the manufacturer as listed in subclause 4.7.

For test contiguous operation configurations used in receiver tests only the outermost carriers need to be generated by the test equipment. For non-contiguous operation test configurations used in receiver tests, outermost carriers for each sub-block need to be generated by the test equipment.

The applicable test models for generation of the carrier transmit test signal are defined in subclause 4.9.2.

NOTE:	In case carriers are shifted to align with the channel raster Foffset, RAT as defined in clauses 4.4.1 and 4.4.2 may be different.

Proposal to modify the note:

NOTE:	If carriers are shifted to align with the channel raster, then Foffset, RAT may be different from the values defined in clauses 4.4.1 to 4.4.3.

### 4.4.1	Band category 1 aspects (BC1)

For each BC1 band, BC1 requirements for receiver and transmitter shall apply with a frequency offset Foffset, RAT from the lowest and highest carriers to the Base Station RF Bandwidth edges and sub-block edges (if any) as defined in Table 4.4.1-1.

Table 4.4.1-1: Foffset, RAT for band category 1

| RAT | Foffset, RAT |
| --- | --- |
| 1.4, 3 MHz E-UTRA | BWChannel/2 + 200 kHz |
| 5, 10, 15, 20 MHz E-UTRA and NR | BWChannel/2 |
| UTRA FDD | 2.5 MHz |
| Standalone NB-IoT | 200 kHz |

### 4.4.2	Band category 2 aspects (BC2)

For each BC2 band, BC2 requirements for receiver and transmitter shall apply with a frequency offset Foffset, RAT from the lowest and highest carriers to the Base Station RF Bandwidth edges and sub-block edges (if any) as defined in Table 4.4.2-1.

Table 4.4.2-1: Foffset, RAT for band category 2

| RAT | Foffset, RAT |
| --- | --- |
| E-UTRA and NR | BWChannel/2 |
| UTRA FDD | 2.5 MHz |
| GSM/EDGE | 200 kHz |
| Standalone NB-IoT | 200 kHz |

### 4.4.3	Band category 3 aspects (BC3)

For each BC3 band, BC3 requirements for receiver and transmitter shall apply with a frequency offset Foffset, RAT from the lowest and highest carriers to the Base Station RF Bandwidth edges and sub-block edges (if any) as defined in Table 4.4.3-1.

Table 4.4.3-1: Foffset, RAT for band category 3

| RAT | Foffset, RAT |
| --- | --- |
| 1.4, 3 MHz E-UTRA | BWChannel /2 + 200 kHz |
| 5, 10, 15, 20 MHz E-UTRA and NR | BWChannel /2 |
| 1.28 Mcps UTRA TDD | 1 MHz |
| Standalone NB-IoT | 200 kHz |

37.145-1 and 37.145-2

Foffset, RAT	Frequency offset from the centre frequency of the highest transmitted/received carrier to the upper RF bandwidth edge, sub-block edge or inter-RF bandwidth edge, or from the centre frequency of the lowest transmitted/received carrier to the lower RF bandwidth edge, sub-block edge or inter-RF bandwidth edge for a specific RAT

The note is completely incomprehensible:

## 4.11	Test signal configurations for AAS BS

### 4.11.1	General

The test configurations shall be constructed using the methods defined below subject to the parameters declared by the manufacturer as listed in subclause 4.10.

For test contiguous spectrum operation configurations used in receiver tests only the carriers in the outermost frequency positions in the TAB connector Base Station RF Bandwidth need to be generated by the test equipment. For non-contiguous spectrum operation test configurations used in receiver tests, outermost carriers for each sub-block need to be generated by the test equipment.

The applicable test models for generation of the carrier transmit test signal are defined in subclause 4.12.2.

NOTE:	In case carriers are shifted to align with the channel raster Foffset.

There is no definition of values for Foffset,RAT as we see for MSR BS in 37.141

Throughout the document in many instances is used FOffset-RAT instead of Foffset, RAT. This needs to be corrected.

Furthermore the term FOffset , which is not defined, is used in at least two instances.

38.141-1 and 38.141-2

There is no definition of the FOffset

There is no definition for values for FOffset

The note is again confusing:

## 4.7	Test configurations

### 4.7.1	General

The test configurations shall be constructed using the methods defined below, subject to the parameters declared by the manufacturer for the supported RF configurations as listed in clause 4.6. The test configurations to use for conformance testing are defined for each supported RF configuration in clauses 4.8.3 and 4.8.4.

The applicable test models for generation of the carrier transmit test signal are defined in clause 4.9.

NOTE:	In case, carriers are shifted to align with the channel raster Foffset.

# Proposal

We propose to agree on the following list of modifications to be implemented across the specifications:

| 36.141 | Need to add the note regarding alignment to channel raster |
| --- | --- |
| 37.141 | Correct the note regarding alignment to channel raster. Final text to be agreed offline. |
| 37.145-x | Add table with definition of values Foffset-RAT Replace Foffset-RAT Correct the note to align with 37.141 |
| 38.141-x | Add definition Foffset Add table with the value for Foffset Correct the note to align with 37.141 |