3GPP TSG-RAN WG4 Meeting #94-e	R4-2002016

Online, 24th February – 6th March 2020

Source:	Ericsson, T-Mobile US

Title:	TP to TR 37.716.21-21: Addition of DC configurations for DC_66_n25-n71

Agenda item:	9.7.2

Document for:	Approval

# Background

This text proposal for TR 37.716-21-21 to add DC_66_n25-n71 configurations as defined in WID [1].

# Text Proposal

##### ---Start of changes---

## 6.x	DC_66_n25-n71

### 6.x.1	Operating bands for DC

Table 6.x.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n25-n71 | 66 | 1710 MHz | – | 1780 MHz | 2210 MHz | – | 2200 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

### 6.x.2	Configuration for DC

Table 5.1.X.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_66A_n25A-n71A | DC_66A_n25A DC_66A_n71A | 66A | CA_n25A-n71A |

### 6.x.3	Co-existence studies

Co-existence analysis for DC_66_n25 shows that there is no impact from DC_66_n25 UL to NR Band n71 DL.

Co-existence analysis for DC_66_n71 shows that there is no impact from DC_66_n71 UL to NR Band n25 DL.

### 6.x.4	∆TIB and ∆RIB values

For DC_66_n25-n71 the TIB,c and RIB,c values are same as for DC_2-66_n71 and are given in the tables below.

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n25-n71 | 66 | 0.5 |
|  | n25 | 0.5 |
|  | n71 | 0.3 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n25-n71 | 66 | 0.3 |
|  | n25 | 0.5 |
|  | n71 | 0 |

### 6.x.4	MSD

MSD is not needed to be defined.

##### ---End of changes---

# Reference

[1]	RP-192592, “Revised WID on EN-DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL)”, LG Electronics