3GPP TSG-RAN WG4 RAN4#94	R4-2002131

E-meeting, 24 Feb. – 6 Mar., 2020

Agenda item:	8.1.4.13

Source: 	Qualcomm Incorporated

Title: 	Discussion regarding NR-U UL timing

Document for:	Discussion

# Introduction

During the last meeting [1], RAN4 discussed UE’s UL transmission timing in NR-U but left one shot UL timing adjustment for future study. We discuss this issue further in this meeting.

# One shot UL timing adjustment

RAN4 made the following agreements regarding UL transmission timing during the previous meetings

Possible WF: - Option 1 is agreed - The UE transmission after one shot adjustment due to changing the reference cell shall meet Te. - The UE transmit timing immediately after the one shot adjustment shall be: T2-(NTA+NTA_offset)Tc + 2*(T1-T2), where - T1 is the DL reception reference time of the old reference cell (e.g. SpCell) at the UE immediately before switching to new reference cell (e.g. SCell). - T2 is the DL reception reference time of the new reference cell (e.g. SCell) at the UE immediately after switching to the new reference cell. - One shot UE transmit timing adjustment to be applied by the UE immediately after switching to the new reference cell = 2*(T1-T2). Agreement: - Scenario B and C: if the current reference cell (i.e. PCell in Scenairo C or PSCell in Scenario B) is unavailable to UE after certain number of DL SSB detection attempts, then the UE can take any activated SCell with SSB available at the UE within this CG as the new reference cell - FFS: The UE applies one shot timing adjustment

RAN4 has been discussing one shot UL timing adjustment for Rel-15 UEs for quite a long time. One discussion point is to determine the value of T where UE performs one shot UL timing adjustment when the transmission timing error between the UE and the reference timing exceeds -+T [2].

Agreement: When the transmission timing error between the UE and the reference timing exceeds ±T then the UE shall adjust its transmission timing in one adjustment to within ±Te1 provided the following conditions are met at the UE: - SSB_RP and SSB Ês/Iot according to Annex B.2.6.1 for a corresponding operating Band, - CSI-RS_RP and CSI-RS Ês/Iot according to Annex B.2.6.2 for a corresponding operating Band, Editor’s note: The values of Te1 and T are FFS.

RAN4 hasn’t yet been able to decide this value of T and one-shot timing adjustment for Rel-15 is still under discussion.

Agreement: - Scenario B and C: if the current reference cell (i.e. PCell in Scenairo C or PSCell in Scenario B) is unavailable to UE after certain number of DL SSB detection attempts, then the UE can take any activated SCell with SSB available at the UE within this CG as the new reference cell FFS: The UE applies one shot timing adjustment

In NR-U networks, if the current reference cell is unavailable to UE after certain number of DL SSB detection attempts, UE can take any activated SCell with SSB available at the UE as the new reference cell. The available SSB within the new reference cell may not be QCLed with the SSB that was tracking in the previous reference cell. The timing difference between the previously tracked SSB and the newly available SSB may be high and will limit UE’s capability to perform one shot UL timing adjustment.

Observation 1: In NR-U networks, the available SSB within the new reference cell may not be QCLed with the SSB that the UE was tracking in the previous reference cell. The timing difference between these two SSBs can be large.

Hence, RAN4 should discuss one shot UL timing adjustment procedures for Rel-16 NR- U only after this issue gets resolved for Rel-15.

Proposal 1: RAN4 does not discuss one shot UL timing adjustment procedures for Rel-16 NR-U networks before this issue gets resolved for Rel-15.

# Conclusion

Observation 1: In NR-U networks, the available SSB within the new reference cell may not be QCLed with the SSB that the UE was tracking in the previous reference cell. The timing difference between these two SSBs can be high.

Proposal 1: RAN4 does not decide one shot UL timing adjustment procedures for Rel-16 NR-U networks before this issue gets resolved for Rel-15.

# References

- Chairman’s notes, 3GPP TSG RAN WG4 93, Reno, Nevada, USA.

- Chairman’s notes, RAN4 90-bis