3GPP TSG-RAN WG4 Meeting #94	R4-2002136

E-meeting, 24 Feb. – 6 Mar., 2020

Agenda item:	8.7.3.2

Source: 	Qualcomm Incorporated

Title: 	Interruption time during MIMO layer adaptation

Document for:	Discussion

# Introduction

RAN4 has been discussing switching time and interruption time for adapting MIMO layers during the last few meetings. The switching time related discussion has been happening almost exclusively in the RF session. We have submitted a contribution [1] regarding this topic to the RF agenda item. Here, we focus on interruption time during MIMO layer adaptation.

# Background

38.133 describes the interruption time during BWP switching through the following tables:

Table 8.2.1.2.7-1 [2]: interruption length X

|  | NR Slot length (ms) | Interruption length X (slotsNote 1) |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 1 |
| 2 | 0.25 | 3 |
| 3 | 0.125 | 5 |
| Note1: If the BWP switch involves changing of SCS, the interruption due to BWP switch is determined by the smaller SCS between the SCS before BWP switch and the SCS after the BWP switch. |  |  |

Table 8.2.1.2.7-2 [2]: Parameters which cause interruption other than SCS

| Parameters | Comment |
| --- | --- |
| locationAndBandwidth | From TS 38.331 [2] |
| nrofSRS-Ports |  |
| Editor’s note: More parameters can be added if identified |  |

Rel-16 power saving work item is currently discussing the interruption time during MIMO layer switching. RAN4 made the following agreement about this topic during a previous meeting [3]:

RAN4 Agreements: - It is RAN4 common understanding that Per-BWP maximum MIMO layer configuration is beneficial - Dynamic adaption to the maximum number of MIMO layers shall comply with Rel-15 per-CC limit configured via RRC signalling. - The current BWP switch delay and interruption requirements for Type 1 and Type 2 UE in 38.133 are reused for MIMO layer adaption when the maximum number of MIMO layers is adapted as a part of the BWP change. (case 1) - Further study the switching delay and interruption requirements when only the number of maximum MIMO layer is changed in the BWPs before and after MIMO layer adaption (case 2) - e.g. the BWP center frequency, bandwidth and SCS … will keep unchanged during MIMO layer adaptation (case 2) - no further relaxation of the requirements compared to BWP switching delay and interruption for Typ1 and Type 2 - The above bullets are for downlink MIMO layer adaption. FFS for UL. - Where, - Case 1: Maximum number of MIMO layers is adapted as a part of the BWP change - Case 2: Only the number of maximum MIMO layer is changed in the BWP before and after MIMO layer adaption

# Discussion

All companies have proposed to keep interruption duration as one slot in FR1. A couple of companies have proposed to reduce the interruption duration in FR2.

In FR2, UE may share a same set of antennas across different component carriers. Also, there might be several filters that are shared between different component carriers. As a result, change in number of MIMO layers in one carrier may impact RX tuning of other carriers.

That’s why, interruption lengths for MIMO layer switching should be same as those for Rel-15 BWP switching.

Observation 1: UE may share a same set of antennas and RF filters across different component carriers. Hence, change in number of MIMO layers in one carrier may impact RX tuning of other carriers.

Proposal 1: The interruption lengths during MIMO layer adaptation (case 2) are same as those during Rel-15 BWP switching.

# Conclusion

Observation 1: UE may share a same set of antennas and RF filters across different component carriers. Hence, change in number of MIMO layers in one carrier may impact RX tuning of other carriers.

Proposal 1: The interruption lengths during MIMO layer adaptation (case 2) are same as those during Rel-15 BWP switching.

# Reference

- R4-2000767, “Switching times for MIMO layer only change”, Qualcomm Incorporated.

- 3GPP TS 38.144, V 16.2.0, Requirements for support of radio resource management (release 16).

- Chairman’s notes, 3GPP TSG RAN WG4 Meeting #92.