3GPP TSG-RAN4 Meeting #94-e	R4-2002138

# Online, , 24th Feb 2020 - 6th Mar 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR |  | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for 38.101-1: Introduction of Power Class 1.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sprint Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_B41_Bn41_PC29dBm-Core |  |  |  |  |  | Date: |  |  | 2020-02-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Draft of Introduction of Power Class 1.5 being provided for discussion |  |  |  |  |  |  |  |  |
| Summary of change: |  | Requirements for Power Class 1.5 added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not crrently up for approval |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1, 6.2.2, 6.2.3.2, 6.2D.1, 6.2D.4, 6.5.2.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.101-3 CR 0168 TS 38.307 CR 0015 |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.521-1 CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of changes>

### 6.2.1	UE maximum output power

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth of NR carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

Table 6.2.1-1: UE Power Class

| NR band | Class 1 (dBm) |  | Tolerance (dB) |  | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  |  |  |  |  | 23 | ±2 |
| n2 |  |  |  |  |  |  |  |  | 23 | ±23 |
| n3 |  |  |  |  |  |  |  |  | 23 | ±23 |
| n5 |  |  |  |  |  |  |  |  | 23 | ±2 |
| n7 |  |  |  |  |  |  |  |  | 23 | ±23 |
| n8 |  |  |  |  |  |  |  |  | 23 | ±23 |
| n12 |  |  |  |  |  |  |  |  | 23 | ±23 |
| n14 | 31 |  | +2/-3 |  |  |  |  |  | 23 | ±23 |
| n18 |  |  |  |  |  |  |  |  | 23 | ±2 |
| n20 |  |  |  |  |  |  |  |  | 23 | ±23 |
| n25 |  |  |  |  |  |  |  |  | 23 | ±2 |
| n28 |  |  |  |  |  |  |  |  | 23 | +2/-2.5 |
| n30 |  |  |  |  |  |  |  |  | 23 | ±2 |
| n34 |  |  |  |  |  |  |  |  | 23 | ±2 |
| n38 |  |  |  |  |  |  |  |  | 23 | ±2 |
| n39 |  |  |  |  |  |  |  |  | 23 | ±2 |
| n40 |  |  |  |  |  |  |  |  | 23 | ±2 |
| n41 |  |  |  |  | 29 | +2/-33 | 26 | +2/-33 | 23 | ±23 |
| n48 |  |  |  |  |  |  |  |  | 23 | +2/-3 |
| n50 |  |  |  |  |  |  |  |  | 23 | ±2 |
| n51 |  |  |  |  |  |  |  |  | 23 | ±2 |
| n65 |  |  |  |  |  |  |  |  | 23 | ±2 |
| n66 |  |  |  |  |  |  |  |  | 23 | ±2 |
| n70 |  |  |  |  |  |  |  |  | 23 | ±2 |
| n71 |  |  |  |  |  |  |  |  | 23 | +2/-2.5 |
| n74 |  |  |  |  |  |  |  |  | 23 | ±2 |
| n77 |  |  |  |  |  |  | 26 | +2/-3 | 23 | +2/-3 |
| n78 |  |  |  |  |  |  | 26 | +2/-3 | 23 | +2/-3 |
| n79 |  |  |  |  |  |  | 26 | +2/-3 | 23 | +2/-3 |
| n80 |  |  |  |  |  |  |  |  | 23 | ±2 |
| n81 |  |  |  |  |  |  |  |  | 23 | ±2 |
| n82 |  |  |  |  |  |  |  |  | 23 | ±2 |
| n83 |  |  |  |  |  |  |  |  | 23 | ±2/-2.5 |
| n84 |  |  |  |  |  |  |  |  | 23 | ±2 |
| n86 |  |  |  |  |  |  |  |  | 23 | ±2 |
| n89 |  |  |  |  |  |  |  |  | 23 | ±2 |
| n91 |  |  |  |  |  |  |  |  | 23 | ±23, 4 |
| n92 |  |  |  |  |  |  |  |  | 23 | ±23, 4 |
| n93 |  |  |  |  |  |  |  |  | 23 | ±23, 4 |
| n94 |  |  |  |  |  |  |  |  | 23 | ±23, 4 |
| n95 |  |  |  |  |  |  |  |  | 23 | ±2 |
|  |  |  |  | NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 4: The maximum output power requirement is relaxed by reducing the lower tolerance limit by 0.3 dB |  |  |  |  |  |  |

If a UE supports a different power class than the default UE power class for the TDD band and the supported power class enables the higher maximum output power than that of the default power class:

-	if the field of UE capability maxUplinkDutyCycle is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 50% (The exact evaluation period is no less than one radio frame); or

-	if the field of UE capability maxUplinkDutyCycle is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or

-	if the IE P-Max as defined in TS 38.331 [7] is provided and set to the maximum output power of the default power class or lower;

-	shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified in clause 6.2.4;

-	else if the UE does not supports Power Class 1.5; or

-	if the field of UE capability maxUplinkDutyCycle is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 25% (The exact evaluation period is no less than one radio frame); or

-	if the field of UE capability maxUplinkDutyCycle is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle/2 (The exact evaluation period is no less than one radio frame); or

-	if the IE P-Max as defined in TS 38.331 [7] is provided and set to the maximum output power of the power class 2 or lower;

-	shall apply all requirements for power class 2 to the supported power class and set the configured transmitted power as specified in clause 6.2.4;

-	else

-	shall apply all requirements for the supported power class and set the configured transmitted power as specified in clause 6.2.4.

<Next changed section>

### 6.2.2	UE maximum output power reduction

UE is allowed to reduce the maximum output power due to higher order modulations and transmit bandwidth configurations. For UE power class 2 and 3, the allowed maximum power reduction (MPR) is defined in Table 6.2.2-2 and Table 6.2.2-1, respectively for channel bandwidths that meets both following criteria:

Channel bandwidth ≤ 100 MHz.

Relative channel bandwidth ≤ 4 % for TDD bands and ≤ 3 % for FDD bands

Where relative channel bandwith = 2*BWChannel / (FUL_low + FUL_high)

The allowed MPR for SRS, PUCCH formats 0, 1, 3 and 4, and PRACH shall be as specified for QPSK modulated DFT-s-OFDM of equivalent RB allocation. The allowed MPR for PUCCH format 2 shall be as specified for QPSK modulated CP-OFDM of equivalent RB allocation.

Table 6.2.2-1 Maximum power reduction (MPR) for power class 3

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.51 | ≤ 1.21 | ≤ 0.21 |
|  |  | ≤ 0.52 | ≤ 0.52 | 02 |
|  | QPSK | ≤ 1 |  | 0 |
|  | 16 QAM | ≤ 2 |  | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3 |  | ≤ 1.5 |
|  | 16 QAM | ≤ 3 |  | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |
| NOTE 1: Applicable for UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and if the IE powerBoostPi2BPSK is set to 1 and 40 % or less slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. The reference power of 0 dB MPR is 26 dBm. NOTE 2: Applicable for UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79 with Pi/2 BPSK modulation and if the IE powerBoostPi2BPSK is set to 0 and if more than 40 % of slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. |  |  |  |  |

Table 6.2.2-2 Maximum power reduction (MPR) for power class 2

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 0.5 | 0 |
|  | QPSK | ≤ 3.5 | ≤ 1 | 0 |
|  | 16 QAM | ≤ 3.5 | ≤ 2 | ≤ 1 |
|  | 64 QAM | ≤ 3.5 | ≤ 2.5 |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 3 | ≤ 1.5 |
|  | 16 QAM | ≤ 3.5 | ≤ 3 | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |

Table 6.2.2-3 Maximum power reduction (MPR) for power class 1.5

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ [TBD] | ≤ [TBD] | [TBD] |
|  | QPSK | ≤ [TBD] | ≤ [TBD] | [TBD] |
|  | 16 QAM | ≤ [TBD] | ≤ [TBD] | ≤ [TBD] |
|  | 64 QAM | ≤ [TBD] | ≤ [TBD] |  |
|  | 256 QAM | ≤ [TBD] |  |  |
| CP-OFDM | QPSK | ≤ [TBD] | ≤ [TBD] | ≤ [TBD] |
|  | 16 QAM | ≤ [TBD] | ≤ [TBD] | ≤ [TBD] |
|  | 64 QAM | ≤ [TBD] |  |  |
|  | 256 QAM | ≤ [TBD] |  |  |

Where the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations:

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1. RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

LCRB  ≤  ceil(NRB/2)

where ceil(x) is the smallest integer greater than or equal to x.

An Edge RB allocation is the one for which the RB(s) is (are) allocated at the lowermost or uppermost edge of the channel with LCRB ≤ 2 RBs.

The RB allocation is an Outer RB allocation for all other allocations which are not an Inner RB allocation or Edge RB allocation.

If CP-OFDM allocation satisfies following conditions, it is considered as almost contiguous allocation

NRB_gap / (NRB_alloc + NRB_gap ) ≤ 0.25

and NRB_alloc + NRB_gap is larger than 106, 51 or 24 RBs for 15 kHz, 30 kHz or 60 kHz respectively where NRB_gap is the total number of unallocated RBs between allocated RBs and NRB_alloc is the total number of allocated RBs. The size and location of allocated and unallocated RBs are restricted by RBG parameters specified in clause 6.1.2.2 of TS 38.214 [10]. For these almost contiguous signals in power class 2 and 3, the allowed maximum power reduction defined in Table 6.2.2-1 is increased by

CEIL{ 10 log10(1 + NRB_gap / NRB_alloc), 0.5 } dB,

where CEIL{x,0.5} means x rounding upwards to closest 0.5dB. The parameters of RBStart,Low and RBStart,High to specify valid RB allocation ranges for Outer and Inner RB allocations are defined as following:

RBStart,Low = max(1, floor((NRB_alloc + NRB_gap)/2))

RBStart,High = NRB – RBStart,Low – NRB_alloc –NRB_gap

For the UE maximum output power modified by MPR, the power limits specified in clause 6.2.4 apply.

<Next changed section>

#### 6.2.3.2	A-MPR for NS_04

For NS_04, A-MPR is not added to MPR. Also, when NS_04 is signalled, MPR shall be set to zero in the PCMAX equations to avoid double counting MPR.

Allowed maximum power reduction is defined as A-MPR = max(MPR, A-MPR'),

Note that A-MPR' = 0 dB means only MPR is applied,

where A-MPR' is defined as

if RBstart ≤ fstart,max,IMD3 / (12SCS) and LCRB ≤ AWmax,IMD3 / (12SCS) and FC - BWChannel/2 < FUL_low + offsetIMD3,
then

the A-MPR' is defined according to Table 6.2.3.2-2 PC3_A2 relative to 23 dBm for power class 3 and PC2_A4 relative to 26 dBm for power class 2,

else,

if RBstart ≤ LCRB/2 + start / (12SCS) and LCRB ≤ AWmax,regrowth / (12SCS) and FC - BWChannel/2 < FUL_low + offsetregrowth,
then

the A-MPR' is defined according to Table 6.2.3.2-2 PC3_A1 relative to 23 dBm for power class 3 and PC2_A3 relative to 26 dBm for power class 2,

else

A-MPR' = 0 dB and apply MPR.

With the parameters defined in Table 6.2.3.2-1.

Table 6.2.3.2-1: Parameters for region edges and frequency offsets

| Parameter | Symbol | Value |  |  | Related condition |
| --- | --- | --- | --- | --- | --- |
|  |  | CP-OFDM |  | DFT-s-OFDM |  |
| Max allocation start in IMD3 region | fstart,max,IMD3 | 0.33 BWChannel |  |  | RBstart ≤ fstart,max,IMD3 / (12SCS) |
| Max allocation BW in IMD3 region | AWmax,IMD3 | 4 MHz |  |  | LCRB ≤ AWmax,IMD3 / (12SCS) |
| Max freq. offset for IMD3 region | offsetmax,IMD3 | BWChannel – 6 MHz |  |  |  |
| Freq. offset required to avoid A-MPR in IMD3 region | offsetIMD3 | offsetmax,IMD3 |  |  | FC - BWChannel/2 ≥ FUL_low + offsetIMD3 |
| Right edge of regrowth region | start | 0.08 BWChannel |  |  | RBstart ≤ LCRB/2 + start / (12SCS) |
| Max allocation BW in regrowth region | AWmax,regrowth | 100 MHz |  |  | LCRB ≤ Min(LCRB,Max, AWmax,regrowth / (12SCS)) |
| Freq. offset required to avoid A-MPR in regrowth region | offsetregrowth | Max (10 MHz, 0.25* BWChannel MHz) | Max (10 MHz, 0.45* BWChannel MHz) |  | FC - BWChannel/2 ≥ FUL_low + offsetregrowth |

Table 6.2.3.2-2: A-MPR' values Access

| Modulation/Waveform |  | A-MPR' (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PC3_A1 |  | PC3_A2 | PC2_A3 | PC2_A4 | PC1.5_A5 | PC1.5_A6 |
| DFT-s-OFDM |  | Pi/2-BPSK | ≤ 3.5 |  | ≤ 3.5 | ≤ 3.5 | ≤ 5.5 | ≤ [TBD] | ≤ [TBD] |
|  |  | QPSK | ≤ 4 |  | ≤ 4 | ≤ 4.5 | ≤ 6 | ≤ [TBD] | ≤ [TBD] |
|  |  | 16 QAM | ≤ 4 |  | ≤ 4 | ≤ 5 | ≤ 6 | ≤ [TBD] | ≤ [TBD] |
|  |  | 64 QAM | ≤ 4 |  | ≤ 4.5 | ≤ 5 | ≤ 6.5 | ≤ [TBD] | ≤ [TBD] |
|  |  | 256 QAM | ≤ 4.5 |  | ≤ 6 | ≤ 6.5 | ≤ 8 | ≤ [TBD] | ≤ [TBD] |
| CP-OFDM |  | QPSK | ≤ 5.5 |  | ≤ 5.5 | ≤ 6.5 | ≤ 7.5 | ≤ [TBD] | ≤ [TBD] |
|  |  | 16 QAM | ≤ 5.5 |  | ≤ 5.5 | ≤ 6.5 | ≤ 7.5 | ≤ [TBD] | ≤ [TBD] |
|  |  | 64 QAM | ≤ 5.5 |  | ≤ 5.5 | ≤ 6.5 | ≤ 7.5 | ≤ [TBD] | ≤ [TBD] |
|  |  | 256 QAM | ≤ 6.5 |  | ≤ 8 | ≤ 7.5 | ≤ 10 | ≤ [TBD] | ≤ [TBD] |
| NOTE: The A-MPR' values in this table apply for both A-MPR relative to 23 dBm for power class 3 and A-MPR relative to 26 dBm for power class 2 and S-MPR relative to 29 dBm for PC 1.5 |  |  |  |  |  |  |  |

<Next changed section>

## 6.2D	Transmitter power for UL MIMO

### 6.2D.1	UE maximum output power for UL MIMO

For power class 2 UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the maximum output power for any transmission bandwidth within the channel bandwidth is specified in Table 6.2D.1-1. The requirements shall be met with the UL MIMO configurations specified in Table 6.2D.1-2. For UE supporting UL MIMO, the maximum output power is measured as the sum of the maximum output power at each UE antenna connector. The period of measurement shall be at least one sub frame (1 ms).

The requirements shall be met with the UL MIMO configurations of using 2-layer UL MIMO transmission with codebook of. DCI Format for UE configured in PUSCH transmission mode for uplink single-user MIMO shall be used.

Table 6.2D.1-1: UE Power Class for UL MIMO in closed loop spatial multiplexing scheme

| NR band | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n41 | 29 | +2/-31 | 26 | +2/-31 | 23 | +2/-31 |  |  |
| n77 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n78 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n79 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| NOTE 1: The transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: Power class 3 is the default power class unless otherwise stated |  |  |  |  |  |  |  |  |

Table 6.2D.1-2: UL MIMO configuration in closed-loop spatial multiplexing scheme

| Transmission scheme | DCI format | Codebook Index |
| --- | --- | --- |
| Codebook based uplink | DCI format 0_1 | Codebook index 0 |

If UE is configured for transmission on single-antenna port, the requirements in clause 6.2.1 apply.

<Next changed section>

### 6.2D.4	Configured transmitted power for UL MIMO

For UE supporting UL MIMO, the transmitted power is configured per each UE.

The definitions of configured maximum output power PCMAX,c, the lower bound PCMAX_L,c, and the higher bound PCMAX_H,c specified in clause 6.2.4 shall apply to UE supporting UL MIMO, where

-	PPowerClass, ΔPPowerClass and ∆TC,c are specified in clause 6.2D.1;

-	MPRc is specified in clause 6.2D.2;

-	A-MPRc is specified in clause 6.2D.3.

The measured configured maximum output power PUMAX,c for serving cell c shall be within the following bounds:

PCMAX_L,c  –  MAX{TL, T LOW(PCMAX_L,c)}  ≤  PUMAX,c  ≤  PCMAX_H,c  +  T HIGH(PCMAX_H,c)

where TLOW(PCMAX_L,c) and THIGH(PCMAX_H,c) are defined as the tolerance and applies to PCMAX_L,c and PCMAX_H,c separately, while TL is the absolute value of the lower tolerance in Table 6.2D.1-1 for the applicable operating band.

For UE with two transmit antenna connectors in closed-loop spatial amultiplexing scheme, the tolerance is specified in Table 6.2D.4-1. The requirements shall be met with UL MIMO configurations specified in Table 6.2D.1-2.

Table 6.2D.4-1: PCMAX,c tolerance in closed-loop spatial multiplexing scheme

| PCMAX,c (dBm) | Tolerance TLOW(PCMAX_L,c) (dB) | Tolerance THIGH(PCMAX_H,c) (dB) |
| --- | --- | --- |
| 26 ≤ PCMAX,c ≤ 29 | 3.0 | 2.0 |
| 23 ≤ PCMAX,c < 26 | 3.0 | 2.0 |
| 22 ≤ PCMAX,c < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX,c < 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX,c < 21 | 6.0 | 4.0 |
| 16 ≤ PCMAX,c < 20 | 5.0 |  |
| 11 ≤ PCMAX,c < 16 | 6.0 |  |
| -40 ≤ PCMAX,c < 11 | 7.0 |  |

If UE is configured for transmission on single-antenna port, the requirements in clause 6.2.4 apply.

<Next changed section>

##### 6.5.2.4.1	NR ACLR

NR Adjacent Channel Leakage power Ratio (NRACLR) is the ratio of the filtered mean power centred on the assigned NR channel frequency to the filtered mean power centred on an adjacent NR channel frequency at nominal channel spacing.

The assigned NR channel power and adjacent NR channel power are measured with rectangular filters with measurement bandwidths specified in Table 6.5.2.4.1-1.

If the measured adjacent channel power is greater than –50 dBm then the NRACLR shall be higher than the value specified in Table 6.5.2.4.1-2.

Table 6.5.2.4.1-1: NR ACLR measurement bandwidth

| NR channel bandwidth / NR ACLR measurement bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| NR ACLR measurement bandwidth | 4.515 | 9.375 | 14.235 | 19.095 | 23.955 | 28.815 | 38.895 | 48.615 | 58.35 | 68.07 | 78.15 | 88.23 | 98.31 |

Table 6.5.2.4.1-2: NR ACLR requirement

|  | Power class 1.5 | Power class 2 | Power class 3 |
| --- | --- | --- | --- |
| NR ACLR | 31 dBm | 31 dB | 30 dB |

<End of changes>