3GPP TSG-RAN WG4 Meeting #94-e	R4-2002144

# Online, 24th February – 6th March, 2020

Source: 	Skyworks Solutions, Inc.

Title: 	FR2 ACLR Measurement Bandwidth Definition

Agenda Item:	6.3.3 Other system parameters maintenance [NR_newRAT-Core]

Document for:	Discussion

# Introduction

This contribution highlights the differences between FR1 and FR2 ACLR Measurement Bandwidth definitions with regards to Sub-Carrier (SC) alignment. We discuss two options as to ensure consistency between FR1 and FR2.

# Discussion

## FR1 ACLR Measurement Bandwidth

Figure 1 reproduced from [1] formed of the original proposal to add 1 Sub-Carrier (SC) so as to ensure that the measurement bandwidth (MBW) is made symmetrical.

Figure 1: Addition of one Sub-Carrier to make measurement bandwidth symmetrical in FR1

Table 1 reproduces the relationship between the maximum Transmission Bandwidth (maxTXBW) configuration and the MBW. The values highlighted in yellow were approved in WF [2].

Table 1: ACLR and wanted signal MBW for FR1 [1]

| Sub-6GHz | SCS [kHz] | 5MHz | 10MHz | 15MHz | 20MHz | 25MHz | 40MHz | 50MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TXBW [MHz] | 15 | 4.5 | 9.36 | 14.22 | 19.08 | 24.3 | 38.88 | 48.6 | N.A | N.A | N.A |
|  | 30 | 3.96 | 8.64 | 13.68 | 18.72 | 23.4 | 38.16 | 47.88 | 58.32 | 78.12 | 98.28 |
|  | 60 | N.A | 8.64 | 12.96 | 17.28 | 23.04 | 37.44 | 46.8 | 56.88 | 77.04 | 97.2 |
| MBW [MHz] | 15 | 4.515 | 9.375 | 14.235 | 19.095 | 24.315 | 38.895 | 48.615 | N.A | N.A | N.A |
|  | 30 | 3.99 | 8.67 | 13.71 | 18.75 | 23.43 | 38.19 | 47.91 | 58.35 | 78.15 | 98.31 |
|  | 60 | N.A | 8.7 | 13.02 | 17.34 | 23.1 | 37.5 | 46.86 | 56.94 | 77.1 | 97.26 |
| maxTXBW [MHz] | lowest | 4.515 | 9.375 | 14.235 | 19.095 | 24.315 | 38.895 | 48.615 | 58.35 | 78.15 | 98.31 |

Note that for FR1:

- The NR Adjacent Channel Leakage power Ratio (NRACLR) is defined as the ratio of the filtered mean power centered on the assigned NR channel frequency to the filtered mean power centered on an adjacent NR channel frequency at nominal channel spacing,

- The minimum guard bands have been calculated using the following equation: (BW Channel x 1000 (kHz) - NRB x SCS x 12) / 2 - SCS/2, where NRB are from 38.101-1 Table 5.3.2-1.

- The same MBW is used to measure the wanted channel power and the adjacent channel power. The MBW is specified in 38.101-1 Table 6.5.2.4.1-1, and reproduced in Table 2 below,

- Even though not explicitly specified in Table 2, the offset for the adjacent channel MBW centre is +/- Channel BW from wanted channel center (in the middle of the Channel) [2].

Table 2: ACLR and wanted channel MBW for FR1

| NR channel bandwidth / NR ACLR measurement bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| NR ACLR measurement bandwidth | 4.515 | 9.375 | 14.235 | 19.095 | 23.955 | 28.815 | 38.895 | 48.615 | 58.35 | 68.07 | 78.15 | 88.23 | 98.31 |

## FR2 ACLR Measurement Bandwidth

In FR2, the following items are exactly identical to FR1:

- The NR Adjacent Channel Leakage power Ratio (NRACLR) is defined as the ratio of the filtered mean power centered on the assigned channel frequency to the filtered mean power centered on an adjacent channel frequency at nominal channel spacing.

- The same MBW is used to measure the wanted channel power and the adjacent channel power,

- The minimum guard bands have been calculated using the following equation: (BW Channel x 1000 (kHz) - NRB x SCS x 12) / 2 - SCS/2, where NRB are from Table 5.3.2-1.

The differences between FR2 and FR1 are:

- The offset for the adjacent channel MBW center is explicitly specified in 38.101-2 Table 6.5.2.3-1,

- The ACLR and wanted channel MBW are set equal to the maxTXBW, i.e., the measurement BW might not be always symmetrical.

Observation 1: MBW in FR2 may not always be symmetrical.

Similarly to FR1, one sub-carrier is needed to ensure MBW is also symmetrical in FR2 as shown in Figure 2 below.

Figure 2: Addition of one Sub-Carrier to make measurement bandwidth symmetrical in FR2

Last row of Table 3 proposes a symmetrical MBW.

Table 3: ACLR and wanted signal MBW for FR2 [1]

| Sub-6GHz | SCS [kHz] | 50MHz | 100MHz | 200MHz | 400MHz |
| --- | --- | --- | --- | --- | --- |
|  | 60 | 66 | 132 | 264 | n-a |
|  | 120 | 32 | 66 | 132 | 264 |
| TXBW [MHz] | 60 | 47.52 | 95.04 | 190.08 | n-a |
|  | 120 | 46.08 | 95.04 | 190.08 | 380.16 |
| Current MBW [MHz] | all | 47.52 | 95.04 | 190.08 | 380.16 |
| Symmetrical TXBW [MHz] | 60 | 47.58 | 95.10 | 190.14 | n-a |
|  | 120 | 46.20 | 95.16 | 190.20 | 380.28 |
| Possible maxMBW |  | 47.58 | 95.16 | 190.20 | 380.28 |

Two possible options may be considered to ensure MBW is centered on TxBW.

- Option 1: modify FR2 MBW according to Table 3

- Option 2: maintain existing FR2 MBW, and change the text defining the center frequency of the measurement BW for the wanted channel so that MBW becomes centered on the TxBW.

# Conclusions

In this contribution we present a possible discrepancy between FR1 MBW and FR2 MBW for wanted channel and adjacent channel power measurements. We note that the current FR2 MBW may not always ensure MBW is centered correctly on the wanted channel. MBW can be either extended by one SC or, current MBW may remain intact if the text defining how the center frequency of the wanted channel is modified, a possible solution consisting in applying a center frequency shift.

# References

[1] R4-1707702, Impact of ACLR Measurement BW and TRX Impairments on NR UE MPR, Skyworks Solutions, Inc., Nokia, Nokia Shanghai Bell, RAN4#84, Berlin, Germany, 21 - 24 August, 2017.

[2] R4-1708912, NR MPR simulation assumptions for sub-6 GHz operation, Nokia, RAN4#84, Berlin, Germany, 21 - 24 August, 2017.