3GPP TSG-RAN WG4 Meeting #94-e	R4-2002145

# Online, 24th February – 6th March, 2020

Source: 	Skyworks Solutions, Inc.

Title: 	n26 PA Back-Off Measurements

Agenda Item:	9.19.1 UE RF (38.101-1) [NR_n26]

Document for:	Discussion

# Introduction

In RAN4#92bis meeting, contribution [1] listed several scenarios for n26 emission requirements. This contribution presents power amplifier (PA) back-off measurements.

# Discussion

## Selected Scenarios

Measurements target the following scenarios.

NS-12:Range 1 (806 ≤ f ≤ 813.5) additional emissions requirements:

- 5 MHz carrier in 814-819 MHz (SouthernLINC)

- 10 MHz carrier in 814-824 MHz (SouthernLINC)

NS-13:Range 2 (806 ≤ f ≤ 816) additional emissions requirements:

- 5 MHz carrier in 817-822 MHz (Sprint)

- 5 MHz carrier in 819-824 MHz (Sprint)

## PA and Waveform calibration

- A low-band power amplifier (PA) is calibrated to meet 1 dB MPR using a 100 RB QPSK DFT-s-OFDM SCS 15kHz with RBs allocated at the channel edge.

- Image and Local Oscillator (LO) rejection is set to -28 dBc,

- Counter-IM3 (C-IM3) set to -60 dBc,

- Counter-IM5 (C-IM5) set to -70 dBc,

- QPSK DFT-S-OFDM SCS 15kHz waveforms for all tests.

## Measurement Results

| Network Signaling | Cases | CBW | Carrier Centre Frequency ‘Fc’ | Range | Target Emission Level | Waveform |  | Back-off |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | [MHz] | [MHz] | [MHz] | [dBm/6.25kHz] | LCRB | RBStart | [dB] |
| NS_12 | 1 | 5 | Fc = 816.5 | 1 | -42 | 1 | 0 | 3.3 |
|  |  |  |  |  |  | 5 | 5 | 2.2 |
|  |  |  |  |  |  | 25 | 0 | 3.6 |
|  | 2 | 10 | Fc = 819 |  |  | 1 | 0 | 3.1 |
|  |  |  |  |  |  | 5 | 0 | 1.8 |
|  |  |  |  |  |  | 10 | 0 | 4.9 |
|  |  |  |  |  |  | 50 | 0 | 3.1 |
| NS_13 | 3 | 5 | Fc = 819.5 | 2 | -42 | 1 | 0 | 2.9 |
|  |  |  |  |  |  | 25 | 0 | 3.3 |
|  | 4 | 5 | Fc = 821.5 |  |  | 1 | 0 | 2.2 |
|  |  |  |  |  |  | 5 | 0 | 1.2 |

# Conclusions

In this contribution, we present our measurement results for n26.

# References

[1] R4-1913078, n26 Emissions requirements, Sprint, SouthernLINC, Dish Network, TSG-RAN WG4 Meeting #92bis, Chongqing, China, 14-18 October 2019.