3GPP TSG-RAN WG4 Meeting #94-e	R4-2002215

Online, 24 February - 6 March, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0444 | rev | 1 | Current version: | 15.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial corrections for 38.133 Perf Part R15 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2020-03-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Multiple errors in tables in clause 10.1 spotted: - duplication of units - typos |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove duplicated units and correct typos. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The aforementioned errors remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

<start of the change 1>

### 10.1.2	Intra-frequency RSRP accuracy requirements for FR1

#### 10.1.2.1	Intra-frequency SS-RSRP accuracy requirements

##### 10.1.2.1.1	Absolute SS-RSRP Accuracy

Unless otherwise specified, the requirements for absolute accuracy of SS-RSRP in this clause apply to a cell on the same frequency as that of the serving cell in FR1.

The accuracy requirements in Table 10.1.2.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each relevant SSB.

Table 10.1.2.1.1-1: SS-RSRP Intra frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 4.5 | 9 | -6 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -70 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -70 |
| 8 | 11 | -6 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_G, NR_FDD_FR1_H | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

##### 10.1.2.1.2	Relative SS-RSRP Accuracy

The relative accuracy of SS-RSRP is defined as the SS-RSRP measured from one cell compared to the SS-RSRP measured from another cell on the same frequency, or between any two SS-RSRP levels measured on the same cell in FR1.

The accuracy requirements in Table 10.1.2.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each relevant SSB.

Table 10.1.2.1.2-1: SS-RSRP Intra frequency relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 2 | 3 | -3 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
| 3 | 3 | -6 | Note 3 | Note 3 | Note 3 | N/A | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

#### 10.1.2.2	Void

### 10.1.3	Intra-frequency RSRP accuracy requirements for FR2

#### 10.1.3.1	Intra-frequency SS-RSRP accuracy requirements

##### 10.1.3.1.1	Absolute SS-RSRP Accuracy

Unless otherwise specified, the requirements for absolute accuracy of SS-RSRP in this clause apply to a cell on the same frequency as that of the serving cell in FR2.

The accuracy requirements in Table 10.1.3.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each relevant SSB.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.3.1.1-1: SS-RSRP Intra frequency absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 2 range |  |  |  |
|  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 1 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |  |
| 6 | 9 | ≥-6 | Same value as SSB_RP in Table B.2.2-2, according to UE Power class, operating band and angle of arrival |  | N/A | -70 |
| 8 | 11 |  | N/A |  | -70 | -50 |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |

##### 10.1.3.1.2	Relative SS-RSRP Accuracy

The relative accuracy of SS-RSRP is defined as the SS-RSRP measured from one cell compared to the SS-RSRP measured from another cell on the same frequency, or between any two SS-RSRP levels measured on the same cell in FR2.

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each relevant SSB.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.3.1.2-1: SS-RSRP Intra frequency relative accuracy in FR2

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 2 range |  |  |
|  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 1 |  | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |
| 6 | 9 | ≥-6 | Same value as SSB_RP in Table B.2.2-2, according to UE Power class, operating band and angle of arrival |  | -50 |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. Note 4: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. |  |  |  |  |  |

#### 10.1.3.2	Void

### 10.1.4	Inter-frequency RSRP accuracy requirements for FR1

#### 10.1.4.1	Inter-frequency SS-RSRP accuracy requirements

##### 10.1.4.1.1	Absolute Accuracy of SS-RSRP in FR1

The requirements for absolute accuracy of SS-RSRP in this clause apply to a cell on a frequency in FR1 that has different carrier frequency from the serving cell.

The accuracy requirements in Table 10.1.4.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for each relevant SSB.

Table 10.1.4.1.1-1: SS-RSRP Inter frequency Absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 4.5 | 9 | -6 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -70 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -70 |
| 8 | 11 | -6 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_G, NR_FDD_FR1_H | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: Void NOTE 3: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

##### 10.1.4.1.2	Relative Accuracy of SS-RSRP in FR1

The relative accuracy of SS-RSRP in inter frequency case is defined as the RSRP measured from one cell on a frequency in FR1compared to the RSRP measured from another cell on a different frequency in FR1.

The accuracy requirements in Table 10.1.4.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] Clause 7.3 for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for each relevant SSB.

-	|SSB_RP1dBm - SSB_RP2dBm| ≤ 27 dB

-	| Channel 1_Io -Channel 2_Io |  20 dB

Table 10.1.4.1.2-1: SS-RSRP Inter frequency relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 4.5 | 6 | -6 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

#### 10.1.4.2	Void

<end of the change 1>

<start of the change 2>

### 10.1.7	Intra-frequency RSRQ accuracy requirements for FR1

#### 10.1.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR1

##### 10.1.7.1.1	Absolute SS-RSRQ Accuracy in FR1

Unless otherwise specified, the requirements for absolute accuracy of SS-RSRQ in this clause apply to a cell on the same frequency as that of the serving cell in FR1.

The accuracy requirements in Table 10.1.7.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each relevant SSB.

Table 10.1.7.1.1-1: SS-RSRQ Intra frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 2.5 | 4 | -3 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
| 3.5 | 4 | -6 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

### 10.1.8	Intra-frequency RSRQ accuracy requirements for FR2

#### 10.1.8.1	Intra-frequency SS-RSRQ accuracy requirements in FR2

##### 10.1.8.1.1	Absolute SS-RSRQ Accuracy in FR2

Unless otherwise specified, the requirements for absolute accuracy of SS-RSRQ in this clause apply to a cell on the same frequency as that of the serving cell in FR2.

The accuracy requirements in Table 10.1.8.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each relevant SSB.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.8.1.1-1: SS-RSRQ Intra frequency absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 2 range |  |  |
|  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 1 |  | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |
| 2.5 | 4 | ≥-3 | Same value as SSB_RP in Table B.2.2-2, according to UE Power class, operating band and angle of arrival |  | -50 |
| 3.5 | 4 | ≥-6 |  |  |  |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |

### 10.1.9	Inter-frequency RSRQ accuracy requirements for FR1

#### 10.1.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR1

##### 10.1.9.1.1	Aboslute Accuracy of SS-RSRQ in FR1

The requirements for absolute accuracy of SS-RSRQ in this clause apply to a cell on a frequency in FR1 that has different carrier frequency from the serving cell.

The accuracy requirements in Table 10.1.9.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for each relevant SSB.

Table 10.1.9.1.1-1: SS-RSRQ Inter frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 2.5 | 4 | -3 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
| 3.5 | 4 | -6 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

##### 10.1.9.1.2	Relative Accuracy of SS-RSRQ in FR1

The relative accuracy of SS-RSRQ in inter frequency case is defined as the RSRQ measured from one cell on a frequency in FR1 compared to the RSRP measured from another cell on a different frequency in FR1.

The accuracy requirements in Table 10.1.9.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for each relevant SSB.

-	|SSB_RP1dBm - SSB_RP2dBm| ≤ 27 dB

-	| Channel 1_Io -Channel 2_Io |  20 dB

Table 10.1.9.1.2-1: SS-RSRQ Inter frequency relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 3 | 4 | -3 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
| 4 | 4 | -6 | Note 3 | Note 3 | Note 3 | Note 3 | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

<end of the change 2>

<start of the change 3>

### 10.1.12	Intra-frequency SINR accuracy requirements for FR1

#### 10.1.12.1	Intra-frequency SS-SINR accuracy requirements in FR1

##### 10.1.12.1.1	Absolute SS-SINR Accuracy in FR1

Unless otherwise specified, the requirements for absolute accuracy of SS-SINR in this clause apply to a cell on the same frequency as that of the serving cell in FR1.

The accuracy requirements in Table 10.1.12.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band.

Table 10.1.12.1.1-1: SS-SINR Intra frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 3 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 3.0 | 4 | -3 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
| 3.5 | 4 | -6 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: The requirements apply for SSB Ês/Iot ≤ 25 dB. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

### 10.1.13	Intra-frequency SINR accuracy requirements for FR2

#### 10.1.13.1	Intra-frequency SS-SINR accuracy requirements in FR2

##### 10.1.13.1.1	Absolute SS-SINR Accuracy in FR2

Unless otherwise specified, the requirements for absolute accuracy of SS-SINR in this clause apply to a cell on the same frequency as that of the serving cell in FR2.

The accuracy requirements in Table 10.1.13.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.13.1.1-1: SS-SINR Intra frequency absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 2 range |  |  |
|  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 1 |  | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |
| 3 | 4 | ≥-3 | Same value as SSB_RP in Table B.2.2-2, according to UE Power class, operating band and angle of arrival |  | -50 |
| 3.5 | 4 | ≥-6 |  |  |  |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. Note 4: The requirements apply for SSB Ês/Iot ≤ 25 dB. |  |  |  |  |  |

### 10.1.14	Inter-frequency SINR accuracy requirements for FR1

#### 10.1.14.1	Inter-frequency SS-SINR accuracy requirements in FR1

##### 10.1.14.1.1	Aboslute Accuracy of SS-SINR in FR1

The requirements for absolute accuracy of SS-SINR in this clause apply to a cell on a frequency in FR1 that has different carrier frequency from the serving cell.

The accuracy requirements in Table 10.1.14.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band.

Table 10.1.14.1.1-1: SS-SINR Inter frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 3 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 3.0 | 4 | -3 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
| 3.5 | 4 | -6 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: The requirements apply for SSB Ês/Iot ≤ 25 dB. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

##### 10.1.14.1.2	Relative Accuracy of SS-SINR in FR1

The relative accuracy of SS-SINR in inter frequency case is defined as the SS-SINR measured from one cell on a frequency in FR1 compared to the SS-SINR measured from another cell on a different frequency in FR1.

The accuracy requirements in Table 10.1.14.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band.

-	|SSB_RP1dBm - SSB_RP2dBm| ≤ 27 dB

-	| Channel 1_Io -Channel 2_Io |  20 dB

Table 10.1.14.1.2-1: SS-SINR Inter frequency relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2,4 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 5 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 120 kHz | SCSSSB = 240 kHz |  |  |
| 3.5 | 4 | -3 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
| 4 | 4 | -6 | Note 3 | Note 3 | Note 3 | Note 3 | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: The requirements apply for SSB Ês/Iot ≤ [25] dB. NOTE 5: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

<end of the change 3>

<start of the change 4>

### 10.1.19	L1-RSRP accuracy requirements for FR1

#### 10.1.19.1	SSB based L1-RSRP accuracy requirements

##### 10.1.19.1.1	Absolute Accuracy

Unless otherwise specified, the requirements for absolute accuracy of SSB based L1-RSRP in this clause apply to all SSBs of the serving cell configured for L1-RSRP measurement.

The accuracy requirements in Table 10.1.19.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to Annex B.2.4.1 for a corresponding Band for each relevant SSB.

Table 10.1.19.1.1-1: SSB based L1-RSRP absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| ±5.0 | ±9.5 | -3 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -70 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -70 |
| ±8.5 | ±11.5 | -3 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_G, NR_FDD_FR1_H, | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

##### 10.1.19.1.2	Relative Accuracy

The relative accuracy of SSB based L1-RSRP is defined as the L1-RSRP measured from one SSB compared to the largest measured value of L1-RSRP among all SSBs of the serving cell.

The accuracy requirements in Table 10.1.19.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to Annex B.2.4.1 for a corresponding Band for each relevant SSB.

Table 10.1.19.1.2-1: SSB based L1-RSRP relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| ±3 | ±4 | -3 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of SSBs to which the requirement applies. NOTE 3: Void NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2.. |  |  |  |  |  |  |  |

#### 10.1.19.2	CSI-RS based L1-RSRP accuracy requirements

##### 10.1.19.2.1	Absolute Accuracy

Unless otherwise specified, the requirements for absolute accuracy of CSI-RS based L1-RSRP in this clause apply to all CSI-RS resources of the serving cell configured for L1-RSRP measurement.

The accuracy requirements in Table 10.1.19.2.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to Annex B.2.4.2 for a corresponding Band for each relevant CSI-RS.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3.

The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 10.1.19.2.1-1.

Table 10.1.19.2.1-1: CSI-RS based L1-RSRP absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSCSI-RS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |  |
| ±5.0 | ±9.5 | -3 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -70 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | -112 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 | N/A | -70 |
| ±8.5 | ±11.5 | -3 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_G, NR_FDD_FR1_H | N/A | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |

##### 10.1.19.2.2	Relative Accuracy

The relative accuracy of CSI-RS based L1-RSRP is defined as the L1-RSRP measured from one CSI-RS compared to the largest measured value of L1-RSRP among all CSI-RS resources of the serving cell.

The accuracy requirements in Table 10.1.19.2.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to Annex B.2.4.2 for a corresponding Band for each relevant CSI-RS.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3.

The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 10.1.19.2.2-1.

Table 10.1.19.2.2-1: CSI-RS based L1-RSRP relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR operating band groups Note 4 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSCSI-RS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |  |
| ±3 | ±4 | -3 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -50 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | -112 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter CSI-RS Ês/Iot is the minimum CSI-RS Ês/Iot of the pair of CSI-RS resources to which the requirement applies. NOTE 3: Void NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |

<end of the change 4>

<start of the change 5>

### 10.1.21	SFTD accuracy requirements

#### 10.1.21.1	SFTD acuracy requirements for NE-DC

The SFN and frame timing difference (SFTD) is measured between PCell and E-UTRAN PSCell under NE-DC.

The accuracy requirements in Table 10.1.21.1-4 are appilicable under the following conditions:

For FR1 PCell SFN and frame timing measurement:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Io range deifined in Table 10.1.21.1-1.

Table 10.1.21.1-1: PCell Io range conditions in FR1

| Parameter | Io Note 1 range |  |  |  |
| --- | --- | --- | --- | --- |
|  | NR operating band groups Note 4, 5 | Minimum Io Note 2, 3 |  | Maximum Io |
|  |  | dBm/ SCSSSB |  | dBm/BWChannel |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A | -121 | -118 | -50 |
|  | NR_FDD_FR1_B | -120.5 | -117.5 | -50 |
|  | NR_TDD_FR1_C | -120 | -117 | -50 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -50 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -50 |
|  | NR_FDD_FR1_G | -118 | -115 | -50 |
|  | NR_FDD_FR1_H | -117.5 | -114.5 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The condition level is increased by ΔRIB,c as defined in clause 7.3B in TS 38.101-3 [54], depending on E-UTRA – NR band combination. NOTE 3: The condition level is increased by MSD as defined in clause 7.3B in TS 38.101-3 [54], if applicable depending on E-UTRA – NR band combination. NOTE 4: NR operating band groups are as defined in clause 3.5. NOTE 5: Only NR bands within EN-DC band combinations as specified in clause 5.5B in TS 38.101-3 [54] are applicable. |  |  |  |  |

For FR2 PCell SFN and frame timing measurement:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Io range deifined in Table 10.1.21.1-2.

Table 10.1.21.1-2: PCell Io range conditions in FR2

| Parameter | Io Note 1 range |  |  |
| --- | --- | --- | --- |
|  | Minimum Io Note 2, 3 |  | Maximum Io |
|  | dBm/ SCSSSB |  | dBm/BWChannel |
|  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | Same value as SSB_RP in Table B.2.4.1-2, according to UE Power class, operating band and angle of arrival | Same value as SSB_RP in Table B.2.4.1-2, according to UE Power class, operating band and angle of arrival | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth and specified at the Reference point. NOTE 2: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |

For E-UTRA PSCell SFN and frame timing measurement:

-	Cell specific reference signals are transmitted either from one, two or four antenna ports.

-	Conditions defined in TS 36.101 [25] Clause 7.3 for reference sensitivity are fulfilled.

-	No changes to the uplink transmission timing are applied during the measurement period.

-	RSRP|dBm according to Annex B.3.5 in TS 36.101 [25] for a corresponding Band.

-	Io range deifined in Table 10.1.21.1-3.

Table 10.1.21.1-3: E-UTRA PSCell Io range conditions

| Parameter | Io Note 1 range |  |  |
| --- | --- | --- | --- |
|  | E-UTRA operating band groups Note 3 | Minimum Io | Maximum Io |
| Conditions |  | dBm/15kHz Note 2 | dBm/BWChannel |
|  | FDD_A, TDD_A | -121 | -50 |
|  | FDD_C, TDD_C | -120 | -50 |
|  | FDD_D | -119.5 | -50 |
|  | FDD_E, TDD_E | -119 | -50 |
|  | FDD_F | -118.5 | -50 |
|  | FDD_G | -118 | -50 |
|  | FDD_H | -117.5 | -50 |
|  | FDD_N | -114.5 | -50 |
| NOTE 1: When in dBm/15kHz, the minimum Io condition is expressed as the average Io per RE over all REs in that symbol. Io may be different in different symbols within a subframe. NOTE 2: The condition level is increased by ∆>0, when applicable, as described in clauses B.4.2 and B.4.3 in TS36.133 [15]. NOTE 3: E-UTRA operating band groups are as defined in clause 3.5 in TS 36.133 [15]. |  |  |  |

Table 10.1.21.1-4: SFTD measurement accuracy

| Accuracy | Conditions |  |
| --- | --- | --- |
|  | Ês/Iot Note 2 | Frequency range |
| Ts Note 1 | dB |  |
| 40*64*Tc | -3 | FR1 |
| 40*64*Tc |  | FR2 |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 2: The parameter Ês/Iot is the minimum Ês/Iot of the pair of cells to which the requirement applies. |  |  |

#### 10.1.21.2	SFTD acuracy requirements for NR-DC

The SFN and frame timing difference (SFTD) is measured between PCell in FR1 and PSCell in FR2 under NR dual connectivity.

The accuracy requirements in Table 10.1.21.2-3 are appilicable under the following conditions:

For FR1 PCell SFN and frame timing measurement:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Io range deifined in Table 10.1.21.2-1.

Table 10.1.21.2-1: PCell Io range conditions in FR1

| Parameter | Io Note 1 range |  |  |  |
| --- | --- | --- | --- | --- |
|  | NR operating band groups Note 2 | Minimum Io |  | Maximum Io |
|  |  | dBm/ SCSSSB |  | dBm/BWChannel |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A | -121 | -118 | -50 |
|  | NR_FDD_FR1_B | -120.5 | -117.5 | -50 |
|  | NR_TDD_FR1_C | -120 | -117 | -50 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -50 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -50 |
|  | NR_FDD_FR1_G | -118 | -115 | -50 |
|  | NR_FDD_FR1_H | -117.5 | -114.5 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups are as defined in clause 3.5.2. |  |  |  |  |

For FR2 PSCell SFN and frame timing measurement:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Io range deifined in Table 10.1.21.2-2.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.21.2-2: PSCell Io range conditions in FR2

| Parameter | Io Note 1 range |  |  |
| --- | --- | --- | --- |
|  | Minimum Io Note 2, 3 |  | Maximum Io |
|  | dBm/ SCSSSB |  | dBm/BWChannel |
|  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | Same value as SSB_RP in Table B.2.4.1-2, according to UE Power class, operating band and angle of arrival | Same value as SSB_RP in Table B.2.4.1-2, according to UE Power class, operating band and angle of arrival | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth and specified at the Reference point. NOTE 2: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |

Table 10.1.21.2-3: SFTD measurement accuracy

| Accuracy | Conditions |  |
| --- | --- | --- |
|  | Ês/Iot Note 2 | Frequency range |
| Ts Note 1 | dB |  |
| 40*64*Tc | -3 | Between FR1 and FR2 |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 2: The parameter Ês/Iot is the minimum Ês/Iot of the pair of cells to which the requirement applies. |  |  |

#### 10.1.21.3	Inter frequency SFTD acuracy requirements

The SFN and frame timing difference (SFTD) is measured between PCell and inter-frequency neighbour cell.

The accuracy requirements in Table 10.1.21.3-3 are appilicable under the following conditions:

For FR1 PCell, inter frequency neighbour cell SFN and frame timing measurement:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Io range deifined in Table 10.1.21.3-1.

Table 10.1.21.3-1: PCell, inter frequency neighbour cell Io range conditions in FR1

| Parameter | Io Note 1 range |  |  |  |
| --- | --- | --- | --- | --- |
|  | NR operating band groups Note 2 | Minimum Io |  | Maximum Io |
|  |  | dBm/ SCSSSB |  | dBm/BWChannel |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A | -121 | -118 | -50 |
|  | NR_FDD_FR1_B | -120.5 | -117.5 | -50 |
|  | NR_TDD_FR1_C | -120 | -117 | -50 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -50 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -50 |
|  | NR_FDD_FR1_G | -118 | -115 | -50 |
|  | NR_FDD_FR1_H | -117.5 | -114.5 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups are as defined in clause 3.5.2. |  |  |  |  |

For FR2 PCell, inter frequency neighbour cell SFN and frame timing measurement:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Io range deifined in Table 10.1.21.3-2.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.21.3-2: PCell, inter frequency neighbour cell Io range conditions in FR2

| Parameter | Io Note 1 range |  |  |
| --- | --- | --- | --- |
|  | Minimum Io Note 2, 3 |  | Maximum Io |
|  | dBm/ SCSSSB |  | dBm/BWChannel |
|  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | Same value as SSB_RP in Table B.2.4.1-2, according to UE Power class, operating band and angle of arrival | Same value as SSB_RP in Table B.2.4.1-2, according to UE Power class, operating band and angle of arrival | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth and specified at the Reference point. NOTE 2: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |

Table 10.1.21.3-3: Inter frequency SFTD measurement accuracy

| Accuracy | Conditions |  |
| --- | --- | --- |
|  | Ês/Iot Note 2 | Frequency range |
| Ts Note 1 | dB |  |
| 40*64*Tc | -3 | FR1, FR2 |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 2: The parameter Ês/Iot is the minimum Ês/Iot of the pair of cells to which the requirement applies. |  |  |

<end of the change 5>