3GPP TSG-RAN WG4 #94bis-e	R4-2003090

Online, 20th – 30th April 2020

Title: 					Considerations for NR DAPS handover

Source: 				Ericsson

Agenda item:	6.3.2.1

Document for:	Discussion

# Introduction

In RAN4#94e, there was discussion about DAPS handover, with a way forward agreed in [1]. Some of the main open issues to resolve are

Issue 1-1: Interruption in intra-frequency DAPS HO D1

Agreement:

X value:

- Option 1: 3 (slots)

- Option 2: 4 (slots)

Issue 1-2: power imbalance

- Agreement: Companies are encouraged to provide further analysis on side condition of power imbalance for intra-frequency DAPS HO.

Other issue: Synchronization assumption

- Agreement: Companies are encouraged to provide analysis on if requirement for intra-frequency DAPS handover are only applied for synchronous case

# Discussion

In our understanding, issue 1-2 and the sychronisation issue are the most critical for DAPS handover, since they relate to the deployment scenarios in which DAPS may be used. In this contribution we focus on these issues, since the value of X is considerably less significant to resolve.

Issue 1-2 is discussed within the context of intra-frequency handover, since the normal ACS and ACLR requirements should secure performance for interfrequency handover with considerable power difference Synchronisation assumptions are discussed for all intraband scenarios, both intrafrequency and interfrequency handover within the same band.

Synchronization assumption

Until RAN4#94e, there was considerable discussion on UE architectures for DAPS handover. A basic, although unwritten assumption of the work has been that the UE will reuse to some extent baseband and RF capability that has been provided for carrier aggregation or dual connectivity to improve mobility robustness by continuing to receive from and transmit to the source cell until such time as it is released by the target cell. A discussion which has explicitly taken place is that if the same FFT is used to receive both source and target, the downlink signal at the UE antenna connector would need to be received from both cells within the cyclic prefix, and on the uplink side a dual TX signal generated by the same FFT has a single timing reference.

Deployment considerations have also been discussed considerably within the context of carrier aggregation and dual connectivity, which we now refer to as MC operation, noting that there is no such thing as intrafrequency MC operation. There is intraband MC operation with contiguous and non-contiguous flavours, and there is also interband CA. In release 10, deployments for CA were discussed at length, and after considerable discussion of different proposals with cell size in the range 3-100km it was agreed that ~10km cell size would be assumed for the purposes of non-collocated deployment, with the corresponding assumption of 33 µS used as the propagation delay component of MRTD and MTTD based on c=29910^6ms-1 since 10000/c ≈ 33µS. The value is a tradeoff between deployment flexibility and limiting excessive UE complexity. To this figure is added the possible misalignment at the transmission antenna for either downlink or uplink.

Exactly the same consideration would apply to mobility enhancement, depending on the colocation assumption of source and target cell. For a general handover, it would be natural to assume similar cell dimensions in deployments, meaning that the downlink misalignment between a perfectly synchronized source and target cell as seen at the UE antenna connector would be 33µS, and the corresponding difference in ideal uplink timing would be the same. Transmission misalignment is typically smaller for CA than it would be for DAPS handover since all cells in CA are provided by the same gNB, whereas in handover, the nodes may belong to separate clock domains.

One difference in NR from LTE is that the configuration of an SMTC window depends on loose synchronization. So any neighbor which the UE measures needs to be synchronized sufficiently such that its SS burst is within the SMTC at the UE, with the start and end of the SMTC based on serving cell timing. However, such synchronization is up to network implementation, and the required accuracy can easily be an order of magnitude or more, looser than even the 33µS considered in CA deployments.

In general are significant additional costs and technical feasibility issues with synchronising network nodes, especially when GPS is not available. For TDD networks it is guaranteed that the cell phase sync requirement (such as 3us) is met between the BS antenna ports. Better synchronisation than this between geographically separate nodes may in some cases be technically challenging or more expensive.

Sync   at the UE antenna connector is very much deployment dependent and for handover we have to consider that source and target are a different distance from the UE, for interfrequency HO for load balancing

We now summarise observations on synchronization for DAPS handover:

Observation 1 : For operation where the UE is assumed to use the same FFT for DL reception, tight synchronization (significantly shorter than CP and also allowing for possible multipath propagation) is necessary.

Observation 2 : For operation where the UE is assumed to use the same FFT for transmission, independent control of UL transmission timing is not possible

Observation 3 : For network synchronization, there are sometimes significant additional costs or feasibility aspects to consider for accurate sync, especially for cases where GPS based solutions cannot be used

Observation 4 : For TDD networks, cell phase sync requirements are met at the basestation antenna connectors

Observation 5 : As neither network nor UE has control over propagation conditions, a tighter syncronisation requirement at the UE antenna connector implies restricted deployment usability for DAPS handover

Based on the observations and discussion in RAN4#94e, it seems that there are at least 2 possible views of syncronisation for DAPS handover, depending on perspective

View 1 : Syncronisation between cells at the UE significantly better than the CP duration is necessary to allow single FFT architectures for reception and transmission

View 2 :  Syncronisation between cells at the UE should allow for DAPS handover to be used in different deployment scenarios considering a maximum cell size such as 10km

Both view 1 and view 2 correspond to what has been described previously as synchronous operation by 3GPP and RAN4; asynchronous operation would consider the case of arbitrary slot offsets, e.g. ±½ slot or 0.5ms in case of 15kHz SCS. When RAN2 asked RAN4 whether syncrounous and asyncronous DAPS handover was feasible, there was no discussion on the meaning  of sycnronous, although the majority of companies were discussing feasibility from the point of view of UE implementation, i.e. view 1.

Our main concern now that RAN4 has entered the specification phase is that the definition of synchronous DAPS handover is clearly specified. Requirements currently have a note such as  “Note 3:	It is assumed that source cell and target cell are synchronous”.  We would comment that this note does not clarify the definition of synchronous, and as it stands it is rather unlikely that a network vendor could confidently implement DAPS handover if different UE chipsets have different definitions of synchronous, and in any rate it is not clear in which deployments DAPS handover could be used. Therefore, we think it is necessary that RAN4 discusses, and specifies a suitable definition of synchronous for this scenario.

3 approaches have been identified for the specification of synchronous DAPS handover. In addition, we assume that there may be an asynchronous DAPS capability and we do not discuss this further here, other than to say that naturally if a UE supports asynchronous DAPS handover the question for the boundary between sycnronous and asynchronous becomes less critical other than to say that there may be different RAN4 requirements for both types of handover. However, we restrict the discussion to the case of UEs which do not support asynchronous operation, at least for the handover type (intra, inter etc.) being considered. The 3 identified alternatives are:

Alt-1 : UE shall support DAPS handover with “loose sync” e.g. ≥33uS time difference

Alt-2 : Investigate feasibility to specify “tight sync” or “loose” sync DAPS handover capabilities for intra-f/intra-band HO.

Alt-3 : Accept the limited use cases for “tight-sync” DAPS handover and that there are many scenarios where such sync cannot be guaranteed. In those cases the NW would fall back to legacy handover

Alternatives 1 and 3 are fairly self-explanatory. Alt-1 would clearly be preferred by operators and network vendors, since it allows more widespread use of DAPS handover in different deployments. Alt-3 could be preferred to allow simpler UE implementation under the assumption that tight-sync is good enough to allow single-FFT implementation of the downlink receiver.

Under alternative 2, the support for loose or tight sync would be a UE capability. In DAPS handover between nodes which are known to be sufficiently synchronized, the network would be able perform DAPS handover with either type of UE. Between less well synchronized nodes, the network would only be able to perform DAPS handover with UEs supporting “loose sync DAPS handover”, and would need to fall back to legacy (R15) handover procedures for less capable UEs only supporting tightly synchronized  DAPS handover. As the mobility enhancement work item is drawing to a close and RAN4 has provided a preliminary view of UE capabilities for DAPS handover, it may be difficult to conclude on alt-2. Moreover, alt-2 only makes sense if there is a realistic possibility that both types of UE will be developed in future. For these reasons, we do not expect that such capability will be introduced, and alt-2 is more of a theoretical possibility than a likely outcome of release 16. Hence, we expect the main discussion to be between alt-1 and alt-3, although alternative 2 would be a useful approach if different UE designs are anticipated.

Proposal 1 : RAN4 should discuss alternatives for syncronisation accuracy in DAPS handover, and explicitly capture the condition in which synchronous DAPS handover is expected to succeed as a side condition of requirements in 38.133

Imbalance assumption

Similarly to the discussion on syncronisation, the imbalance at UE antenna connector depends on two factors

- The imbalance of transmission power at the gNB antenna connectors relative to each other

- The propagation condition between the gNB and the UE, which is not controlled by the basestation

Hence, the issue is also related to deployment considerations. However, there are a few other factors as well:

- For intrafrequency handover, handovers are performed when the UE is at the cell edge, so SS-RSRP of each cell should be similar, and average imbalance should be relatively small. This is not the case for interfrequency HO, which may be performed e.g. for load balancing.

- The consequence of a large imbalance after the DAPS handover is started may not result in complete failure of the handover, for example if the source cell is lost (starts to have large negative imbalance) after the target cell is added, the source release procedure may fail but still the UE may maintain connection to the target cell. Indeed this is the main robustness improvement of DAPS handover.

There has been some discussion and possible confusion on the reason for specifying an imbalance requirement. In our understanding, this requirement is not a demodulation requirement, in which it should be possible to receive both source and target cell simultaneously, since the purpose of DAPS handover is to maintain connectivity to source and target protocol stack so that the failure of either link does not cause the connection to be lost. The network will add the target cell based on some criteria such as an event triggered measurement report, which marks the beginning of the DAPS handover.  After this event and addition, the network has no control over power imbalance, as indeed it has no control over the power imbalance during a legacy handover procedure. As indicated above, even if the imbalance becomes very large the DAPS handover should not fail

- If the source cell (from UE perspective) disappears the UE will no longer be successfully scheduled on the PDCCH of the source cell until it receives the command to release the source. For a period of time the network may not be aware that the source cell has disappeared from the UE’s perspective and may attempt to schedule the UE. However, in this case the UE can still be scheduled by the target cell and there is no impact on data interruption.

- If the DAPS handover fails (target cell connection fails prior to random access completion to the target), then according to RAN2 agreements and running CR, the UE reverts back to original  configuration with the source cell prior to the handover.

- Upon DAPS handover failure, UE reverts back to the original source configuration (including RLC and PDCP state, but do not re-establish PDCP and RLC) for the DRB that is not configured with DAPS. - Upon DAPS HO failure, source RRM configuration is reversed as legacy HO failure. But no specification change is needed.

- At any rate, the RRC reestablishment procedure can be expected to recover from failure as for release 15 handover.

Since either source or target connection may fail during the DAPS handover procedure (from RRC message implying DAPS handover until RRC message implying DAPS source cell release), we do not find a strong motivation to specify imbalance. Clearly requirements in 38.133 will apply only for a successful DAPS handover, however the same is true for a legacy handover which may result in an RRC reestablishment. For DAPS handover failure may result in fallback to the original cell that initiated the handover, or RRC reestablishment.

Proposal 2 : The power imbalance during DAPS handover is not specified in 38.133

# Conclusions

In this contribution, we discuss syncronisation and power imbalance for DAPS handover.

3 approaches have been identified for the specification of synchronous DAPS handover. In addition, we assume that there may be an asynchronous DAPS capability and we do not discuss this further here, other than to say that naturally if a UE supports asynchronous DAPS handover the question for the boundary between sycnronous and asynchronous becomes less critical other than to say that there may be different RAN4 requirements for both types of handover. However, we restrict the discussion to the case of UEs which do not support asynchronous operation, at least for the handover type (intra, inter etc.) being considered. The 3 identified alternatives are:

Alt-1 : UE shall support DAPS handover with “loose sync” e.g. ≥33uS time difference

Alt-2 : Investigate feasibility to specify “tight sync” or “loose” sync DAPS handover capabilities for intra-f/intra-band HO.

Alt-3 : Accept the limited use cases for “tight-sync” DAPS handover and that there are many scenarios where such sync cannot be guaranteed. In those cases the NW would fall back to legacy handover

Proposal 1 : RAN4 should discuss alternatives for syncronisation accuracy in DAPS handover, and explicitly capture the condition in which synchronous DAPS handover is expected to succeed as a side condition of requirements in 38.133

Proposal 2 : The power imbalance during DAPS handover is not specified in 38.133

# References

- R4-2002224	WF on NR Mobility Enhancements RRM requirements	Intel