3GPP TSG-RAN WG4 Meeting #94bis-e                                                     R4-2003205

Electronic Meeting, 20 April – 30 April, 2020

Agenda item:       6.15.1.8

Source:                Intel Corporation

Title:                     Discussion on requirements for spatial relation info switch

Document for:     Discussion

# Introduction

In last meeting, there are some agreements about uplink spatial relation info switch:

- Define MAC CE based spatial relation info switching delay for PUCCH - Define RRC based spatial relation info switching delay for P-SRS - Define DCI based spatial relation info switching delay for A-SRS - When spatial relation info associated with DL-RS - For unknown TCI state, there is no requirement - For known TCI state, refer to RAN1 spec.

In this contribution we continue to provide our views on requirements for uplink spatial relation info switch.

# Discussion

## PUCCH spatial relation switch

In last meeting, it’s agreed that MAC CE based spatial relation info switching delay for PUCCH will be defined. There are still many open issues:

- Delay requirement for MAC CE based spatial relation info switching associated with SRS for PUCCH - Option 1: THARQ +3ms - Option 2: Deprioritize - Option 3: Refer to RAN1 requirement - Delay requirement for MAC CE based spatial relation info switching associated with DL-RS for PUCCH - For known TCI state - Option 1: THARQ +3ms - Option 2: THARQ +3ms + time for time tracking if applicable - Option 3: UE to switch beams at THARQ + 3ms (RAN1 specification). Performance guaranteed (in terms of timing etc) after reception 1 SSB(similar to TCI state switching) - For unknown TCI state - Option 1: THARQ + 3ms + TL1-RSRP - Option 2: THARQ + 3ms + TL1-RSRP + time for time tracking if applicable - Option 3: No requirement - Whether to define delay requirement for RRC based spatial relation info switching with DL-RS for PUCCH

For the case when MAC CE based spatial relation of PUCCH is associated with UL SRS, the UE shall transmit on the same TX beam as used for SRS transmission. the switching delay would depend on DL command decoding time, which is THARQ +3ms.

Proposal 1: Delay requirement for MAC CE based spatial relation info switching associated with SRS for PUCCH is THARQ +3ms.

MAC CE based TCI state switching is defined for known and unknown TCI state and the same would apply to spatial relation switch associated with DL RS. For TCI state switching, additional time is needed for time tracking if the target TCI state doesn’t belong to the active TCI state list tracked by the UE. However, for UL spatial relation switch associated with DL RS, additional time for DL time tracking is not needed. Since only the uplink spatial relation will be changed and there is no impact on the DL timing.

UL timing is dependent on the DL timing. If new UL timing is applied according to the DL timing tracking, then in one UL slot there may be multiple UL timings which are hard for UE to implement. If the UL timing is not changed according to the DL timing tracking, it’s not necessary to track the DL timing. Hence MAC CE based spatial relation info switching delay associated with DL RS could be defined as 									THARQ +3ms for known TCI state										THARQ + 3ms + TL1-RSRP for unknown TCI state

Since some companies have concern on the DL timing tracking, it’s suggested to discuss whether DL timing tracking is needed further.

Proposal 2: MAC CE based spatial relation info switching delay for PUCCH associated with DL RS is defined as: 		THARQ +3ms for known TCI state

THARQ + 3ms + TL1-RSRP for unknown TCI state

Proposal 3:  Discuss whether DL timing tracking is needed when MAC CE based spatial relation info switching is associated with DL-RS for PUCCH.

## PUSCH spatial relation switch

There is some consensus about the requirement for PUSCH spatial relation switch:

- No requirements are defined for spatial relation info switching delay for PUSCH

From RAN1 spec, for PUSCH configured with DCI format 0_0, the Tx beam is based on the PUCCH resource with lowest resource ID; for PUSCH scheduled using DCI format 0_1, the spatial relation is given by SRI (SRS Resource Indicator). There is no need to define requirement for PUSCH spatial relation switch in RAN4.

Proposal 4: Don’t define PUSCH spatial relation switching requirement.

## SRS spatial relation switch

For SRS spatial relation switch, there are some open issues:

- Delay requirement for RRC based spatial relation info switching associated with DL-RS for P-SRS - For known TCI state - Option 1: Define delay based on RRC based TCI state switching requirements - Option 2: TRRCprocessing (timing is not required) - Option 3: No requirements - For unknown TCI state - Option 1: Define delay based on RRC based TCI state switching requirements - Option 1a: TRRCprocessing + TL1-RSRP - Option 2: No requirements - Delay requirement for RRC based spatial relation info switching associated with SRS for P-SRS - Option 1: TRRCprocessing - Option 2: No requirements - Whether to define delay requirement for MAC CE based spatial relation info switching with DL-RS for SP-SRS - Option 1: Yes - Option 2: No

For delay requirement for RRC based spatial relation info switching associated with DL-RS for P-SRS, similar with the analysis for spatial relation info switching associated with DL-RS for PUCCH, no DL tracking timing is needed. The only delay time is RRC processing time which has been defined in RAN2. There is no other extra delay time needed in RAN4.

For delay requirement for RRC based spatial relation info switching associated with SRS for P-SRS, UE can directly use the SRS TX beam, then the spatial relation switching delay is TRRC_processing. No additional timing is required except for RRC processing time as well. Therefore, it’s suggested to not define requirement in RAN4 for this case.

Proposal 5: For delay requirement for RRC based spatial relation info switching associated with SRS or DL-RS for P-SRS, no requirement is defined in RAN4.

The spatial relation info for SP-SRS could be indicated via MAC-CE. In case of SP-SRS, MAC CE is configuring the spatial relation info for SRS transmission and not switching. Hence, MAC CE based spatial relation info switch requirements could not be defined for SP-SRS.

Proposal 6: For SP-SRS, MAC CE is configuring the spatial relation info rather than switching, prefer to not define requirement for this case.

## UL signal has spatial relation to an unknown TCI-state

There is still open issues for UL signal which has spatial relation to an unknow TCI-state:

- UE behavior of when the UL signal has spatial relation to an unknown TCI-state - Option 1: UE transmits with previous TX beam/ arbitrary beam - Option 1a: UE transmits using previous TX beam - Option 2: Up to UE implementation - Option 3: UE behaviour shall not be specified for the period until TCI state becomes known.

when the UL signal has spatial relation to an unknown TCI-state, the UE might not know the suitable spatial filter to use and network might not be able to receive the UL transmission in the slot scheduled for uplink transmission of SRS. Additional time is needed for UE to perform RX beam sweep in order to determine suitable RX beam for the DL RS and use it for uplink transmission. the UE should also be able to drop the transmission on the UL until suitable RX beam is determined. Another option would be to use the best-known beam or an arbitrary beam for transmission. Among the several options, drop UL transmission until TCI is known seems more reasonable.

Proposal 7: When UL transmission is configured with spatial relation info associated with DL RS and the TCI state of the DL RS is unknown, the UE shall drop UL transmission until TCI state is known.

# Conclusion

In this paper we provide our views on introducing requirements for UL spatial relation info switching. Our observations and proposals are captured below:

Proposal 1: Delay requirement for MAC CE based spatial relation info switching associated with SRS for PUCCH is THARQ +3ms.

Proposal 2: MAC CE based spatial relation info switching delay for PUCCH associated with DL RS is defined as: 		THARQ +3ms for known TCI state

THARQ + 3ms + TL1-RSRP for unknown TCI state

Proposal 3:  Discuss whether DL timing tracking is needed when MAC CE based spatial relation info switching is associated with DL-RS for PUCCH.

Proposal 4: Don’t define PUSCH spatial relation switching requirement.

Proposal 5: For delay requirement for RRC based spatial relation info switching associated with SRS or DL-RS for P-SRS, no requirement is defined in RAN4.

Proposal 6: For SP-SRS, MAC CE is configuring the spatial relation info rather than switching, prefer to not define requirement for this case.

Proposal 7: When UL transmission is configured with spatial relation info associated with DL RS and the TCI state of the DL RS is unknown, the UE shall drop UL transmission until TCI state is known.

# Reference