3GPP TSG-RAN WG4 Meeting #94bis-e                                                         R4-2003122

Online, 20th April-30th April, 2020

Source:	Samsung, KDDI

Title:	TP for TR 37.716-21-11: DC_1-41_n3

Agenda item:			8.4.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.716-21-11 to include DC_1-41_n3 according to the request in [1].

# 2. Reference

- RP-200271, Revised WID on Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL).

# 3. Text Proposal

<Start of Text Proposal>

## 5.1.x	DC_1-41_n3

### 5.1.x.1		Operating bands for DC

Table 5.1.x.1-1: Band combination EN-DC (three bands)

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-41_n3 | CA_1-41 | n3 | No |

### 5.1.x.2	Configuration for DC

Table 5.1.x.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-41A_n3A | DC_41A_n3A | CA_1A-41A | n3A |
| DC_1A-41C_n3A | DC_41A_n3A DC_41C_n3A | CA_1A-41C | n3A |

### 5.1.x.3	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes, there is no harmonic or IMD issues for this band combination.

### 5.1.x.4	∆TIB and ∆RIB values

For DC_1-41_n3, the TIB,c and RIB,c values are given in the tables below. Numbers come from LTE CA_1A-3A-41A

Table 5.1.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-41_n3 | 1 | 0.5 |
|  | 41 | 0.31/0.82 |
|  | n3 | 0.5 |
| NOTE 1: The requirement is specified for the frequency range of 2545-2690MHz. NOTE 2: The requirement is specified for the frequency range of 2496-2545MHz. |  |  |

Table 5.1.x.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-41_n3 | 1 | 0 |
|  | 41 | 01/0.52 |
|  | n3 | 0 |
| NOTE 1: The requirement is specified for the frequency range of 2545-2690MHz. NOTE 2: The requirement is specified for the frequency range of 2496-2545MHz. |  |  |

### 5.1.x.5	REFSENS requirements

There is no additional requirement for this band combination.

<End of Text Proposal>