3GPP TSG-RAN WG4 Meeting #94bis-e                                                    R4-2003127

Online, 20th April-30th April, 2020

Source:	Samsung, KDDI

Title:	TP for TR 37.716-21-11: DC_3-41_n77-n77

Agenda item:			8.4.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.716-21-11 to include DC_3-41_n77-n77 according to the request in [1].

# 2. Reference

- RP-200271, Revised WID on Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL).

# 3. Text Proposal

<Start of Text Proposal>

## 5.1.4	DC_3-41_n77 and DC_3-41_n77-n77

### 5.1.4.1	Operating bands for DC

Table 5.1.4.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-41_n77 | CA_3-41 | n77 | DC_3_n77 |
| DC_3-41_n77-n77 | CA_3-41 | CA_n77-n77 | DC_3_n77 |

### 5.1.4.2	Configurations for DC

Table 5.1.4.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A-41A_n77A | DC_3A_n77A DC_41A_n77A | CA_3A-41A | n77A |
| DC_3A-41C_n77A | DC_3A_n77A DC_41A_n77A | CA_3A-41C | n77A |
| DC_3A-41A_n77(2A) | DC_3A_n77A DC_41A_n77A | CA_3A-41A | CA_n77(2A) |
| DC_3A-41C_n77(2A) | DC_3A_n77A DC_41A_n77A DC_41C_n77A | CA_3A-41C | CA_n77(2A) |

### 5.1.4.3	Co-existence studies

Based on co-existence studies of Band 3 + Band n77 and Band-41 + Band n77 captured in TR 37.863-01-01, own Rx impact of the 3rd band is the followings.

-	IMD generated by dual uplink of Band 3 + Band n77 doesn’t fall into own Rx of band 41.

-	3rd order IMD generated by dual uplink of Band 41 + Band n77 may fall into own Rx of band 3.

### 5.1.4.4	∆TIB and ∆RIB values

Table 5.1.4.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-41-n77 DC_3-41_n77-n77 | 3 | 0.6 |
|  | 41 | 0.31 |
|  |  | 0.82 |
|  | n77 | 0.8 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

Table 5.1.4.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-41-n77 DC_3-41_n77-n77 | 3 | 0.2 |
|  | 41 | 01 |
|  |  | 0.52 |
|  | n77 | 0.5 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

### 5.1.4.5	REFSENS requirements

Table 5.1.4.5-1 lists the required MSD test configurations and Table 5.1.4.5-2 proposed the MSD levels from interested companies for the EN-DC 3A-41A_n77A, EN-DC 3A-41C_n77A, EN-DC_3A-41A_n77(2A) and EN-DC_3A-41C_n77(2A) band combination.

Table 5.1.4.5-1: MSD test configurations

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-41A_n77A DC_3A-41C_n77A DC_3A-41A_n77(2A) DC_3A-41C_n77(2A) | 3 | IMD5 | ¦3*fB3 -2*fn77¦ | 1720 | 5 | 25 | 1815 | 5 |
|  | n77 |  |  | 3900 | 10 | 52 | 3900 | 10 |
|  | 41 |  |  | 2640 | 5 | 25 | 2640 | 5 |
|  | 41 | IMD3 | ¦2*fB41 –fn77¦ | 2620 | 5 | 25 | 2620 | 5 |
|  | n77 |  |  | 3400 | 10 | 52 | 3400 | 10 |
|  | 3 |  |  | 1745 | 5 | 25 | 1840 | 5 |
|  | 41 | IMD4 | ¦2*fB41 –2*fn77¦ | 2520 | 5 | 25 | 2520 | 5 |
|  | n77 |  |  | 3445 | 10 | 52 | 3445 | 10 |
|  | 3 |  |  | 1755 | 5 | 25 | 1850 | 5 |

Table 5.1.4.5-2: MSD results from each company

| E-UTRA and NR Band |  |  |  |
| --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | MSD (LGE) | Source of IMD |
| DC_3A-41A_n77A DC_3A-41C_n77A DC_3A-41A_n77(2A) DC_3A-41C_n77(2A) | 3 | 5.3 | IMD5 ¦3*fB3-2*fn77¦ |
|  | n77 |  |  |
|  | 41 |  |  |
|  | 41 | 16.4 | IMD3 ¦2*fB41 - fn77¦ |
|  | n77 |  |  |
|  | 3 |  |  |
|  | 41 | 10.9 | IMD4 ¦2*fB41 - 2*fn77¦ |
|  | n77 |  |  |
|  | 3 |  |  |

Table 5.1.4.5-3 propose the required MSD levels for the dual connectivity configuration as average manner.

Table 5.1.4.5-3: Proposed MSD requirements

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | CF (dB) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-41A_n77A DC_3A-41C_n77A DC_3A-41A_n77(2A) DC_3A-41C_n77(2A) | 3 | IMD5 | ¦3*fB3 -2*fn77¦ | 1720 | 5 | 25 | 1815 | 5 | 0.3 | N/A |
|  | n77 |  |  | 3900 | 10 | 52 | 3900 | 10 |  |  |
|  | 41 |  |  | 2640 | 5 | 25 | 2640 | 5 |  | 5.3 |
|  | 41 | IMD3 | ¦2*fB41 –fn77¦ | 2620 | 5 | 25 | 2620 | 5 | 1.4 | N/A |
|  | n77 |  |  | 3400 | 10 | 52 | 3400 | 10 |  |  |
|  | 3 |  |  | 1745 | 5 | 25 | 1840 | 5 |  | 16.4 |

<End of Text Proposal>