3GPP TSG-RAN WG4 Meeting #94bis-e                                                       R4-2013167

Online, 20th April-30th April, 2020

Source:	Samsung, Verizon

Title:	TP for TR 37.716-21-11: DC_2-48_n66

Agenda item:			8.4.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.716-21-11 to include DC_2-48_n66 according to the request in [1].

# 2. Reference

- RP-200271, Revised WID on Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL).

# 3. Text Proposal

<Start of Text Proposal>

## 5.1.x	DC_2-48_n66

### 5.1.x.1	Operating bands for DC

Table 5.1.x.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-48_n66 | CA_2-66 | n48 | No |

### 5.1.x.2	Configurations for DC

Table 5.1.x.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A-48A_n66A | DC_2A_n66A DC_48A_n66A | CA_2A-48A | n66A |

### 5.1.x.3	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes, and the impact to own Rx of the 3rd band is shown as the followings.

- 2nd order IMD generated by dual uplink of Band 2 + Band n66 may fall into own Rx of band 48, it can reuse the MSD of DC_2A_n66A-n78A.

- 2nd order IMD generated by dual uplink of Band 48+ Band n66 may fall into own Rx of band 2, it can reuse the MSD of DC_2A-66A_n48A.

### 5.1.x.4	∆TIB and ∆RIB values

For DC_2-48_n66, the TIB,c and RIB,c values reused the values for DC_2A_n48A-n66A, as given in the tables below.

Table 5.1.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-48_n66 | 2 | 0.6 |
|  | 48 | 0.8 |
|  | n66 | 0.6 |

Table 5.1.x.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-48_n66 | 2 | 0.3 |
|  | 48 | 0.5 |
|  | n66 | 0.3 |

### 5.1.x.5	REFSENS requirements

Table 5.1.x.5-1 shows the required MSD levels for the DC configuration.

Table 5.1.x.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_2A-48A_n66A | 2 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | 48 | 3620 | 10 | 50 | 3620 | 29.4 | FDD | IMD2 ¦fB2+Bn66¦ |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | TDD | N/A |
| DC_2A-48A_n66A | 2 | 1880 | 5 | 25 | 1960 | 28.3 | FDD | IMD2 ¦fBn48-fB66¦ |
|  | 48 | 3695 | 5 | 25 | 3695 | N/A | FDD | N/A |
|  | n66 | 1735 | 5 | 25 | 2135 | N/A | TDD | N/A |

<End of Text Proposal>