3GPP TSG-RAN WG4 Meeting #94bis-e                                R4-2003172

Online, 20th April-30th April, 2020

Source:	Samsung

Title:	Discussion on how to handle the NE-DC band combinations

Agenda item:			4.1

Document for:	Approval

- Introduction

Currently, there is no related WID for NE-DC band combination and no procedure agreed on how to add the new NE-DC band combinations. In this contribution, we discussed how to handle the new NE-DC band combinations for inter-band NE-DC within FR1 and inter-band NE-DC including FR2 considering corresponding NE-DC is requested by market.

# 2. Discussion

In RAN4 #91 meeting, inter-band NE-DC within FR1 feature was introduced into TS 38.101-3 and one example band combination case DC_n1A_28A was captured into Spec 38.101-3 [1]. Table 2-1 summary the requirements for inter-band NE-DC within FR1.

Table 2-1 The requirements of inter-band NE-DC within FR1

| Requirements |  |
| --- | --- |
| 5.5B.4a Configurations for inter-band NE-DC within FR1 | Need introduce respectively for each case |
| 6.2B.1.3a UE maximum output power for inter-band NE-DC within FR1 | Need introduce respectively for two bands UL NE-DC |
| 6.2B.2.3a UE maximum output power reduction for inter-band NE-DC within FR1 | Apply E-UTRA requirements and NR requirements respectively |
| 6.2B.4.1.3a Configured output power level for inter-band NE-DC within FR1 | Already defined for NE-DC |
| 6.2B.4.2.3a ΔTIB,c for inter-band NE-DC within FR1 | Apply the requirements of corresponding specified EN-DC configuration or specific NE-DC exceptions |
| 6.3B.1a Output power dynamics for NE-DC with UL sharing from UE perspective | Apply E-UTRA requirements and NR requirements respectively |
| 6.4B.1.3a Frequency error for inter-band NE-DC within FR1 |  |
| 6.4B.2.3a Transmit modulation quality for Inter-band NE-DC within FR1 |  |
| 6.5B.2.3a Out-of-band emissions for inter-band NE-DC within FR1 |  |
| 6.5B.3.3a.1 General spurious emissions for inter-band NE-DC within FR1 | Apply E-UTRA requirements and NR requirements respectively |
| 6.5B.3.3a.2 Spurious emission band UE co-existence for inter-band NE-DC within FR1 | Apply the requirements of corresponding specified EN-DC configuration or specific NE-DC exceptions |
| 6.5B.5.3a Transmit intermodulation for inter-band NE-DC within FR1 | Apply E-UTRA requirements and NR requirements respectively |
| 7.3B.2.3a Reference sensitivity for inter-band NE-DC within FR1 | Apply the requirements of correspondingly specified EN-DC configuration or specific NE-DC exceptions |
| 7.3B.3.3a ΔRIB,c forInter-band NE-DC within FR1 | Apply E-UTRA requirements and NR requirements respectively |
| 7.4B.3a Maximum input level for inter-band NE-DC within FR1 | Apply E-UTRA requirements and NR requirements respectively |
| 7.5B.3a Adjacent channel selectivity for inter-band NE-DC within FR1 |  |
| 7.6B.3.3a Out-of-band blocking for inter-band NE-DC within FR1 |  |
| 7.7B.3a Spurious response for inter-band NE-DC within FR1 |  |
| 7.8B.2.3a Wide band Intermodulation inter-band NE-DC within FR1 |  |
| 7.9B.3a Spurious emissions for inter-band NE-DC within FR1 |  |

Based on above summary, the requirements for inter-band NE-DC within FR1 can be classed as

- Update in general section, i.e., NE-DC configurations.

- Band combination specific requirements, i.e., UE maximum output power, ΔTIB,c, ΔRIB,c, Spurious emission band UE co-existence and REFSENs.

- Band combination agnostic requirement for NE-DC, i.e., Pcmax

- Requirement to apply E-UTRA requirements and NR requirements for LTE and NR carriers respectively.

For inter-band NE-DC including FR2, there is no related clause in Spec 38.101-3 currently. According to the requirements of inter-band EN-DC including FR2, Table 2-2 lists what requirements need introduce and how to introduce these requirements for inter-band NE-DC including FR2.

Table 2-2 The needed requirements for inter-band NE-DC including FR2

| Requirements |  |
| --- | --- |
| 5.5B.5a Configurations for for inter-band EN-DC including FR2 | Need introduce respectively for each case |
| 6.2B.1.4a UE maximum output power for inter-band EN-DC including FR2 | Apply E-UTRA requirements and NR requirements respectively |
| 6.2B.2.4a UE maximum output power reduction for inter-band EN-DC including FR2 |  |
| 6.2B.4.1.4a Configured output power level for inter-band EN-DC including FR2 |  |
| 6.2B.4.2.4a ΔTIB,c for inter-band EN-DC including FR2 | Apply the requirements of corresponding specified EN-DC configuration or specific NE-DC exceptions |
| 6.4B.1.4a Frequency error for inter-band EN-DC including FR2 | Apply E-UTRA requirements and NR requirements respectively |
| 6.4B.2.4a Transmit modulation quality for inter-band EN-DC including FR2 |  |
| 6.5B.2.4a Out-of-band emissions for inter-band EN-DC including FR2 |  |
| 6.5B.3.4a Spurious emissions for inter-band EN-DC including FR2 |  |
| 6.5B.3.4a.1 Spurious emission band UE co-existence for inter-band EN-DC including FR2 | Apply the requirements of corresponding specified EN-DC configuration or specific NE-DC exceptions |
| 6.5B.5.4a Transmit intermodulation for inter-band EN-DC including FR2 | Apply E-UTRA requirements and NR requirements respectively |
| 6.6B.4a Beam correspondence for inter-band EN-DC including FR2 | Apply NR FR2 bands requirements |
| 7.3B.3.4a ΔRIB,c for inter-band EN-DC including FR2 | Apply the requirements of corresponding specified EN-DC configuration or specific NE-DC exceptions |
| 7.4B.4a Maximum input level for inter-band EN-DC including FR2 | Apply E-UTRA requirements and NR requirements respectively |
| 7.5B.4a Adjacent channel selectivity for inter-band EN-DC including FR2 |  |
| 7.6B.3.4a Out-of-band blocking for inter-band EN-DC including FR2 |  |
| 7.7B.4a Spurious response for inter-band EN-DC including FR2 |  |
| 7.8B.2.4a Wide band Intermodulation inter-band EN-DC including FR2 |  |
| 7.9B.4a Spurious emissions for inter-band EN-DC including FR2 |  |

Based on above table, we can class the needed requirements for inter-band NE-DC including FR2 as

- Update needed in general section, i.e., NE-DC configurations.

- Band combination specific requirement, i.e., ΔTIB,c, ΔRIB,c, Spurious emission band UE co-existence.

- Requirement to apply E-UTRA requirements and NR requirements for LTE and NR carriers respectively.

Therefore, for the new NE-DC band combinations request for inter-band NE-DC within FR1 and inter-band NE-DC including FR2, the requirements could handle in two different cases:

Case 1: with correspondingly EN-DC configurations have been completed:

- Only update needed is to introduce NE-DC configurations and UE maximum output power for inter-band NE-DC within FR1

- Only update needed is to introduce NE-DC configurations for inter-band NE-DC including FR2.

Case 2: without correspondingly EN-DC configurations, ΔTIB,c, ΔRIB,c, Spurious emission band UE co-existence and REFSENs also need further study.

Hence, when correspondingly EN-DC configurations have been completed, there is no necessity to repeat the study and conclusion of the correspondingly specified EN-DC configuration for the NE-DC configurations.

To offload RAN4 work, we proposed:

Proposal 1: If corresponding EN-DC band combination has already been defined in specification, the NE-DC band combinations can be introduced into the Spec 38.101-3 directly for inter-band NE-DC within FR1 and inter-band NE-DC including FR2.

Proposal 2: When purely new NE-DC comes to RAN4 new WID can be considered accordingly.

- Conclusion

In this contribution, we discussed and summarized the requirements for inter-band NE-DC, and proposed

Proposal : If corresponding EN-DC band combination has already been defined in specification, the NE-DC band combinations can be introduced into the Spec 38.101-3 directly for inter-band NE-DC within FR1 and inter-band NE-DC including FR2.

Proposal 2: When purely new NE-DC comes to RAN4 new WID can be considered accordingly.

The related draft CR introducing new inter-band NE-DC band combinations corresponding EN-DC band combinations have already been defined in specification and necessary requirements into 38.101-3 are available [2][3].

# Reference

[1] R4-1907808 Draft CR to 38.101-3 NE-DC introduction, InterDigital Communications

[2] R4-2003173, Draft CR for 38.101-3 to introduce new inter-band NE-DC band combinations including FR2, Samsung

[3] R4-2003174, Draft CR for 38.101-3 to introduce new inter-band NE-DC band combinations within FR1, Samsung