3GPP TSG RAN WG4 Meeting #94-e BIS                   	R4-2003235

Online, 20th – 30th April, 2020

Agenda Item:	6.4.4

Source:	LG Electronics

Title:	TP on remaining issues for NR V2X UE RF requirements

Document for:	Approval

- Introduction

In the last RAN4 #94-e meetings, RAN4 agreed the NR V2X endorsed draft CR and most UE RF core requirements are agreed for single CC V2X operating and inter-band con-current V2X operating for 5G V2X service.

Based on last RAN4 e-meeting, the following items are listed up to cover the open issues for NR V2X UE RF requirements.

- Switching period and interruption position between LTE SL and NR SL at n47

  - In Switched period between case1 and case2, the follow switching period were agreed in last RAN4 meeting

    - For SUL and UL-CA UE: {35us, 140us, 210us} or {1,4, and 6} OFDM symbols for 30 kHz SCS

    - For EN-DC UE: {35us, 140us} or {1,4} OFDM symbols for 30 kHz SCS

      - Based on the agreements, RAN4 can remove the 210us for NR V2X TDM operation between LTE SL and NR SL at n47.

- MPR for S-SSB transmission : RAN1 agreed the transmission power as follow

  - The transmission power is same for S-PSS, S-SSS, and PSBCH symbol

- NR V2X_nX-n47 treatment  Change to NR V2X_n71-n47

- Remove [ ] in the concreted RF requirements

So, we propose updated TP contents to reflect the above agreements.

****************** Start of the TP in clause 8 of TR38.886 ************************

7	Operating bands and channel arrangement

7.1	Operating bands

7.1.1	Operating bands in FR1

NR V2X communication is designed to operate in the operating bands in FR1 defined in Table 7.1.1-1.

Table 7.1.1-1 NR V2X operating bands in FR1

| NR V2X Operating Band | Sidelink (SL) Transmission operating band |  |  | Sidelink (SL) Reception operating band |  |  | Duplex Mode | Interface |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |  |
| n381 | 2570 MHz | - | 2620 MHz | 2570 MHz | - | 2620 MHz | TDD | PC5 |
| n47 | 5855 MHz | - | 5925 MHz | 5855 MHz | - | 5925 MHz | TDD | PC5 |
| Note 1: n38 operating restriction will be captured based on agreement in R4-1913959 |  |  |  |  |  |  |  |  |

7.1.2	Operating bands in FR2

7.1.3	Operating bands for inter-band con-current operation in FR1

NR V2X operation is designed to operate concurrent with NR uplink/downlink on the operating bands combinations listed in Table 7.1.3-1.

Table 7.1.3-1: NR V2X inter-band con-current operation

| V2X con-current operating Band | V2X Band | Interface |
| --- | --- | --- |
| NR V2X_n71-n47 | n71 | Uu |
|  | n47 | PC5 |
| NOTE 1: NR licensed Band is operated for legacy NR operation and/or V2X service |  |  |

## 7.2	Channel bandwidth

### 7.2.1	Channel bandwidth in FR1

The NR V2X Communication channel bandwidths and operating bands are shown in Table 7.2.1-1. The same (symmetrical) channel bandwidth is specified for both the TX and RX path.

Table 7.2.1-1 NR V2X Communication channel bandwidth

|  |  | NR V2X band / V2X channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR V2X Operating Band | SCS kHz | 10 MHz | 20 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n38 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 30 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 60 | Yes | Yes | Yes | Yes |  |  |  |  |  |
| n47 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 30 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 60 | Yes | Yes | Yes | Yes |  |  |  |  |  |

### 7.2.2	Channel bandwidth in FR2

### 7.2.3	Channel bandwidth for inter-band con-current operation in FR1

For NR V2X inter-band con-current operation in FR1, the NR V2X channel bandwidths for each operating band is specified in Table 7.2.3-1.

Table 7.2.3-1: NR V2X inter-band con-current configurations

| V2X con-current operating band Configuration | NR Bands | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 30 MHz | 40 MHz | 50 MHz | Maximum bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR V2X_nXA-n47A | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  | 60 | 0 |
|  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |
|  | n47 | 15 |  | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  | 30 |  | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  | 60 |  | Yes |  | Yes | Yes | Yes |  |  |  |

<Unchanged sections are omitted>

8	Transmitter characteristics

8.1	NR V2X UE Tx requirements in FR1

When a NR V2X UE is TDM operating between NR SL and LTE SL at n47, the NR V2X UE need to satisfy the individual V2X Tx requirements of each RAT.

<Unchanged sections are omitted>

When UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions for NR V2X operating bands specified in Table 8.1-1, this subclause specifies the allowed Maximum Power Reduction (MPR) power for NR V2X physical channels and signals due to PSCCH and PSSCH simultaneous transmission.

8.1.2.1	MPR for Power class 3 V2X UE

For contiguous allocation of PSCCH and PSSCH simultaneous transmission, the allowed MPR for the maximum output power for NR V2X physical channels PSCCH and PSSCH shall be specified as inner/outer RB allocations in Table 8.1.2.1-1 for power class 3.

Table 8.1.2.1-1: Maximum Power Reduction (MPR) for power class 3 NR V2X (Contiguous PSCCH and PSSCH transmission)

| Modulation |  | Channel bandwidth/MPR (dB) |  |
| --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |
| CP-OFDM | QPSK/ 16QAM | ≤ [4.0] | ≤ [2.0] |
|  | 64 QAM | ≤ [4.5] | ≤ [3.5] |
|  | 256 QAM | ≤ [6.0] |  |

Where the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations:

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1 in TS38.101-1.

RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

LCRB  ≤  ceil(NRB/2)

where ceil(x) is the smallest integer greater than or equal to x.

For non-contiguous allocation for simultaneous PSFCH transmission for NR V2X will be specified as follow

MPR_PSFCH = CEIL {MA_PSFCH, 0.5}

Where MA is defined as follows

MA_PSFCH =	TBD		; 0.00< A ≤ TBD

TBD		; TBD< A ≤TBD

TBD		; TBD< A ≤1.00

Where

A = NRB_alloc / NRB.

CEIL{MA, 0.5} means rounding upwards to closest 0.5dB.

The allowed A-MPR for the maximum output power for V2X physical channels on S-SSB shall be applied the legacy NR Uu requirements in subclause 6.2.3 in TS38.101-1 for the corresponding modulation and transmission bandwidth.

.

<Unchanged sections are omitted>

### 8.1.4	Configured transmitted power for NR V2X UE

The NR V2X UE is allowed to set its configured maximum output power PCMAX,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,c is set within the following bounds:

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c with

PCMAX_L,f, c = MIN {PEMAX,c – TC,c,  (PPowerClass – ΔPPowerClass) –– MAX(MAX(MPRc , A-MPRc)+ ΔTIB,c + TC,c + ∆TRxSRS, P-MPRc), PRegulatory,c }

PCMAX_H,f, c = MIN {PEMAX,c,  (PPowerClass– ΔPPowerClass),  PRegulatory,c }

where

- For the total transmitted power PCMAX,c of PSSCH and PSCCH, PEMAX,c is the value given by IE maxTxPower, defined by [TS 38.331], when the UE is not associated with a serving cell on the NR V2X carrier .

-	For PCMAX,S-SSB, PEMAX,c is the value given by the IE maxTxPower in [TS 38.331] when the UE is not associated with a serving cell on the V2X carrier.

- PPowerClass is the maximum UE power specified in Table 6.2.1-1 in TS38.101-1 without taking into account the tolerance specified in the Table 6.2.1-1 in TS38.101-1;

-	 MPRc and A-MPRc for serving cell c are specified in subclause 6.2.2 and subclause 6.2.3 in TS38.101-1, respectively;

-	TIB,c, TC,c, ∆TRxSRS, PPoweclass and P-MPRc are specified in subclause 6.2.4 in TS38.101-1

- PRegulatory,c= 10 dBm when the CEN DSRC tolling system is nearby NR V2X UE; PRegulatory,c= 33 dBm otherwise.

The maximum output power PCMAX,PSSCH and PCMAX,PSCCH are derived from PCMAX,c based on 0dB offset between PSSCH and PSCCH.

For the measured configured maximum output power PUMAX,c for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions, the same requirement as in subclause 6.2.4 in TS38.101-1 shall be applied.

<Unchanged sections are omitted>

8.1.7	ON/OFF time mask for NR V2X UE

For NR V2X service, additional requirements on ON/OFF time masks for V2X physical channels and signals are specified in this clause.

#### 8.1.7.1		General time mask for NR V2X UE

The General ON/OFF time mask defines the observation period between the Transmit OFF and ON power and between Transmit ON and OFF power for PSCCH, and PSSCH transmissions in a subframe wherein the last symbol is punctured to create a guard period.

Figure 8.1.7.1-1: General PSCCH/PSSCH time mask for NR V2X UE

#### 8.1.7.2	S-SSB time mask

The S-SSB time mask for NR V2X UE defines the observation period between transmit OFF and ON S-PSS power and between transmit ON PSBCH and OFF power in a slot, with last symbol punctured to create a guard period.

Figure 8.1.7.2-1: S-SSB time mask for NR V2X UE

#### 8.1.7.3		Additional Time mask for TDM operation between NR SL and LTE SL at n47

When a NR V2X UE is operated with TDM between NR SL and LTE SL at n47 without dual PA capability, the maximum UL switching time is defined as [150] us and SL reception interruption is allowed during UL switching time masks in Figure 8.1.7.3-1 and Figure 8.1.7.3-2 shall apply.

RAN4 will not specify the RF specific requirements during the switched period.

For the switching period position, the NR V2X would complement LTE V2X for advanced V2X services. It means legacy LTE V2X should be not impacted by adding NR V2X. So it is clear that the switching period position will be allocated in NR slot as shown in Figure 8.1.7.3-1 and Figure 8.1.7.3-2..

Figure 8.1.7.3-1: E-UTRA V2X to NR V2X switching time mask at n47 without dual PA capability

Figure 8.1.7.3-2: NR V2X to E-UTRA V2X switching time mask at n47 without dual PA capability

<Unchanged sections are omitted>

### 9.1.4	Blocking characteristics

#### 9.1.4.1	In-band blocking

In EN 302 571, the blocking level shall not be less than -30 dBm. And the blocking is tested at ± 50 MHz, ± 100 MHz, and ± 200 MHz frequency offset. Blocking testing shall be performed at least at 6 different frequency offset positions. The blocking level is larger than exiting case 2 blocking interferer in the specification, which is in the second alternate channel offset from the wanted signal. Considering the total 70MHz band width of the V2X band, ± 50 MHz is already outside the operating band or close at the band edge depends on the placement of the carrier.

RAN4 reuse the In-band blocking requirements in LTE V2X UE as shown in Table 9.1.4.1-1 and Table 9.1.4.1-2 NR V2X UE in Table 8.1-1.

Table 9.1.4.1-1: In band blocking parameters for V2X

| Rx parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 30 MHz | 40MHz |
| Power in Transmission Bandwidth Configuration | dBm | PREFSENS_V2X + channel bandwidth specific value below |  |  |  |  |  |  |  |
|  |  |  |  |  | 6 |  | 9 | 11 | 12 |
| BWInterferer | MHz |  |  |  | 10 |  | 10 | 10 | 10 |
| FIoffset, case 1 | MHz |  |  |  | 15 |  | 15 | 15 | 15 |
| FIoffset, case 2 | MHz |  |  |  | 25 |  | 25 | 25 | 25 |
| NOTE 1: The interferer is QPSK modulated PUSCH containing data and reference symbols. Normal cyclic prefix is used. |  |  |  |  |  |  |  |  |  |

Table 9.1.4.1-2: In-band blocking for V2X

| NR band | Parameter | Unit |  | Case 1 | Case 2 |
| --- | --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -44 |  | -44 |
|  | FInterferer (offset) | MHz | =-BW/2 – FIoffset,case 1 & =+BW/2 + FIoffset,case 1 |  | ≤-BW/2 – FIoffset,case 2 & ≥+BW/2 + FIoffset,case 2 |
| n47 | FInterferer | MHz |  | (NOTE 2) | FDL_low – 30 to FDL_high + 30 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band. NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -BW/2 - FIoffset, case 1 and b. the carrier frequency +BW/2 + FIoffset, case 1 NOTE 3: FInterferer range values for unwanted modulated interfering signal are interferer center frequencies NOTE 4: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. |  |  |  |  |  |

#### 9.1.4.2	Out-of-band blocking

Out-of-band band blocking in existing specification for licensed bands is defined for an unwanted CW interfering signal falling more than 15 MHz below or above the UE receive band. For the first 15 MHz below or above the UE receive band the appropriate in-band blocking or adjacent channel selectivity shall be applied.

RAN4 reuse the Out-of- band blocking of LTE V2X UE as shown in Table 9.1.4.2-1 and Table 9.1.4.2-2 for NR V2X UE in Table 8.1-1.

Table 9.1.4.2-1: Out-of-band blocking parameters

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 30 MHz | 40 MHz |
| Power in Transmission Bandwidth Configuration | dBm | PREFSENS_V2X + channel bandwidth specific value below |  |  |  |  |  |  |  |
|  |  |  |  |  | 6 |  | 9 | 11 | 12 |
| NOTE 1: Reference measurement channel is FFS |  |  |  |  |  |  |  |  |  |

Table 9.1.4.2-2: Out of band blocking

| NR band | Parameter | Units | Frequency |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Range 1 | Range 2 | Range 3 |
|  | PInterferer | dBm | -44 | -30 | -15 |
| n47 | FInterferer (CW) | MHz | FDL_low -30 to FDL_low -60 | FDL_low -60 to FDL_low -85 | FDL_low -85 to 1 MHz |
|  |  |  | FDL_high +30 to FDL_high + 60 | FDL_high +60 to FDL_high +85 | FDL_high +85 to +12750 MHz |
| NOTE: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 4400 MHz. |  |  |  |  |  |

### 9.1.5	Spurious response

Spurious response is a measure of the receiver's ability to receive a wanted signal on its assigned channel frequency without exceeding a given degradation due to the presence of an unwanted CW interfering signal at any other frequency at which a response is obtained.

For V2X, 10MHz, 20MHz, 30MHz and 40MHz channel bandwidths are considered. RAN4 reuse the spurious response of LTE V2X UE as shown in Table 9.1.5-1 and Table 9.1.5-2 for NR V2X UE in Table 8.1-1.

Table 9.1.5-1: Spurious response parameters for V2X

| Rx parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz |  | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 30 MHz | 40 MHz |
| Power in Transmission Bandwidth Configuration | dBm | PREFSENS_V2X + channel bandwidth specific value below |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 6 |  | 9 | 11 | 12 |
| NOTE 1: Reference measurement channel is FFS |  |  |  |  |  |  |  |  |  |  |

Table 9.1.5-2: Spurious response for V2X

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -44 |
| FInterferer | MHz | Spurious response frequencies |

### 9.1.6	Intermodulation characteristics

Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal.

The wide band intermodulation requirement is defined using modulated NR carrier and CW signal as interferer. 10MHz channel bandwidth will be chosen as the modulated interfering signal for 10MHz, 20MHz, 30MHz and 40MHz channel bandwidth.

RAN4 reuse the intermodulation characteristics of LTE V2X UE as shown in Table 9.1.6-1 for NR V2X UE in Table 8.1-1.

Table 9.1.6-1: Wide band intermodulation

| NR band | Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz |  | 15 MHz |  | 20 MHz | 30 MHz | 40 MHz |
| n47 | Power in Transmission Bandwidth Configuration | dBm | PREFSENS_V2X + channel bandwidth specific value below |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 6 |  |  |  | 9 | 11 | 12 |
|  | PInterferer 1 (CW) | dBm | -46 |  |  |  |  |  |  |  |  |  |
|  | PInterferer 2 (Modulated) | dBm | -46 |  |  |  |  |  |  |  |  |  |
|  | BWInterferer 2 |  |  |  |  |  | 10 |  |  | 10 | 10 | 10 |
|  | FInterferer 1 (Offset) | MHz |  |  |  |  | -BW/2 – 15 / +BW/2 + 15 |  |  | -BW/2 – 15 / +BW/2 + 15 | -BW/2 – 15 / +BW/2 + 15 | -BW/2 – 15 / +BW/2 + 15 |
|  | FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |  |  |  |  |  |  |  |  |  |
| NOTE 1: Reference measurement channel is FFS NOTE 2: The interferer is QPSK modulated PUSCH containing data and reference symbols. Normal cyclic prefix is used. |  |  |  |  |  |  |  |  |  |  |  |  |

<Unchanged sections are omitted>

# 10	Tx/Rx characteristics of concurrent operation

## 10.1	NR V2X SL operation in 5.9GHz or other potential ITS spectrum and LTE/NR Uu operation in licensed band for FR1

### 10.1.1	Tx requirements for inter-band con-current NR V2X operation

      - Maximum output power

For the inter-band con-current NR V2X operation, the following NR V2X UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth. The period of measurement shall be at least one sub frame (1ms).

Table 10.1.1.1-1: Con-current NR V2X UE Power Class for uplink inter-band combination (two bands)

| NR V2X con-current operating band Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR V2X_n71A-n47A |  |  |  |  | 23 | +2/-34 |  |  |
| NOTE 1: The con-current band combinations is used for NR V2X Service. NOTE 2: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 3: For inter-band con-current aggregation the maximum power requirement apply to the total transmitted power over all component carriers (per UE). NOTE 4: 4 refers to the transmission bandwidths (Figure 5.6-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |  |  |  |  |  |  |

<Unchanged sections are omitted>

      - Spurious emission band UE co-existence

This clause specifies the spurious emission requirements of the inter-band con-current V2X operation, for UE-to-UE coexistence to protect legacy protected bands

Table 10.1.1.13-1: Requirements

| NR V2X con-current operating band cofiguration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| NR V2X_n71A-n47A | E-UTRA Band 1, 5, 7, 8, 26, 28, 34, 39, 40, 44, 45, 65, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 22, 41, 42, 52 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 1 |
|  | Frequency range | 5925 | - | 5950 | -30 | 1 | 3, 4 |
|  | Frequency range | 5815 | - | 5855 | -30 | 1 | 3 |
| NOTE 1: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. In case the exceptions are allowed due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3 or 4 for the 2nd, 3rd or 4th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 2: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the aggregated channel bandwidth. NOTE 3: Applicable when NS_XX is configured by the pre-configured radio parameters for power class 3 V2X UE. NOTE 4: In the frequency range x-5950MHz, SE requirement of -30dBm/MHz should be applied; where x = max (5925, fc + 15), where fc is the channel centre frequency. |  |  |  |  |  |  |  |

<End of Changes>