3GPP TSG-RAN WG4 Meeting # 94-e-Bis                                                       R4-2003269

Electronic Meeting, 20 – 30 Apr., 2020

Source: 	CATT

Title: 	Simulation results for NR PUSCH UL timing

Agenda item:	6.17.2.2.3

Document for:	Discussion

- Introduction

In the recent RAN4#93 and RAN4#94-e meeting, the simulation assumptions for NR PUSCH UL timing in WFs [1,2] were approved. This paper will provide our evaluation results for NR PUSCH UL timing for simulation alignment based on agreed simulation asumptions .

- Evaluation results

The agreed test parameters[1,2] are summarized and shown in Table A-1 in Annex.

The evaluation results for  NR PUSCH UL timing are shown in Table 2-1.

Table 2-1 Evaluation results for NR PUSCH UL timing

| Scenario | SRS transmission(optional) | CBW | SCS | Waveform | Tx/Rx | DMRS | l0 | MCS | PUSCH mapping type | SNR @70% of maximum throughput |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | Ideal | Impairment |
| Scenario Y | Enabled or Disabled | 10MHz | 15KHz | CP-OFDM | 1T2R | 1+1+1 | 2 | 16 | Type A | 5.8 | 8.3 |
|  |  | 10MHz | 15KHz | CP-OFDM | 1T2R | 1+1+1 | 0 | 16 | Type B | 5.78 | 8.28 |
|  | Enabled or Disabled | 40MHz | 30KHz | CP-OFDM | 1T2R | 1+1+1 | 2 | 16 | Type A | 5.86 | 8.36 |
|  |  | 40MHz | 30KHz | CP-OFDM | 1T2R | 1+1+1 | 0 | 16 | Type B | 5.84 | 8.34 |

- Conclusion

This paper provides our evaluation results for NR PUSCH UL timing for simulation alignment based on agreed simulation asumptions.

- References

- R4-1915886，Way forward on NR HST PUSCH demodulation requirements，Huawei, HiSilicon，RAN4#93

- R4-2002405，WF on Rel-16 NR HST BS demodulation requirements，Nokia, Nokia Shanghai Bell, RAN4#94-e

- Annex

Table A-1 Test parameters for NR PUSCH UL timing

| Parameter | value |
| --- | --- |
| Transform precoding | Disabled |
| Number of Tx | 1 |
| Number of Rx | 2 |
| Number of layers | 1 |
| Transmission scheme | Identity matrix (TPMI index 0) |
| symbols length | 14 |
| start symbol index | 0 |
| Time domain resource allocation type | type A and type B |
| MCS index | MCS 16 |
| PTRS | Not configured |
| Timing offset | 0 |
| Frequency offset | 0 |
| Code block group, Frequency hopping, Limited buffer rate matching | Disabled |
| Number of HARQ transmissions | 4 |
| RV | {0,2,3,1} |
| DMRS type | type 1 |
| Number of DMRS symbols | 1+1+1 |
| Front loaded DMRS start symbol | Type A: l0 = 2 Type B: l0 = 0 |
| SCS and BW | 15kHz: 10MHz; 30kHz: 40MHz |
| RB assignment | 15kHz SCS/10MHz CBW: 25 continuously allocated RBs for each UE 30kHz SCS/40MHz CBW: 50 continuously allocated RBs for each UE |
| Starting PRB index | 15kHz SCS/10MHz CBW: Moving UE 0, Stationary UE 25 30kHz SCS/40MHz CBW: Moving UE 0, Stationary UE 50 |
| Carrier frequency (GHz) | 15kHz SCS: 2.1GHz 30kHz SCS: 3.6GHz |
| Propagation condition | Scenario Y: AWGN 15kHz SCS: A=10us, △ω=0.13s-1 30kHz SCS: A=5us, △ω=0.26s-1 |
| SRS transmission(optional) | Disabled |
| Testing metric | SNR @70% of maximum throughput |