3GPP TSG-RAN4  #94bis-e															     	R4-2003433

20th Apr – 30th Apr 2020

Source: 	Qualcomm Incorporated

Title: 	MPR for NR V2X PSSCH PSCCH transmission all channel bandwidths TP

Agenda item:	6.4.4.1

Document for:	Approval

# Introduction

We study the required MPR and propose a specification structure and values.

# Simulation description and findings

Waveforms were simulated at  fc = 5860, 5870, …, 5920 MHz for 15,30, and 60 kHz SCS. Plots of select simulations are shown in the text proposal section, along with a proposed formulation for MPR.

All valid waveforms were simulated: channel bandwidths 10, 20, 30, and 40 MHz; subcarrier spacings 15, 30,  and 60 kHz ;  modulations QPSK, 16QAM, 64QAM, and 256 QAM ; and all valid subchannel RB widths. RBstart was constrained to subchannel boundaries.

Select results are shown in the figures below.

The results for all simulations are summarized in the table below, categorized by channel bandwidth, subcarrier spacing, and inner/outer allocation. There are no inner allocations for 10MHz with 30 and 60 kHz SCS and 20MHz with 60kHz SCS. Note that EVM-limited backoff is not included in this table and must be considered when writing the spec.

|  |  | 15kHz |  | 30kHz |  | 60kHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Modulation Order | Inner | Outer | Inner | Outer | Inner | Outer |
| 10MHz | 2,4,6,8 | 1.5 | 4.7 | na | 4.6 | na | 4.5 |
| 20MHz | 2,4,6,8 | 2 | 4.8 | 1.5 | 4.7 | na | 4.7 |
| 30MHz | 2,4,6,8 | 2.5 | 4.8 | 2 | 4.8 | 1.5 | 4.6 |
| 40MHz | 2,4,6,8 | 2.5 | 4.8 | 2 | 4.8 | 1.5 | 4.8 |

V2X PSSCH PSCCH Emissions-limited MPR (does not include EVM-limited MPR)

<begin changes to TR 38.886 draft v0.6.0>

#### 8.1.2.1	MPR for Power class 3 V2X UE

For contiguous allocation of PSCCH and PSSCH simultaneous transmissions, the allowed MPR for the maximum output power for the NR V2X physical channels PSCCH and PSSCH shall be specified as inner/outer RB allocations in Table 8.1.2.1-1 for power class 3.

Table 8.1.2.1-1: Maximum Power Reduction (MPR) for power class 3 NR V2X (Contiguous PSCCH and PSSCH transmission)

| Modulation |  | Channel bandwidth/MPR (dB) |  |
| --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |
| CP-OFDM | QPSK/ 16QAM | ≤ [4.0] | ≤ [2.0] |
|  | 64 QAM | ≤ [4.5] | ≤ [3.5] |
|  | 256 QAM | ≤ [6.0] |  |

|  | MPR (dB) SCS = 15 kHz |  | MPR (dB) SCS = 30 kHz |  | MPR (dB) SCS = 60 kHz |  |
| --- | --- | --- | --- | --- | --- | --- |
| Channel BW (MHz) | Inner | Outer | Inner | Outer | Inner | Outer |
| 10 | ≤1.5 | ≤4.8 | na | ≤4.8 | na | ≤4.8 |
| 20 | ≤2 |  | ≤1.5 |  | na |  |
| 30, 40 | ≤2.5 |  | ≤2 |  | ≤1.5 |  |
| Note 1: MPR is the maximum of the value in this table and the values in Table 6.2.2-1 |  |  |  |  |  |  |

Where the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations:

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1 in TS38.101-1.

RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

LCRB  ≤  ceil(NRB/2)

where ceil(x) is the smallest integer greater than or equal to x.

For non-contiguous allocation for simultaneous PSFCH transmission for NR V2X will be specified as follow

MPR_PSFCH = CEIL {MA_PSFCH, 0.5}

Where MA is defined as follows

MA_PSFCH =	TBD		; 0.00< A ≤ TBD

TBD		; TBD< A ≤TBD

TBD		; TBD< A ≤1.00

Where

A = NRB_alloc / NRB.

CEIL{MA, 0.5} means rounding upwards to closest 0.5dB.

The allowed MPR for the maximum output power for V2X physical channels PSBCH and PSSS shall be applied the legacy NR Uu requirements in subclause 6.2.2 in TS38.101-1 for the corresponding modulation and transmission bandwidth.

The allowed MPR for the maximum output power for NR V2X physical signal SSSS is FFS.

<end changes>