3GPP TSG-RAN WG4 Meeting #94-e-Bis                                R4-2003544

Electronic Meeting, 20th– 30th April, 2020

Agenda item:			6.11.2.4

Source:			Samsung

Title:	Discussion on Multi-TRP/Panel Transmission on MRTD/MTTD

Document for:		Approval

# 1	Introduction

The Rel-16 MIMO enhancement work item was approved in RAN#80 with a list of enhancement areas including enhancements on multi-TRP/panel transmission, i.e.,

<Captured in latest Rel-16 eMIMO WID> - Enhancements on multi-TRP/panel transmission including improved reliability and robustness with both ideal and non-ideal backhaul: - Specify downlink control signalling enhancement(s) for efficient support of non-coherent joint transmission - Perform study and, if needed, specify enhancements on uplink control signalling and/or reference signal(s) for non-coherent joint transmission - Multi-TRP techniques for URLLC requirements are included in this WI

Although RAN4 have not yet identified RRM impact due to enhancement on multi-TRP/panel transmission, some offline discussion is triggered on its potential impact on MRTD/MTTD requirement. In this discussion paper, we would like to further provide our analysis and view on this impact.

# Discussion

During offline discussion, some company propose that the following MRTD/MTTD requirement may have potential limitation on multi-TRP/panel transmission. Specifically, for intra-band EN-DC and intra-band NR CA, co-located deployment is assumed in Rel-15 RAN4 RRM discussion, with the following highlighted contents specified in TS38.133:

<Captured from TS38.133 v15.8.0> ### 7.5.3 Minimum Requirements for intra-band EN-DC For intra-band EN-DC, only co-located deployment is applied. The UE shall be capable of handling a maximum uplink transmission timing difference between E-UTRA PCell and PSCell as shown in Table 7.5.2-1 for E-UTRA FDD-NR FDD intra-band EN-DC provided the UE indicates that it is capable of asynchronous EN-DC operation [2]. The UE shall be capable of handling a maximum uplink transmission timing difference between E-UTRA PCell and PSCell as shown in Table 7.5.3-1 for E-UTRA TDD-NR TDD and E-UTRA FDD-NR FDD intra-band EN-DC provided the UE does not indicate that it is capable of asynchronous FDD-FDD EN-DC operation [16]. Table 7.5.3-1: Maximum uplink transmission timing difference requirement for intra-band synchronous EN-DC Sub-carrier spacing in E-UTRA PCell (kHz); UL Sub-carrier spacing for data in PSCell (kHz); Maximum uplink transmission timing difference (µs). 15; 15; 5.21Note1,Note 2. 15; 30; 5.21Note 2. 15; 60; 5.21 Note 2. NOTE 1: This is not applicable for a UE which indicates the capability of only supporting single UL timing (ul-TimingAlignmentEUTRA-NR is signalled). Single UL timing for E-UTRA and NR cell is assumed for this UE. NOTE 2: If the transmission timing difference exceeds the cyclic prefix length of the UL Sub-carrier spacing for data in PSCell, NR UE Tx EVM degradation is expected for the symbol that is overlapping the LTE subframe boundary. …… ### 7.6.3 Minimum Requirements for intra-band EN-DC For intra-band EN-DC, only co-located deployment is applied. The UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from a E-UTRA cell belonging to the MCG and slot timing of signal from a cell belonging to the SCG as shown in Table 7.6.2-1 for E-UTRA FDD-NR FDD intra-band EN-DC provided the UE indicates that it is capable of asynchronous EN-DC operation [2]. The UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from a E-UTRA cell belonging to the MCG and slot timing of signal from a cell belonging to the SCG as shown in Table 7.6.3-1 for E-UTRA FDD-NR FDD and E-UTRA TDD-NR TDD intra-band EN-DC provided the UE does not indicate that it is capable of asynchronous FDD-FDD EN-DC operation [16]. Table 7.6.3-1 Maximum receive timing difference requirement for intra-band synchronous EN-DC Sub-carrier spacing of E-UTRA cell in MCG (kHz); DL Sub-carrier spacing of cell in SCG (kHz) Note1; Maximum receive timing difference (µs). 15; 15; 3. 15; 30; 3. 15; 60; 3. NOTE 1: DL Sub-carrier spacing is min{SCSSS, SCSDATA}.. Table 7.6.3-2 Void ### 7.6.4 Minimum Requirements for NR Carrier Aggregation For intra-band CA, only co-located deployment is applied. For intra-band non-contiguous NR carrier aggregation, the UE shall be capable of handling at least a relative receive timing difference between slot timing of different carriers to be aggregated at the UE receiver as shown in Table 7.6.4-1 below. Table 7.6.4-1: Maximum receive timing difference requirement for intra-band non-contiguous NR carrier aggregation Frequency Range; Maximum receive timing difference (µs). FR1; 31. FR2; 0.26. Note 1: In the case of different SCS on different CCs, if the receive time difference exceeds the cyclic prefix length of that SCS, demodulation performance degradation is expected for the first symbol of the slot.. For inter-band NR carrier aggregation, the UE shall be capable of handling at least a relative receive timing difference between slot timing of all pairs of carriers to be aggregated at the UE receiver as shown in Table 7.6.4-2 below. Table 7.6.4-2: Maximum receive timing difference requirement for inter-band NR carrier aggregation Frequency Range of the pair of carriers; Maximum receive timing difference (µs). FR1; 33. FR2; 8. Between FR1 and FR2; 25.

The problem of how to handle this Rel-15 restriction to enable multi-TRP/panel discussion should be addressed in Rel-16 eMIMO work item.

#### 2.1 Intra-band EN-DC MRTD/MTTD

For intra-band EN-DC, only co-located deployment is assumed in Rel-15, which means the signals from eNB and gNB should comes from the same base station site with no propagation delay observed. However, if the NR cell formed by gNB consist of two TRPs with different geographic location, the aforementioned propagation delay could be observed as below figure illustrates.

On the hand, when multi-TRP transmission is discussed in RAN4 Demod session, the following RAN1 agreement is followed, i.e.,

Note that for the sake of discussion, the UE may assume that the UE may receive DL transmission from multiple TRP within a CP with single/multiple FFT windows. Companies are encouraged to clarify time/frequency synchronization assumptions for proposed multi-TRP/panel DL transmission.

Accordingly, RAN4 is discussing in the direction to specify Demod requirement with timing offset less than one CP length, while it is still under discussion.

Therefore we suggest the following proposal with text proposal to not only removing this restriction but also take UE’s performance degradation into account:

Proposal-1: To remove Rel-15 co-located deployment assumption for intra-band EN-DC, the following three text proposals is adopted to MRTD/MTTD requirement in TS38.133: 
  - “For intra-band EN-DC, only co-located deployment is applied.” is changed to “For intra-band EN-DC without multi-TRP transmission in NR PCell, only co-located deployment is applied.” In Section 7.5.3/7.6.3.
  - Additional Note is captured in Table 7.5.3-1, i.e., “Note 3: In the case of multi-TRP transmission deployed, the requirement of maximum transmission timing difference shall not be applicable to NR signals to multiple TRPs.”
  - Additional Note is captured in Table 7.6.3-1, i.e., “Note 2: In the case of multi-TRP transmission deployed, the requirement of maximum receive timing difference shall not be applicable to NR signals from multiple TRPs. If the receive time difference exceeds the cyclic prefix length of that SCS, demodulation performance degradation shall be expected.”

#### 2.2 Intra-band CA MRTD

Similar story applies to intra-band FR1 CA scenario, in which MRTD requirement is defined under the consumption of co-located deployment. To remove the restriction for multi-TRP deployment and also take UE performance degradation into account, the following proposal is provided:

Proposal-2: To remove Rel-15 co-located deployment assumption for intra-band CA, the following two text proposals is adopted to MRTD requirement in TS38.133: 
  - “For intra-band CA, only co-located deployment is applied.” is changed to “For intra-band CA without multi-TRP transmission, only co-located deployment is applied.” In Section 7.6.4.
  - Additional Note is captured in Table 7.6.4-1, i.e., “Note 2: In the case of multi-TRP transmission deployed, the requirement of maximum receive timing difference shall not be applicable to NR signals from multiple TRPs.”

For FR2 intra-band CA scenario, based on Rel-16 FR2 RF enhancement WI, it is single beam management scheme assumed for all intra-band FR2 CA case. In other words, multiple TRPs is hard to be applied for intra-band FR2 CA. Therefore, we expect no change for FR2 intra-band CA requirement.

Proposal-3: No need to change MRTD requirement for FR2 intra-band CA scenario.

# Conclusion

In this paper, we provided our analysis and view on the potential impact on MRTD/MTTD requirement due to the newly introduced multi-TRP/panel transmission, with following observations and proposals:

Proposal-1: To remove Rel-15 co-located deployment assumption for intra-band EN-DC, the following three text proposals is adopted to MRTD/MTTD requirement in TS38.133: 
  - “For intra-band EN-DC, only co-located deployment is applied.” is changed to “For intra-band EN-DC without multi-TRP transmission in NR PCell, only co-located deployment is applied.” In Section 7.5.3/7.6.3.
  - Additional Note is captured in Table 7.5.3-1, i.e., “Note 3: In the case of multi-TRP transmission deployed, the requirement of maximum transmission timing difference shall not be applicable to NR signals to multiple TRPs.”
  - Additional Note is captured in Table 7.6.3-1, i.e., “Note 2: In the case of multi-TRP transmission deployed, the requirement of maximum receive timing difference shall not be applicable to NR signals from multiple TRPs. If the receive time difference exceeds the cyclic prefix length of that SCS, demodulation performance degradation shall be expected.”

Proposal-2: To remove Rel-15 co-located deployment assumption for intra-band CA, the following two text proposals is adopted to MRTD requirement in TS38.133: 
  - “For intra-band CA, only co-located deployment is applied.” is changed to “For intra-band CA without multi-TRP transmission, only co-located deployment is applied.” In Section 7.6.4.
  - Additional Note is captured in Table 7.6.4-1, i.e., “Note 2: In the case of multi-TRP transmission deployed, the requirement of maximum receive timing difference shall not be applicable to NR signals from multiple TRPs.”

Proposal-3: No need to change MRTD requirement for FR2 intra-band CA scenario.

# Text Proposal

Since the topic is firstly treated in RAN4#94Bis-e meeting, we prepare the text proposal as follows for the purpose of triggering the detailed wording discussion:

----------------------------------------------Start of TP to MTTD ------------------------------------------------

### 7.5.3	Minimum Requirements for intra-band EN-DC

For intra-band EN-DC without multi-TRP transmission in NR PCell, only co-located deployment is applied.

The UE shall be capable of handling a maximum uplink transmission timing difference between E-UTRA PCell and PSCell as shown in Table 7.5.2-1 for E-UTRA FDD-NR FDD intra-band EN-DC provided the UE indicates that it is capable of asynchronous EN-DC operation [2].

The UE shall be capable of handling a maximum uplink transmission timing difference between E-UTRA PCell and PSCell as shown in Table 7.5.3-1 for E-UTRA TDD-NR TDD and E-UTRA FDD-NR FDD intra-band EN-DC provided the UE does not indicate that it is capable of asynchronous FDD-FDD EN-DC operation [16].

Table 7.5.3-1: Maximum uplink transmission timing difference requirement for intra-band synchronous EN-DC

| Sub-carrier spacing in E-UTRA PCell (kHz) | UL Sub-carrier spacing for data in PSCell (kHz) | Maximum uplink transmission timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 5.21Note1,Note 2,Note 3 |
| 15 | 30 | 5.21Note 2, Note 3 |
| 15 | 60 | 5.21 Note 2, Note 3 |
| NOTE 1: This is not applicable for a UE which indicates the capability of only supporting single UL timing (ul-TimingAlignmentEUTRA-NR is signalled). Single UL timing for E-UTRA and NR cell is assumed for this UE. NOTE 2: If the transmission timing difference exceeds the cyclic prefix length of the UL Sub-carrier spacing for data in PSCell, NR UE Tx EVM degradation is expected for the symbol that is overlapping the LTE subframe boundary NOTE 3: In the case of multi-TRP transmission deployed, the requirement of maximum transmission timing difference shall not be applicable to NR signals to multiple TRPs. |  |  |

---------------------------------------------End of TP to MTTD --------------------------------------------------

----------------------------------------------Start of TP to MRTD ------------------------------------------------

### 7.6.3	Minimum Requirements for intra-band EN-DC

For intra-band EN-DC without multi-TRP transmission in NR PCell, only co-located deployment is applied.

The UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from a E-UTRA cell belonging to the MCG and slot timing of signal from a cell belonging to the SCG as shown in Table 7.6.2-1 for E-UTRA FDD-NR FDD intra-band EN-DC provided the UE indicates that it is capable of asynchronous EN-DC operation [2].

The UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from a E-UTRA cell belonging to the MCG and slot timing of signal from a cell belonging to the SCG as shown in Table 7.6.3-1 for E-UTRA FDD-NR FDD and E-UTRA TDD-NR TDD intra-band EN-DC provided the UE does not indicate that it is capable of asynchronous FDD-FDD EN-DC operation [16].

Table 7.6.3-1 Maximum receive timing difference requirement for intra-band synchronous EN-DC

| Sub-carrier spacing of E-UTRA cell in MCG (kHz) | DL Sub-carrier spacing of cell in SCG (kHz) Note1 | Maximum receive timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 3 |
| 15 | 30 | 3 |
| 15 | 60 | 3 |
| NOTE 1: DL Sub-carrier spacing is min{SCSSS, SCSDATA}. NOTE 2: In the case of multi-TRP transmission deployed, the requirement of maximum receive timing difference shall not be applicable to NR signals from multiple TRPs. If the receive time difference exceeds the cyclic prefix length of that SCS, demodulation performance degradation shall be expected. |  |  |

Table 7.6.3-2	Void

### 7.6.4	Minimum Requirements for NR Carrier Aggregation

For intra-band CA without multi-TRP transmission, only co-located deployment is applied. For intra-band non-contiguous NR carrier aggregation, the UE shall be capable of handling at least a relative receive timing difference between slot timing of different carriers to be aggregated at the UE receiver as shown in Table 7.6.4-1 below.

Table 7.6.4-1: Maximum receive timing difference requirement for intra-band non-contiguous NR carrier aggregation

| Frequency Range | Maximum receive timing difference (µs) |
| --- | --- |
| FR1 | 3Note 1, Note 2 |
| FR2 | 0.26 |
| Note 1: In the case of different SCS on different CCs, if the receive time difference exceeds the cyclic prefix length of that SCS, demodulation performance degradation is expected for the first symbol of the slot. Note 2: In the case of multi-TRP transmission deployed, the requirement of maximum receive timing difference shall not be applicable to NR signals from multiple TRPs. |  |

For inter-band NR carrier aggregation, the UE shall be capable of handling at least a relative receive timing difference between slot timing of all pairs of carriers to be aggregated at the UE receiver as shown in Table 7.6.4-2 below.

Table 7.6.4-2: Maximum receive timing difference requirement for inter-band NR carrier aggregation

| Frequency Range of the pair of carriers | Maximum receive timing difference (µs) |
| --- | --- |
| FR1 | 33 |
| FR2 | 8 |
| Between FR1 and FR2 | 25 |

---------------------------------------------End of TP to MRTD --------------------------------------------------

# Reference

[1] RP-192271,	“Revised WID: Enhancements on MIMO for NR”, Samsung.