3GPP TSG RAN WG4 Meeting #94bis-e	R4-2003549

Electronic meeting, 	20th – 30th April 2020

Agenda Item:	6.4.2

Source:	Futurewei

Title:	TP on Indevice Coexistence

Document for:	Approval

# Introduction

This contribution provides text proposal for the section 6 Evaluation of Indevice Coexistence in the TR 38.886 [1]. The scenario description for evaluation are described.

************* Start of the TP in sub clause 6 of TR 38.886 ***************

# 6	Evaluation of In-device Coexistence

## 6.2 UE architecture considerations

For in-device coexistence, the UE architecture and implementation assumptions need to be considered.  For this short-term switching period analysis, intra-band operation without dual PA capability can be the baseline.   It may be possible that a better switching delay performance is possible by considering separate RF chains with dual PAs. However, for this analysis the baseline performance considering single PA is considered.

## 6.2 Switching time analysis

The switching time between NR SL and LTE SL for short-term TDM operation was analyzed as part of this study.  The general ON / OFF time mask requirements in subclause 6.3.3.2 in TS 38.101-1 is applicable for NR sidelink also.  In addition to the above general ON / OFF time masks, the additional switching period of about [TBD] is applicable between the NR SL and LTE SL.  Considering that the NR V2X CBW is 10/20/30/40 MHz and that of LTE V2X is 10/20MHz, the spectrum span for contiguous and non-contiguous transmissions would be similar, the switching time can be similar.

For this short term TDM operation of NR SL to LTE SL or LTE SL to NR SL, the switching period position can be on either on NR SL or LTE SL. The switching period power requirements can be different to OFF power period and this switching power period will not be separately defined.

<End of Changes>

# References

TR 38.886. Title: V2X Services based on NR; User Equipment (UE) radio transmission and reception