3GPP TSG RAN WG4 Meeting #94bis-e	R4-2003578

Electronic meeting, 	20th – 30th April 2020

Agenda Item:	6.4.4.1

Source:	Futurewei

Title:	On UL-SL Prioritization

Document for:	Discussion

# Introduction

In the LS R4-1910714 [1], RAN2 asked RAN1/RAN4 regarding UL-SL prioritization for various scenarios. RAN1 has provided a reply in R4-1913107 [2]. During RAN4#93 meeting, RAN4 provided reply LS in R4-1915985 [3], however the case 1 of Q1 was left to be answered until details became available.  This contribution provides some analysis and considerations before drafting a further Reply LS to [1].

# On UL-SL Prioritization

## NR UL/NR SL Prioritization cases

In the LS [1], for Q1, there are two scenarios: (1) UL-TX/SL-TX in shared/single carrier, (2) UL-TX/SL-TX in different carriers. The scenario and questions provided from RAN2 are about only a TX-TX overlap scenario. When the UL-TX and SL-TX are in the same carrier or in different carriers, the final decision of whether to drop, prioritize or even adjust the transmit power is up to higher layers. RAN1 and RAN4 analysis would consider the feasibility of this TX –TX overlap. RAN4 should note that in [2], RAN1 has confirmed that this is a VALID scenario. RAN4 can analyze the scenarios from the perspective of RF requirements and coexistence.

For scenario (1) where the UL-TX and SL-TX in single /shared carrier: in LTE V2X, the solution adopted for is to drop the lower priority transmission. In NR V2X, RAN4 has not concluded the coexistence analysis in the licensed bands yet. Please note that LTE UL-TX and SL-TX are DFT-S-OFDM, and in NR, because UL-TX and NR SL-TX are CP-OFDM, NR V2X has more flexibility for this transmission in the same carrier. It is certainly possible for UL-TX and SL-TX in single /shared carrier as this is no different than intra-band CA cases.  In the event of UL-TX and SL-TX, options whether to drop the lower priority or to adjust the Tx power are possible.  So, the scenario is VALID. The purpose of the discussion is whether the scenario is valid, so RAN4 can inform RAN2 about the validity.

If the UE supports simultaneous transmission on UL BWP and SL BWP and if the maximum transmission power of UL and SL is not exceeded, then case 1 of Q1 scenario of  UL-TX/SL-TX in shared/single carrier is valid.

For scenario (2) where the UL-TX and SL-TX are on different carriers, in LTE V2X, the solution adopted is to adjust the Tx power. This simultaneous transmission in different carriers is a common scenario in CA / DC cases and it is no different for V2X.  This is clearly a valid scenario. Of course, RAN4 still require more investigations on timing advance of UL TX /SL TX, mixed numerology transmissions, etc. but the scenario is a valid one. RAN4 can also indicate to RAN2 that for Q1 both the cases are valid.

Proposal 1: For Q1, two scenarios: (1) UL-TX/SL-TX in shared/single carrier, (2) UL-TX/SL-TX in different carriers, RAN4 can indicate to RAN2 that both the scenarios are valid

# Conclusion

This contribution provides some analysis and considerations in drafting a Reply LS to R4-1910710.

Proposal 1: For Q1, two scenarios: (1) UL-TX/SL-TX in shared/single carrier, (2) UL-TX/SL-TX in different carriers, RAN4 can indicate to RAN2 that both the scenarios are valid

# References

- [1] R4-1910714 LS on UL-SL prioritization, RAN4#92-bis, October, 2019.

- [2] R4-1913107 Reply LS on UL-SL prioritization, RAN1, RAN4#93, November, 2019.

- [3] R4-1915985 Reply LS to RAN2 on UL-SL Prioritization RAN4#93, November, 2019.