3GPP TSG-RAN WG4 Meeting #94-e-Bis	R4-2003600

Electronic Meeting, 20th – 30th Apr., 2020

Agenda item:		6.7.2.1

Source:	MediaTek Inc.

Title:	Discussion on RRM measurement relaxation in IDLE/INACTIVE mode

Document for:		Discussion

# 1	Introduction

In last meeting [1], RAN4 agreed on the following WF for the RRM measurement relaxation in RRC IDLE/INACTIVE mode

- Applicability of RRM relaxation methods for scenario#1 and scenario#2: - Scenario #1: - Agreement - RRM measurement relaxation with longer intervals - Scenario #2: - Agreement - RRM measurement relaxation with longer intervals - FFS on the scaling factor of measurement interval - Option 1: Fixed value - FFS on one fixed value for both scenario#1 and scenario #2 - FFS on separate values for scenario#1 and scenario #2 - Option 2: Network configurable value - FFS on the transition period between scenario#1 or scenario#2 and scenario#3 - Option1: - When switching from scenario#2 to scenario#3, UE shall fulfill the requirements corresponding to scenario#2 for N DRX cycles and thereafter switch to requirements corresponding to scenario#3 - When switching from scenario#3 to scenario#2, UE shall fulfill the requirements corresponding to scenario #2 immediately. - Other option is not precluded. - RRM measurement relaxation for inter-frequency layer with higher priority - Option 1: (CATT, Vivo, CMCC, Huawei, LGE, OPPO, Apple, MediaTek) - When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, no relaxation of the current measurement delay requirement is expected for inter-frequency measurement with higher priority. - When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, the relaxed requirement for the frequency layer of higher priority shall use the same relaxed measurement requirement as those for the frequency layer of equal/lower priority. - Option 2: (Ericsson) - Measurements of higher priority carriers shall not be relaxed in high mobility scenarios (scenario #2) - Option 3: (Nokia) - Measurements of higher priority carriers shall not be relaxed - RRM measurement relaxation by reducing the number of frequency layer to be measured - No consensus to introduce the RRM measurement relaxation by reducing the number of frequency layer in RAN4 94e. - Interested companies are encouraged to provide analysis on achievable power saving from reducing number of frequency layers for intra/inter-frequency measurements in RAN4 94bis - EMR impact in power saving mode - Option 1: Up to UE’s implementation(CATT, Vivo) - Option 2: EMR frequency layer shall not be relaxed if T331 is running (LGE, Nokia, Qualcomm, CATT, OPPO, CMCC, Apple, Ericsson, ZTE, Intel) - Option 3: EMR frequency layer shall be relaxed (Huawei, MediaTek) - RRM measurement relaxation threshold for inter-frequency measurement - Option 1: Introduce carrier specific search thresholds for measurement relaxation(Vivo, Nokia, LGE) - Option 2: Up to RAN2’s decision(CATT, Huawei, OPPO,CMCC, Apple, Ericsson, ZTE, MediaTek, Qualcomm)

RAN4 has already agreed that RRM measurement relaxation with longer measurement intervals will be applied when UE is only in the low mobility condition or UE is only not at cell edge condition. In this paper, we will discuss the remaining issues, e.g., the relaxation scenarios and corresponding UE behaviors, the setting of the threshold value, and the scaling factors. Besides, EMR impact in power saving mode is also considered.

# 2	Discussion

RRM measurement relaxation threshold for inter-frequency measurement

We firstly would like to discuss whether RAN4 should introduce the carrier specific search threshold for inter-frequency measurement. In last meeting, it was agreed in the RAN2 CR [2] that the thresholds of both low mobility condition and not at cell edge condition are scalar. It means that all target intra-frequency or inter-frequency will apply the same threshold of low mobility condition to conduct power saving, and also all target intra-frequency or inter-frequency will apply the same threshold of not at cell edge condition to conduct power saving.

Proposal 1: RAN 4 to confirm that RRM measurement relaxation threshold is the same for all frequency layers

SIB2 ::= SEQUENCE { cellReselectionInfoCommon SEQUENCE { nrofSS-BlocksToAverage INTEGER (2..maxNrofSS-BlocksToAverage) OPTIONAL, -- Need S absThreshSS-BlocksConsolidation ThresholdNR OPTIONAL, -- Need S rangeToBestCell RangeToBestCell OPTIONAL, -- Need R q-Hyst ENUMERATED { dB0, dB1, dB2, dB3, dB4, dB5, dB6, dB8, dB10, dB12, dB14, dB16, dB18, dB20, dB22, dB24}, speedStateReselectionPars SEQUENCE { mobilityStateParameters MobilityStateParameters, q-HystSF SEQUENCE { sf-Medium ENUMERATED {dB-6, dB-4, dB-2, dB0}, sf-High ENUMERATED {dB-6, dB-4, dB-2, dB0} } } OPTIONAL, -- Need R ..., [[ relaxedMeasurement-r16 SEQUENCE { lowMobilityEvalutation-r16 SEQUENCE { s-SearchDeltaP-r16 ENUMERATED { dB3, dB6, dB9, dB12, dB15, spare3, spare2, spare1} OPTIONAL, -- Need S t-SearchDeltaP-r16 ENUMERATED { s5, s10, s20, s30, s60, s120, s180, s240, s300, spare7, spare6, spare5, spare4, spare3, spare2, spare1} OPTIONAL -- Need S } OPTIONAL, -- Cond OptMandatory cellEdgeEvalutation-r16 SEQUENCE { s-SearchThresholdP-r16 ReselectionThreshold OPTIONAL, -- Need R s-SearchThresholdQ-r16 ReselectionThresholdQ OPTIONAL -- Need R } OPTIONAL, -- Cond OptMandatory relaxedMeasCondition-r16 ENUMERATED { lowMobilityOrNotAtCellEdge, lowMobilityAndNotAtCellEdge} OPTIONAL, -- Cond MultRelaxCriteria highPriorityMeasRelax-r16 ENUMERATED {true} OPTIONAL -- Need R } OPTIONAL -- Need R ]] },

Threshold mismatch between not at cell edge condition and cell center condition

As we have seen, the ENUMERATED of s-SearchThresholdP-r16 and s-SearchThresholdQ-r16              for not at cell edge condition are ReselectionThreshold and ReselectionThresholdQ, respectively. According to the current RAN2 spec, it is possible that threshold of not at cell edge condition might be configured with a value no less than threshold of cell center condition (ENUMERATED of s-IntraSearchP and s-IntraSearchQ are also ReselectionThreshold and ReselectionThresholdQ, respectively). This kind of configurations exist indication conflict. For example, for an high priority inter-frequency, when the cell center condition is fulfilled, UE should conduct the normal measurement. However, if threshold of not at cell edge condition is larger than that of the cell center condition, the not at cell edge condition might also be fulfilled at the same time, and it implies that UE can be allowed to measure with extended measurement interval. UE behavior has ambiguity for this case. Thus, we suggest to preclude this scenario in the RAN4 spec.

Proposal 2: No RAN4 requirements are specified when the configured threshold of not at cell edge condition is no less than the configured threshold of cell center condition.

RRM measurement relaxation for inter-frequency layer with higher priority

If aforementioned proposal is agreed, we can divide the UE behaviors based on 3 different UE locations shown in Figure 1, i.e., the cell center, middle area, and cell edge.

Figure 1: 3 UE locations in ILDE RRM measurement

The scenarios and the corresponding UE behaviors for IDLE mode power saving are listed in Table 1, where scaling factors Krelax_LowMobility and Krelax_NotCellEdge are used to denote the relaxation when only the low mobility condition is fulfilled and only the not at cell edge condition is fulfilled, respectively. However, there exists a scenario in this table that hasn’t been agreed in RAN4, it is marked by the gray color.

Table 1: Relaxation principles of R-16 power saving

| High priority NBR cell | Low mobility | Not low mobility |  | Equal/Low priority NBR cell | Low mobility | Not low mobility |
| --- | --- | --- | --- | --- | --- | --- |
| Cell center | No requirement | Normal measurement (already long interval) |  | Cell center | No requirement | No requirement |
| Middle area | No requirement | Krelax_NotCellEdge |  | Middle area | No requirement | Krelax_NotCellEdge |
| Cell edge | Krelax_LowMobility | Normal measurement |  | Cell edge | Krelax_LowMobility | Normal measurement |

Considering that when UE is located in the cell center, the search time for inter-frequency measurement with higher priority is very long already ( Thigher_priority_search = (60 * Nlayers) seconds), we do not think a further extension is needed. So we suggest that

Proposal 3: When UE is located in the cell center, the search time for inter-frequency measurement with higher priority will not be relaxed.

Scenarios for scaling factor apply on

Furthermore, RAN4 also needs to confirm how the delay requirements of high/equal/low priority intra/inter-frequency neighboring cell are scaling up. According to the current spec [3], UE is required to measure within Tdetect for the unknown cell and Tevaluate for the known cell, and two measurements shall be spaced by at least Tmeasure,NR/2 within the set of measurements used for the filtering. We propose that

Proposal 4: Scaling factor of RRM measurement relaxation with longer intervals should apply to Tdetect, Tevaluate, and Tmeasure.

Scaling factor of measurement interval

As for whether to apply same value for the scaling factors Krelax_LowMobility and Krelax_NotCellEdge. From UE perspective, the simple solution will benefit the UE measurement scheduling, especially that the relaxation principles shown in Table 1 are already very complicated. So we support to apply same setting for these 2 scaling factors. Besides, a fixed value is preferred.

Proposal 5: RAN4 to apply same fixed scaling factor for IDLE mode RRM relaxation when only the low mobility condition is fulfilled or only the not at cell edge condition is fulfilled.

EMR impact in power saving mode

We would like to also provide our own view on the impact of RRM measurement relaxation on early measurement reporting. For UE who supports the RRM measurement relaxation in IDLE mode, the measurement period might be extended for the neighboring cell measurement for re-selection purpose. Since the re-selection requirement are re-used to specify the requirement of EMR overlapping carrier, it is better to relax EMR at the same time. Otherwise, it becomes complicated when measurement relaxation is only applied for re-selection while not for the EMR. So we suggest that

Proposal 6: For UE who supports both the IDLE mode RRM relaxation and EMR, UE shall also be allowed to relax the measurement period of the EMR carriers.

# 3	Summary

In this contribution, we have the following proposals:

Proposal 1: RAN 4 to confirm that RRM measurement relaxation threshold is the same for all frequency layers

Proposal 2: No RAN4 requirements are specified when the configured threshold of not at cell edge condition is no less than the configured threshold of cell center condition

Proposal 3: When UE is located in the cell center, the search time for inter-frequency measurement with higher priority will not be relaxed

Proposal 4: Scaling factor of RRM measurement relaxation with longer intervals should apply to Tdetect, Tevaluate, and Tmeasure

Proposal 5: RAN4 to apply same fixed scaling factor for IDLE mode RRM relaxation when only the low mobility condition is fulfilled or only the not at cell edge condition is fulfilled

Proposal 6: For UE who supports both the IDLE mode RRM relaxation and EMR, UE shall also be allowed to relax the measurement period of the EMR carriers

# Reference

- R4-2002239, WF on RRM measurement relaxation for Power Saving, CATT, Feb., 2020.

- R2-2002389, CR for 38.331 for Power Savings, MediaTek Inc., Mar., 2020.

- TS38.133 v15.8.0, 3GPP, Jan., 2020.