3GPP TSG-RAN WG4 Meeting #94-e-Bis	R4-2003603

Electronic Meeting, 20th – 30th Apr., 2020

Agenda item:		6.6.3.2.2

Source:	MediaTek Inc.

Title:	Discussion on NR SSB based measurement in LTE IDLE/INACTIVE mode

Document for:		Discussion

# 1	Introduction

In RAN4 #94 meeting, it is agreed that companies are encouraged to provide input and views on following early measurement reporting (EMR) issues [1]:

Agreements - UE requirements scenarios: - RAN4 to continue the discussion related to UE requirements for supporting NR Inter-RAT measurements for EMR. Later RAN4 can, if needed, return to the capability discussion. - RAN4 defines the LTE NR Inter-RAT measurements requirements for EMR regardless of the capability, which will be discussed separately - Number of LTE EMR carriers to support: - At least agree that the total number of carriers for EMR and mobility should not exceed the corresponding Rel-15 capability - RAN4 discussion to continue to decide the actual number. - Number of NR inter-RAT carriers: - At least agree that the total number of carriers for EMR and mobility should not exceed the corresponding Rel-15 capability. - RAN4 discussion to continue to decide the actual number. - Cell detected status at transitioning from connected mode - Adopt the LTE concept: - An NR inter-RAT cell which is a detected cell in Connected mode prior to connection release, shall remain detected after UE has entered Idle/Inactive mode - Open: conditions when the detected cell status remain detected during transitioning from connected to idle or inactive mode. Way Forward - Measurement requirements for NR Inter-RAT EMR carrier - RAN4 will discuss and if needed define requirements for an NR Inter-RAT EMR carrier: - UE performs periodic cell detection and measurement for LTE – NR inter-RAT EMR measurement - UE measurement requirements

In this paper, we discuss the all the related issues for LTE IDLE/INACTIVE DC measurement.

# 2	Discussion on scenarios of the early measurement reporting

We would like to firstly discuss the scenarios that RAN4 has to consider for the early measurement reporting as shown in Table 1. The first one is LTE CRS based measurement in LTE IDLE/INACTIVE mode and RAN4 had already discussed the corresponding requirements in the LTE euCA section. The remaining 3 scenarios are:

- NR SSB based measurement in LTE IDLE/INACTIVE mode for DC purpose;

- LTE CRS based measurement in NR IDLE/INACTIVE mode for DC purpose,

- NR SSB based measurement in NR IDLE/INACTIVE mode for both DC and CA purposes.

Table 1: Measurements of inter-frequency CA candidate cells

| scenarios | EMR requirement | Purpose | Spec | Requirement Framework | Capability |
| --- | --- | --- | --- | --- | --- |
| 1 | LTE CRS based measurement in LTE IDLE/INACTIVE | CA | 36.133 | Done | ca-IdleModeMeasurements-r15 in LTE RRC |
| 2 | NR SSB based measurement in LTE IDLE/INACTIVE | DC (EN-DC) | 36.133 | TBD | New proposed in LTE RRC (RAN2) |
| 3 | LTE CRS based measurement in NR IDLE/INACTIVE | DC (NE-DC) | 38.133 | TBD | New proposed in NR RRC (RAN2) |
| 4 | NR SSB based measurement in NR IDLE/INACTIVE | DC ,CA (NR-DC, NR-SA) | 38.133 | TBD | New proposed in NR RRC (RAN2) |

In this contribution, we will focus on the scenario 2.

# 3	Discussion on requirement framework of LTE IDLE/INACTIVE DC measurement

In this section, we will provide our own view on the rest of 2 issues (issues #7 and #8).

- Number of carriers:

In RAN4 #94 meeting, companies already agreed in related WF [2] that “The total number of carriers that the UE must measure for mobility and for EMR (overlapping and non-overlapping) should not exceed UE’s NR idle mode mobility measurement capability.” We believe that the similar principles can also be applied in LTE IDLE/INACTIVE DC and CA measurement. The number of reported carriers of CRS based and SSB based measurement in LTE IDLE/INACTIVE mode, including both overlapping and non-overlapping carriers, should not extend the measurement capability defined in Rel-15, i.e., Section 4.2.2.9 and 4.2.2.9a in TS 36.133.

Proposal 1: The number of reported carriers of CRS based and/or SSB based measurement in LTE IDLE/INACTIVE mode (overlapping and non-overlapping carriers) should not extend the measurement capability defined in Rel-15

- UE measurement requirements

In euCA, RAN4 reused the measurement period and measurement accuracy of cell re-selection in EMR overlapping carriers. Besides, RAN4 didn’t specified measurement period for EMR non-overlapping carriers. We suggest to reuse this principles in NR IDLE/INAVTIVE mode CA and DC measurement.

Proposal 2: RAN4 to reuse the measurement period and accuracy of cell re-selection to specify the requirements of overlapping EMR carriers

Proposal 3: RAN4 will not specify the measurement period of non-overlapping EMR carriers

In [3], a CRS based and SSB Based EMR requirement framework in NR IDLE/INACTIVE mode is proposed. We also see the benefit for RAN4 to apply similar EMR requirement framework for SSB based EMR requirement framework in LTE IDLE/INACTIVE mode. Similar methodologies can be reused here if RAN4 can achieve the consensus for NR IDLE/INACTIVE mode DC and CA measurement. So we suggest that

Proposal 4: In LTE IDLE/INACTIVE mode DC and CA measurement, UE at least shall be able to measure 3 carriers for EMR when UE is in cell center and UE at least shall be able to measure 1 carriers for EMR when UE is not in cell center

Proposal 5: RAN4 to adopt the SSB based EMR requirement framework shown in following table as a baseline to further discuss the corresponding measurement delay and accuracy requirements

Table 2: SSB based EMR requirement framework in LTE IDLE/INACTIVE mode

| Serving cell signal quality | Srxlev ≤ Threshold |  | Srxlev > Threshold |  |
| --- | --- | --- | --- | --- |
| Overlapping with carriers for re-selection | Overlapping | Non-overlapping | Overlapping | Non-overlapping |
| # of early measurement reporting carriers | At least [1] inter-freq. carriers |  | At least [3] inter-freq. carriers |  |
| SSB based Measurement delay | FFS | FFS | FFS | FFS |
| SSB based Measurement accuracy | FFS | FFS | FFS | FFS |

# 4	Summary

In this contribution, we have the following proposals:

Proposal 1: The number of reported carriers of CRS based and/or SSB based measurement in LTE IDLE/INACTIVE mode (overlapping and non-overlapping carriers) should not extend the measurement capability defined in Rel-15

Proposal 2: RAN4 to reuse the measurement period and accuracy of cell re-selection to specify the requirements of overlapping EMR carriers

Proposal 3: RAN4 will not specify the measurement period of non-overlapping EMR carriers

Proposal 4: In LTE IDLE/INACTIVE mode DC and CA measurement, UE at least shall be able to measure 3 carriers for EMR when UE is in cell center and UE at least shall be able to measure 1 carriers for EMR when UE is not in cell center

Proposal 5: RAN4 to adopt the SSB based EMR requirement framework shown in following table as a baseline to further discuss the corresponding measurement delay and accuracy requirements

Table 3: SSB based EMR requirement framework in LTE IDLE/INACTIVE mode

| Serving cell signal quality | Srxlev ≤ Threshold |  | Srxlev > Threshold |  |
| --- | --- | --- | --- | --- |
| Overlapping with carriers for re-selection | Overlapping | Non-overlapping | Overlapping | Non-overlapping |
| # of early measurement reporting carriers | At least [1] inter-freq. carriers |  | At least [3] inter-freq. carriers |  |
| SSB based Measurement delay | FFS | FFS | FFS | FFS |
| SSB based Measurement accuracy | FFS | FFS | FFS | FFS |

# Reference

- R4-2002235, WF on NR Inter-RAT EMR requirements, Nokia, Feb., 2020.

- R4-2002234, WF on MR-DC RRM requirements, Nokia, Feb., 2020.

- R4-2003601, Discussion on LTE CRS SSB based and NR SSB based measurement in NR IDLE/INACTIVE mode, Mediatek Inc., Apr., 2020.