3GPP TSG-RAN WG4 Meeting #94bis-e	R4-2003626

Electronic Meeting, April 20 – 30, 2020

Agenda item:	4.6.2

Source: 		NTT DOCOMO, INC.

Title: 	Additional unwanted emission requirements for the EESS protection from band n257 and n258

Document for:	Approval

- Introduction

In ITU WRC-19, the limits for the Earth exploration-satellite service (EESS) (passive) protection in the frequency band 23.6-24GHz were agreed [1]. In the last RAN4 meeting, the following agreements were made in WF [2]:

RAN4 #94 [2] - Limits will based on Resolution 750 and be expressed in dBm/200 MHz. - A phased approached will be applied, with two sets of limits listed on a equal basis, each with a Note. The note to the lower limit would indicate that it applies to BS brought into use after 1 September 2027. The note to the higher limit would indicate that it applies to BS brought into use on or before 1 September 2027. - Applicability in terms of frequency range: Limits apply when any part of the BS transmitted carrier(s) falls within 24.25 GHz to 27.5 GHz. - Limits will be expressed as both OBUE limits and spurious emission limits - Limits will also apply for Receiver spurious emissions - The test tolerance in TS 38.141-2 will be TT=0, based on the EESS protection limit being a regulatory requirement.

In this contribution, we further discuss on how to incorporate the limits into the BS RF specifications.

2.	Discussion

2.1.	Limits agreed by ITU WRC-19

The unwanted emission limits for the EESS (passive) protection in the frequency band 23.6-24 GHz was concluded as follows [1]:

2.2.	Requirement for the EESS (passive) protection

Figure 1 shows the relationship among the EESS (passive) band, NR FR2 operating bands and corresponding OBUE and spurious domains, and table 1 compares the RAN4 emission requirements with the EESS protection requirements.

Figure 1: The relationship among EESS (passive) band, NR operating bands and corresponding OBUE and Spurious domain.

Table 1: Comparison of OBUE, spurious emissions and EESS protection limits

| Frequency range | OBUE |  | General TX spurious emissions |  | General RX spurious emissions (Band n257) | Limits agreed by WRC-19 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Category A (Band n258) | Category B (Band n258) | Category A (Band n257) | Category B (Band n257) |  |  |
| 23.6-24GHz | f < fmax: -20dBm/MHz ∼ -5dBm/MHz | f fB: -20dBm/MHz ∼ -5dBm/MHz fB f < fmax: -10dBm/10MHz ∼ -5dBm/10MHz | -13dBm/MHz | -10dBm/10MHz | -10dBm/10MHz | After Sep. 1, 2027: -33dBW/200MHz = -3dBm/200MHz ≃ -26dBm/MHz ≃ -36dBm/10MHz Before Sep. 1, 2027: -39dBW/200MHz = -9dBm/200MHz ≃ -32dBm/MHz ≃ -42dBm/10MHz |

For frequency domain, from figure 1, EESS band overlaps with OBUE domain of band n258 and spurious domain of band n257. Table 1 shows that EESS protection limits are stricter than RAN4 emission requirements including OBUE, TX, and RX spurious emissions. Therefore, it is reasonable to consider OBUE, TX and RX spurious emissions requirements for EESS protection.

Observation 1: The limits for EESS protection are stricter than RAN4 OBUE, TX and RX spurious emissions requirements.

For time domain, basically, the EESS protection limit is considered to apply for any time period. According to TS38.104, the applicable period for RX spurious emissions is explicitly stated that the requirement applies only during the transmitter OFF period, while the applicable period for OBUE and TX spurious emissions are not described. That is, OBUE and TX spurious emissions apply for any period regardless of the state of the transmitter (transmitter ON period, transmitter OFF period or transient period). If this understanding is correct, we may not need to consider Rx spurious emissions for EESS protection. However, the correct understanding on applicable period for OBUE and TX spurious emissions requirements need be further checked and clarified.

Observation 2: In TS 38.104, there is no description about applicable period for OBUE and TX spurious emissions requirements.

Proposal 1: 	Clarify the applicable period of OBUE and TX spurious emissions requirements.

Proposal 2: 	If TX spurious emissions is applied regardless of transmitter ON, transmitter OFF or transient period, RAN4 introduces EESS protection requirements as OBUE and TX spurious emissions requirements. Otherwise, RAN4 introduces EESS protection requirements as OBUE, TX spurious emissions and RX spurious emissions requirements.

An example of text for EESS protection is shown below. Note that for spurious emissions, we can refer to the OBUE requirements to avoid redundant specifications.

------------------------------Text example for additional OBUE requirements for EESS protection-------------------------------

9.7.4.3.4	Earth exploration-satellite service (EESS) (passive) protection

When any part of BS transmitted carrier(s) falls within 24.25 – 27.5 GHz, the level of emissions in the band 23.6 – 24 GHz, measured in a 200 MHz filter bandwidth, shall not exceed the maximum emission TRP level shown in table 9.7.4.3.4-1. This requirement applies in the frequency range 23.6 – 24 GHz even though part of the range falls in the spurious domain, and applies during the transmitter ON period, the transmitter OFF period and the transient period.

Table 9.7.4.3.4-1: Operating band unwanted emission basic limits for EESS (passive) protection

| Frequency range | Basic limit | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 23.6 – 24 GHz | -9 dBm | 200 MHz | This requirement applies BSs brought into use before 1st September 2027. |
|  | -3 dBm | 200 MHz | This requirement applies BSs brought into use prior 1st September 2027. |

------------------------------------------------------------------------------------------------------------------------------------------------

3.	Conclusion

In this contribution, the approach to incorporate the EESS (passive) protection applicable to band n257 and n258. The following observations and proposals are obtained.

Observation 1: The limits for EESS protection are stricter than RAN4 OBUE, TX and RX spurious emissions requirements.

Observation 2: In TS 38.104, there is no description about applicable period for OBUE and TX spurious emissions requirements.

Proposal 1: 	Clarify the applicable period of OBUE and TX spurious emissions requirements.

Proposal 2: 	If TX spurious emissions is applied regardless of transmitter ON, transmitter OFF or transient period, RAN4 introduces EESS protection requirements as OBUE and TX spurious emissions requirements. Otherwise, RAN4 introduces EESS protection requirements as OBUE, TX spurious

References

- World Radiocommunication Conference 2019 (WRC-19) Provisional Final Acts (https://www.itu.int/en/ITU-R/conferences/wrc/2019/Documents/PFA-WRC19-E.pdf)

- R4-202466, “Way forward on EESS protection for NR BS operation in Band n257 and n258”, Ericsson, RAN4 #94, February 2020