3GPP TSG-RAN WG4 Meeting #94bis-e	R4-2003679

# Electronic Meeting, 20th Apr – 30th Apr, 2020

Title: 	Discussion and simulation for URLLC UE PDSCH demodulation requirements for high reliability

Source: 	Huawei, HiSilicon

Agenda item:	6.9.1.2

Document for:	Discussion

Background

When considering the high reliability of URLLC PDSCH performance requirements, RAN4 agreed to test the ultra-low BLER target (10^-5) and a higher BLER target separately. In this paper, the higher BLER target test is discussed. From #94 e-meeting discussion results, the higher BLER target of 10^-2 was agreed. Based on the agreements on #94 e-meeting, most of the parameters have been decided and open issues will be analysed.  The agreements and open issues are listed below:

Agreement

  - TDD pattern: 7D1S2U, S=6D: 4G: 4U for 30 kHz SCS.

  - MCS: MCS 5 in table 3.

  - Propagation condition: TDLA30-10

  - SCS & CBW:

    - FDD: 15 kHz & 10 MHz

    - TDD: 30 kHz & 40 MHz

  - PDSCH configuration: Mapping type A, symbol length 12, starting symbol 2.

  - Antenna configuration: 2x2 and 2x4, ULA low

  - Target BLER: 1%

  - Target Confidence level: 99%

  - BLER is calculated after all transmission

  - Max number of HARQ transmissions: 4

According to the WF [1], two open issues are remained:

- Whether to define UE FR2 URLLC requirements for high reliability

- PDSCH aggregation level

  - FDD

    - Option 1: 4

    - Option 2: 8

    - Option 3: 2

  - TDD

    - Option 1: 4

    - Option 2: 2

In this paper, open issues are discussed based on the agreed parameters. Our proposals are given.

Discussion

Whether to define UE FR2 URLLC requirements for high reliability

From current deployment request, FR2 is not a common use case for URLLC. For the link level evaluation studies from RAN1, the common carrier frequency for different use cases is 4 GHz and 700 MHz. The following table lists some sources:

Table 2.1-1: Carrier frequency for URLLC use cases

| Use case | Carrier frequency for evaluation | Source |
| --- | --- | --- |
| Electrical power distribution | 4 GHz | R1-1901248, R1-1900077, R1-1901350 |
|  | 700 MHz | R1-1901352 |
| Factory automation | 4 GHz | R1-1901555, R1-1903400, R1-1903447 |
| Rel-15 enabled use case | 4 GHz | R1-1900079, R1-1903451, R1-1903451, R1-1901250 |
|  | 700 MHz | R1-1901250 |
| Transport industry | 4 GHz | R1-1901247, R1-1900080, R1-1901351 |
|  | 700 MHz | R1-1901553, R1-1900238 |

At this stage, we think it is better to focus on FR1 firstly and deprioritize FR2.

Proposal 1: No need to define UE FR2 URLLC requirements for high reliability.

PDSCH aggregation level

Regarding to the aggregation level, K= [2, 4, 8] are supported. K= 8 is not the typical use case for URLLC. Between K=2 and K=4, we prefer the aggregation level is 4 as it ensures the high reliability of the transmission. Based on the agreed and proposed parameters, PDSCH performance with different aggregation levels for TDD and FDD were simulated.

- FDD

Table below illustrates the simulation parameters for FDD:

Table 2.2-1: Test parameters for UE PDSCH FDD performance requirements

| Parameter |  | Value |
| --- | --- | --- |
| Frequency range |  | FR1 |
| Duplex mode |  | FDD |
| Antenna configuration |  | 2x2 and 2x4, ULA low |
| PDSCH configuration | Mapping type | Type A |
|  | Starting symbol (S) | 2 |
|  | Length (L) | 12 |
|  | PDSCH aggregation factor | 4 |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | Number of additional DMRS | 1 |
| CSI-RS configuration | Periodicity and offset | 5 slots, 0 slots |
|  | Number of ports | 2 |
|  | Frequency domain allocation | Row3, ‘000001 |
|  | First OFDM symbol in time domain | 7 |
|  | Cdm Type | CDM2 |
|  | Density | 1 |
| Maximum number of HARQ transmission |  | 4 |
| Propagation condition |  | TDLA30-10 |
| MCS Table |  | Table 3, MCS 5 |
| SCS and BW |  | 15KHz & 10MHz |
| Frequency domain resource |  | Full BW |
| Testing metric |  | Target BLER: 1% |

The simulation results with antenna configuration 2x2 (a) and 2x4 (b) are shown in Figure 2.2-1.

- Antenna configuration 2x2                                                                (b) Antenna configuration 2x4

Figure 2.2-1: PDSCH performance with different aggregation level

With higher aggregation level, the PDSCH can achieve better performance. The SNR values with aggregation level = 2, 4 and 8 when BLER=0.01 for 2x2 and 2x4 are shown in table below:

Table 2.2-2: SNR values for PDSCH FDD performance requirements

| Antenna configuration SNR AL | 2x2 | 2x4 |
| --- | --- | --- |
| 2 | -2.8 | -9.3 |
| 4 | -6.1 | -10.8 |
| 8 | -11.6 | N/A |

- TDD

Table below illustrates the simulation parameters for TDD, as the aggregation level for TDD relates to the TDD pattern, so K=2 and 4 are simulated here:

Table 2.2-3: Test parameters for PDSCH TDD performance requirements

| Parameter |  | Value |
| --- | --- | --- |
| Frequency range |  | FR1 |
| Duplex mode |  | TDD |
| Antenna configuration |  | 2x2 and 2x4, ULA low |
| PDSCH configuration | Mapping type | Type A |
|  | Starting symbol (S) | 2 |
|  | Length (L) | 12 |
|  | PDSCH aggregation level | 4 |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | Number of additional DMRS | 1 |
| CSI-RS configuration | Periodicity and offset | 5 slots, 0 slots |
|  | Number of ports | 2 |
|  | Frequency domain allocation | Row3, ‘000001 |
|  | First OFDM symbol in time domain | 7 |
|  | Cdm Type | CDM2 |
|  | Density | 1 |
| Maximum number of HARQ transmission |  | 4 |
| Propagation condition |  | TDLA30-10 |
| MCS Table |  | Table 3, MCS 5 |
| SCS and BW |  | 30KHz & 40MHz |
| Frequency domain resource |  | Full BW |
| TDD pattern |  | 7D1S2U (S=6:4:4) |
| Testing metric |  | Target BLER: 1% |

The simulation results with antenna configuration 2x2 (a) and 2x4 (b) are shown in Figure 2.2-2.

- Antenna configuration 2x2                                             (b) Antenna configuration 2x4

Figure 2.2-2: PDSCH performance with different aggregation level 2, 4

The SNR values with aggregation level = 2, 4 when BLER=0.01 for 2x2 and 2x4 are shown in table below:

Table 2.2-4: Test parameters for PDSCH TDD performance requirements

| Antenna configuration SNR AL | 2x2 | 2x4 |
| --- | --- | --- |
| 2 | -3.8 | -9.9 |
| 4 | -9.9 | -13.6 |

Proposal 2: We propose the aggregation level is 4.

Proposal

In this paper, the requirements for PDSCH demodulation performance are defined. Proposals are listed below:

Proposal 1: No need to define UE FR2 URLLC requirements for high reliability.

Proposal 2: We propose the aggregation level is 4.

Reference

[1] R4-2002428, Way forward for NR UE URLLC performance requirements, #94, Intel Corporation.