3GPP TSG-RAN WG4 Meeting #94e-bis	R4-2003707

# Electronic Meeting, 20th - 30th Apr, 2020

Title: 	Discussion on the performance requirements for NR DL 256QAM for FR2

Source: 	Huawei, HiSilicon

Agenda item:	6.12.4

Document for:	Discussion

Background

As per TR 38.883 [1], the conclusion is made about the feasibility for NR DL 256QAM for FR2 which is derived as below.

The feasibility of FR2 DL 256QAM has been evaluated, including link level simulation, system level simulation and implementation study. Based on the study output, it can be concluded that FR2 DL 256QAM is feasible and beneficial as concluded in section 5.2.1.10, 5.2.2.6 and 5.3, and corresponding requirements shall be discussed and specified afterwards.

As per status report [2], performance requirements for NR DL 256QAM for FR2 need to be initially discussed in this meeting. In this contribution, we share our views on demodulation requirements for NR DL 256QAM for FR2.

Discussion

Testability

The mainly factor of testability is the testable SNR, which is in a certain range due to the limitation of the transmitted signal power of TE. The following Table derived from TS 38.810 shows the maximum SNR for DNF test method with 43.5GHz carrier frequency.

Table 2.1-1: Maximum SNR for DNF test method with 43.5GHz carrier frequency

|  | Channel Bandwidth | Maximum SNR |
| --- | --- | --- |
| Single band UE | 100 MHz | [22.6 dB] |
|  | 200 MHz | [19.6 dB] |
| Multi-band UE (Note) | 100 MHz | [20.6 dB] |
|  | 200 MHz | [17.6 dB] |
| Note: For ∑MBp from TS 38.101-2 [16] Table 6.2.1.3-4 allow up to 2 dB |  |  |

Note that the performance requirements are defined with maximum carrier frequency not exceeding 40000MHz as per current TS 38.101-4 [3]. With the carrier frequency decrease, free space path loss can be reduced and UE Refsens can be improved and then leads to the improvement of the maximum testable SNR.

To ensure the UE performance under 256QAM for FR2, RAN4 should evaluate the required SNR for different scenario and then define the performance requirements for NR DL 256QAM for FR2 based on the scenario satisfied the demand that required SNR is less than maximum testable SNR.

Proposal 1: To ensure the UE performance under 256QAM for FR2, RAN4 should evaluate the required SNR for different scenario and then define the performance requirements for NR DL 256QAM for FR2 based on the scenario satisfying the demand that required SNR is less than maximum testable SNR.

Simulation assumption

The principle of determining setup for simulation assumption is that select typical values to reduce the testing cost and satisfy the demand that required SNR is less than maximum testable SNR simultaneously.

Carrier frequency

As mentioned above, the carrier frequency value is negatively correlated with the maximum testable SNR. Therefore we propose to select a lower carrier frequency, i.e. 29GHz (n257) comparing to 39GHz (n260) to meet the demand of the maximum testable SNR.

Channel bandwidth

Similar as carrier frequency, the lower bandwidth the lower maximum of the total power of the bandwidth which means increase of the maximum testable SNR. Therefore we propose to select 50MHz as channel bandwidth.

EVM

At the BS side, an agreement is achieved that the EVM requirement for FR2 DL 256QAM is 3.5% for BS type 2-O. For UE side, we can assume that the EVM is the same 3.5% for simulation considering the component quality.

Phase noise model

Whether the phase models used for the evaluations for the feasibility of DL 256QAM for FR2 can resemble the real UE implementation and whether to explicitly model the phase noise by using the example model in TR 38.803 may need further discussion.

FRC

The selection of MCS and rank is based on the simulation results to meet the demand of the maximum testable SNR.

Other parameters

For other parameters, typical values can be selected to reduce the test cost.

Based on above discussion, we can get the following proposal:

Proposal 2: Define new performance requirements for NR DL 256QAM for FR2 based on the evaluations by using the following simulation assumptions:

| Parameters | Value |
| --- | --- |
| Carrier frequency | 29GHz |
| Channel bandwidth | 50MHz |
| EVM | 3.5% txEVM |
| Phase noise model | FFS |
| FRC | Rank 1/2, MCS 21/23/25/27 |
| Propagation condition | TDL-D30-75 |
| Antenna configuration | 2x2 ULA low |
| SCS | 120kHz |
| SSB configuration | Periodicity 20 ms, Allocated in first slot within 20ms |
| PDSCH configuration | Type A, Start symbol 1, Duration 13 |
| DMRS configuration | Type 1, 1 additional DMRS, Single symbol |
| TRS configuration | 20 ms periodicity, 2 slots, Offset 10 ms |
| PTRS configuration | Frequency density 2, Time density 1 |
| TDD UL-DL pattern | 3D1S1U with S=10:2:2 |
| HARQ process number | 8 |
| Transform precoding | CP-OFDM |
| Allocated RBs | Full BWP |
| PRB bundling | 2 |
| Precoding model | Random Precoding, per slot, WB granularity (codebook configuration Single panel Type 1) |
| Receiver type | MMSE-IRC |
| Test metric | 70% of max TP |

Proposal

In this contribution, we discuss on the performance requirements for NR DL 256QAM for FR2. Our observations and proposals are:

Proposal 1: To ensure the UE performance under 256QAM for FR2, RAN4 should evaluate the required SNR for different scenario and then define the performance requirements for NR DL 256QAM for FR2 based on the scenario satisfying the demand that required SNR is less than maximum testable SNR.

Proposal 2: Define new performance requirements for NR DL 256QAM for FR2 based on the evaluations by using the following simulation assumptions:

| Parameters | Value |
| --- | --- |
| Carrier frequency | 29GHz |
| Channel bandwidth | 50MHz |
| EVM | 3.5% txEVM |
| Phase noise model | FFS |
| FRC | Rank 1/2, MCS 21/23/25/27 |
| Propagation condition | TDL-D30-75 |
| Antenna configuration | 2x2 ULA low |
| SCS | 120kHz |
| SSB configuration | Periodicity 20 ms, Allocated in first slot within 20ms |
| PDSCH configuration | Type A, Start symbol 1, Duration 13 |
| DMRS configuration | Type 1, 1 additional DMRS, Single symbol |
| TRS configuration | 20 ms periodicity, 2 slots, Offset 10 ms |
| PTRS configuration | Frequency density 2, Time density 1 |
| TDD UL-DL pattern | 3D1S1U with S=10:2:2 |
| HARQ process number | 8 |
| Transform precoding | CP-OFDM |
| Allocated RBs | Full BWP |
| PRB bundling | 2 |
| Precoding model | Random Precoding, per slot, WB granularity (codebook configuration Single panel Type 1) |
| Receiver type | MMSE-IRC |
| Test metric | 70% of max TP |

Reference

TR 38.883, Study on support of NR downlink 256 Quadrature Amplitude Modulation (QAM) for frequency range 2 (FR2)

RP-200458, SR for NR DL 256QAM for FR2, RAN#87, China Telecom

TS 38.101-4, User Equipment (UE) radio transmission and reception, Part 4: Performance requirements

TR 38.803, Study on new radio access technology: Radio Frequency (RF) and co-existence aspects