3GPP TSG-RAN WG4 Meeting #94-e-bis	R4-2003734

# Electronic Meeting, 20 – 30 Apr., 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR |  | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.101-3 on time masks for ULSUP in R16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1 |  |  |  |  |  | Date: |  |  | 2020-04-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The existing timing mask requirement for ULSUP-TDM specified in Rel-15 applies for the case where there is no significant uplink timing difference between LTE and NR. According to the updated WID on RF requirements for NR frequency range 1 (FR1), the new timing mask requirement for ULSUP-TDM will be specified. The figures in clause 6.3B.1 blurs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify the timing mask requirements for ULSUP-TDM with uplink timing difference up to 3us. Replace the figures in clasue 6.3B.1 with clear ones. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No requirement for the objective to timing maks in WID on RF requirements for NR frequency range 1 (FR1). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New 6.3B.1.1b |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of change >>

6.3B.1	Output power dynamics for EN-DC with UL sharing from UE perspective

6.3B.1.1	E-UTRA and NR switching time mask for TDM based UL sharing from UE perspective without uplink transmission timing difference

The E-UTRA and NR switching time mask is applicable for non-simultaneous transmissions between E-UTRA and NR in TDM based UL sharing from the UE perspective in the same channel, which is shared by E-UTRA and NR.

For UEs reporting E-UTRA and NR switching time capability of type 1 with switching time < 0.5 us for TDM based UL sharing from UE perspective within FR1 time masks in Figure 6.3B.1.1-1 and Figure 6.3B.1.1-2 shall apply. For UEs reporting E-UTRA and NR switching time capability of type 2 with switching time < 20 us for TDM based UL sharing from UE perspective within FR1, time masks in Figure 6.3B.1.1-3 and Figure 6.3B.1.1-4 shall apply.

20µs

Transient period

E

-

UTRA

subframe

10µs

ON power

requirement

NR slot/mini-slot

ON power

requirement

Figure 6.3B.1.1-1: E-UTRA to NR switching time mask for type 1 for TDM based UL sharing from UE perspective within FR1

20µs

Transient period

E-UTRA subframe

NR slot/mini-slot

10µs

ON power

Requirement

ON power

Requirement

Figure 6.3B.1.1-2: NR to E-UTRA switching time mask for type 1 for TDM based UL sharing from UE perspective within FR1

20µs

Transient period

NR slot/mini-slot

10µs

ON power

requirement

ON power

requirement

OFF power requirement

Transient period

E-UTRA subframe

20µs

Figure 6.3B.1.1-3: E-UTRA to NR switching time mask for type 2 for TDM based UL sharing from UE perspective within FR1

20µs

Transient period

NR slot/mini

-

sl

ot

10µs

ON power

requirement

ON power

requirement

Transient period

OFF power requirement

20µs

NR slot/mini-slot

E-UTRA subframe

Figure 6.3B.1.1-4: NR to E-UTRA switching time mask for type 2 for TDM based UL sharing from UE perspective within FR1

6.3B.1.2	E-UTRA and NR switching time mask for TDM based UL sharing from UE perspective with uplink timing difference up to 3us

The E-UTRA and NR switching time mask is applicable for non-simultaneous transmissions between E-UTRA and NR in TDM based UL sharing from the UE perspective in the same channel, which is shared by E-UTRA and NR with uplink transmission timing difference up to 3us.

For UEs reporting E-UTRA and NR switching time capability of type 1 with switching time < 0.5 us for TDM based UL sharing from UE perspective within FR1 time masks in Figure 6.3B.1.2-1 and Figure 6.3B.1.2-2 shall apply. When there is any uplink transmission timing misalignment between E-UTRA subframe and NR slot, the additional 3us is allowed for the transient period for the succeeding NR slot or E-UTRA subframe.

For UEs reporting E-UTRA and NR switching time capability of type 2 with switching time < 20 us for TDM based UL sharing from UE perspective within FR1, time masks in Figure 6.3B.1.2-3 and Figure 6.3B.1.2-4 shall apply. When there is any uplink transmission timing misalignment between E-UTRA subframe and NR slot, the additional 3us is allowed for the transient period for the succeeding NR slot or E-UTRA subframe.

20µs

Transient period

E

-

UTRA

subframe

13µs

ON power

Requirement

NR slot/mini-slot

ON power

Requirement

Figure 6.3B.1.2-1: E-UTRA to NR switching time mask for type 1 for TDM based UL sharing from UE perspective within FR1

23µs

Transient period

E-UTRA subframe

NR slot/mini-slot

10µs

ON power

Requirement

ON power

Requirement

Figure 6.3B.1.2-2: NR to E-UTRA switching time mask for type 1 for TDM based UL sharing from UE perspective within FR1

20µs

Transient period

NR slot/mini-slot

13µs

ON power

requirement

ON power

requirement

OFF power requirement

Transient period

E-UTRA subframe

20µs

Figure 6.3B.1.2-3: E-UTRA to NR switching time mask for type 2 for TDM based UL sharing from UE perspective within FR1

20µs

Transient period

NR slot/mini

-

sl

ot

10µs

ON power

requirement

ON power

requirement

Transient period

OFF power requirement

23µs

NR slot/mini-slot

E-UTRA subframe

Figure 6.3B.1.2-4: NR to E-UTRA switching time mask for type 2 for TDM based UL sharing from UE perspective within FR1

<< End of change >>