3GPP TSG-RAN WG4 Meeting #94bis-e                                R4-2003925

Electronic Meeting, 20th April – 30th April, 2020

Source: 	Huawei, HiSilicon

Title: 	TP for TR 37.716-11-11 DC_(n)38AA

Agenda Item:	8.3.2

Document for:	Approval

Introduction

The WID for NR DC was updated in RAN #87 meeting. This contribution provides a TP for DC_(n)38AA for TR 37.716-11-11.

- References

[1]	RP-200221, “Revised WID: Dual Connectivity (EN-DC) of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL)”, NTT DOCOMO, INC.

- Text Proposal

<TP for TR 37.716-11-11>

### 6.3.x	DC_(n)38

6.3.x.1	Channel bandwidths per operating band for CA

Table 6.3.x.1-1: Band combinations for intra-band contiguous EN-DC

| EN-DC Band Uplink Combination | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_(n)38 | 38 | n38 | Yes |

Table 6.3.x.1-2: Supported channel bandwidths per DC configuration

|  |  | E-UTRA – NR configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | BCS |
|  |  | Channel bandwidths for LTE carrier (MHz) | Channel bandwidths NR for carrier (MHz) | Channel bandwidths for LTE carrier (MHz) |  |  |
| DC_(n)38AA | DC_(n)38AA1 | 5, 10, 15, 20 | 5, 10, 15, 20, 40 |  | 50 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | 5, 10, 15, 20 |  |  |
| NOTE1: Only single switched UL is supported |  |  |  |  |  |  |

6.3.x.2	Configuration for DC

Table 6.3.x.2-1:  Intra-band contiguous EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_(n)38AA | DC_(n)38AA2 | 38A | n38A |
| NOTE2: Only single switched UL is supported |  |  |  |

6.3.x.3	Maximum output power for DC

Table 6.3.x.3-1: Maximum output power for  Intra-band contiguous EN-DC

| DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_(n)38AA2 |  |  | 23 | +2/-3 |
| NOTE 2: Only single switched UL is supported |  |  |  |  |

6.3.x.4	Spurious emission band UE co-existence for DC

Note that only Single Tx Switched UL mode is supported for this combination, no spurious emission band UE co-existence requirement is needed

6.3.x.5	MSD analysis for DC

Note that only Single Tx Switched UL mode is supported for this combination, no MSD requirement is needed

6.3.x.6	MPR, AMPR requirements

Note that only Single Tx Switched UL mode is supported for this combination, no MPR or A-MPR requirement is needed

<End of TP >